3GPP TSG-RAN WG3 Meeting #118	R3-226611

Toulouse, France, 14-18 November 2022

Title: 	(TP for AI&ML BLCR for 38.423) Further discussions on remaining open issues for energy saving using AI/ML

Source: 	Huawei

Agenda item:	12.2.2.1

Document Type:	Discussion and decision

1. Introduction

In last RAN3#117e and RAN3#117b-e meetings, the support of energy saving (ES) using AI&ML was discussed and the following agreements were reached [1][2].

RAN3 focus on the cell-level energy saving strategy as a start point, to avoid overlapped discussion for network energy saving SI. Predicted resource status information of neighbouring NG-RAN node(s) generated by the current NG-RAN node is internally used, and no standard impacts. Regarding AI/ML based Energy Saving, the following information should be specified as a start point on the basis of TR37.817: - Predicted resource status information over Xn - Current/Predicted Energy Efficiency over Xn, FFS on the calculation of Energy Efficiency - UE performance (e.g, UL/DL throughput, packet delay, packet loss) Current Energy Efficiency metric can be exchanged between RAN nodes for the energy saving use case. FFS what the exact node behaviour at reception of the Energy Efficiency metric will be. Energy Efficiency constitutes a metric that reflects the energy consumption of a cell or a node. It is FFS what the granularity and exact coding of this metric is. FFS on how is energy efficiency represented in the Xn message.

The feasibility, interpretability and encoding of the EE metrics is FFS. It is FFS how to transfer current Energy Efficiency metric It is FFS whether EE metric is per node or per cell and how per cell EE metric can be calculated. RAN3 to discuss the scenarios where predicted energy efficiency is exchanged. - FFS on detailed energy efficiency metric and corresponding E1/F1 impact after the sufficient work for non-split architecture.

In this paper, we discuss the left issues on Energy Efficiency (EE).

2. Discussion

2.1 Encoding of EE

The following two ways of encoding EE metric were discussed in last RAN3#117e meeting:

- detailed metric, i.e., calculated as DV / EC;

- abstract metric, i.e., using abstract values e.g. 0 to 100.

For the second way, how to interpret the abstract values could be left to implementation. For example, OAM could configure the detailed mapping of each abstract value to the actual EE value, or OAM could configure the maximum and minimum EE values and a scaling method (e.g. linear scale, logarithmic scale), based on which the actual EE value can be derived. The exact coding of the abstract values could be INTEGER (0..N), where candidate values of N include e.g. 10, 100, 1000, and the value 100 seems reasonable.

For the above two ways, though both could work, the second way is slightly preferred since: in the first way, the maximum value of EE needs to be specified, however the determination of a reasonable value that covers all possible configurations and scenarios is difficult. An improperly defined value may limit the uses of the EE metric, especially considering the possible boost in EE by using new hardware, algorithms and network topology in the future. For the second away, the maximum value of EE does not need to be defined, and the OAM can determine such value according to the measurement or statistics result, which is more flexible.

Proposal 1: The EE metric is encoded in an abstract way, e.g. INTEGER (0..100) , and how to interpret each value of the abstract metric is left to implementation.

2.2 Granularity of EE

The candidate granularities include per cell and per node level. The controversial case here is that multiple cells share (part or all elements of) the same node, e.g. a 3-cell gNB where the three cells share the same BBU and PA. In our understanding, the metric “EE per cell” is more proper since the ES actions are mainly taken on a cell level basis, e.g. offloading UEs to target neighboring cells. If “EE per node” is indicated, the neighbors would not know how to make ES decisions, e.g. which cell(s) to offload the UEs. Of course, the energy consumption of each cell cannot be directly measured in the node sharing case. This requires some methods to approximately calculate the energy consumption of each cell, e.g. calculating based on the proportion of their data volume, which is left to implementation.

Proposal 2: The EE metric is collected on a per cell basis.

2.3 Use of predicted EE

Use case 1

If a node can predict the EE of its cell corresponding to the possible ES actions to take by neighbours and indicate the predictions to neighbours, the neighbours can make more accurate ES decisions based on such information. For example, in Fig. 1, node A may take two possible ES actions: 1) switch-off the cell and offload all UEs to node B (with average throughput of X Mbps), and 2) switch-off the cell and offload no UEs to node B (may offload UEs to other nodes). Node B can make predictions of EE for each of these two cases and send the predictions to node A. Such type of predictions can provide explicit knowledge on how the EE will change after some specific ES actions are taken, which is much helpful for node A to choose a proper ES action.

Fig. 1 Illustration of how ES related info is used between neighbour nodes – use case 1

The exchange of the prediction related information could reuse the agreed new AI/ML procedure. In the class 1 AI info reporting initiation procedure, a list of possible ES actions and an indication for requesting prediction of EE is indicated. As the starting point, in this release the offloading related ES action is defined, and the information for the ES action could include:

- Number of UEs to Offload: This IE indicates the number of UEs that are expected to be offloaded to cell B.

- UE Average Bit Rate Downlink: This IE indicates the average bit rate of these UEs in downlink.

- UE Average Bit Rate Uplink: This IE indicates the average bit rate of these UEs in uplink.

In the class 2 AI info reporting procedure, a list of predicted EEs is transferred, each of which corresponds to one ES action.

Proposal 3: Over Xn, the ES related prediction information include a list of possible ES actions (node A  node B) and, a list of predicted EEs each of which corresponds to one ES action (node B  node A). As the starting point, the offloading related ES action is defined.

Use case 2

The L1/L2 related ES decisions, e.g. energy efficient antenna selection and scheduling, are made by gNB-DU. To make such decisions, the gNB-DU needs to collect information locally as well as from the gNB-CU. The following information collected from the gNB-CU may be useful for ES decision making by the gNB-DU (Fig. 2):

- predicted Radio Resource Status,

- current/predicted EE.

Fig. 2 Illustration of how ES related info is used between CU and DU – use case 2

As to the predicted Radio Resource Status (e.g. predicted PRB usage), the gNB-DU may use it for deciding whether to switch off some antennas/radio units for ES. For example, if both the current PRB usage (measured by gNB-DU locally) and the predicted PRB usage (indicated by gNB-CU) are low, the gNB-DU can select part of the antennas/radio units (e.g. 32 out of 64 antennas/radio units) to serve the UEs and switch off other antennas/radio units; on the other hand, if the current PRB usage is low while the predicted PRB usage is high, the gNB-DU may decide not to switch off antennas/radio units, though the current PRB usage is low, to avoid the insufficient capacity for serving UEs in the future.

As to the predicted EE, the gNB-DU may use it for adjusting the objective of scheduling. For example, if both the current and predicted EE is high, the scheduler may would like to maximize the spectrum efficiency (SE) to improve the network throughput without increasing the energy consumption remarkably; on the other hand, if the current EE is high while the predicted EE is low, the scheduler may tend to maximize the EE to balance network performance and energy consumption.

A new procedure, based on a requested way, can be introduced for transferring the above AI/ML related information over F1, which is aligned with Xn.

Proposal 4: Over F1, the following AI/ML related information is signalled to gNB-DU: predicted Radio Resource Status, current EE and predicted EE. A new procedure is introduced for transferring the above information.

2.4 Transfer of current EE over Xn

A remaining question is, over Xn, in which procedure the current EE is transferred.

In fact, both the Resource Status procedure and the new AI/ML procedure could be used for transferring the current EE. The new AI/ML procedure is slightly preferred, since: i) the EE metric is a newly introduced information, and it is mainly AI/ML specific, and ii) the current EE is expected to be used jointly with the predicted EE, so it is reasonable to indicate the two information in the same procedure/message.

Proposal 5: Over Xn, the current EE is transferred through the new AI/ML procedure.

- The corresponding TPs to TS 38.423 is attached below in Annex part and CR to TS 38.473 are provided in [3].

3. Conclusion

Based on the discussion in this paper, we make the following proposals.

Proposal 1: The EE metric is encoded in an abstract way, e.g. INTEGER (0..100) , and how to interpret each value of the abstract metric is left to implementation.

Proposal 2: The EE metric is collected on a per cell basis.

Proposal 3: Over Xn, the EE prediction information including a list of possible ES actions (node A  node B) and a list of predicted EEs each corresponding to one ES action (node B  node A) is exchanged. As the starting point, the offload related ES action is defined.

Proposal 4: Over F1, the following AI/ML related information is signalled to gNB-DU: predicted Radio Resource Status, current EE and predicted EE. A new procedure is introduced for transferring the above information.

Proposal 5: Over Xn, the current EE is transferred through the new AI/ML procedure.

4. Reference

- RAN3_117-e_agenda_20220825_EOM

- RAN3_117bis-e_agenda_20221018_EOM

- R3-226615,	CR to 38.473 On the introduction of RAN AI/ML, Huawei

5. Annex – TP to 38.423

The TP below tries to introduce the following proposals:

-	Introduce new procedures for AI/ML related information.

-	Introduce the following information in the new procedures:

-		Predicted resource status information,

-		Current/predicted Energy Efficiency.

-		UE trajectory prediction.

- Introduce UE performance in ACCESS AND MOBILITY INDICATION.

- Introduce Intended Timestamp for Handover Execution in HANDOVER REQUEST.

- Introduce Cell-level UE performance in RESOURCE STATUS UPDATE

- Introduce Handover Desirable for ML/AI Inference in Cause.

//////////////////////////////////////////////////////////////TP Starts//////////////////////////////////////////////////////////////////////////

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| IAB Transport Migration Management | IAB TRANSPORT MIGRATION MANAGEMENT REQUEST | IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE | IAB TRANSPORT MIGRATION MANAGEMENT REJECT |
| IAB Transport Migration Modification | IAB TRANSPORT MIGRATION MODIFICATION REQUEST | IAB TRANSPORT MIGRATION MODIFICATION RESPONSE |  |
| IAB Resource Coordination | IAB RESOURCE COORDINATION REQUEST | IAB RESOURCE COORDINATION RESPONSE |  |
| Partial UE Context Transfer | PARTIAL UE CONTEXT TRANSFER | PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE | PARTIAL UE CONTEXT TRANSFER FAILURE |
| AI/ML Information Reporting Initiation | AI/ML INFORMATION REQUEST | AI/ML INFORMATION RESPONSE | AI/ML INFORMATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| RAN Multicast Group Paging | RAN MULTICAST GROUP PAGING |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Retrieve UE Context Confirm | RETRIEVE UE CONTEXT CONFIRM |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| AI/ML Information Reporting | AI/ML INFORMATION UPDATE |

-------------------------------------------------------------Next change-------------------------------------------------------------

8.4.x	AIML Information Reporting Initiation

8.4.x.1	General

This procedure is used by an NG-RAN node to request the reporting of AI/ML related information to another NG-RAN node.

The procedure uses non UE-associated signalling.

8.4.x.2	Successful Operation

Figure 8.4.x.2-1: AIML Information Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the AIML INFORMATION REQUEST message to NG-RAN node2 to start a measurement, stop a measurement or add cells to report for a measurement. Upon receipt, NG-RAN node2:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the AIML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific measurements, the Cell To Report List IE shall be included.

If Registration Request IE is set to "add" in the AIML INFORMATION REQUEST message, the Cell To Report List IE shall be included.

If NG-RAN node2 is capable to provide all requested AI/ML related information, it shall initiate the measurement as requested by NG-RAN node1 and respond with the AIML INFORMATION RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AIML INFORMATION REQUEST indicates the type of objects NG-RAN node2 shall perform measurements on. For each cell, NG-RAN node2 shall include in the AIML INFORMATION UPDATE message:

-	the Predicted Radio Resource Status IE, if the first bit, "Predicted PRB Periodic" of the Report Characteristics IE included in the AIML INFORMATION REQUEST message is set to "1". If NG-RAN node2 is a gNB and if the cell for which Predicted Radio Resource Status IE is requested to be reported supports more than one SSB, the Predicted Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall include the requested SSB Area Radio Resource Status List IE; If the cell for which Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall, if supported, include the requested Slice Radio Resource Status Item IE;

-	the Predicted TNL Capacity Indicator IE, if the second bit, "Predicted TNL Capacity Ind Periodic" of the Report Characteristics IE included in the AIML INFORMATION REQUEST message is set to "1". The received Predicted TNL Capacity Indicator IE represents the predicted lowest TNL capacity available for the cell, only taking into account interfaces providing user plane transport.

-	the Predicted Composite Available Capacity Group IE, if the third bit, "Predicted Composite Available Capacity Periodic" of the Report Characteristics IE included in the AIML INFORMATION REQUEST message is set to "1". If the Predicted Cell Capacity Class Value IE is included within the Predicted Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If NG-RAN node2 is a gNB and if the cell for which Predicted Composite Available Capacity Group IE is requested to be reported supports more than one SSB, the Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value. If the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE.

If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

- 	the Predicted Number of Active UEs IE, if the fourth bit, "Predicted Number of Active UEs" of the Report Characteristics IE included in the AIML INFORMATION REQUEST message is set to "1";

-	the Predicted RRC Connections IE, if the fifth bit, "Predicted RRC Connections" of the Report Characteristics IE included in the AIML INFORMATION REQUEST message is set to "1".

- the Energy Efficiency IE, if the sixth bit, "Energy Efficiency Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

- 	the Predicted Energy Efficiency IE, if the seventh bit, "Predicted Energy Efficiency" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

- 	the UE trajectory prediction IE, if the eighth bit, " UE trajectory prediction " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

If the Reporting Periodicity IE in the AIML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. the NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity IE.

8.4.x.3	Unsuccessful Operation

Figure 8.4.x.3-1: AIML Information Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, NG-RAN node2 shall send the AIML INFORMATION FAILURE message with an appropriate cause value.

8.4.x.4	Abnormal Conditions

For the same Measurement ID, if the initiating NG-RAN node1 does not receive either the AIML INFORMATION RESPONSE message or the AIML INFORMATION FAILURE message, the NG-RAN node1 may reinitiate the AIML Information Reporting Initiation procedure towards the same NG-RAN node, provided that the content of the new AIML INFORMATION REQUEST message is identical to the content of the previously unacknowledged AIML INFORMATION REQUEST message.

If the NG-RAN node2 receives an AIML INFORMATION REQUEST message which includes the Registration Request IE set to "add" or "stop" and if the NG-RAN node2 Measurement ID value received in the AIML INFORMATION REQUEST message is not used, the NG-RAN node2 shall initiate AIML INFORMATION FAILURE message with an appropriate cause value.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the AIML INFORMATION REQUEST message then NG-RAN node2 shall initiate an AIML INFORMATION FAILURE message with an appropriate cause value.

If the NG-RAN node2 receives an AIML INFORMATION REQUEST message which includes the Registration Request IE set to "start" and the NG-RAN node1Measurement ID IE corresponding to an existing on-going load measurement reporting, then NG-RAN node2 shall initiate a AIML INFORMATION FAILURE message with an appropriate cause value.

8.4.y	AIML Information Reporting

8.4.y.1	General

This procedure is initiated by an NG-RAN node to report the result of measurements admitted by the NG-RAN node following a successful AIML Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

8.4.y.2	Successful Operation

Figure 8.4.y.2-1: AIML Information Reporting, successful operation

NG-RAN node2 shall report the results of the admitted measurements in AIML INFORMATION UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding AIML Information Reporting Initiation procedure.

8.4.y.3	Unsuccessful Operation

Not applicable.

8.4.y.4	Abnormal Conditions

Void

9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Intended Timestamp for Handover Execution | O |  | INTEGER (0..1000000, …) | Expressed in units of 1 ms. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

-------------------------------------------------------------Next change-------------------------------------------------------------

9.1.3.x	AIML INFORMATION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested AI/ML related information according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the NG-RAN node2 is requested to report. First Bit = Predicted PRB Periodic, Second Bit = Predicted TNL Capacity Ind Periodic, Third Bit = Predicted Composite Available Capacity Periodic, Fourth Bit = Predicted Number of Active UEs, Fifth Bit = Predicted RRC connections, Sixth Bit = Energy Efficiency, Seventh Bit = Predicted Energy Efficiency, Eighth= UE trajectory prediction Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | – |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB-Index | M |  | INTEGER (0..,63..) |  | – |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | – |  |
| >>>Slice To Report Item |  | 1 .. < maxnoofBPLMNs > |  |  | – |  |
| >>>>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN | – |  |
| >>>>S-NSSAI List |  | 1 |  |  | – |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | – |  |
| >>>>>>S-NSSAI | M |  | S-NSSAI 9.2.3.21 |  | – |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of Predicted PRB Periodic, Predicted TNL Capacity Ind Periodic, Predicted Composite Available Capacity Periodic. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

9.1.3.y	AIML INFORMATION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested AI/ML related information is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.3.z	AIML INFORMATION FAILURE

This message is sent by the NG-RAN node2 to NG-RAN node1 to indicate that any of the requested AI/ML related information cannot be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.3.xx	AIML INFORMATION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested AI/ML related information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Prediction Result |  | 1 |  |  | YES | ignore |
| > Time ID | M |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [25]. It indicates the start time of the collecting period of the included Usage Count UL IE and Usage Count DL IE. |  |  |
| >Cell Prediction Result Item | O | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Predicted Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>Predicted TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Predicted Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Predicted Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>Predicted RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>Current Energy Efficiency | O |  | 9.2.2.xx |  | – |  |
| >>Predicted Energy Efficiency | O |  | 9.2.2.xx |  | – |  |
| UE Trajectory Prediction | O |  |  |  |  |  |
| > UE Trajectory Prediction Item |  |  |  |  |  |  |
| >> UE ID | M |  | 9.2.3.16 | NG-RAN node UE XnAP ID |  |  |
| >> Predicted UE Trajectory | M |  | 9.2.3.xx |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

-------------------------------------------------------------Next change-------------------------------------------------------------

9.1.3.21	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | YES | ignore |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>Radio Resource Status | O |  | 9.2.2.50 |  | – |  |
| >>TNL Capacity Indicator | O |  | 9.2.2.49 |  | – |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.51 |  | – |  |
| >>Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| >>Number of Active UEs | O |  | 9.2.2.62 |  | –- |  |
| >>RRC Connections | O |  | 9.2.2.56 |  | – |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | – |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | The NR-U channel utilised in the last reporting period | – |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 corresponds to the duration between consecutive reporting. | – |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | – |  |
| Cell-based UE performance Information | O |  |  | Cell-based UE performance Information for AI based load balancing |  |  |
| > DL Average Throughput Periodic |  |  |  |  |  |  |
| > UL Average Throughput Periodic |  |  |  |  |  |  |
| > DL PDCP Packet Average Delay Periodic |  |  |  |  |  |  |
| > UL PDCP Packet Average Delay Periodic |  |  |  |  |  |  |
| > DL Packet Average Loss Rate Periodic |  |  |  |  |  |  |
| > UL Packet Average Loss Rate Periodic |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U channel IDs in a cell. Value is 16. |

9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |
| Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | – |  |
| >>Successful HO Report Container | O |  | OCTET STRING | SuccessHO-Report-r17 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| UE performance Information | O |  |  | Per UE performance Information for AI based mobility optimization |  |  |
| > UE Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  |  |  |
| > DL Throughput Periodic |  |  |  |  |  |  |
| > UL Throughput Periodic |  |  |  |  |  |  |
| > DL PDCP Packet Delay Periodic |  |  |  |  |  |  |
| > UL PDCP Packet Delay Periodic |  |  |  |  |  |  |
| > DL Packet Loss Rate Periodic |  |  |  |  |  |  |
| > UL Packet Loss Rate Periodic |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |

9.2.2.xx	Energy efficiency

The Energy Efficiency IE indicates the energy efficiency.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Energy Efficiency | M |  | - INTEGER (0..100) |  |

9.2.3.xx	Predicted UE Trajectory

The Predicted UE Trajectory IE contains information about predicted cells from AI/ML model inference results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Predicted Cell List |  | 1..<maxnoofCellsinUEPredictedInfo> |  | Most recent information is added to the top of this list |
| > Predicted NG-RAN Cell Information | M |  | 9.2.3.yy |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEPredictedInfo | Maximum number of Predicted cell information records that can be reported in the IE. Value is 16. |

9.3.1.97	Predicted NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of predicted NR cells and expected time of stay into the cells. The information is to be used for mobility enhancements using AI & ML purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | 9.2.2.27 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |

9.2.3.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the XnAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Set ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Resources not available for the slice(s), UE Maximum integrity protected data rate reason, CP Integrity Protection Failure, UP Integrity Protection Failure, Slice(s) not supported by NG-RAN, MN Mobility, SN Mobility, Count reaches max value, Unknown Old NG-RAN node UE XnAP ID, PDCP Overload, DRB ID not available, Unspecified, …, UE Context ID not known, Non-relocation of context, CHO-CPC resources to be changed, RSN not available for the UP, NPN access denied, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, UE Power Saving, Not existing NG-RAN node2 Measurement ID, Insufficient UE Capabilities, Normal Release, Value out of allowed range, SCG activation deactivation failure, SCG deactivation failure due to data transmission, Handover Desirable for ML/AI Inference), |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

Change Ends