3GPP TSG-RAN WG3 Meeting #118                                                                        R3-226711

14 – 18 November, 2022

Agenda Item:	12.2.2.1

Source:	CMCC

Title:	Discussion on event-based reporting

Document for:	Discussion and Decision

1	Introduction

In RAN3#117 e-meeting, it was agreed that a new procedure is introduced for AI/ML related information:

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information.

The new procedure for reporting of AI/ML related information, e.g., predicted information, should be based in a requested way, like resource status report procedure.

Then in last RAN3#117 e-meeting, following further agreements achieved [1]:

Introduce a new Class 1 procedure for initiating the reporting of AI/ML Related Information and a Class 2 procedure for Data Reporting of AI/ML Related Information.

Reporting options for the new procedure used for AI/ML Related Information to be evaluated on a case-by-case basis. Possible reporting options are one-time and periodic reporting.

And Event-based reporting is ffs:

Event-based reporting and how to determine an event are FFS.

In this contribution, we provide considerations on event-based reporting.

2	Discussion

Event-based reporting was proposed since periodic reporting may provide unnecessary measurements in some cases, e.g., when the only intention is to monitor events or updates in model inference. Where event-based reporting could be introduced to capture changes or updates in the reported predictions [2].  Furthermore, following event-based reporting scenarios were proposed by companies:

- Scenario 1: previous prediction becomes invalid, or the accuracy of predicted information is not good enough

- Scenario 2: a special event happens, e.g., overload, source NG-RAN node cannot provide the predicted information

In our opinion, event-based reporting could reduce the reductant measurements reporting and suitable for at least above two scenarios, which could be used as the start point.

Proposal 1: Event-based reporting is introduced for following scenarios:

- Scenario 1: Previous prediction becomes invalid, or the accuracy of predicted information is not good enough

- Scenario 2: A special event happens, e.g., overload, source NG-RAN node cannot provide the predicted information

3	Summary

In this contribution, we provide considerations on remaining issues on event-based reporting. Following proposals are made:

Proposal 1: Event-based reporting is introduced for following scenarios:

- Scenario 1: Previous prediction becomes invalid, or the accuracy of predicted information is not good enough

- Scenario 2: A special event happens, e.g., overload, source NG-RAN node cannot provide the predicted information

4	Reference

[1] 	RAN3_117bis-e_agenda_20221018_EOM1

[2]	R3-225917, Summary of email Discussion on CB: # AIRAN2_XnImpact, Nokia