3GPP TSG-RAN3 Meeting #118	R3-226922

Toulouse, France, 14th - 18th November 2022

Title:	Reply LS on the re-establishment of the MBS context during mobility registration update or service request procedure

Response to:	R3-226172

Release:	Rel-18

Work Item:	FS_5MBS_Ph2

Source:	CATT

To:	SA2

Cc:	CT1, CT4

Contact Person:

Name:                Aijuan Liu

E-mail Address:   liuaijuan (at) catt (dot) cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks SA2 for the LS on the re-establishment of the MBS context during mobility registration update or service request procedure. RAN3 would like to provide the following feedback for SA2’s questions:

Q1: Considering the related N1SM and N2SM information needs to be sent to UE and NG-RAN during the multicast session join/leave procedures as specified in Rel-17, is it possible and desirable for the SMF to handle the procedure as described in the two solutions? For example, how is the MBS session related context information is sent to UE and NG-RAN without SM message?

Q2: Separating the MM and the SM signaling is a design principle of the 5GS from beginning. In the solutions, the UE includes multicast MBS session information container, which is information for SM handling, in the MM signalling (i.e. Registration Request and Service Request). Do CT1/CT4/RAN3 see problems with adopting this proposed procedure in Rel-18, considering the potential issue to be solved, e.g. coordinating the MM/SM handling?

Answer to Q1 and Q2: RAN3 understanding is that this question depends on discussions in other groups than RAN3.

2. Actions:

To SA2:

ACTION: RAN3 kindly asks SA2 to take above into account.

3. Date of Next TSG-RAN Meetings:

TSG-RAN3 Meeting #119-e	             27 Feb-3 March 2023                 Athens

TSG-RAN3 Meeting #119bis-e	         17-26 April 2023                      Online