3GPP TSG-RAN WG3 Meeting #119	R3-230886

# Athens, Greece, 27th Feb – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0274 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Network-Controlled Repeater |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Core |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add the support for Network-Controlled Repeater |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are added: - Add a new section for NCR integration procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not able to support NCR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: updated during RAN3#119 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 8.x	Overall procedures for Network Controlled Repeater

## 8.x.y	NCR- Integration Procedure

A high-level flow chart for NCR integration is shown below:

Figure 8.x.y-1: The integration procedure for NCR

Phase 1: NCR-MT setup. In this phase, the NCR-MT of the new NCR connects to the network in the same way as a UE, by performing RRC connection setup procedure with gNB-CU, and authentication with the core network. The NCR-MT selects a cell for access based on NCR support indication in SIB1, and includes the NCR indication in RRCSetupComplete message to assist the gNB to select an AMF supporting NCR. Upon receiving the NCR authorization information from 5GC, the gNB-CU provides the authorization information to the gNB-DU.

NOTE 1: The signalling flow for UE initial access procedure as shown in Figure 8.1-1/Figure 8.9.1-1 is used for the setup of the NCR-MT.

Phase 2: The gNB-CU may configure the NCR via the RRC.

Editor’s NOTE: The information to be configured via RRC pends on RAN2.

Phase 3: After the NCR is configured, it may start serving the UEs.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>