3GPP TSG-RAN WG3 Meeting #119	R3-230103

Athens, Greece, February 27th –March 3rd, 2023

Title: 	(TP to TS 38.423 BL CR) Consideration on selective activation of SCGs

Source: 	Huawei

Agenda item:	14.4

Document Type:	other

# Introduction

In this contribution, on selective activation of SCGs, taking into consideration of both RAN3 and RAN2 conclusions for the agreed solutions and FFSs, we provide our views on the potential solutions and their impacts on RAN3 and provide related TP to TS 38.423.

# Background

For selective activation of SCGs related objective, the following agreements/WA were made during previous RAN3 meetings:

RAN3 considers SCG selective activation is prioritized in the Rel-18 work. It can be revisited based on RAN2 progress.

RAN3 considers the Inter-CU and Intra-CU cases with equal priority, and studies both the F1 and Xn signaling aspects. It can be revisited based on RAN2 progress.

From RAN3 point of view, Rel-16/Rel-17 CPAC procedures are considered as start point for the Rel-18 work.

The following scenarios are depending on RAN2 progress.

SCG failure handling enhancements to enable PSCell addition and PSCell change after SCG failure.

Signaling support for inclusion of CPC configuration within a CPC or CPA configuration, in case CPC/CPA configuration is supported within CHO configuration.

WA: A primary focus of the objective is to enable subsequent cell changes by keeping conditional reconfigurations after a cell change. RAN3 to pursue study of the Xn/F1 signaling changes required to support this objective.

WA: RAN3 will work to enable both indirect and direct early data forwarding in Selective Activation. At this moment, RAN3 does not foresee any scenarios where direct forwarding is not feasible/desired.

WA (up to RAN2’s discussion): RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation.

WA: Enhance signalling for Selective Activation.

In RAN2#119bis-e meeting, there were some progresses on the scenarios to be studied for selective activation of SCG:

Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

Confirm that we aim to support delta configuration, i.e. that there need to be known reference.

In RAN2#120 meeting, agreements have been made as below:

Delta configuration

A UE stores the reference configuration as a separate configuration.

The reference configuration is managed separately.

# Discussion

## 3.1 Selective activation indication for subsequent CPAC

SCG selective activation is introduced in Rel-18 to reduce signalling overhead and increase mobility robustness.

To reduce signalling overhead, we would like to reduce the signalling over both Uu and Xn interfaces. To increase the mobility robustness, we would like to reduce the possibility of SCG failure, and SCG interruption time.

The basic scenario for SCG selective activation is UE move to one of the prepared candidate PSCells after the initial CPA/CPC execution. To support the subsequent CPC execution within the prepared PSCells, it requires the UE to keep/store the CPAC configuration after successful (conditional) PSCell addition/change. As the release of CPAC configuration is performed by Rel-17 UEs after successful (conditional) PSCell addition/change, the Rel-18 UEs should only keep the conditional configurations only in case some specific indication is received from the network, and the C-SN should only provide rel-18 selective SCG activation configuration (e.g. subsequent CPC configuration within the CPAC configuration) in case the MN indicates it to do so.

For support of C-SN delta config relative to reference config agreed in last RAN2 meeting, the candidate SN needs to generate the candidate configuration using the reference config instead of source SCG config as baseline in case of UE in DC. In case of UE in SA (standalone), it might require C-SN to even prepare the candidate configuration of CPA in delta config based on the reference configuration, since RAN2 has confirmed that “CPA” selective activation of cell groups will be supported for this WI objective, as well as subsequent CPC.

Proposal 1: In the preparation phase of selective activation, it is required that MN indicates candidate SNs of selective activation of SCG to distinguish from legacy/conditional SN addition request.

## 3.2 Reference configuration generation and exchanging

For support of C-SN delta config relative to reference config, the candidate SN should be provided with reference config. However, RAN2 has not decided yet whether there is only one reference configuration or multiple reference configuration for all the C-SNs, and which node to generate the reference configuration.

In case the reference configuration is generated by the C-SN, the MN needs to ask the C-SN to provide reference configuration in the SN Addition procedure. In case the reference configuration is generated by the S-SN, the MN needs to ask source SN to provide reference configuration in e.g. MN initiated SN modification procedure.

In case the selective activation of SCGs are triggered in case the UE is served by the MN in standalone case, i.e. there is no S-SN, we think that the reference configuration should not be generated by the MN, therefore, regardless of RAN2 progress and whether there is a reference config or multiple reference configs, there is a need to support the case in which the C-SN provides reference configuration in SN Addition Request ACK. Taking into account proposal 1, it is FFS on whether explicit reference configuration request indication is needed from MN in SN Addition Request.

Proposal 2: The C-SN should be able to provide reference configuration to the MN in SN Addition Request ACK, FFS on whether explicit reference config request indication is needed from MN in SN Addition Request.

Proposal 3: The support of providing reference configuration from S-SN is pending to RAN2 progress, i.e. whether the S-SN need to generate the reference configuration(s) for C-SNs.

# Conclusion

In this paper, we discuss miscellaneous enhancements for selective activation of SCGs, and we have the following observations and proposals:

Proposal 1: In the preparation phase of selective activation, it is required that MN indicates candidate SNs of selective activation of SCG to distinguish from legacy/conditional SN addition request.

Proposal 2: The C-SN should be able to provide reference configuration to the MN in SN Addition Request ACK, FFS on whether explicit reference config request indication is needed from MN in SN Addition Request.

Proposal 3: The support of providing reference configuration from S-SN is pending to RAN2 progress, i.e. whether the S-SN need to generate the reference configuration(s) for C-SNs.

To support proposal 1, the corresponding TP is provided in section 6.

# Reference

- RAN3_118_agenda_20221118_1710

- R2_120 Chair Notes EOM rev1

# TP to TS 38.423 BL CR

------------Start of the First Change----------------

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

//skip the unchanged part

If the Conditional PSCell Addition Information Acknowledge is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, shall, if supported, consider the indicated PSCells are selected by the target SN as candidate PSCells for CPAC.

If the CG-CandidateList is included in the S-NG-RAN node to M-NG-RAN node Container IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it for the purpose of CPAC.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

If the S-NG-RAN node UE Slice Maximum Bit Rate IE for a specific S-NSSAI is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

If the Selective SCG Activation Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node may use it to configure SCG resources as specified in TS 37.340 [8].

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

------------Start of the Next Change----------------

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| //skip the unchanged part |  |  |  |  |  |  |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| Selective SCG Activation Request | O |  | 9.2.3.xxx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

------------Start of the Next Change----------------

#### 9.2.3.xxx	Selective SCG Activation Request

This IE indicates whether the resources for selective activation of SCG are required.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Selective SCG Activation Request | M |  | ENUMERATED (true, …) |  |

Editor’s Note: the name and definition of the IE is FFS.

------------Start of the Next Change----------------

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

//skip unchanged part

NRPagingeDRXInformationforRRCINACTIVE,

SDTSupportRequest,

SDT-Termination-Request,

SDTPartialUEContextInfo,

SDTDataForwardingDRBList,

PEIPSassistanceInformation,

UESliceMaximumBitRateList,

PagingCause,

MDTPLMNModificationList,

F1-terminatingIAB-donorIndicator,

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

SelectiveSCG-ActivationRequest

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AdditionalDRBIDs,

id-AMF-Region-Information,

id-AMF-Region-Information-To-Add,

//skip unchanged part

id-SDT-Termination-Request,

id-SDTPartialUEContextInfo,

id-SDTDataForwardingDRBList,

id-PEIPSassistanceInformation,

id-UESliceMaximumBitRateList,

id-S-NG-RANnodeUE-Slice-MBR,

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-SelectiveSCG-ActivationRequest,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

//skip unchanged part

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- The IE shall be present if there is at least one  PDUSessionResourceSetupInfo-SNterminated included --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional }|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-UERadioCapabilityID					CRITICALITY reject		TYPE UERadioCapabilityID							PRESENCE optional }|

{ ID id-SourceNG-RAN-node-ID				CRITICALITY ignore		TYPE GlobalNG-RANNode-ID							PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY ignore		TYPE MDTPLMNList									PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore		TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-PSCellChangeHistory					CRITICALITY ignore		TYPE PSCellChangeHistory							PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject		TYPE IABNodeIndication								PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore		TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-CHOinformation-AddReq				CRITICALITY reject		TYPE CHOinformation-AddReq						PRESENCE optional }|

{ ID id-SCGActivationRequest				CRITICALITY ignore		TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CPAInformationRequest				CRITICALITY reject		TYPE CPAInformationRequest						PRESENCE optional }|

{ ID id-S-NG-RANnodeUE-Slice-MBR				CRITICALITY reject		TYPE UESliceMaximumBitRateList					PRESENCE optional }|

{ ID id-F1-terminatingIAB-donorIndicator		CRITICALITY reject		TYPE F1-terminatingIAB-donorIndicator			PRESENCE optional }|

{ ID id-SelectiveSCG-ActivationRequest		CRITICALITY ignore		TYPE SelectiveSCG-ActivationRequest			PRESENCE optional },

...

}

------------Start of the Next Change----------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

//skip unchanged part

#### -- S

SCGreconfigNotification ::= ENUMERATED {executed, ...}

S-NSSAIListQoE ::= SEQUENCE (SIZE(1..maxnoofSNSSAIforQMC)) OF S-NSSAI

S-BasedMDT ::= SEQUENCE {

ng-ran-TraceID				NG-RANTraceID,

iE-Extension				ProtocolExtensionContainer { {S-BasedMDT-ExtIEs} }	OPTIONAL,

...

}

S-BasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SelectiveSCG-ActivationRequest ::= ENUMERATED {true, ...}

ServiceType ::= ENUMERATED{

qMC-for-streaming-service,

qMC-for-MTSI-service,

qMC-for-VR-service,

...

}

SecondarydataForwardingInfoFromTarget-Item::= SEQUENCE {

secondarydataForwardingInfoFromTarget				DataForwardingInfoFromTargetNGRANnode,

iE-Extensions		ProtocolExtensionContainer { { SecondarydataForwardingInfoFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

SecondarydataForwardingInfoFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

------------Start of the Next Change----------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

//skip unchanged part

id-PositioningInformation																			ProtocolIE-ID ::= 360

id-UEAssistantIdentifier																			ProtocolIE-ID ::= 361

id-ManagementBasedMDTPLMNModificationList															ProtocolIE-ID ::= 362

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-TAINSAGSupportList																				ProtocolIE-ID ::= 364

id-SCGreconfigNotification																			ProtocolIE-ID ::= 365

id-earlyMeasurement																					ProtocolIE-ID ::= 366

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-SelectiveSCG-ActivationRequest																	ProtocolIE-ID ::= xxx

END

-- ASN1STOP

------------End of the Changes----------------