3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231196

Electronic, 17 - 26 April 2023

Agenda item:	23.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for TS 38.413 BL CR) RAN feedback for low latency communication

Document for:	Discussion and Decision

# 1	Introduction

At RAN#99, a new WID on NR Timing Resiliency and URLLC enhancements was approved in [1]. It includes the following objective:

3. Adapting downstream and upstream scheduling based on RAN feedback for low latency communication [RAN3, RAN2]: a. RAN enhancements in order for application to adapt scheduling based on RAN feedback (e.g., feedback regarding burst arrival time, periodicity) for low latency communication. Note 3: Reactive RAN feedback for upstream scheduling is pending RAN2 conclusion on burst arrival time (BAT) offset derivation.

In this paper, we evaluate the RAN3 specification impacts of objective 3 (Adapting downstream and upstream scheduling based on RAN feedback for low latency communication), and provide an initial set of proposals.

# 2	Discussion

SA2 has agreed on two mechanisms for RAN feedback:

From TS 23.501 v18.1.0: ##### 5.27.2.5.1 Overview […] NG-RAN may support the following feedback mechanisms: - Proactive RAN feedback for adaptation of Burst Arrival Time and Periodicity: NG-RAN may provide a Burst Arrival Time offset and an adjusted Periodicity as part of QoS flow establishment or modification as illustrated in clause 5.27.2.5.2; - Reactive RAN feedback for Burst Arrival Time adaptation: NG-RAN may provide a Burst Arrival Time offset after QoS flow establishment as illustrated in clause 5.27.2.5.3.

Proactive RAN feedback applies if an application does not tolerate mid-stream burst timing adaptation and when application and 5GS are not in time synch.

Reactive RAN feedback applies when application and 5GS are not in time sync, and application session tolerates mid-stream timing adjustments.

## 2.1	Proactive RAN feedback

For proactive RAN feedback SA2 has agreed on the following solution:

From TS 23.501 v18.1.0: ##### 5.27.2.5.2 Proactive RAN feedback for adaptation of Burst Arrival Time and Periodicity If the NG-RAN receives a Burst Arrival Time and Burst Arrival Time Window in the TSCAI for a QoS Flow, the 5GS will perform the following actions: - The NG-RAN can determine a BAT offset in order to align the expected arrival of the traffic bursts (as indicated in the BAT) with the time when the next transmission over the air interface in each direction (i.e. DL or UL) is expected. The BAT offset shall always be provided by NG-RAN and it shall be within the BAT Window. The BAT offset is calculated with reference to the BAT. - If the BAT offset is provided from NG-RAN to the SMF in the response to the QoS Flow establishment or modification request, the SMF provides the BAT offset to the PCF and the PCF notifies the AF as described in clause 6.1.3.23a of TS 23.503 [45]. - The SMF may adjust the BAT offset received from NG-RAN based on the clock drifting report from UPF as specified in clause 4.4.3.4 of TS 23.502 [3]. NOTE: The feedback from RAN implies that the RAN accepts the BAT offset. If the AF requested BAT is acceptable for NG-RAN, the NG-RAN provides a BAT offset of zero. - If the RAN also receives a Periodicity Range along with the Periodicity in the TSCAI for a QoS flow, the 5GS will further perform the following actions: - The RAN may determine an adjusted periodicity in order to align the periodicity of the traffic bursts with the expected time interval between subsequent transmission opportunities over the air interface in each direction (i.e. DL or UL). If the RAN determines an adjusted periodicity, the RAN provides it together with a BAT offset mentioned above. The adjusted periodicity shall be within the Periodicity Range and the BAT offset is based on the adjusted periodicity. - The adjusted periodicity is forwarded to the AF via the SMF and the PCF together with a BAT offset in the same way it is described above.

An end to end signaling flow for proactive RAN feedback is illustrated in figure 1 below.

Figure 1: Proactive RAN feedback signaling flow.

As seen from figure 1, from RAN perspective the proactive RAN feedback is provided as follows:

Step 1:		gNB receives RAN feedback information from the AMF in the TSC Assistance Information (TSCAI).

-	If the TSCAI contains Burst Arrival Time (BAT), Burst Arrival Time Window (BAT Window) and optionally Periodicity Range, gNB determines that proactive feedback is expected and whether the adjusted periodicity is to be provided together with the BAT offset.

Step 2:		gNB determines requested RAN feedback based on the radio configuration.

-	Which RAN nodes (gNB-CU and/or gNB-DU) need to be involved in RAN feedback determination depends on the used radio configuration information. For instance, if feedback is determined based on the TDD configuration then the CU has all the relevant information for feedback determination, but if scheduling information needed then the DU hosts all the relevant information.

Step 3:	gNB sends feedback (BAT offset and optionally Adjusted Periodicity) to SMF “in the response to the QoS flow establishment or modification request”, meaning that feedback is sent in the SMF container in the NGAP response message from gNB to AMF.

Based on the above description, the following is proposed:

Proposal 1:	For proactive RAN feedback, enhance the NGAP TSC Assistance Information IE to include the Burst Arrival Time Window IE and Periodicity Range IE. Whether F1AP enhancements are needed is FFS.

Proposal 2:	If gNB supports proactive RAN feedback, it provides the requested Burst Arrival Time Offset IE and Adjusted Periodicity IE in the SMF response container (i.e. within the PDU Session Resource Setup Response Transfer IE or PDU Session Resource Modify Response Transfer IE).

## 2.2	Reactive RAN feedback

For reactive RAN feedback SA2 has agreed on the following solution:

From TS 23.501 v18.1.0: ##### 5.27.2.5.3 Reactive RAN feedback for Burst Arrival Time adaptation If the RAN receives the capability for BAT adaptation without a Burst Arrival Time in the TSCAI and notification control is enabled for this QoS Flow, the 5GS will perform the following actions: - If NG-RAN determines that the PDB of the QoS flow cannot be fulfilled in DL and UL direction, then if supported, NG-RAN shall determine a BAT offset value which reduces the time between the arrival of the traffic bursts and the time of the next possible transmission over the air interface for DL and UL, respectively. NG-RAN shall not provide a BAT offset with the same value until the PDB of the QoS Flow can be fulfilled again. NOTE: NG-RAN determines BAT offset value in reference to the current arrival time of the bursts experienced by RAN in DL and by UE in UL. Further details on BAT offset determination for DL and UL will be defined by RAN WG2. - The BAT offset is provided from NG-RAN to the SMF when sending the notification towards the SMF that the "GFBR can no longer be guaranteed" described in clause 5.7.2.4. The SMF provides the BAT offset to the PCF and the PCF provides the BAT offset to the AF as part of notifying the AF as described in clause 6.1.3.23a of TS 23.503 [45]

An end to end signaling flow for reactive feedback is illustrated in figure 2 below.

Figure 2. Reactive RAN feedback signaling flow.

The same three steps as in proactive RAN feedback can be defined for reactive RAN feedback:

Step 1:	As in the proactive RAN feedback mechanism, the gNB receives RAN feedback information from the AMF in the TSCAI.

-	The TSCAI contains Capability for BAT Adaptation but no BAT if reactive feedback is requested.

-	Also, reactive feedback is applicable only when notification control is enabled for the QoS Flow.

Step 2: 	When the gNB determines that the Packet Delay Budget (PDB) of the QoS flow cannot be met with the existing BATs, but the PDB could be fulfilled with better burst timing, the gNB determines the required BAT offset.

-	Similar to the proactive feedback mechanism, which RAN nodes (gNB-CU and/or gNB-DU) need to be involved in feedback determination depends on the used radio configuration information.

Step 3: 	gNB sends feedback (BAT offset) when sending a “not fulfilled” notification towards the CN, i.e. the feedback is sent in the SMF container using the PDU Session Resource Notify procedure.

Comparing figure 2 with figure 1, the main difference is that reactive feedback is provided using QoS notification control. Based on the above description, the following is proposed:

Proposal 3:	For reactive RAN feedback, enhance the NGAP TSC Assistance Information IE to include the Capability for BAT Adaptation IE. Whether F1AP enhancements are needed is FFS.

Proposal 4:	If gNB supports reactive RAN feedback and notification control is enabled, it provides the requested Burst Arrival Time Offset IE in the PDU SESSION RESOURCE NOTIFY message (i.e. within the PDU Session Resource Notify Transfer IE) when sending a “not fulfilled” notification.

# 3	Conclusions

The following is proposed:

Proposal 1:	For proactive RAN feedback, enhance the NGAP TSC Assistance Information IE to include the Burst Arrival Time Window IE and Periodicity Range IE. Whether F1AP enhancements are needed is FFS.

Proposal 2:	If gNB supports proactive RAN feedback, it provides the requested Burst Arrival Time Offset IE and Adjusted Periodicity IE in the SMF response container (i.e. within the PDU Session Resource Setup Response Transfer IE or PDU Session Resource Modify Response Transfer IE).

Proposal 3:	For reactive RAN feedback, enhance the NGAP TSC Assistance Information IE to include the Capability for BAT Adaptation IE. Whether F1AP enhancements are needed is FFS.

Proposal 4:	If gNB supports reactive RAN feedback and notification control is enabled, it provides the requested Burst Arrival Time Offset IE in the PDU SESSION RESOURCE NOTIFY message (i.e. within the PDU Session Resource Notify Transfer IE) when sending a “not fulfilled” notification.

A text proposal for TS 38.413 is provided in Annex A (with encoding details left FFS).

# References

- RP-230754 New WID on NR Timing Resiliency and URLLC enhancements, Nokia, Nokia Shanghai Bell

# Annex	A: Text Proposal for TS 38.413

First Modification

#### 9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  | - |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  | - |  |
| Survival Time | O |  | 9.3.1.221 |  | YES | ignore |
| Burst Arrival Time Window | O |  | 9.3.1.x1 | This IE may be present if the Burst Arrival Time IE is included; otherwise, it is ignored. | YES | ignore |
| Capability for BAT Adaptation | O |  | 9.3.1.x2 | This IE may be present if the Burst Arrival Time IE is omitted; otherwise, it is ignored. | YES | ignore |
| Periodicity Range | O |  | 9.3.1.x3 | This IE may be present if both the Burst Arrival Time IE and Burst Arrival Time Window IE are included; otherwise, it is ignored. | YES | ignore |

Next Modification

#### 9.3.1.x1	Burst Arrival Time Window

This IE indicates the burst arrival time window of the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

#### 9.3.1.x2	Capability for BAT Adaptation

This IE indicates the capability for BAT adaptation for the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

#### 9.3.1.x3	Periodicity Range

This IE indicates the periodicity range for the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

#### 9.3.1.x5	RAN Feedback for TSC Traffic

This IE provides the RAN feedback for adaptation of a TSC QoS flow (see TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TSC Feedback Information Downlink | O |  | TSC Feedback Information 9.3.1.x6 |  |
| TSC Feedback Information Uplink | O |  | TSC Feedback Information 9.3.1.x6 |  |

#### 9.3.1.x6	TSC Feedback Information

This IE provides the TSC feedback information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time Offset | M |  | [FFS] |  |
| Adjusted Periodicity | O |  | [FFS] | Not applicable to reactive RAN feedback. |

Next Modification

#### 9.3.4.2	PDU Session Resource Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL QoS Flow per TNL Information | M |  | QoS Flow per TNL Information 9.3.2.8 | NG-RAN node endpoint of the NG-U transport bearer for delivery of DL PDUs, together with associated QoS flows. | - |  |
| Additional DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows and corresponding to the Additional UL NG-U UP TNL Information IE in the PDU Session Resource Setup Request Transfer IE. | - |  |
| Security Result | O |  | 9.3.1.59 |  | - |  |
| QoS Flow Failed to Setup List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |
| Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information 9.3.2.8 | NG-RAN node endpoint of the NG-U transport bearer(s) for delivery of DL PDUs of the indicated Redundant QoS Flow(s) and corresponding to the Redundant UL NG-U UP TNL Information IE in the PDU Session Resource Setup Request Transfer IE. | YES | ignore |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of redundant DL PDUs for split PDU session, together with associated QoS flows and corresponding to the Additional Redundant UL NG-U UP TNL Information IE in the PDU Session Resource Setup Request Transfer IE. | YES | ignore |
| Used RSN Information | O |  | Redundant PDU Session Information 9.3.1.136 |  | YES | ignore |
| Global RAN Node ID of Secondary NG-RAN Node | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| MBS Support Indicator | O |  | 9.3.1.210 |  | YES | ignore |
| MBS Session Setup Response List | O |  | 9.3.1.213 |  | YES | ignore |
| MBS Session Failed to Setup List | O |  | 9.3.1.214 |  | YES | ignore |
| QoS Flow TSC Feedback List |  | 0..1 |  |  | YES | ignore |
| >QoS Flow TSC Feedback Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>RAN Feedback for TSC Traffic | M |  | 9.3.1.x5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

Next Modification

#### 9.3.4.4	PDU Session Resource Modify Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the 5GC node. | - |  |
| QoS Flow Add or Modify Response List |  | 0..1 |  |  | - |  |
| >QoS Flow Add or Modify Response Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.152 | Index to the currently fulfilled alternative QoS parameters set | YES | Ignore |
| >>RAN Feedback for TSC Traffic | O |  | 9.3.1.x5 |  | YES | Ignore |
| Additional DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows. | - |  |
| QoS Flow Failed to Add or Modify List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |
| Additional NG-U UP TNL Information | O |  | UP Transport Layer Information Pair List 9.3.2.11 | NG-RAN node endpoint of the NG-U transport bearer corresponding to the modified UPF endpoint received in the PDU Session Resource Modify Request Transfer IE in case of PDU session split. | YES | ignore |
| Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs for the redundant transmission. | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the 5GC node for the redundant transmission. | YES | ignore |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of redundant DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Additional Redundant NG-U UP TNL Information | O |  | UP Transport Layer Information Pair List 9.3.2.11 | NG-RAN node endpoint of the NG-U transport bearer for delivery of redundant DL PDUs corresponding to the modified UPF endpoint(s) received in the UL NG-U UP TNL Modify List IE of the PDU Session Resource Modify Request Transfer IE in case of PDU session split. | YES | ignore |
| Secondary RAT Usage Information | O |  | 9.3.1.114 |  | YES | ignore |
| MBS Support Indicator | O |  | 9.3.1.210 |  | YES | ignore |
| MBS Session Setup or Modify Response List | O |  | MBS Session Setup Response List 9.3.1.213 |  | YES | ignore |
| MBS Session Failed to Setup or Modify List | O |  | MBS Session Failed to Setup List 9.3.1.214 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.3.4.5	PDU Session Resource Notify Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow Notify List |  | 0..1 |  |  | - |  |
| >QoS Flow Notify Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Notification Cause | M |  | ENUMERATED (fullfilled, not fulfilled, …) |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Notify Index 9.3.1.153 | Index to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameters set. | YES | Ignore |
| >>RAN Feedback for TSC Traffic | O |  | 9.3.1.x5 | Applicable when the Notification Cause IE is set to “not fulfilled”. | YES | ignore |
| QoS Flow Released List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |
| Secondary RAT Usage Information | O |  | 9.3.1.114 |  | YES | ignore |
| QoS Flow Feedback List |  | 0..1 |  |  | YES | ignore |
| >QoS Flow Feedback Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Update Feedback | O |  | BIT STRING { CN PDB DL(0), CN PDB UL(1)} (SIZE(8, …)) | Each position in the bitmap represents a QoS parameter. If a bit is set to "1", the respective parameter was not updated. If a bit is set to "0", the respective parameter was successfully updated. Bits 2-7 reserved for future use. | - |  |
| >>CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.135 | Indicates when the packet delay budget downlink was not updated in path switch that NG-RAN can offer this value | - |  |
| >>CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.135 | Indicates when the packet delay budget uplink was not updated in path switch that NG-RAN can offer this value | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

End Modification