3GPP TSG-RAN WG3 Meeting #119bis-e                                                                R3-231278

Online, April 17th – 26th, 2023

Source:	CATT

Title:	(TP for BLCR) Discussion on Paging Enhancement

Agenda Item:	24.2

Document for:	Discussion and Decision

1 Introduction

According to the WID [1], objectives of Rel-18 NR Network energy savings related to RAN3 are listed as below:

Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

- Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]

In the last meeting[2], enhancement on restricting paging in a limited area was discussed but no agreement was made.

In this contribution, we provide our understanding on the paging enhancement for network energy saving, and the TPs for TS 38.423 and TS 38.473 are provided accordingly.

2 Discussion

3GPP has studied the paging optimization since the LTE R13 MTC WI. It is used for idle mode UE. When releasing the UE context, the eNB reports the UE recent movement trajectory (cell id, eNB id, etc.) to the CN, and then the CN can decide to perform paging optimization according to the UE type, capability and other information. By the information obtained from eNB, the CN can indicate the assistance information (Recommended Cells for Paging, Recommended eNBs for Paging) for paging to the eNB in the paging message via S1AP. As a result, the paging can be restricted in the recommended area, i.e., omitted outside the recommended area, that paging signaling can be reduced over air interface.

There is similar paging optimization mechanism in NR. At UE context release, the NG-RAN node may provide the AMF with a list of recommended cells and NG-RAN nodes as the assistance info for subsequent paging.

Observation 1: It has already been supported by LTE/NR to restrict the paging in a limited area for idle mode UE.

It is assumed beneficial for network energy saving to limit the paging area for UE even to beam level based on the cell/node level paging optimization which is already supported. That is, the paging message can include the information of recommended beams for paging, so that the paging for UE can be restricted within the recommended beams.

In the last meeting, the on-line discussion focused on what kind of UE should the beam level paging optimization apply to. Whether the beam level paging is only applied to inactive mode UE or applied to both idle and inactive UE is discussed.

- Apply to inactive mode UE

As the context of inactive UEs is stored at the gNB-CU, paging over F1 interface needs to be enhanced to support beam level paging. For the gNB-DU is aware of the information about which beam the UE visited and the time the UE stayed at each visited beam, when the UE context is released at gNB-DU, the gNB-DU can report the list of recommended beams or SSBs as the assistance information to gNB-CU. The recommend beams or SSBs can include the UE’s visited or non-visited beams or SSBs. The gNB-CU can keep the information of recommended beams until receiving the user data for the UE. Then the gNB-CU determines the area which the paging should be performed based on the list of recommended beams or SSBs, UE’s movement trajectory, movement status or other information. When sending F1AP paging message to gNB-DU, the paging assistance information (i.e., recommended beams for paging) can be included, that gNB-DU can only send paging in the beams recommended by the gNB-CU.

- Apply to idle mode UE

As the context of inactive UEs is stored at the CN, not only the F1 interface but also the NG interface has to introduce the beam information to achieve the beam level paging. Also, gNB-DU should report the list of recommended beams or SSBs to gNB-CU. When the UE context is released at gNB-CU, the gNB-CU may convey the list of recommended beams or SSBs received from the gNB-DU as the assistance information to CN for subsequent paging. The CN has to keep the information of recommended beams until the DL data coming for the UE. It is CU to determine the beams which the paging should be performed (not only cell and nodes), based on the list of recommended beams, cells and nodes sent by the gNB-CU. Then CN can send the CN-paging message to gNB-CU with the paging assistance information including the recommended cells and beams. When gNB-CU sending F1AP paging message to gNB-DU, the recommended beams can be included, that gNB-DU can only send paging in the beams recommended.

Several companies supported the beam level paging should apply to both idle and inactive mode UE. However, for idle mode UE, CN should keep the beam level information for UE and determine not only the recommended cells/nodes but also the recommended beams in each cell of paging. This will impose much complexity on CN and it seems CN is involved in beam level management which in principle belongs to NG-RAN. Thus, we think the beam level paging should not apply to idle mode UE.

Proposal 1: Enhancement on restricting the paging in limited beams should only apply to inactive mode UE.

Proposal 2: To apply the paging enhancement to inactive UEs, enhance the F1AP messages:

- Introduce the information on recommended beams for paging in the UE CONTEXT RELEASE COMPLETE message;

- Introduce the recommended beams for paging in the PAGING message.

Also, the mobility status was discussed, e.g., whether the beam level paging is only applied to the stationary or applied to both stationary UE and low speed UE. We assume the mobility status of UE should not be restricted. If low speed UE is not supported, the gNB-DU only has to report the last visited beam for UE to the gNB-CU and the gNB-CU recommends just one beam in the F1AP paging message for the stationary UE. This can be a feasible solution, however, the efficiency of network energy saving will be very poor due to paging for UEs of all other mobility status is still cell level. To support the low speed UEs, the information of recommended beams from the gNB-DU can include the UE visited beams as well as the non-visited beams whose area is close to the visited beams, hence the gNB-CU can determine several paging beams for the low-speed UE base on its mobility status, movement trajectory or other information. This will not introduce much impact to spec.

Proposal 3: The paging enhancement should not be restricted to stationary UEs.

Proposal 4: The information on recommended beams UE CONTEXT RELEASE COMPLETE message includes the visited beams as well as the non-visited beams of UE.

For INACTIVE mode UE, currently, it is already been supported that NG-RAN node indicates paging for an inactive UE to the neighbor node in limited RAN notification areas or cells. To further decrease network energy consumption, paging optimization over XnAP is needed, it is similar as the paging optimization over F1AP. As the last serving node has obtained the information on UE’s movement trajectory and also be aware of UE capability and mobility status, it could determine whether to request the neigbhour node to page the UE within limited beams by including the recommended beams in the RAN paging message to the neighour nodes. For example, for the stationary UEs or UEs with low speed, the last serving node can request the neigbhour node to page UE only in the beams which cover the areas close to or overlapped with area of the last serving beam for the UE.

Proposal 5: Introduce the recommended beams for paging in the RAN PAGING message.

3 Conclusion

In the previous sections we made the following proposals:

Observation 1: It has already been supported by LTE/NR to restrict the paging in a limited area for idle mode UE.

Proposal 1: Enhancement on restricting the paging in limited beams should only apply to inactive mode UE.

Proposal 2: To apply the paging enhancement to inactive UEs, enhance the F1AP messages:

- Introduce the information on recommended beams for paging in the UE CONTEXT RELEASE COMPLETE message;

- Introduce the recommended beams for paging in the PAGING message.

Proposal 3: The paging enhancement should not be restricted to stationary UEs.

Proposal 4: The information on recommended beams in the UE CONTEXT RELEASE COMPLETE message includes the visited beams as well as the non-visited beams of UE.

Proposal 5: Introduce the recommended beams for paging in the RAN PAGING message.

4 Reference

- RP-223540, New WID: Network energy savings for NR, Huawei, RAN#98

- Meeting Minutes, RAN3 Chairman, RAN3#119

5 Test proposal

5.1 Text proposal for 38.423

<<<<<<<<<<<<<<<<<<<< Start of change >>>>>>>>>>>>>>>>>>>>

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards an ng-eNB (e.g. NG-RAN node2).

When available, the NG-RAN node1 shall include the E-UTRA Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the E-UTRA Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the NR Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the NR Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the NR Paging eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

When available, the NG-RAN node1 shall include the Paging Cause IE in the RAN PAGING message towards the NG-RAN node2. If the Paging Cause IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.331 [10].

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the Recommended SSBs for paging IE is included in the RAN PAGING message and if the NG-RAN is a gNB, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.38	RAN Paging Area

The RAN Paging Area IE defines the paging area within a PLMN for RAN paging a UE in RRC_INACTIVE state.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.2.4 |  |
| CHOICE RAN Paging Area Choice | M |  |  |  |
| >Cell List |  |  |  |  |
| >>Cell List Item |  | 1 .. < maxnoofCellsinRNA> |  |  |
| >>>NG-RAN Cell Identity | M |  | 9.2.2.9 | In this version of the specification, the RAN paging area should contain NG-RAN cells of the same RAT type. |
| >>> Recommended SSBs for paging |  | 0..1 |  | Includes the recommended SSBs where the paging is performed. |
| >>>> Recommened SSBs Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>> SSB Index | M |  | INTEGER (0..63) |  |
| >RAN Area ID List |  |  |  |  |
| >>RAN Area ID List Item |  | 1 .. <maxnoofRanAreasinRNA> |  |  |
| >>>RAN Area ID | M |  | 9.2.3.39 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

ASN.1 will be added after the solution is agreed.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

5.2 Text proposal for 38.473

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

### 8.3.3	UE Context Release (gNB-CU initiated)

#### 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message. If the CG-SDT Kept Indicator IE is contained in the UE CONTEXT RELEASE COMMAND message and set to "true", the gNB-DU shall, if supported, consider that the UE is sent to RRC_INACTIVE state with CG-SDT configuration and store the configured CG-SDT resources, C-RNTI, CS-RNTI, the CG-SDT related RLC configurations and F1-U connections associated with the SDT bearers while releasing the UE context.

If the old gNB-DU UE F1AP ID IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall additionally release the UE context associated with the old gNB-DU UE F1AP ID.

If the UE CONTEXT RELEASE COMMAND message contains the RRC-Container IE, the gNB-DU shall send the RRC container to the UE via the SRB indicated by the SRB ID IE.

If the UE CONTEXT RELEASE COMMAND message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall consider that the gNB-CU is cancelling only the conditional handover or conditional PSCell addition or conditional PSCell change associated to the cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Positioning Context Reservation Indication IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall not release the positioning context including the SRS configuration for the UE.

If the Information on Recommended SSBs for Paging IE is included in the UE CONTEXT RELEASE COMPLETE message, the gNB-CU shall, if supported, store it and may use it for subsequent paging.

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU, e.g. when gNB-CU rejects UE access it shall trigger UE Context Release procedure with the cause value of UE rejection.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.7.1	Paging

#### 8.7.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the gNB-DU to page a UE. The procedure uses non-UE associated signalling.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: Paging procedure. Successful operation.

The gNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it to determine the final paging cycle for the UE.

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

At the reception of the PAGING message, the gNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Origin IE may be included in the PAGING message, and if present the gNB-DU shall transfer it to the UE.

The RAN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The CN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information for RRC INACTIVE IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Paging Cause IE may be included in the PAGING message. If present the gNB-DU shall, if supported, send it to UE according to TS 38.331 [8].

The PEIPS Assistance Information IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [6].

The UEID Subgrouping Support Indication IE may be included in UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [6].

The RedCap Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging of RedCap UEs.

The Last Used Cell Indication IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, consider the cell identified by the NR CGI IE as the last used cell of the paged UE, and use it as specified in TS 38.331 [8].

The PEI Subgrouping Support Indication IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging early indication as described in TS 38.300 [6] and TS 38.304 [24].

The UE Paging Capability IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, take it into account when paging the UE.

The Recommended SSBs for Paging IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging of the UE, as specified in TS 38.300 [6].

#### 8.7.1.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the UE-associated logical F1 connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Information on Recommended SSBs for Paging | O |  | 9.3.1.X |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.6.1	PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.3.1.39 |  | YES | reject |
| CHOICE Paging Identity | M |  |  |  | YES | reject |
| >RAN UE Paging identity |  |  |  |  |  |  |
| >>RAN UE Paging identity | M |  | 9.3.1.43 |  | - |  |
| >CN UE paging identity |  |  |  |  |  |  |
| >>CN UE paging identity | M |  | 9.3.1.44 |  | - |  |
| Paging DRX | O |  | 9.3.1.40 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| Paging Priority | O |  | 9.3.1.41 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Last Used Cell Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >> Recommended SSBs for Paging | O |  | 9.3.1.X |  | - |  |
| Paging Origin | O |  | 9.3.1.79 |  | YES | ignore |  |
| RAN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the RAN paging cycle as defined in TS 38.304 [24]. | YES | ignore |  |
| CN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the UE specific paging cycle as defined in TS 38.304 [24]. | YES | ignore |  |
| NR Paging eDRX Information | O |  | 9.3.1.258 |  | YES | ignore |  |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.3.1.259 |  | YES | ignore |  |
| Paging Cause | O |  | ENUMERATED(voice, …) | This IE indicates the paging cause is IMS voice, refer to TS 23.501[21]. | YES | ignore |  |
| PEIPS Assistance Information | O |  | 9.3.1.269 |  | YES | ignore |  |
| UE Paging Capability | O |  | 9.3.1.270 |  | YES | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.X	Recommended SSBs for Paging

This IE contains recommended SSBs for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended SSB List |  | 1 |  |  |
| >Recommended SSB Item |  | 1..<maxnoofSSBAreas> |  | Includes visited and non-visited SSBs, where visited SSBs are listed in the order the UE visited them with the most recent SSB being the first in the list. Non-visited SSBs are included immediately after the visited SSB they are associated with. |
| >> SSB Index | M |  | INTEGER (0..63) |  |
| >>Time Stayed in SSB | O |  | INTEGER (0..4095) | This is included for visited SSBs and indicates the time a UE stayed in a SSB in seconds. If the UE stays in a SSB more than 4095 seconds, this IE is set to 4095. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

ASN.1 will be added after the solution is agreed.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>