3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231279

Online, April 17 – 26, 2023

Agenda item:	23.2

Source: 	Qualcomm Incorporated

Title: 	Enhancements for LPHAP

Document for:		Discussion

# 1.	Introduction

At RAN#98-e, the Work Item on "Expanded and Improved NR Positioning" was agreed which includes the following objective for "Low-Power, High-Accuracy Positioning (LPHAP)" [1]:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

Since multiple objectives of this Work Item may have RAN3 impacts, there is also a general objective for RAN3 [1]:

- Define extensions of signalling, protocol, and procedure for NR positioning enhancement, as needed for the above objectives [RAN3].

During the Study Item Phase it was agreed at RAN2#119e [2]:

"RAN2 to consider at least the 'Low Power Periodic and Triggered 5GC-MT-LR Procedures' in TS 23.273.  Other procedures are not excluded from discussion."

A simplified/RAN-centric procedure of the 'Low Power Periodic and Triggered 5GC-MT-LR Procedures' in RRC_INACTIVE state as defined in section 6.7 of TS 23.273 [3] is shown in Annex A of this contribution  (according to [4]) and is used as baseline for the discussion.

# 2.	SRS for positioning configuration enhancements

## 2.1	Pre-configuration of positioning SRS

At RAN1#112 and RAN2#121, some progress has been made on pre-configuration of positioning SRS [5]; in particular:

-	An SRS for positioning validity area is introduced.

-	When configured with SRS configuration along with SRS validity area, if the UE reselects to another cell within the SRS validity area during SRS transmission, the UE continues the SRS transmission, subject to validation for SRS transmission.

-	When the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

-	An SRS for positioning validity area consists of cells configured in the same band and the same carrier with common BWP information for the SRS for positioning (locationAndBandwidth, subcarrierSpacing, cyclicPrefix).

-	A SRS for positioning validity area specific TA timer may be configured. For TA validation, area specific RSRP change threshold may be used.

-	For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, some SRS for positioning parameter can commonly be configured across cells within the validity area, incl.

-	SRS-PosResourceSet

-	srs-PosResourceSetId

-	srs-PosResourceIdList

-	resourceType

-	SRS-PosResource

-	srs-PosResourceId

-	transmissionComb

-	resourceMapping

-	freqDomainShift

-	freqHopping

-	groupOrSequenceHopping

-	resourceType

-	The "Pathloss Reference Information" and "Spatial Relation Information" of an SRS for positioning configuration may or may not be included in an SRS for positioning pre-configuration. (FFS RAN1)

The SRS for positioning request should be independent on whether the SRS for positioning is associated with an area validity or not. When an event has been detected at the target device and the positioning method configured was an UL or UL+DL method, there would then be no need to perform the Steps 5-7 in Figure A.1 (see Annex) each time an event has been detected. An LMF would be aware of the pre-configured SRS and would only require an identifier of the activated SRS. Therefore, Step 3 in Figure A.1 does not need to be a NAS message to an LMF (Event Report) but could be an RRC message or MAC-CE to request SRS activation.

A modification of the deferred MT-LR procedure for the UL positioning part is shown in Figure 2.1-1 below.

Figure 2.1-1: Deferred MT-LR with positioning SRS pre-configuration.

1a.	During the deferred MT-LR configuration phase (e.g., during step 15 of Figure 6.3.1-1 in TS 23.273 [3]), the LMF sends a NRPPa Positioning Information Request message to the serving gNB including a request for preconfigured positioning SRS. The request may include one or more "Requested SRS Transmission Characteristics" IEs [6], each defining a positioning SRS configuration (e.g., Number Of Periodic Transmissions, Resource Type, Bandwidth, Resource Set List, SRS Frequency, etc.).

1b.	The serving gNB determines one or more positioning SRS configurations and provides these configurations to the target device for transmission of positioning SRS by the target device at a later time (i.e., the target device does not transmit any positioning SRS when pre-configured). 
Each positioning SRS configuration has an associated identifier.
Each positioning SRS may have a validity time and/or a validity area.

1c.	The serving gNB provides the set of preconfigured positioning SRS to the LMF (e.g., one or more "SRS Configuration" IEs [6] where each positioning SRS has an associated ID, and probably an validity area indication).

NOTE 1:	During the deferred MT-LR configuration phase, an LMF may also provide DL-PRS assistance data to the target device for UL+DL positioning. The pre-configured DL-PRS assistance data may consist of multiple instances, where each instance may be applicable to a different area within the network.

NOTE 2:	A target device may obtain the DL-PRS assistance data also via posSI.

NOTE 3:	The SRS for positioning validity area ID and the area ID for the pre-configured DL-PRS assistance data are independent of each other.

3.	After (or slightly before) an event is detected, the UE sends an RRC SRS Activation Request message along with the RRC Resume Request to the receiving gNB. 
The SRS Activation Request message may include the IDs of a desired preconfigured SRS configuration (e.g., in priority order) to be activated.

4.	The receiving gNB identifies the last serving gNB using the I-RNTI and retrieves the UE context (incl. the preconfigured positioning SRS information) by means of Xn-AP Retrieve UE Context procedure.

The receiving gNB determines the positioning SRS configuration based on the pre-configuration during step 1. The receiving gNB may determine the Pathloss Reference Information (e.g., alpha, p0, pathlossReferenceRS-Pos [7]) or Spatial Relation Information (e.g., SRS-SpatialRelationInfoPos [7]) for the positioning SRS valid for the receiving gNB. The receiving gNB may also determine the TA timer and RSRP-change-threshold (e.g., inactivePosSRS-TimeAlignmentTimer, inactivePosSRS-RSRP-changeThreshold [7]). The receiving gNB uses the assistance information received from the LMF at Step 1a to determine the above SRS parameter.

NOTE 4:	For the spatial relation/pathloss reference information of an SRS for positioning configuration in multiple cells, RAN1 is considering two options [5]:
Option 1: Spatial relation/pathloss reference information is absent in the pre-configuration.
Option 2: Spatial relation/pathloss reference information is provided in the pre-configuration.

5.	The receiving gNB may then send an RRC SRS Activation message to the UE which includes the ID of the preconfigured SRS to be activated, and possibly the pathloss reference, spatial relation, the TA timer and RSRP-change-threshold.

6.	The receiving gNB sends a NRPPa Positioning Information Update message to the LMF including the ID of the activated positioning SRS and the cell ID/cell portion ID of the current UE serving cell.

7.	The LMF sends a NRPPa Measurement Request message to a group of gNBs/TRPs incl. the positioning SRS measurement configuration (i.e., based on the ID received at Step 6).

8.	The receiving gNB sends an RRC Release message to the UE to release the UE to RRC_INACTIVE state. If step 5 did not occur, the RRC Release message includes the SRS Activation message.

Observation 1:	The pre-configuration of positioning SRS can occur during the location preparation phase of the 'Low Power Periodic and Triggered 5GC-MT-LR Procedures' (e.g., together with the pre-configuration of DL-PRS assistance data, if applicable). One or more positioning SRS configurations can be provided to the gNB/UE during the location preparation phase using modified NRPPa Positioning Information Exchange procedures.

Proposal 1:	The NRPPa Positioning Information Exchange procedures are extended to support pre-configuration of SRS for positioning. One or more Requested SRS Transmission Characteristics IE can be provided in a POSITIONING INFORMATION REQUEST message to the NG-RAN node, each defining a desired positioning SRS characteristics (e.g., Number Of Periodic Transmissions, Resource Type, Bandwidth, Resource Set List, SRS Frequency, etc.). 
The NG-RAN node provides the SRS configurations pre-configured in the UE in the POSITIONING INFORMATION RESPONSE message to the LMF.

Proposal 2:	The pre-configured SRS for positioning information can be stored in the UE context and can be retrieved from the old NG-RAN node by the new NG-RAN node where the UE RRC connection is resumed.

Proposal 3:	After activation of a pre-configured SRS for positioning in the target device, the NG-RAN node sends a NRPPa POSITIONING INFORMATION UPDATE message to the LMF, including information of the currently activated SRS for positioning and the Cell-ID/Cell Portion ID of the UE's current serving cell to enable the LMF to request UL measurements from TRPs.

The 'SRS deactivation request' could be UE triggered (e.g., included at Step 12 in Figure A.1 when measurements are completed) or LMF triggered (e.g., after Step 14 in Figure A.1 when TRP measurements have been received). For LMF triggered deactivation, the NRPPa Positioning Deactivation [6] procedures could be reused as shown in Figure 2.1-2 below.

For UE triggered deactivation request, the UE could send an RRC message along with the RRC Resume Request at Step 12 in Figure 2.1-3. The receiving gNB could then inform the LMF using the NRPPa Positioning Information Update message as shown in Figure 2.1-3 below.

NOTE:	For UL-only positioning, the RRC 'SRS Deactivation' in Figure 2.1-2 and 2.1-3 may not be 'Subsequent DL SDT', but MT SDT.

Figure 2.1-2: Deferred MT-LR with LMF-triggered positioning SRS deactivation request.

Figure 2.1-3: Deferred MT-LR with UE-triggered positioning SRS deactivation request.

Proposal 4:	The NRPPa POSITIONING DEACTIVATION message is also used to deactivate pre-configured and activated SRS for positioning in the UE.

Proposal 5:	The NRPPa POSITIONING INFORMATION UPDATE message is also used to inform the LMF about UE or NG-RAN node triggered SRS for positioning deactivation.

# 3.	LMF Assistance Information

There are several Work Item objectives where the NG-RAN and/or AMF would require or would benefit from assistance information from an LMF, incl.:

-	Measurements for a UE in RRC_IDLE state;

-	Extending eDRX cycle/alignment between eDRX and PRS configurations.

## 3.1	Assistance Information for RRC State Transition

The 'Low Power Periodic and Triggered 5GC-MT-LR Procedure' in RRC_INACTIVE state illustrated in Figure A.1 in the Annex and defined in greater detail in TS 23.273 clause 6.7 [3] is also applicable to positioning measurements in RRC_IDLE state but with Event Reporting without SDT. Instead of sending an Event Report with SDT at Steps 3 and/or 12 in Figure A.1 in the Annex, the UE would perform a UE Triggered Service Request as defined in clause 4.2.3.2 of TS 23.502 [8] in order to establish a signalling connection with the AMF.

After the Step 1 in Figure A.1, the NG-RAN node may decide to move the UE to RRC_INACTIVE or RRC_IDLE state as illustrated in Figure 3-1.

Figure 3-1: Deferred MT-LR with Event Detection in RRC_INACTIVE or RRC_IDLE state.

The RRC State transition after Step 1 in Figure 3-1 depends on several factors, incl.:

-	UE reporting activity. The UE reporting activity depends on the event type configured during Step 1 of Figure 3-1, e.g., periodicLocation, areaEventReporting, or motionEventReporting [3],[9]. In the case of very frequent reporting has been configured in the UE, it may be more appropriate to keep the UE in RRC_CONNECTED state. For very infrequent reporting, e.g., once per day, RRC_IDLE mode may be more efficient, etc.

-	Positioning method. If the positioning method configured in the UE during Step 1 in Figure 3-1 is an UL or UL+DL method, the UE should be in either RRC_CONNECTED or RRC_INACTIVE state, because SRS transmission in RRC_IDLE state is not supported.

-	UE capabilities. The UE positioning capabilities are taken into account by an LMF when configuring the deferred MT-LR in the UE at Step 1. However, the LMF is agnostic to the UE RRC state and other radio capabilities. If a UE does not support SDT-SRB2 or positioning SRS in RRC_INACTIVE state, the UE may better not be transitioned to RRC_INACTIVE state.

-	Expected UE Behaviour. There would be several non-positioning/non-LCS specific criteria which impacts the decision of proper RRC state transition, incl. expected UE mobility, HO interval, etc., as described in TS 23.501 [10]. However, the positioning/LCS activity would be additional criteria which should be considered.

As described in [11], there would be two general options for providing positioning/LCS specific information to assist RRC state transition to the NG-RAN, as illustrated in Figure 3-2:

-	Option 1: Using the existing Core Network Assistance Information for RRC INACTIVE as specified in [12].

-	Option 2:	Defining a NRPPa message.

In Option 1, the LMF provides the configured LCS information to the AMF in the Nlmf_Location_DetermineLocation Response service operation once the deferred MT-LR has been configured in the UE at Step 18 in Figure 3-2. The AMF includes the LCS information (together with other criteria for state transition) in the NGAP Core Network Assistance Information for RRC INACTIVE [12] to the NG-RAN node at Step 19a.1.

In Option 2, the LMF provides the configured LCS information to the NG-RAN in a NRPPa message, transparent to the AMF (e.g., Position Information Request/Response).

In both options, the NG-RAN node may use this information together with the UE capability for positioning measurements in RRC_INACTIVE state and the UE SDT-SRB2 capability to decide on a proper RRC state for the UE. Similar to the pre-configured SRS for positioning discussed in section 2.1 above, the LCS information could be stored in the UE context which can then be retrieved from the old NG-RAN node by the new NG-RAN node where the UE RRC connection is resumed.

Since the LCS Information is not the only information which a NG-RAN node may take into account when deciding on a proper RRC state for the UE, it seems more appropriate to use the existing Core Network Assistance Information for RRC INACTIVE (Option 1) for this purpose, as also further discussed in section 3.2.

Proposal 6:	Provide the UE configured LCS Reporting Activity (e.g., periodicLocation, areaEventReporting, or motionEventReporting [3],[9]) and Positioning Method (e.g., DL-only, UL-only, UL+DL) to the NG-RAN to assist the NG-RAN node with RRC state transition using one of the following options:

Option 1: Include the UE configured LCS Reporting Activity and Positioning Methods in the NGAP Core Network Assistance Information for RRC INACTIVE.

Option 2:	Include the UE configured LCS Reporting Activity and Positioning Methods in a NRPPa message (e.g., Position Information Exchange).

Figure 3-2: Providing Assistance Information for RRC State Transition.

## 3.2	Assistance Information for Alignment between eDRX and PRS Configurations

### 3.2.1	Background

Extended DRX (eDRX) for RRC idle and inactive states was introduced for RedCap devices in Rel-17 to enable longer battery lifetimes for e.g., IoT devices. eDRX with DRX cycles up to 10485.76 seconds (roughly 3 hours) in RRC idle state and up to 10.24 seconds in the RRC inactive state were specified for RedCap in Release 17. For the RRC inactive state, eDRX cycles longer than 10.24 seconds are considered feasible from the RAN perspective. However, certain CN aspects needed further study, and therefore, introduction of the longer cycles in inactive state was postponed for consideration in Rel-18 [13].

According to the results in [14], the target requirement of 6-12 months battery life for LPHAP can not be achieved with an eDRX cycle of 10.24 seconds. It was found that extending eDRX cycle beyond 10.24 seconds can be beneficial for meeting the battery life requirement for LPHAP. However, no specific values for eDRX cycle lengths beyond 10.24 seconds were recommended by RAN1 in [14].

For the objective of "Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP" (see section 1), the eDRX cycle lengths for idle mode (CN paging) can serve as baseline, if considered feasible as part of the eRedCap work.

Proposal 7:	Support eDRX for RRC_INACTIVE state with max. value of eDRX cycle up to 10485.76 seconds (1024 hyper-frames), if considered feasible in eRedCap WI. The eDRX design for eRedCap is also applicable for LPHAP.

Currently, DL-PRS and (e)DRX are independently configured for a target device, and therefore, it may happen that there are no DL-PRS measurement occasions available during the 'active time' as illustrated in Figure 3.2.1-1.  A UE may have to wake-up during the eDRX 'sleep duration' to perform the DL-PRS measurements, which adversely affects the UE power consumption.

Figure 3.2.1-1: DL-PRS configuration and DRX cycle.

Compared to LTE, the eDRX configuration for RRC_IDLE and RRC_INACTIVE states is independently configured. For examples, two RRC states may have different lengths for their eDRX cycles.

For RRC idle state, eDRX for CN paging is configured by the AMF. In RRC idle state the UE monitors only CN paging.

For RRC inactive state, eDRX configuration for RAN paging is decided and configured by NG-RAN. In RRC inactive state the UE monitors both RAN and CN paging. The AMF may provide to the NG-RAN node the Core Network Assistance Information to assist the NG-RAN node's decision whether the UE can be sent to RRC inactive state, and to assist UE configuration and paging in RRC inactive state. The 'Core Network Assistance Information for RRC INACTIVE' includes (among others) the UE specific DRX and the NR Paging eDRX Information ([12], section 9.3.1.15).

Information on whether eDRX for CN paging and RAN paging is allowed on the cell is provided separately in system information.

In CM_CONNECTED with RRC inactive state, the UE applies either the DRX cycle negotiated with AMF, or the DRX cycle broadcast by RAN or the UE specific DRX cycle configured by RAN, as defined in TS 38.300 [15] and TS 38.304 [16].

In general, there are two possibilities for the alignment between eDRX and PRS configurations:

(a)	eDRX alignment with configured DL-PRS;

(b)	DL-PRS alignment with configured eDRX.

### 3.2.2	eDRX alignment with configured DL-PRS

The AMF and/or NG-RAN could take the configured PRS information (e.g., PRS periodicity) and configured UE reporting activity and positioning method into account when configuring eDRX in the target device.

For RRC_INACTIVE state, the AMF already provides assistance information for RRC state transition and (e)DRX information to the NG-RAN in the IE 'Core Network Assistance Information for RRC INACTIVE' [12]. An LMF could provide the necessary assistance to an AMF as discussed in section 3.1 above and illustrated in Figure 3.2.2-1 (Option 1, Step 18). The LCS Activity information could include the information of the configured (deferred) location request (e.g., PeriodicEventInfo, AreaEventInfo, MotionEventInfo, configured positioning method, etc.) together with PRS information (e.g., periodicity). The AMF could then include the LCS specific information received from an LMF in the 'Core Network Assistance Information for RRC INACTIVE' in Step 19a, Option 1 in Figure 3.2.2-1. The AMF/NG-RAN Node may then use this information to decide on a suitable RRC state (as discussed in section 3.1) and (e)DRX cycle, taking the expected UE reporting interval/time and PRS periodicity into account.

Figure 3.2.2-1 also illustrates the option where the LMF provides the assistance information directly to the NG-RAN using NRPPa (Option 2, Step 19a), as also discussed in  section 3.1 above. However, this has the disadvantage that CN paging (e)DRX for RRC_IDLE can not be considered for the alignment with PRS (since the NRPPa message is transparent to an AMF and the (e)DRX configuration for RRC_INACTIVE and RRC_IDLE are independent). Also, all the information needed from the CN for RRC state transition would be in a single IE/procedure if the 'Core Network Assistance Information' is also used for LCS/positioning.

Proposal 8:	For the (e)DRX alignment with configured DL-PRS, support adding the UE configured LCS reporting activity and configured DL-PRS information (e.g., DL-PRS periodicity) to the IE 'Core Network Assistance Information for RRC INACTIVE' (TS 38.413).

Figure 3.2.2-1: Initiation of Deferred MT-LR with LMF assistance to the AMF (Option 1) or NG-RAN (Option 2).

### 3.2.3	DL-PRS alignment with configured eDRX

In section 3.2.2 above, the LMF provides assistance information to the AMF/NG-RAN to assist the NG-RAN with the state transition to RRC_INACTIVE state and to assist the AMF/NG-RAN with possible alignment of eDRX with already configured DL-PRS.

The opposite scenario, where the UE/NG-RAN/AMF assists the LMF to configure DL-PRS aligned with already configured eDRX is also possible but seems generally an anti-causal problem.  The DL-PRS Assistance Data are typically configured while the UE is in RRC_CONNECTED state (e.g., during Step 1 in Figure A.1 (see Annex A), or step 15 in Figure 3.2.2-1 above), or may receive the assistance data via broadcast (posSI). At this point of the procedure, an LMF does not know whether the NG-RAN will move the UE to RRC_INACTIVE or RRC_IDLE and if so, may not know any eDRX configuration information which can be used for possible alignment with DL-PRS.

Observation 2:	The "DL-PRS alignment with configured eDRX" is generally an anti-causal problem. The DL-PRS Assistance Data are typically configured while the UE is in RRC_CONNECTED state (e.g., during Step 1 in Figure A.1 (see Annex A), or step 15 in Figure 3.2.2-1 above), or may receive the assistance data via broadcast (posSI). At this point of the procedure, an LMF does not know whether the NG-RAN will move the UE to RRC_INACTIVE or RRC_IDLE and if so, may not know any eDRX configuration information which can be used for possible alignment with DL-PRS.

Therefore, DL-PRS alignment with configured eDRX seems generally only possible after the UE has already received a location request with DL-PRS assistance data and is in RRC_INACTIVE or RRC_IDLE state with configured eDRX. The UE, NG-RAN, or AMF could then inform the LMF after Step 1 in Figure A.1 (i.e., after the UE has been moved to RRC_INACTIVE or RRC_IDLE) about the configured eDRX information, which an LMF may consider when providing updated assistance data (e.g., after Step 3/4 or Step 12/13 in Figure A.1).

For e.g., CN buffering of MT data when the UE is in RRC_INACTIVE state with long eDRX [17], RAN3 made already an early agreement to use a Class 1 procedure to report to the AMF when the UE enters RRC_INACTIVE state with eDRX > 10.24s [18]. Although, RAN3 clarified that this new procedure is for eDRX cycle lengths larger than 10.24s [19], this new procedure could be reused for providing eDRX information from the AMF to the LMF, as illustrated in Figure 3.2.3-1, Option 1.  Note, a new NG-RAN triggered NRPPa procedure may also possible (Option 2 in Figure 3.2.3-1), but similar to the discussion in section 3.2.2, may not provide any CN DRX information for idle mode to the LMF. However, as indicated in [20], it is up to NG-RAN implementation whether and when to send this N2 message with eDRX indication to the AMF.

Figure 3.2.3-1: Providing eDRX information to the LMF.

Therefore, a more reliable solution would be that a UE informs the LMF about configured (e)DRX (either directly or indirectly), e.g., at Step 3 or 12 in Figure A.1 using LPP. A UE would always know the periodic or triggered location request configuration (i.e., when a report would be needed) and all configured (e)DRX information independent on RRC/CM state. The LMF may then take the (e)DRX information into account when requesting DL-PRS from the TRPs using the on-demand DL-PRS procedures [6] as illustrated in Figure 3.2.3-2.

The LPP Request Assistance Data for on-demand DL-PRS can already include a desired DL-PRS periodicity [21]. A UE may determine the desired DL-PRS periodicity and set the dl-prs-ResourceSetPeriodicityReq in IE NR-On-Demand-DL-PRS-Information accrding to its (e)DRX cycle and configured LCS activity.  The NRPPa procedures for on-demand DL-PRS (PRS Configuration Exchange) could then be used to configure DL-PRS aligned with the UE configured (e)DRX. Any additional parameter (e.g., slot offset) if needed could be easily added.

Proposal 9:	For the DL-PRS alignment with configured (e)DRX, the UE sends a LPP Request Assistance Data for on-demand DL-PRS to the LMF. The NRPPa procedures for on-demand DL-PRS (PRS Configuration Exchange) could then be used to configure DL-PRS aligned with the UE configured (e)DRX. FFS whether additional parameter in the Requested DL PRS Transmission Characteristics are needed (e.g., DL-PRS slot offset).

Figure 3.2.3-2: On-demand DL-PRS procedure for aligning DL-PRS with UE DRX Active Time.

# 4.	Summary

In this contribution, we discussed the enhancements for enabling the LPHAP use case #6 as defined in TS 22.104. The following proposals were made.

SRS for positioning configuration enhancements

Observation 1:	The pre-configuration of positioning SRS can occur during the location preparation phase of the 'Low Power Periodic and Triggered 5GC-MT-LR Procedures' (e.g., together with the pre-configuration of DL-PRS assistance data, if applicable). One or more positioning SRS configurations can be provided to the gNB/UE during the location preparation phase using modified NRPPa Positioning Information Exchange procedures.

Proposal 1:	The NRPPa Positioning Information Exchange procedures are extended to support pre-configuration of SRS for positioning. One or more Requested SRS Transmission Characteristics IE can be provided in a POSITIONING INFORMATION REQUEST message to the NG-RAN node, each defining a desired positioning SRS characteristics (e.g., Number Of Periodic Transmissions, Resource Type, Bandwidth, Resource Set List, SRS Frequency, etc.). 
The NG-RAN node provides the SRS configurations pre-configured in the UE in the POSITIONING INFORMATION RESPONSE message to the LMF.

Proposal 2:	The pre-configured SRS for positioning information can be stored in the UE context and can be retrieved from the old NG-RAN node by the new NG-RAN node where the UE RRC connection is resumed.

Proposal 3:	After activation of a pre-configured SRS for positioning in the target device, the NG-RAN node sends a NRPPa POSITIONING INFORMATION UPDATE message to the LMF, including information of the currently activated SRS for positioning and the Cell-ID/Cell Portion ID of the UE's current serving cell to enable the LMF to request UL measurements from TRPs.

Proposal 4:	The NRPPa POSITIONING DEACTIVATION message is also used to deactivate pre-configured and activated SRS for positioning in the UE.

Proposal 5:	The NRPPa POSITIONING INFORMATION UPDATE message is also used to inform the LMF about UE or NG-RAN node triggered SRS for positioning deactivation.

Idle mode positioning / Alignment between eDRX and PRS configurations

Observation 2:	The "DL-PRS alignment with configured eDRX" is generally an anti-causal problem. The DL-PRS Assistance Data are typically configured while the UE is in RRC_CONNECTED state (e.g., during Step 1 in Figure A.1 (see Annex A), or step 15 in Figure 3.2.2-1 above), or may receive the assistance data via broadcast (posSI). At this point of the procedure, an LMF does not know whether the NG-RAN will move the UE to RRC_INACTIVE or RRC_IDLE and if so, may not know any eDRX configuration information which can be used for possible alignment with DL-PRS.

Proposal 6:	Provide the UE configured LCS Reporting Activity (e.g., periodicLocation, areaEventReporting, or motionEventReporting [3], [9]) and Positioning Method (e.g., DL-only, UL-only, UL+DL) to the NG-RAN to assist the NG-RAN node with RRC state transition using one of the following options:

Option 1: Include the UE configured LCS Reporting Activity and Positioning Methods in the NGAP Core Network Assistance Information for RRC INACTIVE.

Option 2:	Include the UE configured LCS Reporting Activity and Positioning Methods in a NRPPa message (e.g., Position Information Exchange).

Proposal 7:	Support eDRX for RRC_INACTIVE state with max. value of eDRX cycle up to 10485.76 seconds (1024 hyper-frames), if considered feasible in eRedCap WI. The eDRX design for eRedCap is also applicable for LPHAP.

Proposal 8:	For the (e)DRX alignment with configured DL-PRS, support adding the UE configured LCS reporting activity and configured DL-PRS information (e.g., DL-PRS periodicity) to the IE 'Core Network Assistance Information for RRC INACTIVE' (TS 38.413).

Proposal 9:	For the DL-PRS alignment with configured (e)DRX, the UE sends a LPP Request Assistance Data for on-demand DL-PRS to the LMF. The NRPPa procedures for on-demand DL-PRS (PRS Configuration Exchange) could then be used to configure DL-PRS aligned with the UE configured (e)DRX. FFS whether additional parameter in the Requested DL PRS Transmission Characteristics are needed (e.g., DL-PRS slot offset).

# References

[1]		RP-223549, "New WID on Expanded and Improved NR Positioning".

[2]		RP-222982, "Status Report to TSG".

[3]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[4]	R2-2203949, "LS to SA2 on Positioning in RRC_INACTIVE State", RAN2.

[5]	RP-230327, "Status Report to TSG", NR_pos_enh2.

[6]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[7]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[8]		3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[9]	3GPP TS 24.080: "Mobile radio interface layer 3; Supplementary services specification; Formats and coding".

[10]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[11]	R3-225548, "Enhancements to Positioning in RRC_INACTIVE State for LPHAP".

[12]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[13]	RP-223544, "Revised WID on Enhanced support of reduced capability NR devices".

[14]	3GPP TR 38.859: "Study on expanded and improved NR positioning".

[15] 	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[16] 	3GPP TS 38.304: "User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[17]	R3-226170, "LS On long eDRX support for RRC_INACTIVE", SA2.

[18]	R3-226776, "Reply LS on long eDRX support for RRC_INACTIVE", RAN3.

[19]	R3-230803, "Reply LS on long eDRX support for RRC_INACTIVE", RAN3.

[20]	S2-2301858, "Reply LS on long eDRX support for RRC_INACTIVE", SA2.

[21]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

# Annex A:	Low Power Deferred MT-LR Procedure with Event Reporting in RRC_INACTIVE state for UL+DL Positioning

Figure A.1: Low Power Deferred MT-LR Procedure with Event Reporting in RRC_INACTIVE state for UL+DL Positioning.

1.	Steps 1-21 for the deferred 5GC-MT-LR procedure for periodic or triggered location events specified in TS 23.273, clause 6.3.1 are performed.

The LMF may perform one or more positioning procedures at Step 15 of TS 23.273, clause 6.3.1 to request and obtain the UE positioning capabilities or provide any necessary assistance data to the target device.

The LCS Periodic-Triggered Location Invoke at Step 16 of TS 23.273, clause 6.3.1 includes an embedded LPP Request Location Information message which indicates the allowed or required Multi-RTT location measurements for each location event reported.

The UE is released by the last serving gNB from RRC_CONECTED to RRC_INACTIVE by RRCRelease with SuspendConfig.

2.	The UE monitors for occurrence of the trigger or periodic event requested during step 1.

3.	When an event is detected (or slightly before) the UE sends an RRC UL Information Transfer message containing an UL NAS Transport message along with the RRC Resume Request with SDT. The UE includes an LCS Event Report in the payload container of the UL NAS Transport message, and the Deferred Routing Identifier received during Step 1 in the Additional Information of the UL NAS Transport message as defined in TS 24.501.

The LCS Event Report includes an embedded LPP Request Assistance Data message with IE NR-Multi-RTT-RequestAssistanceData and nr-AdType set to 'ul-srs' to request an UL-SRS for Multi-RTT positioning as specified in TS 37.355.

NOTE:	The receiving gNB of the UE when UE performs step 3 might be the same or different from the last serving gNB where the UE is released to the RRC_INACTIVE state.

4.	The receiving gNB sends the LCS Event Report with the LPP Request Assistance Data message in an NGAP Uplink NAS Transport message to the serving AMF. The AMF determines the LMF from the Deferred Routing Identifier received in the Additional Information IE of the UL NAS TRANSPORT message and forwards the LCS Event Report with embedded LPP message via triggering Namf_Communication_N1MessageNotify service operation towards the LMF. The AMF also includes the Payload Container Type and the Correlation Identifier set to the Deferred Routing Identifier.

NOTE:	If the anchor gNB is not changed from the last serving gNB to the receiving gNB, the LCS event report is forwarded form the receiving gNB to the last serving gNB via XnAP message RRC TRANSFER as in TS 38.423. Subsequent downlink/uplink messages are also forwarded between last serving gNB to the receiving gNB via XnAP message RRC TRANSFER.

5.	The LMF sends a NRPPa Positioning Information Request message to the receiving gNB to request UL-SRS for the target device.

6.	The receiving gNB determines the resources available for UL-SRS.

7.	The receiving gNB provides the UL-SRS configuration information to the LMF in a NRPPa Positioning Information Response message.

8.	The LMF sends a NRPPa Measurement Request to a group of gNBs incl. the UL-SRS measurement configuration.

9.	The LMF sends a SS LCS Event Report Acknowledgement to the receiving gNB. The receiving gNB then provides the SS Event Report Acknowledgement to the UE at Step 9b via Subsequent DL SDT.

10.	The receiving gNB sends a RRCRelease message with suspendConfig to keep the UE in RRC_INACTIVE state. The RRCRelease message includes the UL-SRS Configuration.

11.	The UE performs DL-PRS measurements and each configured TRP performs UL-SRS measurements.

12.	The UE sends an RRC UL Information Transfer message containing an UL NAS Transport message along with the RRC Resume Request with SDT. The UE includes the LCS Event Report and LPP Provide Location Information message in the payload container of the UL NAS Transport message, and the Deferred Routing Identifier received during Step 1 in the Additional Information of the UL NAS Transport message as defined in TS 24.501.

13.	The receiving gNB sends the LCS Event Report with the LPP Provide Location Information message in an NGAP Uplink NAS Transport message to the serving AMF. The AMF determines the LMF from the Deferred Routing Identifier received in the Additional Information IE of the UL NAS TRANSPORT message and forwards the LCS Event Report with embedded LPP message via triggering Namf_Communication_N1MessageNotify service operation towards the LMF. The AMF also includes the Payload Container Type and the Correlation Identifier set to the Deferred Routing Identifier.

14.	After performing the UL-SRS measurements, the gNBs provide the UL measurements to the LMF in a NRPPa Measurement Response message.

15. When all LPP Provide Location Information messages have been received, the LMF sends a SS LCS Event Report Acknowledgement to the receiving gNB. The receiving gNB then provides the SS Event Report Acknowledgement to the UE at Step 15b via Subsequent DL SDT.

16.	The receiving gNB sends a RRCRelease message with suspendConfig to keep the UE in RRC_INACTIVE state.

17.	Steps 28-31 for the deferred 5GC-MT-LR procedure for periodic or triggered location events specified in TS 23.273, clause 6.3.1 are performed.