3GPP TSG-RAN WG3 #119bis-e		                                   R3-231281

Online, 17th – 26th April, 2023

Source:	CATT, ZTE, Huawei, CMCC, China Telecom

Title:	Discussion on multiple Trace Session activation

Agenda Item:	9.2.5

Document for:	Discussion and Decision

Introduction

In last RAN3 meeting, the case that NG-RAN may receive multiple Trace Session activations for the same UE has been discussed but no agreement was reached. In this contribution, we make further analysis and provide proposals accordingly.

Discussion

Currently, in RAN3 spec, on the behavior of NG-RAN node when receiving Trace Start message, it is described as below:

The AMF initiates the procedure by sending a TRACE START message. Upon reception of the TRACE START message, the NG-RAN node shall initiate the requested trace session as described in TS 32.422 [11].

Then, there is corresponding description in 32.422 as follow:

If the Trace Reference is the same as an existing Trace Session for the same subscriber or equipment, and the Trace Recording Session Reference is the same as the existing Trace Recording Session in the existing Trace Session having the same Trace Reference, the NG-RAN node shall not start a new Trace Recording Session and shall continue with the existing trace session and ignore the second request.

If the Trace Reference is the same as an existing Trace Session for the same subscriber or equipment, and the Trace Recording Session Reference is not the same as the existing Trace Recording Session in the existing Trace Session having the same Trace Reference, the NG-RAN node shall continue with the existing trace session and ignore the second request.

So, it is clearly that NG-RAN node would ignore the received Trace activation request if the trace reference of the new trace session is the same with the existing one,

Then, in last RAN3 meeting, on how NG-RAN node react when NG-RAN node receives a second trace activation request whose trace reference is different with the current one, there is no consensus. The possible options are listed as below:

Option 1: NG-RAN node also ignore the received Trace activation request message and continue with the existing trace session.

Option 2: NG-RAN node starts a new Trace session and deactivates the existing trace session.

Option 3: Completely depends on NG-RAN node implementation i.e. NG-RAN node could either ignore the second trace activation request or start a new trace session and deactivates the existing trace session.

Since this issue has been discussed in RAN3 for several meetings and no conclusion was made, we propose to send a LS to SA5 on what should be the behavior of NG-RAN node and also in which cast the NG-RAN node would receive a second Trace Activation request.

Proposal: It is proposed to send a LS to SA5 on the behavior of NG-RAN node when NG-RAN node receives a second trace activation request whose trace reference is different with the current one.

The draft LS is in [1]

Conclusion

Proposal: It is proposed to send a LS to SA5 on the behavior of NG-RAN node when NG-RAN node receives a second trace activation request whose trace reference is different with the current one.

The draft LS is in [1]

Reference

[1] R3-231282 LS to SA5 on multiple Trace activation