3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231297

Electronic Meeting, April 17th – 26th, 2023

Agenda item:		24.2

Source:	Intel Corporation

Title:	Considerations on network energy savings

Document for:		Discussion and Decision

# Introduction

A new WID on Network energy saving for NR [1] was approved in RAN#98e meeting, and the following objectives, which have RAN3 impacts, were included.

Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]

This contribution further discusses the above three issues and also provides the detailed specification changes.

# Discussion

## Inter-node cell DTX/DRX information exchange

According to the latest study progress in RAN2[2], at least the parameters of periodicity, start slot/offset and on duration can be configured per Cell DTX/DRX configuration. Besides providing the Cell DTX/DRX configuration from gNB to UE, RAN3 achieved an agreement in #117bis meeting that inter-node exchange of the cell DTX/DRX (if defined by RAN1/RAN2) is considered necessary [3].

The network node receiving cell DTX/DRX information from its neighbor cells can use this information to help determine its own cell DTX/DRX configuration. This coordination of cell DTX/DTX information between nodes can further help with load and coverage optimization of the network. For example, this information can provide a node the knowledge when a neighbour cell is in on/off cycle and thus decide whether or when to handover a UE to that neighbour cell, and that node can also adjust some parameters to cover the coverage hole because its neighbour cell(s) is in off cycle.

In CU-DU split deployment, for the same reason as state above, CU and DU also need to synchronize their cell DTX/DRX configuration. Considering CU has more comprehensive information about neighbour cells of other gNBs or other DUs, CU can coordinate with neighbour nodes and make an optimized DTX/DRX configuration, especially for a better coverage reason. Besides, RAN2 already agreed that periodic cell DTX/DRX pattern is configured by RRC which resides in CU. Therefore, it is reasonable that CU decides the cell DTX/DRX pattern configuration and then informs it to DU(s).

Proposal 1: Support exchange of the Cell DTX/DRX configuration (including periodicity, start slot/offset, on duration) over Xn and F1.

For Xn interface, the existing NG-RAN node Configuration Update procedure, which is used to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface [4], can be reused for cell DTX/DRX configuration exchange.

For F1 interface, the gNB-CU Configuration Update, which is used to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface [5], can be reused for cell DTX/DRX configuration exchange from CU to DU.

Proposal 2: To reuse NG-RAN node Configuration Update procedure for cell DTX/DRX configuration exchange over Xn.

Proposal 3: To reuse gNB-CU Configuration Update procedure for cell DTX/DRX configuration exchange over F1.

As confirmed in last RAN2 meeting, Cell DTX and Cell DRX modes can be configured either separately or together, therefore the configurations for Cell DTX and Cell DRX needs to be listed separately in the NG-RAN node Configuration Update message and the gNB-CU Configuration Update message. In Annex of this contribution, we use the three agreed cell DTX/DRX parameters in TR 38.864, i.e. periodicity, start slot/offset and on duration, as an example for the detailed message structure and IEs for TS 38.423 and TS 38.473.

## Inter-node beam activation

## Beam activation cases

In last RAN3 meeting, we also reached an agreement that Support beam level activation over Xn and F1 [6].

According to the above agreement and also the statement in [2], an NG-RAN node can request a neighbouring NG-RAN node to switch on certain SSB beams which have been deactivated. When a request is executed, a NG-RAN node may notify its neighbouring NG-RAN node with the list of SSB beams, which are previously reported as inactive due to NES and now need to be activated.

Similarly, in CU-DU split deployment, a CU shall be able to request a connected DU to switch on one or more SSB beams with it for reason of coverage enhancement or traffic offloading. One open issue is whether DU can request CU for beam activation.

There are some cases where DU may prefer CU to activate its SSB beam(s) in the same coverage. For example, when DU plans to switch off the beam(s) with it and needs CU’s help with the coverage, or when DU plans to offload traffic to CU, it can request CU to activate its SSB beams. In these cases, CU can decide whether to activate the beam(s) or not. Therefore, it is beneficial that DU can also initialize the beam activation procedure to CU.

Proposal 4: Beam Activation procedure shall be supported in F1 interface between gNB-CU and gNB-DU.

## Beam activation messages

As for the messages used for beam activation, there are two options. One is to define new beam activation messages, the other one is to extend the existing cell activation messages.

Option 1: new beam activation messages

As show in figure 1, upon receipt of this BEAM ACTIVATION REQUEST message, the neighbouring NG-RAN node will take the SSB beams list contained in it into account for beam activation and activate the SSB beam(s) indicated in the message. Then the neighbouring NG-RAN node shall send the sending NG-RAN node a BEAM ACTIVATION RESPONSE message to indicate which beams the request was fulfilled or a BEAM ACTIVATION FAILURE message if no beam indicated in the BEAM ACTIVATION REQUEST message can be activated. An appropriate cause value should be also included in the BEAM ACTIVATION FAILURE message.

The NG-RAN node1 and NG-RAN node2 can be a gNB, or a CU, or a DU.

Figure 1: Beam Activation procedures

In the BEAM ACTIVATION REQUEST message and the BEAM ACTIVATION RESPONSE message, a list of SSB Index and an Activation ID are included. In the BEAM ACTIVATION FAILURE message, the Activation ID assigned by the sending NG-node and a Cause value are included.

Option 2: extended cell activation messages

The existing CELL ACTIVATION REQUEST message over Xn is used for an NG-RAN node to request a neighbouring NG-RAN node to switch on one or more cells, previously reported as inactive due to energy saving [4]. There is a list of cell IDs contained in it indicating the cells need to be activated. Over F1, a Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message for CU to activate the cells in DU.

For the purpose of beam activation, a SSB Beams List can be added to the CELL ACTIVATION REQUEST message and GNB-CU CONFIGURATION UPDATE message.

As for DU request to CU, the same rationale could be applied and thus a SSB Beams List can be added to the GNB-DU CONFIGURATION UPDATE message, which explicitly indicates to CU the beams preferred to be activated. Or instead, a SSB Beams to be Deactivated List Item IE can be added, which can indicate to CU which beams in DU will be out of service, and then CU can decide, based on this information, to activate other beams.

In comparison of option 2, option 1 applies a unified procedure for both Xn and F1, and meanwhile can support cell activation and beam activation to be performed respectively, therefore we slightly prefer option 2 for beam activation.

Proposal 5: To define new beam activation messages for inter-node beam activation for Xn and F1.

The detailed message structure and IEs for TS 38.423 are shown in R3-231298 [7] and the related TP for TS 38.473 are shown in Annex.

Proposal 6: To agree on the CR for TS 38.423 and the TP for TS 38.473 in Annex.

## Paging enhancemnet

Considering the main intention of this WI is to improve network energy savings in terms of both BS transmission and reception, the techniques studied should be able to achieve this goal.

In order to save power through restricting paging in a limited area of certain beam(s), the network must have the accurate location information of a UE in advance, but not through exchanging additional signaling among network nodes. This can be achieved for stationary UEs. Besides, considering only RAN nodes has the beam information for a UE, this enhancement only benefits for RAN paging. Therefore, we suggest to only focus on stationary UE in inactive mode for this issue.

Proposal 7: To focus on stationary UE in inactive mode for paging enhancement for NES.

In CU-DU deployment, in order to enable DU to page a UE in certain beams, CU can send the serving SSB index of the UE in the paging message over F1.

Proposal 8: To add serving SSB index of the UE in paging message over F1.

# Conclusion

In this contribution we analyzed the three main issues in RAN3 scope for NES, and the following proposals were drawn from the analysis.

Proposal 1: Support exchange of the Cell DTX/DRX configuration (including periodicity, start slot/offset, on duration) over Xn and F1.

Proposal 2: To reuse NG-RAN node Configuration Update procedure for cell DTX/DRX configuration exchange over Xn.

Proposal 3: To reuse gNB-CU Configuration Update procedure for cell DTX/DRX configuration exchange over F1.

Proposal 4: Beam Activation procedure shall be supported in F1 interface between gNB-CU and gNB-DU.

Proposal 5: To define new beam activation messages for inter-node beam activation for Xn and F1.

Proposal 6: To agree on the CR for TS 38.423 and the TP for TS 38.473 in Annex.

Proposal 7: To focus on stationary UE in inactive mode for paging enhancement for NES.

Proposal 8: To add serving SSB index of the UE in paging message over F1.

# References

- RP-223540, New WID: Network energy savings for NR, Huawei

- TR 38.864, Study on network energy savings for NR

- RAN3_117bis-e_agenda_20221018_EOM

- TS 38.423, NG-RAN; Xn application protocol (XnAP)

- TS 38.473, NG-RAN; F1 application protocol (F1AP)

- RAN3_119_agenda_20230303_EOM1

- R3-231298, Baseline CR to 38.423 on NES, Intel Corporation

# Annex	TP to TS 38.473

<Start of change>

### 8.2.x	Beam Activation

#### 8.2.x.1	General

The purpose of the Beam Activation procedure is to enable a gNB-CU to request a connected gNB-DU or a gNB-DU to request a connected gNB-CU node to switch on one or more beams, previously reported as inactive due to energy saving.

The procedure uses non-UE associated signalling.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: Beam Activation, successful operation

The gNB-CU initiates the procedure by sending the BEAM ACTIVATION REQUEST message to the connected gNB-DU, or vice versa.

Upon receipt of this message, the gNB-DU or the gNB-CU should activate the beam/s indicated in the BEAM ACTIVATION REQUEST message and shall indicate in the BEAM ACTIVATION RESPONSE message for which beams the request was fulfilled.

#### 8.2.x.3	Unsuccessful Operation

Figure 8.2.X.3-1: Beam Activation, unsuccessful operation

If the gNB-DU or the gNB-CU cannot activate any of the beams indicated in the BEAM ACTIVATION REQUEST message, it shall respond with the BEAM ACTIVATION FAILURE message with an appropriate cause value.

#### 8.2.x.4	Abnormal Conditions

Void.

<End of change>

<Start of change>

### 9.2.x	Beam Activation messages

#### 9.2.x.1	BEAM ACTIVATION REQUEST

This message is sent by the gNB-CU to the connected gNB-DU to request a previously switched-off beam/s to be re-activated, or vice versa.

Direction: gNB-CU ® gNB-DU or gNB-DU ® gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE SSB Beams To Activate | M |  |  |  | YES | reject |
| >SSB Beams |  |  |  |  |  |  |
| >>SSB Beams List |  | 1 |  |  | – |  |
| >>>SSB Beams item |  | 1 .. <maxnoofSSBBeams> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be activated. | – |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the gNB-CU or the gNB-DU | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBBeams | Maximum no. beams that can be activated by an NG-RAN node. Value is FFS. |

#### 9.2.x.2	BEAM ACTIVATION RESPONSE

This message is sent by a gNB-DU to a connected gNB-CU to indicate that one or more beam(s) previously switched-off has (have) been activated, or vice versa.

Direction: gNB-DU ® gNB-CU or gNB-CU ® gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE SSB Beams To Activate | M |  |  |  | YES | reject |
| >SSB Beams |  |  |  |  |  |  |
| >>SSB Beams List |  | 1 |  |  | – |  |
| >>>SSB Beams item |  | 1 .. <maxnoofSSBBeams> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be activated. | – |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the gNB-CU or the gNB-DU | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBBeams | Maximum no. beams that can be activated by an NG-RAN node. Value is FFS. |

#### 9.2.x.3	BEAM ACTIVATION FAILURE

This message is sent by a gNB-DU to a connected gNB-CU to indicate beam activation failure, or vice versa.

Direction: gNB-DU ® gNB-CU or gNB-CU ® gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the gNB-CU or the gNB-DU | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<End of change>