3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231300

Electronic Meeting, April 17th – 26th, 2023

Agenda item: 		10.2.3

Source:	Intel Corporation

Title:	(TP for SON BL CR for TS 38.473) RACH enhancements

Document for:		Discussion and Decision

# Introduction

This contribution makes a further analysis on RACH enhancements, including RACH indication and RACH partitioning parameters for RA report.

# Discussion

## RACH indication

In last meeting, RAN3 agreed to use non-UE associated signaling for RACH Indication over F1 [1] and the stage 2 description was agreed in [2]. For the stage 3 CR for TS 38.473, we think the same message structure as for Xn interface can be reused.

The TP for TS 38.473 is included in Annex.

Proposal 1: To agree the TP for TS 38.473 for in Annex.

## RA report enhancements

Two parameters, i.e. feature or feature combination that triggered the RACH and used feature combination, were agreed to be added into RA report for RACH partitioning [3]. Besides, there are some other parameters proposed to be added into the RA report for RACH partitioning in the previous RAN3 meetings, e.g. feature priorities configured by network, RACH partition configuration information.

These parameters are configured by network nodes which could be the same node the UE sends the RA report to. Even if this is not the case, for example, the UE sends the RA report after connecting to a different node that is not the one who configured the UE, the configuration information still can be retrieved between network nodes. A Retrieve UE Context-like procedure, with a UE ID included to identify the UE, can be used to retrieve these configuration information from the old gNB. If proven necessary, RAN3 can further discuss the retrieval mechanism.

Considering these additional parameters are optional for configuration optimization, it is still acceptable even if these information are not available or retrieval failure happens.

Therefore, in order to save air interface resources, we think it is not needed to include any network configured parameters in the RA report.

Proposal 2: Network configured parameters are not necessary to be included in the RA report.

# Conclusion

In this contribution, we provided the TP for F1 interface on RACH indication and also analyzed the RA report optimization due to RACH partitioning, and the following proposals were drawn from the analysis.

Proposal 1: To agree the TP for TS 38.473 for in Annex.

Proposal 2: Network configured parameters are not necessary to be included in the RA report.

# References

- RAN3_119_agenda_20230303_EOM2

- R3-230996, (TP for SON BLCRs for TS 38.401): RACH report retrieval, Huawei, Orange, China Telecom, CMCC

- R2-2211101, Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online

# Annex TP to TS 38.473

<Start of change>

## 8.1	List of F1AP Elementary procedures

//////////////////////////////////////////////////////////////Irrelevant parts skipped/////////////////////////////////////////////////////////////////////

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| Multicast Group Paging |  | MULTICAST GROUP PAGING |  |
| Broadcast Context Release Request |  | BROADCAST CONTEXT RELEASE REQUEST |  |
| Multicast Context Release Request |  | MULTICAST CONTEXT RELEASE REQUEST |  |
| PDC Measurement Report |  | PDC MEASUREMENT REPORT |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| PDC Measurement Failure Indication |  | PDC MEASUREMENT FAILURE INDICATION |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| Measurement Activation |  | MEASUREMENT ACTIVATION |  |
| QoE Information Transfer |  | QOE INFORMATION TRANSFER |  |
| Positioning System Information Delivery |  | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |  |
| RACH Indication |  | RACH INDICATION |  |

<End of change>

<Start of change>

### 8.2.x	RACH Indication

#### 8.2.x.1	General

The purpose of the RACH Indication procedure is informing the gNB-CU about the occurrences of successful random access procedures in the gNB-DU. The procedure uses non-UE associated signalling.

#### 8.2.x.2	Successful Operation

Figure 8.2.x.2-1: RACH Indication procedure

If the RACH Indication List IE in the RACH INDICATION message indicates the occurrences of successful random access procedures in the gNB-DU, the gNB-CU may retrieve the RA Report from the UE(s) based on the RACH indication received.

#### 8.2.x.3	Abnormal Conditions

Not applicable.

<End of change>

<Start of change>

#### 9.2.1.x	RACH INDICATION

This message is sent by gNB-DU and is used to indicate that the occurrences of successful random access procedures in the gNB-DU.

Direction: gNB-DU ® gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RACH Indication List |  | 0..1 |  |  | YES | reject |
| >RACH Indication Item |  | 1 .. <maxnoofRACHIndication> |  |  | EACH | reject |
| >> gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - | - |
| >> gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHIndication | Maximum no. of indications of successful RACH occurrences. Value is FFS. |

<End of change>