3GPP TSG-RAN WG3 Meeting #119bis-e                                                             R3-231308

Electronic meeting, April 17 – 26, 2023

Agenda item: 		13.1

Source: 		Qualcomm Incorporated

Title: 	Discussion on issues related to SA2 VMR

Document for:	Discussion

# 1	Introduction

This document addresses issues related to SA2 SI/WI on VMR that have RAN3 impact. This includes the “mobile IAB-node indication” in the NGAP Initial UE message. The chair notes from last meeting state:

To be continued: NGAP Initial UE message to include an optional “mobile IAB-node indication”.

This further includes the discussion on SA2’s request to support positioning of the UE connected to the MBSR via NRPPa. RAN3’s Replay LS to SA2 in R3-23011 states related to this topic:

- For point#6 (regarding KI#5): RAN3 needs to conduct further discussions to converge on a solution. RAN3 will inform SA2 for any progress.

# 2	Discussion

## 2.1 Mobile IAB-node indication in NGAP

SA2 has captured the following normative text in TS 23.501, vs. 18.0, section 5.35A.1:

For a MBSR node, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform mobile IAB authorization.

Based on TS 23.501, vs. 18.0, section 5.35A.1, RAN3 has two options:

Option 1: Support “mobile IAB-node indication” in NGAP Initial UE message.

Option 2: Send an LS to SA2 to indicate that RAN3 is not willing to support such indication in NGAP.

Proposal 1: RAN3 to discuss whether to either include a mobile-IAB indicator into the NGAP Initial UE Message or to send on liaise SA2 that such indicator will not be supported.

## 2.2 UE positioning via NRPPa

SA2’s request can be accommodated with some minor enhancements to NRPPa and F1AP. The overall procedure for the positioning of a UE using a mobile TRP would require the following enhancements:

- According to RAN3 agreement from last meeting, the CU includes the mIAB-MT’s cell ID into the ULI of a UE that is connected to the mIAB-DU.

- According to TR 23.700-05, the AMF serving the UE includes this mIAB-MT’s cell ID into a location request for this UE to the LMF. RAN3 expects that SA2/CT will capture this enhancement during the normative stage based on RAN3’s reply to point#6.

- Based on the mIAB-MT’s cell ID in the AMF’s location request for the UE, the LMF can initiate an NRPPa TRP Information Exchange procedure to obtain information on mobile TRPs in case it doesn’t have this information yet.

- All NRPPa messages to the LMF and all F1AP positioning messages to the gNB that carry TRP information need to be enhanced to optionally include the following information for mobile TRP:

  - mIAB-MT ID, e.g., the GPSI as defined in TS 29.571,

  - The current geolocation of the TRP (the IE is defined in TS 37.355 itself),

  - The current velocity of TRP (the IE can be copied from TS 37.355).

  - The timestamp of the geolocation and velocity measurement (the IE is defined in TS 38.455 itself).

This includes the NRPPa and F1AP messages that are invoked when the LMF requests TRP information from the RAN, which are:

  - NRPPa TRP INFORMATION RESPONSE

  - F1AP TRP INFORMATION RESPONSE

This further includes the NRPPa and F1AP messages that carry measurement information for a specific TRP toward the LMF, which are:

  - NRPPa MEASUREMENT RESPONSE

  - NRPPa MEASUREMENT REPORT

  - F1AP POSITIONING MEASUREMENT RESPONSE

  - F1AP POSITIONING MEASUREMENT REPORT

- The NRPPa TRP INFORMATION Request may include an optional TRP Information Type of “mobile TRP” to allow confining the request to mobile TRPs only.

Proposal 2a: SA2’s request for positioning of onboard UEs via NRPPs using TRPs of the mobile IAB-node can be accommodated by optionally including to the TRP information in upstream F1AP and NRPPa messages:

(1) an mIAB-MT such as GPSI, e.g., as defined in TS 29.571,

(2) the TRP’s current geolocation, e.g., as defined in TS 37.355,

(3) the TRP’s current velocity, e.g., as defined in TS 37.355,

(4) a timestamp of when the measurement was taken, e.g., as defined in TS 38.455.

Proposal 2b: This information to be included in the following upstream NRPP and F1AP messages:

  - NRPPa TRP INFORMATION RESPONSE

  - NRPPa MEASUREMENT RESPONSE

  - NRPPa MEASUREMENT REPORT

  - F1AP TRP INFORMATION RESPONSE

  - F1AP POSITIONING MEASUREMENT RESPONSE

  - F1AP POSITIONING MEASUREMENT REPORT

Proposal 2c: The NRPPa TRP INFORMAION REQUEST to include “MBSR” as an additional TRP Information Type.

Proposal 2d: RAN3 to expect that SA2/CT will capture on normative stage that AMF serving the UE includes the mIAB-MT’s cell ID into a location request for this UE to the LMF.

# Conclusion

This document addressed issues related to SA2 SI/WI on VMR that have RAN3 impact. This included the “mobile IAB-node indication” in the NGAP Initial UE message. It further included the discussion on SA2’s request to support positioning of the UE connected to the MBSR via NRPPa. The following proposals have been made:

Proposal 1: RAN3 to discuss whether to either include a mobile-IAB indicator into the NGAP Initial UE Message or to send on liaise SA2 that such indicator will not be supported.

Proposal 2a: SA2’s request for positioning of onboard UEs via NRPPs using TRPs of the mobile IAB-node can be accommodated by optionally including to the TRP information in upstream F1AP and NRPPa messages:

(1) an mIAB-MT such as GPSI, e.g., as defined in TS 29.571,

(2) the TRP’s current geolocation, e.g., as defined in TS 37.355,

(3) the TRP’s current velocity, e.g., as defined in TS 37.355,

(4) a timestamp of when the measurement was taken, e.g., as defined in TS 38.455.

Proposal 2b: This information to be included in the following upstream NRPP and F1AP messages:

  - NRPPa TRP INFORMATION RESPONSE

  - NRPPa MEASUREMENT RESPONSE

  - NRPPa MEASUREMENT REPORT

  - F1AP TRP INFORMATION RESPONSE

  - F1AP POSITIONING MEASUREMENT RESPONSE

  - F1AP POSITIONING MEASUREMENT REPORT

Proposal 2c: The NRPPa TRP INFORMAION REQUEST to include “MBSR” as an additional TRP Information Type.

Proposal 2d: RAN3 to expect that SA2/CT will capture on normative stage that AMF serving the UE includes the mIAB-MT’s cell ID into a location request for this UE to the LMF.

# References

- Chair Notes; TSG RAN WG3 Meeting #119, Athens, Greece, February/March, 2023