3GPP TSG-RAN WG3 #119bis-e		          		    	     	    	  R3-231318

17th – 26th April 2023

Agenda Item:	11.2

Source:	CATT

Title:	Discussion on NR QoE in RRC_INACTIVE/RRC_IDLE states

Document for:	Discussion

- Introduction

In last RAN3 meeting, the following agreements have been achieved and some open issue is identified. And it is captured in chair Notes [1]:

The RRC state info when UE collects the uploaded QoE data shall not be reported in QoE report for MBS BC. MBS MC can be discussed later.

MBS BC QoE measurements can proceed after the UE switches from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED.

RAN3 to discuss which configuration information related to QoE measurement needs to be available in the new gNB.

At least the following QoE configuration related information for MBS broadcast service should be available in the new gNB:

- QoE reference

- Measurement Collection Entity Information, the detail information can be further discussed

RAN3 shall discuss which of other QoE configuration info for MBS BC QoE shall be available in the new gNB.

Measurement Configuration Application Layer ID (RRC level ID)

Service Type

Container for Application Layer Measurement Configuration (config container)

MDT Alignment Information

Area Scope of QMC (area scope)

S-NSSAI Information (slicing info)

RVQoE Information

QoE measurement type (signalling based, management based)

FFS whether to support RVQoE measurements in RRC_IDLE and RRC_INACTIVE

FFS whether new gNB can re-configure MBS BC QoE.

In this contribution, we provide some analysis on open issues.

- Discussion

In RAN2 meeting, on how to provide QoE configuration to UE, the agreement is as below:

1: Rel-18 QoE configuration can be provided to UE as in Rel-17 (RRCreconfiguration, RRCresume).

FFS if RRCRelease can be used – proponents should provide detailed proposals on what is in RRCRelease, why it is needed, how to handle RRCReconfiguration + RRCRelease together

As for other messages used to provide QoE configuration to UE, it still needs further discussion.

RAN3 also have agreed to not enhance paging to configure UE with QoE measurement. Except for paging, there are many other new mechanisms which may be used for QoE configuration, e.g. MCCH, SIB, but no agreement has been achieved. Here we discuss the pros and cons.

Pros:

The advantage of using broadcast to configure QoE measurement is the configuration can be sent to UE at the first time when UE accessing to the cell. But we think the benefit is so limited because it is not an urgent task especially for m-based QoE measurement task. Operator triggers m-based QoE measurement to collect QoE measurement result for certain area for a period of time. It is usually a relatively long time, i.e. it is not in a hurry to configure UE at the first time.

Cons:

1. It wastes more Uu interface resource due to MCCH is used to transfer QoE configuration which may be a large amount of data.

2. Network cannot select UE when configure QoE measurement. If overloaded, most of time network only configure part of UE to perform QoE measurement which cannot be supported by broadcast.

3. Alignment of MDT and QoE reporting is not supported because QoE measurement is initiated at the first while MDT measurement has to be configured when UE enter RRC_CONNECTED state. It is hard to achieve alignment.

For the above reasons, we do not support new QoE configuration mechanism for QoE measurements in INACTIVE/IDLE RRC states, but it is finally up to RAN2.

Proposal 1: we do not support new QoE configuration mechanism, e.g. MCCH, SIB, for QoE measurements in INACTIVE/IDLE RRC states, but it is finally up to RAN2.

Here we continue discussing how to keep QoE configuration when UE switches to RRC_IDLE and RRC_INACTIVE.

In RAN3 #117 meeting, we have agreed that UE shall keep the QoE configuration for MBS broadcast service configured in RRC_CONNECTED even when UE switches to RRC_IDLE and RRC_INACTIVE.

UE shall keep the QoE configuration for MBS broadcast service configured in RRC_CONNECTED even when UE switches to RRC_IDLE and RRC_INACTIVE

UE has to perform QoE measurement in RRC_IDLE and RRC_INACTIVE, so necessary QoE configuration shall be kept by UE.

Observation 1: UE has to keep necessary QoE configuration to perform QoE measurement in RRC_IDLE and RRC_INACTIVE no matter which solution is adopted to store QoE configuration when UE in the RRC_IDLE state.

We think one solution is for UE to keep all configurations; the other solution is UE only keeps part of configuration in inactive/idle state, while the other part of configuration such as MCE IP address which is security sensitive shall be kept in CN.

The open issue we discussed in last RAN3 meeting is as below:

Whether UE or CN stores the network instance of QoE configuration when UE in the RRC_IDLE state needs further discussion.

Option 1 (CN-based solution): Old gNB stores the entire network instance QoE configuration at AMF before going to RRC_IDLE and new gNB retrieves the stored QoE configuration from AMF during reconnection.

Option 2 (UE-based solution): New gNB doesn’t need to know the QoE configuration of old gNB upon reconnection. It is sufficient if new gNB is informed by UE via QoE report.

Considering UE has to keep part of QoE configuration, the option 1 keeping entire QoE configuration at AMF is unnecessary, i.e. AMF may store part of QoE configuration at most.

Besides above reason, we prefer option 2 for the following reasons:

1. MCE ID can be introduced which is same as TCE ID in logged MDT. There is no security issue on MCE ID.

2. Anyway UE has to keep QoE configuration in order to perform QoE measurement. If UE specific QoE configuration is also kept in CN, it is hard to align QoE configuration between CN and UE. Especially when QoE configuration is removed by UE for exceeding 48 hours, NG-RAN is not informed and cannot trigger CN to release QoE configuration. So, it may be impossible to keep QoE configuration alignment between CN and UE.

3. For m-based QoE configuration, there may be many UEs to be selected to perform QoE measurement which needs lots of storage resource for CN to store many per UE m-based QoE configuration.

Therefore, we support option 2, i.e. UE based solution.

Proposal 2: it is proposed for UE to store QoE configuration when UE in the RRC_IDLE state, i.e. option 2.

In last RAN3 meeting, we have agreed following information shall be send to new gNB

At least the following QoE configuration related information for MBS broadcast service should be available in the new gNB:

- QoE reference

- Measurement Collection Entity Information, the detail information can be further discussed

Since we support UE to store QoE configuration, QoE reference and Measurement Collection Entity Information shall be sent to UE.

Here we discuss the following FFS. First we discuss the information provided to UE and then discuss which information shall be sent to new gNB.

RAN3 shall discuss which of other QoE configuration info for MBS BC QoE shall be available in the new gNB.

- Measurement Configuration Application Layer ID (RRC level ID)

- Service Type

- Container for Application Layer Measurement Configuration (config container)

- MDT Alignment Information

- Area Scope of QMC (area scope)

- S-NSSAI Information (slicing info)

- RVQoE Information

- QoE measurement type (signalling based, management based)

For Measurement Configuration Application Layer ID, it is used to indicate QoE reference in Uu interface. So, it is not needed due to QoE reference is available.

For Service Type, it belongs to MBS information. Besides that, MBS session ID indicate which MBS shall be measured. And MBS area scope is also necessary MBS information. MBS information is needed for UE to collect QoE measurement.

For Container for Application Layer Measurement Configuration (config container), it is used to configure QoE measurement and shall be sent to UE.

For MDT Alignment Information, it used to support alignment of MDT and QoE measurement.

For S-NSSAI Information, it is up to whether to support per slice QoE measurement collection. It is FFS now.

For RVQoE Information, we shall first decide whether to support RVQoE for RRC_INACTIVE/RRC_IDLE states UE. It may be FFS now.

For QoE measurement type (signalling based, management based), QoE measurement type shall be provided to new NG-RAN to prohibit m-based QoE overriding s-based QoE. The related open issue is as below:

Whether/how to handle the potential overriding issue for MBS broadcast QoE configurations after UE switches from RRC_IDLE to RRC_CONNECTED.

It is similar as the solution on logged MDT overriding issue. QoE may be just aligned with it. But it is finally up to RAN2.

Proposal 3: we think following information shall be sent to UE when configuring QoE measurement:

- QoE reference and Measurement Collection Entity Information;

- Container for Application Layer Measurement Configuration (config container);

- Alignment Information of MDT and QoE measurement: aligned MDT trace ID and QoE reference;

- QoE measurement type: s-based or m-based configuration.

- MBS information: MBS session ID, MBS area scope, MBS Service Type.

For MBS information, we think operator shall decide them when initiate QoE task. But we are not sure whether to include MBS information explicitly in NG interface. For s-based QoE configuration, NG-RAN just forward QoE configuration to UE no matter whether UE is receiving the configured MBS session or whether UE is in or out of the MBS service area. We think NG-RAN may not care about MBS information.

Proposal 4: we think NG-RAN just forward QoE configuration to UE without considering MBS information. But we can further discuss whether NG-RAN needs MBS information.

When sending QoE report to the new gNB, besides the QoE reference and Measurement Collection Entity Information, we think UE shall also provide the following information explicitly to the new gNB.

1. Alignment Information of MDT and QoE measurement: aligned MDT trace ID and QoE reference.

2. QoE measurement type: s-based or m-based configuration.

For 1), since MDT is aligned with QoE, it is better for new gNB to retrieve MDT and QoE measurement together and sends them to MCE.

For 2), it is used to solve overriding issue just like MDT.

Proposal 5: it is proposed to also send alignment Information of MDT and QoE measurement and QoE measurement type from UE to new gNB.

As for the issue on which information shall be sent to MCE in QoE report, the open issue is as below:

FFS whether RAN add QoE reference as an explicit IE in QoE report from gNB to MCE.

We think this issue is up to SA5, we can ask SA5 for it.

Proposal 6: it is proposed to ask SA5 for the information sent explicitly to MCE in QoE report.

For UE keeping QoE configuration, there is an open issue as below:

How long UE shall keep the QoE configuration for MBS broadcast service.

We have discussed the detailed QoE configuration UE shall keep as above, and then we continue discuss how long UE shall keep them.

1. For the necessary information in order to send QoE measurement report to MCE (QoE reference and Measurement Collection Entity Information), UE shall keep them till QoE measurement report is sent to new gNB.

2. For the alignment of MDT and QoE measurement, if alignment is configured and MDT and QoE measurement is performing, UE shall keep them.

3. For necessary information used to perform QoE measurement (config container, MBS information), UE shall keep them till there is no QoE measurement report generated any more.

4. For s-based or m-based QoE configuration type, UE shall keep it when the QoE configuration exists.

So, we think UE shall keep the QoE configuration until QoE configuration is deactivated.

Proposal 7: it is proposed for UE to keep the QoE configuration until QoE configuration is deactivated.

But we notice that there is an obvious difference between legacy R17 QoE configuration deactivation and R18 inactive/idle QoE configuration deactivation. Network may not able to deactivate inactive/idle QoE configuration immediately.

For example, for s-based QoE configuration, in order to find UE to deactivate QoE configuration, CN may have to trigger paging procedure. After UE setting up RRC connection, CN can forward deactivation message to RAN. This procedure has also been agreed in RAN2 meeting as below:

RAN2 thinks existing paging can be used to bring UE to CONNECTED, where NW can release QoE configuration. This requires no specification changes.

Although it has been supported by current specification, it is too complicated to perform the above procedures. We do not think it is worth spending important resource, for example paging resource, for not urgent QoE configuration deactivation. What is more, if CN triggers paging to NG-RAN in order for QoE configuration deactivation, but it is the common case that UE cannot be found. CN has to trigger deactivation again after a period of time until the UE is founded. So, we do not think it is suitable to use paging to deactivate QoE configuration.

In logged MDT, time duration is introduced to control the inactive/idle measurement time. For the same reason, we propose to introduce timer during for inactive/idle QoE measurement.

Proposal 8: it is proposed to introduce timer during for inactive/idle QoE measurement as in logged MDT.

RAN3 also discuss whether to support RVQoE for broadcast service type in RRC_INACTIVE/RRC_IDLE states. We do not think it is needed for the following reason:

- RVQoE is used by RAN to optimize QoE measurement in R17 due to RVQoE measurement result can be provided to RAN in time for UE in RRC_CONNECTED state. While for broadcast service, when UE in RRC_INACTIVE/RRC_IDLE states, NG-RAN cannot retrieve RVQoE measurement result immediately because it is not suitable to trigger UE entering RRC_CONNECTED state to just to send RVQoE measurement result.

- If RVQoE measurement result is retrieved by other NG-RAN node, it shall be provided to original NG-RAN when RVQoE measurement result generates which would cost much interface resource.

- After receiving RVQoE measurement result from other NG-RAN, the original NG-RAN can optimize RVQoE, but it is hard to decide whether the optimization is appropriate because UE may have leaved this NG-RAN and cannot provide feedback.

In one word, we do not support RVQoE for broadcast service type because UE cannot provide real time RVQoE to network as in R17.

Proposal 9: it is proposed not to introduce RVQoE for broadcast service type.

- Conclusion

According to the analysis in section 2, we have:

Observation 1: UE has to keep necessary QoE configuration to perform QoE measurement in RRC_IDLE and RRC_INACTIVE no matter which solution is adopted to store QoE configuration when UE in the RRC_IDLE state.

Proposal 1: we do not support new QoE configuration mechanism, e.g. MCCH, SIB, for QoE measurements in INACTIVE/IDLE RRC states, but it is finally up to RAN2.

Proposal 2: it is proposed for UE to store QoE configuration when UE in the RRC_IDLE state, i.e. option 2.

Proposal 3: we think following information shall be sent to UE when configuring QoE measurement:

- QoE reference and Measurement Collection Entity Information;

- Container for Application Layer Measurement Configuration (config container);

- Alignment Information of MDT and QoE measurement: aligned MDT trace ID and QoE reference;

- QoE measurement type: s-based or m-based configuration.

- MBS information: MBS session ID, MBS area scope, MBS Service Type.

Proposal 4: we think NG-RAN just forward QoE configuration to UE without considering MBS information. But we can further discuss whether NG-RAN needs MBS information.

Proposal 5: it is proposed to also send alignment Information of MDT and QoE measurement and QoE measurement type from UE to new gNB.

Proposal 6: it is proposed to ask SA5 for the information sent explicitly to MCE in QoE report.

Proposal 7: it is proposed for UE to keep the QoE configuration until QoE configuration is deactivated.

Proposal 8: it is proposed to introduce timer during for inactive/idle QoE measurement as in logged MDT.

Proposal 9: it is proposed not to introduce RVQoE for broadcast service type.

- Reference

[1] RAN3_119_agenda_20230303_1830