3GPP TSG-RAN WG3 #119bis-e																		R3-231319

17th – 26th Apr 2023

Online

Agenda item:		11.3

Source:				CATT

Title:	Discussion on Support for legacy QoE in NR-DC

Document for:		Discussion and Decision

1		Introduction

In RAN3#119 meeting, the NR QoE supporting in NR-DC was discussed. The legacy QoE related agreements and open issues are captured in chair Note[1] as below:

If the SN is interested in configuring a UE with an m-based QoE measurement configuration, it should send the request to the MN via a UE-associated procedure.

The MN should inform the SN that a UE is configured with an m-based QoE measurement.

For an m-based QoE configuration in the case SN is interested in configuring a UE with an m-based QoE measurement configuration, the MN can decide and notify the SN whether:

The MN shall send the configuration information to the UE, or

The SN should send the configuration to the UE directly, or

The SN should send the configuration information to the UE via the MN (inside a container).

In NR-DC, a node can configure the UE with an m-based QoE configuration only if it has received this configuration from the OAM, and if it serves the UE by a cell within the area scope.

The MN is responsible for RRC ID allocation for m-based sessions configured by the MN or SN, and notifies the allocated RRC ID(s) to the SN.

FFS on whether a pool of RRC ID is splitted between MN and SN or whether it is per measurement.

This contribution will further discuss the supporting on the legacy QoE in NR-DC. And provide the view on the open issues .

2		Discussion

2.1 Support for QoE configuration and reporting over MN/SN for NR-DC

In last RAN3 meeting, we made many agreements for the legacy QoE in NR-DC. The framework for legacy QoE in NR-DC is almost completely designed for M-based QMC. Summarized as below:

- The MN can configure the UE for its received M-based QMC from OAM and inform the SN about the configuration.

- The SN can configure the UE for its received M-based QMC from OAM and inform the SN about the configuration. Then the MN decide whether the UE is selected and how to send the configuration to UE

- The MN allocated the RRC ID for the M-based QMC configured by MN and SN.

Base on the framework, we may discuss some open issues:

- RRC ID pool split, we may have several options:

Option1a: SN can configure the UE for its received M-based QMC from OAM and inform the SN about the configuration without RRC ID. And then MN response the accept decision on the SN configuration with the allocated RRC ID. The MN selects the RRC ID from the whole available RRC IDs for MN and SN

Option1b: SN can configure the UE for its received M-based QMC from OAM and inform the SN about the configuration without RRC ID. And then MN response the accept decision on the SN configuration with the allocated RRC ID. The MN selects the RRC ID from the list of SN specific RRC IDs for the SN QMC.

Option2: The MN pre-configured the RRC ID list for both MN and SN. MN sends the list of available of RRC ID to SN for SN used. SN selects one of the RRC ID from the list when it configures the QMC for the UE. MN can select one of available of RRC ID for MN used when it configured its selected UE QMC

Consider the node specific RRC ID can be used to differentiate the configuration from which node and the report for which node. The option2 is more direct and simple. It avoids the MN selecting the RRC ID for each SN QMC configuration. The SN may direct select one RRC ID from the pool.

Proposal 1: split the RRC IDs to the pool for MN specific and SN specific

Proposal 2: MN sends available of RRC ID to SN for SN specific

- Reporting leg switch

In last RAN2 meeting, they agree RRC configuration determines to which node UE sends the QoE report.  It is possible to change the reporting leg via RRC signalling after it has been configured. So UE may switches the reporting leg based on indication from network on explicit way. When the node sends the switch command to the UE, the node should inform another node about the switching of report sending. And may include some assist information for the report receiving from UE and sending report to MCE

Proposal 3: The report leg switching command and assist information may be sent to another node for the report receiving from UE and sending report to MCE when switching command sent to UE

Both the MN node and SN node may encounter the overload status, to avoid introducing delay and reduce the overhead in interface, we should allow both MN and SN can send the indication of switching reporting leg based on its overload status.. The SN may send the command via SRB3 or SRB1

Proposal 4: Both MN and SN can command the UE to switch the reporting leg based on its overload status

Proposal 5: The SN may send the switching command to UE via SRB3 or SRB1

2.2 Support for alignment of QoE measurements and radio related measurement

In NR-DC, the immediate MDT can be configured in the MN and SN. But the logged MDT can be configured only in MN. The logged MDT aligns with the QoE in idled and inactive state should be discussed in the work item of the idle QoE and the NR-DC will not impact the alignment. For the QoE in connected state align with the immediate MDT in MN and SN, aligned immediate MDT in MN and SN may be the same MDT or different MDT. To simple the specification, we should restrict the alignments to only supporting the same MDT. How to trigger the MDT configured should be enhanced base on the current specification. The QoE start indication should send to SN for the SN MDT configuration.

Proposal 6: aligned immediate MDT in MN and SN should be the same MDT

Proposal 7: The QoE start indication should send to SN for the SN MDT configuration initial

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: split the RRC IDs to the pool for MN specific and SN specific

Proposal 2: MN sends available of RRC ID to SN for SN specific

Proposal 3: The report leg switching command and assist information may be sent to another node for the report receiving from UE and sending report to MCE when switching command sent to UE

Proposal 4: Both MN and SN can command the UE to switch the reporting leg based on its overload status

Proposal 5: The SN may send the switching command to UE via SRB3 or SRB1

Proposal 6: aligned immediate MDT in MN and SN should be the same MDT

Proposal 7: The QoE start indication should send to SN for the SN MDT configuration initial

4		Reference

- RAN3_119_agenda_20230303_EOM2