3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231368

Electronic meeting, 17 Apr – 26 Apr, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1135 | rev | 1 | Current version: | 17.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SIType List |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT, Nokia, Nokia Shanghai Bell, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2023-04-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-CU sends the SI list of other SI to the gNB-DU, with the following semantics for SI Type IE. - The SI Type value of other SI starts from 2 However, according to TS 38.331, the idle/inactive UE can request 32-bit bit string, where each bit can correspond to one item in the schedulingInfoList IE. When the UE requests the first SI, the network should broadcast the first SI configured by schedulingInfoList IE. when the UE requests the second SI, the network should broadcast the second SI, and so on. But with the above semantics for SI type IE, - the CU cannot indicate the first SI to the DU, such that the DU cannot broadcast the first SI when the UE requests the first SI in the other SI. - in case the UE requests overall 32 SIs, the existing SIType List can only indicate 31 SIs to the gNB-DU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In SYSTEM INFORMATION DELIVERY COMMAND message, - for idle/inactive UE on-demand SI request, update the semantic descriptions of the SI Type IE so that the SI type value starts from 1. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the SYSTEM INFORMATION DELIVERY COMMAND message for on-demand SI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For idle/inactive UE on-demand SI request, the first SI in the other SI cannot be broadcasted on demand. Inconsistence between F1AP specification and RRC specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.62 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-230667 Rev1: R3-231368 Revert the changes, and change to update the semantics descriptions of SI Type IE. Update the cover page. |  |  |  |  |  |  |  |  |

Change Begins

<Below is provided for reference without changes>

#### 9.2.5.1	SYSTEM INFORMATION DELIVERY COMMAND

This message is sent by the gNB-CU and is used to enable the gNB-DU to broadcast the requested other SI.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 | NR cell identifier | YES | reject |
| SIType List | M |  | 9.3.1.62 |  | YES | reject |
| Confirmed UE ID | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  | YES | reject |

<Unchanged Text Omitted>

#### 9.3.1.62	SIType List

This IE is used by gNB-CU to provide SI list of other SI for gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SI type item IEs |  | 1.. <maxnoofSITypes> |  |  |
| >SI Type | M |  | INTEGER (1..32, ...) | Indicates a certain SI type required to be broadcasted by the gNB-DU. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSITypes | Maximum no. of SI types, the maximum value is 32. |

<Unchanged Text Omitted>

Change Ends