3GPP TSG-RAN WG3 Meeting #R3-119bis	R3-231374

# Online, 17 April – 21 April, 2023

Agenda Item:	16.3

Source:	NEC

Title:	Discussion on service continuity enhancement

Document for:	Discussion and agreement

1	Introduction

During the previous RAN3 meetings, some details of the support of service continuity had been discussed and the following agreements have been made so far from previous meetings:

Reuse the existing network procedures to support single-hop L2 U2N Relay in Rel-18. Source gNB decides to trigger path switching for the L2 U2N remote UE. Current signaling can support Scenario C, i.e., intra-gNB indirect to indirect path switch. RAN3 focuses on the XnAP and possible F1AP impacts to support the basic scenarios. Regarding the support of lossless data delivery during path switch, RAN3 would wait for RAN2’s progress first. For direct/indirect to indirect path switching, enhance Xn: HANDOVER REQUEST to include at least the Remote UE L2 ID and Relay UE L2 ID. For inter-gNB path switching scenarios, RAN3 should specify mechanisms to support service continuity for L2 U2N relays in NG based handovers as well after supporting service continuity for L2 U2N relays in Xn based handovers, If there is some conclusion from SA2, and then to support NG based HO. WA: During inter-gNB path switching, source gNB can signal the serving cell of the relay UE to target gNB via existing IE Target Cell Global ID. RAN3#118: Source gNB selects the target path type (direct or indirect). Focus on the following two ways for the future discussion, - Way1: to go for Op1, and Op2 can be further discussed. - Way2: accept Op2, or at least as a compromise. No more discussion on Op3 in RAN3. For Op2, continue discussion on following: - FFS on which node (source node or target node) decides the target cell in case of inter-gNB path switching Proponents of Option 2 should provide more details on the whole mechanism, e.g., - Whether source node can choose candidate relay UEs belonging to multiple target cells or can we restrict to candidate relays belonging to one target cell - Whether source node can choose candidate relay?UEs belonging to multiple target gNBs or can we restrict to candidate relays belonging to one target gNB - Potential stage-3 impacts (e.g., number of candidate relays that needs to be signaled to target gNB) During direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO REQ message. At least Remote UE L2 ID and a list of candidate target relay UE IDs should be included in the XnAP HANDOVER REQUEST message. Stage-3 details can be figured out next meeting, for example, whether the Remote UE L2 ID is already included in the inter-node message. Following legacy HO procedure, RAN3 does not set any restriction on whether the target candidate relay UEs belong to the same target gNB or not. The same applies for the target cell. No LS is sent to RAN2 this meeting.

In this contribution, we will continue to address the remaining issues in this topic.

2	Discussion

According to the below agreement made during last meeting in below:

- During direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO REQ message. - At least Remote UE L2 ID and a list of candidate target relay UE IDs should be included in the XnAP HANDOVER REQUEST message

Currently, it could be observed that source gNB currently is taking the responsibility of cell selection for the path switch of relay UE. But still there are some open issues needs to be finalized so that to complete the whole path switch procedure.

Firstly, whether it is allowed that source gNB can only send a single candidate relay UE in the HO REQ message. Due to the principle of discovery procedure defined in RAN2 in Rel-17, it is possible that remote UE can only report one suitable relay UE according to the measurement of SD-RSRP, i.e. there is only one candidate relay UE whose measured SD-RSRP is above the configured threshold. Correspondingly, it is not reasonable for source gNB to report a list of candidate relay UEs all the time.

Source gNB may either send a single candidate relay UE or a list of candidate relay UEs to the same target cell in the HANDOVER REQUEST message

In this case, it is actually source gNB performing both cell selection and relay UE selection for the path switch of remote UE. There is beneficial to separately define responding messages for target gNB when it is receiving a single candidate relay UE and a list of candidate relay UEs, e.g. to reduce latency.

To separately define responding messages for target gNB when it is receiving a single candidate relay UE or a list of candidate relay UEs from source gNB, to reduce latency.

Secondly, so far there is no restriction on whether the candidate relay UEs belong to the same target gNB or not. Whereas, it means source gNB could send multiple HANDOVER REQUEST messages towards multiple target gNBs, as legacy handover procedure. However, if to allow this case, severe latency may be caused due to redundant signalling exchange. In details, firstly, source gNB may send HANDOVER REQUEST message towards target gNB1,target gNB2, target gNB3, etc, multiple candidate relay UEs may be carried in each HANDOVER REQUEST message. Then each target gNB may send back the candidate relay UE selection result towards source gNB. In the following, source gNB may receive multiple candidate relay UEs from multiple target gNBs. Therefore, it needs to select the final relay UE and send the indication of selection result towards the corresponding target gNB. Optionally, the target gNB may need to confirm the selection result.

As we can see, this whole procedure cause three rounds of signalling exchange between source and target gNB to complete the path switch. Therefore, it will cause a very critical latency issue.

Thus, it is suggested to restrict that all candidate relay UEs selected by source gNB should belong to the same target gNB.

If there is no restriction on whether the candidate relay UEs belong to the same target gNB, severe latency issue may be caused during the whole path switch procedure for U2N remote UE.

It is suggested to adopt restriction that all candidate relay UEs selected by source gNB should belong to the same target gNB.

3	Conclusion and Proposals

In this contribution, we have discussed the remaining issues about service continuity enhancement for UE to network relay, the following observations are given out:

Observation 1.	Source gNB may either send a single candidate relay UE or a list of candidate relay UEs to the same target cell in the HANDOVER REQUEST message

Observation 2.	If there is no restriction on whether the candidate relay UEs belong to the same target gNB, severe latency issue may be caused during the whole path switch procedure for U2N remote UE.

and the following proposals are given out:

Proposal1.	To separately define responding messages for target gNB when it is receiving a single candidate relay UE or a list of candidate relay UEs from source gNB, to reduce latency.

Proposal2.	It is suggested to adopt restriction that all candidate relay UEs selected by source gNB should belong to the same target gNB.

4	References