3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231395

Electronic meeting, 17 Apr – 26 Apr, 2023

Title: 	(TPs to TS 38.300, 38.423 BL CRs) Consideration on MT-SDT

Source: 	Huawei

Agenda item:	20.2

Document Type:	Other

# Introduction

At RAN#98-e meeting, the updated WID on MT-SDT for NR was approved in [1], with the following objectives related to RAN2 and RAN3:

Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3] - MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RACH procedure based and CG-SDT procedure based UL response; - MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE. Note: Data transmission in DL within paging message is not in scope of this WI.

In this document we are going to discuss the potential RAN3 impacts for MT-SDT based on the latest progress of RAN2 and RAN3, and provide the TPs for TS 38.300, 38.423 BLCRs.

# Discussion

  - MT-SDT Information IE in XnAP: RAN PAGING message

In last RAN3 meeting, we got the following agreements and FFSs below:

- When receiving DL SDT data, the anchor gNB may send MT-SDT information IE to the neighbour gNBs within the RNA, via XnAP RAN paging message. The encoding and the name of MT-SDT information IE is FFS.

- The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging.

- FFS: MT-SDT assistance information sent from the anchor in the XnAP: RAN paging message and other alignment with RAN2 progress.

Based on the achieved agreements above, it could be understood that it is up to each gNB to make decision on whether Uu paging with MT-SDT indicator or not for its own cells. Therefore, in case of XnAP: RAN Paging, the anchor gNB should provide the assistance information to the neighbour gNBs to help them to make such decision. Based on the discussion so far, we think the MT-SDT information IE should include an MT-SDT indicator (to indicate the data/signalling is SDT only) and optionally the data size of the SDT Data/signalling.

Proposal 1: The MT-SDT information IE carried in the XnAP: RAN PAGING message includes an MT-SDT indicator (to indicate the data/signalling is SDT only) and optionally the data size of the SDT Data/signalling.

  - Detailed design of the Data Size IE

In the agreed TPs of last meeting, the detailed design of Data Size IE is still FFS. In LTE MT-EDT, the Data Size IE included in the S1AP: PAGING message is defined as INTEGER (1..4095, …) and the unit is: bit.

For MO-SDT, the sdt-DataVolumeThreshold-r17 is provided over SIB1, and used by the UE to determine whether the MO-SDT can be initiated, and it is designed as ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000, byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000}.

Considering that the detailed data size could be provided in case of MT-SDT, similar to the Data Size IE in the S1AP: PAGING message, we can also use INTEGER type to provide the accurate data size, and maybe we can use the max value of the MO-SDT data volume threshold as the maximum size.

Proposal 2: Design the Data Size IE in the MT-SDT information IE as INTEGER (1..96000, …) and the unit is byte.

  - Impact on XnAP: RETRIEVE UE CONTEXT REQUEST message

In last meeting discussion, there is an agreement made by RAN2 and an FFS made by RAN3 as follows:

- RAN2 agreement： New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

- RAN3 FFS：FFS: whether and how XnAP RTRV UE CTXT REQ message (that carries MT-SDT resume indication)

- In last meeting, there were some proposals on providing the MT-SDT RRC cause or an MT-SDT indicator from the new gNB to the anchor gNB in XnAP: RETRIEVE UE CONTEXT REQUEST message. Please note that currently we normally not forward the RRC cause received from the UE to the anchor gNB, except the periodic RAU scenario.

Based on the current BL CRs, in case the UE resumes for MT-SDT (e.g. upon receiving the new resume cause ‘mt-SDT’ from UE), the network procedures of the RA-SDT w/o anchor relocation or CG-SDT procedures defined in Rel-17 are used as the baseline. We agreed that MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling, then:

- for DL SDT user Data packet, it should be sent via DL GTP-U tunnels established after receiving Retrieve UE Context Request message by last serving gNB, i.e. via the Xn-U Address Indication procedure, and then the subsequent UL/DL SDT data could be transmitted over the GTP-U tunnels as well.

- for DL SDT signalling, it should be carried via existing XnAP: RRC TRANSFER message after Retrieve UE Context procedure.

For all these handling, we do not see the need to providing either providing the MT-SDT RRC cause or an MT-SDT indicator to the anchor gNB side, especially if we take the RAN2 agreement “RBs configured for SDT are common for MO-SDT and MT-SDT” into account.

Proposal 3: there is no need to provide the MT-SDT RRC cause or an MT-SDT indicator from the new gNB to the anchor gNB in XnAP: RETRIEVE UE CONTEXT REQUEST message.

  - Non-SDT data arrival during MT-SDT

While the MT-SDT procedure is ongoing, if DL non-SDT data/signalling arrives, the network can straightaway move the UE to RRC_CONNECTED state by sending the RRCResume message if the UE responds to paging and resumes in the last serving gNB.

However, since both with and without UE context relocation procedures will be supported for MT-SDT, and if the DL non-SDT data arrives in the last serving gNB while it is anchoring the MT-SDT procedure, it will not be able to move the UE to RRC_CONNECTED state straight away by sending RRCResume message. Instead, the gNB will have to send RRCRelease message to the UE to terminate the ongoing MT-SDT procedure and then again perform paging in the RNA. When the UE responds to Paging, the UE’s context will be relocated and the UE will be moved to RRC_CONNECTED state and only then the new serving gNB will be able to send the DL non-SDT data to the UE. Such procedure will clearly lead to unnecessary signaling overhead and latency in sending, time critical, DL data.

To avoid this, it would be worth allowing the last serving gNB to include an indication in the RRCRelease message informing the UE it should initiate a new resume procedure right-away.

When the UE initiates a new legacy resume procedure, the UE context can be relocated form the last serving gNB to the receiving gNB and the UE can be transitioned to the RRC_CONNECTED state by reusing the legacy procedure. Hence, we propose the following.

Proposal 4: When DL non-SDT data arrives during the ongoing MT-SDT procedure, the last serving gNB shall end the ongoing SDT procedure by sending RRCRelease message to the UE with an indication to initiate a new Resume procedure to avoid paging there by reducing the signaling overhead and latency for time critical DL non-SDT data transfer.

# Conclusion

Based on the discussion in section 2, the following proposals are made for aggregated architecture:

Proposal 1: The MT-SDT information IE carried in the XnAP: RAN PAGING message includes an MT-SDT indicator (to indicate the data/signalling is SDT only) and optionally the data size of the SDT Data/signalling.

Proposal 2: Design the Data Size IE in the MT-SDT information IE as INTEGER (1..96000, …) and the unit is byte.

Proposal 3: there is no need to provide the MT-SDT RRC cause or an MT-SDT indicator from the new gNB to the anchor gNB in XnAP: RETRIEVE UE CONTEXT REQUEST message.

Proposal 4: When DL non-SDT data arrives during the ongoing MT-SDT procedure, the last serving gNB shall end the ongoing SDT procedure by sending RRCRelease message to the UE with an indication to initiate a new Resume procedure to avoid paging there by reducing the signaling overhead and latency for time critical DL non-SDT data transfer.

- To support these proposals, the TPs for TS 38.300 and TS 38.423 are provided in section 5 and 6.

# Reference

- R3-231395 (TPs to TS 38.401, 38.473 and 37.483 BL CRs) MT-SDT in disaggregated architecture, Huawei

- TP to TS 38.300 BL CR

--------Start of the First Change-------------

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

//skip unchanged part

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MT-SDT	Mobile Terminated Small Data Transmission

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

NCD-SSB	Non Cell Defining SSB

NCGI	NR Cell Global Identifier

NCL	Neighbour Cell List

--------Start of the Next Change-------------

## 18.x	MT-SDT overall procedure

The overall procedure for MT-SDT is illustrated in the figure 18.X-1.

Figure 18.X-1. MT-SDT overall procedure

1.	A RAN paging trigger event occurs (incoming DL SDT data and/or signalling from 5GC, etc.)

2.	The last serving gNB triggers RAN paging for MT-SDT, either in the cells controlled by the last serving gNB, or also by means of XnAP: RAN Paging (with MT-SDT Information which includes MT-SDT indicator, and may also the volume of DL SDT data and DL SDT signalling from 5GC) in cells controlled by other gNBs, configured to the UE in the RNA.

3.	Upon receiving the XnAP: RAN Paging, the receiving gNB takes the received MT-SDT Information into account to make decision on whether to page UE over the radio with MT-SDT indicator.

NOTE 1: it is up to the last serving gNB and the receiving gNB to make decision on whether the RAN Paging over the cells controlled by itself is triggered with MT-SDT indicator or not.

4.   Based on the MT-SDT indicator received in paging, the UE initiates MO-SDT procedure as described in clause 18.2 or 18.3 with the following differences:

-	the UE sends RRCResumeRequest message with the resume cause ‘mt-SDT’, and may be without user data and/or signalling.

--------End of the Changes-------------

- TP to TS 38.423 BL CR

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

//skip unchanged part

When available, the NG-RAN node1 shall include the Paging Cause IE in the RAN PAGING message towards the NG-RAN node2. If the Paging Cause IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.331 [10].

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

>>>>>>NEXT CHANGE<<<<<<<

#### 9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34]. | YES | ignore |
| E-UTRA Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.2.3.161 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.2.3.162 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED (voice, …) |  | YES | ignore |
| PEIPS Assistance Information | O |  | 9.2.3.166 |  | YES | ignore |
| MT-SDT Information | O |  | 9.2.3.xxx |  | YES | ignore |

>>>>>>NEXT CHANGE<<<<<<<

#### 9.2.3.xxx	MT-SDT Information

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Indicator | M |  | ENUMERATED (true, ...) |  |
| Data Size | O |  | INTEGER (1..96000, …) | The Unit is: byte. |

>>>>>>NEXT CHANGE<<<<<<<

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

*** skip unchanged ***

-- M

MaxNrofRS-IndexesToReport::= INTEGER (1..64, ...)

*** skip unchanged ***

MessageOversizeNotification ::= SEQUENCE {

maximumCellListSize								MaximumCellListSize,

iE-Extension				ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs}}	OPTIONAL,

...

}

MessageOversizeNotification-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MaximumCellListSize ::= INTEGER(1..16384, ...)

MT-SDT-Information ::= SEQUENCE {

mt-SDT-Indicator		::= ENUMERATED {true, ...}

dataSize				::= INTEGER (1..96000, ...) 				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs} } OPTIONAL,

...

}

MT-SDT-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MultiplexingInfo 	::=	SEQUENCE{

iAB-MT-Cell-List 	IAB-MT-Cell-List,

iE-Extensions		ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL,

...

}

MultiplexingInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

>>>>>>END OF CHANGEs<<<<<<<

# Background

Agreements in RAN3#119:

MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling.

Upon reception of DL SDT user data, the gNB-CU-UP may include the assistance information (e.g., Data size) in E1AP DL Data Notification message to gNB-CU-CP. FFS on MT-SDT indicator.

When receiving DL SDT data, the anchor gNB may send MT-SDT information IE to the neighbour gNBs within the RNA, via XnAP RAN paging message.

The encoding and the name of MT-SDT information IE is FFS.

The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging.

Upon reception of MT-SDT information via XnAP RAN paging message from the anchor gNB-CU, the gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message.

FFS on F1AP MT-SDT information

FFS: whether Data volume should also be provided from the CU to the DU in F1AP PAGING message.

FFS: whether and how XnAP RTRV UE CTXT REQ message (that carries MT-SDT resume indication)

FFS: whether MT-SDT support indication in E1 Bearer Context procedure should be defined to enable the gNB-CU-UP to include the DL data size in the E1AP DL DATA NOTIFICATION message.

FFS: whether receiving gNB-CU or receving gNB-DU decides MT-SDT Uu paging, if we agree that it is the receving gNB to make the final decision on triggering MT-SDT Uu paging.

FFS: MT-SDT assistance information sent from the anchor in the XnAP: RAN paging message and other alignment with RAN2 progress.

Agreements in RAN2#120:

1.	For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed FFS what the indication will be called.  FFS signalling details

2.	Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

3.	UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

4.	The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE.

5.	MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

6.	It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.  FFS stage 3 details

7.	New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

Agreements in RAN2#121:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.   When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6. 	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)