3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231400

Electronic meeting, 17 Apr – 26 Apr, 2023

Title: 	(TP to TS 38.423 BL CR) Consideration on selective activation of SCGs

Source: 	Huawei

Agenda item:	14.4

Document Type:	other

# Introduction

In this contribution, on selective activation of SCGs, taking into consideration of both RAN3 and RAN2 conclusions for the agreed solutions and FFSs, we provide our views on the potential solutions and their impacts on RAN3 and provide related TP to TS 38.423.

# Background

RAN3 Agreements/WAs:

RAN3 considers SCG selective activation is prioritized in the Rel-18 work. It can be revisited based on RAN2 progress.

RAN3 considers the Inter-CU and Intra-CU cases with equal priority, and studies both the F1 and Xn signaling aspects. It can be revisited based on RAN2 progress.

From RAN3 point of view, Rel-16/Rel-17 CPAC procedures are considered as start point for the Rel-18 work.

The following scenarios are depending on RAN2 progress.

SCG failure handling enhancements to enable PSCell addition and PSCell change after SCG failure.

Signaling support for inclusion of CPC configuration within a CPC or CPA configuration, in case CPC/CPA configuration is supported within CHO configuration.

WA: A primary focus of the objective is to enable subsequent cell changes by keeping conditional reconfigurations after a cell change. RAN3 to pursue study of the Xn/F1 signaling changes required to support this objective.

WA: RAN3 will work to enable both indirect and direct early data forwarding in Selective Activation. At this moment, RAN3 does not foresee any scenarios where direct forwarding is not feasible/desired.

WA (up to RAN2’s discussion): RAN3 assumes the last serving (source) PSCell may remain prepared within the prepared cells for Selective Activation.

Enhance XnAP and F1AP signaling to support NR Selective Activation.

Introduce a new indicator to the S-NODE ADDITION REQUEST message over Xn to indicate that the request is for Selective Activation.

RAN2#119bis-e agreements:

Confirm that “CPA” selective activation of cell groups will be supported for this WI objective

Confirm that we aim to support delta configuration, i.e. that there need to be known reference.

RAN2#120 agreements:

Delta configuration

A UE stores the reference configuration as a separate configuration.

The reference configuration is managed separately.

RAN2#121 agreements:

Assume to support the following scenarios of SCG selective activation:

      - SN initiated intra-SN SCG selective activation

      - MN initiated inter-SN SCG selective activation

      - SN initiated inter-SN SCG selective activation

It is assumed that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell.

In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

R2 assumes that a CPA conditional configuration can be used for CPC (but with different triggering conditions)

For inter-SN CPC, MN should provide the reference configuration to all candidate T-SNs (in order to generate the T-SN candidate configuration).

R2 understands that A target SN may include an indication in SN Addition Request Ack for each candidate target PSCell, denoting whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

# Discussion

## 3.1 Selective SCG Activation Request in SN Addition Request

SCG selective activation is to be introduced in Rel-18 to reduce signalling overhead and increase mobility robustness.

In previous meetings, RAN3 agreed to Introduce a new indicator to the S-NODE ADDITION REQUEST message over Xn to indicate that the request is for Selective Activation, based on the papers submitted last meeting, there were three options:

- Option 1: include a new Selective SCG Activation Request IE in the S-NODE ADDITION REQUEST message [R3-230103]

- Option 2: include Selective Activation Indicator IE as a sub IE of Rel-17 Conditional PSCell Addition Information Request IE in the S-NODE ADDITION REQUEST message [R3-230271]

- Option 3: include a Selective Activation IE in the CHO Information SN Addition IE in the S-NODE ADDITION REQUEST message [R3-230115]

Assuming we will not change MN for selective activation of SCG, the option 3 to capture the new IE in CHO related IE should not be considered. For option 1 and 2, the main difference is whether it is allowed to configure both Rel-17 CPAC and Rel-18 Selective Activation of SCGs at the same time for a UE. Based on the discussion in RAN2/3 so far, there is no limitation to support both, e.g. in case both are configured, upon execution of R18 selective activation of SCG, the R17 CPAC configurations are released, but R17 CPAC execution, the configuration of R18 selective activation of SCG are still maintained.

Observation 1: so far there is no limitation to support R17 CPAC and R18 Selective activation of SCG at the same time. (subject to RAN2 discussion)

Considering of ob1, it is better for RAN3 to choose option 1 as the baseline, and to be updated based on RAN2 progress, if needed.

Proposal 1-1: introduce a new Selective SCG Activation Request IE in the S-NODE ADDITION REQUEST message.

Similar to R17 CPAC, we could also include the Maximum Number of PSCells To Prepare IE and Estimated Arrival Probability IE in the new Selective SCG Activation Request IE

Proposal 1-2: include the Maximum Number of PSCells To Prepare IE and Estimated Arrival Probability IE in the new Selective SCG Activation Request IE.

Besides the information above, RAN2 agreed that for inter-SN CPC, MN should provide the reference configuration to all candidate T-SNs (in order to generate the T-SN candidate configuration). One possibility is to include the reference configuration in the existing M-NG-RAN node to S-NG-RAN node Container IE, which means RAN2 should include the reference configuration in the CG-ConfigInfo message. Another possibility is to include the reference configuration as a separate IE in the new Selective SCG Activation Request IE, the details is pending to RAN2 decision.

Proposal 1-3: include a Reference SCG Configuration IE in the new Selective SCG Activation Request IE, mark is as FFS pending to RAN2 progress.

## 3.2 Selective SCG Activation Request ACK in SN Addition Request ACK

Considering of the analyses of above proposal 1-1, we should also include a Selective SCG Activation Request Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, instead of the sub IE of Rel-17 Conditional PSCell Addition Information Acknowledge IE.

Proposal 2-1: introduce a new Selective SCG Activation Request Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

As mentioned in section 2, RAN2 understands that a target SN may include an indication in SN Addition Request Ack for each candidate target PSCell, denoting whether the associated SCG configuration is a delta with respect to the reference SCG configuration. To support this, in the new Selective SCG Activation Request Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, we should provide a per prepared PSCell indication about whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

Proposal 2-2: introduce a new per prepared PSCell optional indication in the new Selective SCG Activation Request Acknowledge IE about whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

## Support of SN initiated SCG selective activation

Based on the information in section 2, RAN2 assume to support the following scenarios of SCG selective activation:

- SN initiated intra-SN SCG selective activation

- MN initiated inter-SN SCG selective activation

- SN initiated inter-SN SCG selective activation

To support SN initiated intra-SN SCG selective activation, it is needed to introduce the Selective SCG Activation Request IE in the XnAP: S-NODE MODIFICATION REQUIRED message.

To support SN initiated inter-SN SCG selective activation, it is also needed to introduce the Selective SCG Activation Request IE in the XnAP: S-NODE CHANGE REQUIRED message, and introduce the Selective SCG Activation Request Acknowledge IE in the XnAP: S-NODE CHANGE CONFIRM message.

Proposal 3-1: introduce a new Selective SCG Activation Required IE in the S-NODE CHANGE REQUIRED REQUEST message.

Proposal 3-2: introduce a new Selective SCG Activation Confirm IE in the S-NODE CHANGE CONFIRM message.

Proposal 3-3: introduce a new Selective SCG Activation Request IE in the S-NODE MODIFICATION REQUIRED message.

The details of these IEs could be further discussed afterwards.

# Conclusion

In this paper, we discuss miscellaneous enhancements for selective activation of SCGs, and we have the following observations and proposals:

For SN Addition Request procedure:

Proposal 1-1: introduce a new Selective SCG Activation Request IE in the S-NODE ADDITION REQUEST message.

Proposal 1-2: include the Maximum Number of PSCells To Prepare IE and Estimated Arrival Probability IE in the new Selective SCG Activation Request IE.

Proposal 1-3: include a Reference SCG Configuration IE in the new Selective SCG Activation Request IE, mark is as FFS pending to RAN2 progress.

Proposal 2-1: introduce a new Selective SCG Activation Request Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

Proposal 2-2: introduce a new per prepared PSCell indication in the new Selective SCG Activation Request Acknowledge IE about whether the associated SCG configuration is a delta with respect to the reference SCG configuration.

SN initiated SCG selective activation

Proposal 3-1: introduce a new Selective SCG Activation Required IE in the S-NODE CHANGE REQUIRED REQUEST message.

Proposal 3-2: introduce a new Selective SCG Activation Confirm IE in the S-NODE CHANGE CONFIRM message.

Proposal 3-3: introduce a new Selective SCG Activation Request IE in the S-NODE MODIFICATION REQUIRED message.

The corresponding TP on SN Addition Request procedure is provided in section 6.

# Reference

- RAN3_118_agenda_20221118_1710

- R2_120 Chair Notes EOM rev1

# TP to TS 38.423 BL CR

------------Start of the First Change----------------

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

//skip the unchanged part

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Addition Information Request IE included in the S-NODE ADDITION REQUEST message, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

If the S-NG-RAN node UE Slice Maximum Bit Rate IE for a specific S-NSSAI is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use the received S-NG-RAN node UE Slice Maximum Bit Rate for all PDU sessions associated with the S-NSSAI for the concerned UE as defined in TS 23.501 [7].

If the Selective SCG Activation Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node may use it to configure SCG resources as specified in TS 37.340 [8]. Accordingly, the S-NG-RAN node shall, if supported, include the Selective SCG Activation Request Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the Selective SCG Activation Request Acknowledge IE is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, consider the indicated PSCells are prepared by the target SN as candidate PSCells for selective activation of SCGs.

------------Start of the Next Change----------------

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| //skip the unchanged part |  |  |  |  |  |  |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| Selective SCG Activation Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >Reference SCG Configuration | M |  | OCTET STRING | FFS | – |  |

------------Start of the Next Change----------------

#### 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| //skip the unchanged part |  |  |  |  |  |  |
| SCG Activation Status | O |  | 9.2.3.155 |  | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| Selective SCG Activation Request Acknowledge | O |  |  |  | YES | ignore |
| >Candidate PSCell List |  | 1 |  |  | – |  |
| >>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>>Delta to Reference | O |  | ENUMERATED (true, ...) | Indicates the associated SCG configuration is a delta with respect to the reference SCG configuration | – |  |

------------Start of the Next Change---------------

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

//skip unchanged part

UESliceMaximumBitRateList,

PagingCause,

MDTPLMNModificationList,

F1-terminatingIAB-donorIndicator,

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

SelectiveSCGActivationRequest,

SelectiveSCGActivationRequestACK

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AdditionalDRBIDs,

id-AMF-Region-Information,

id-AMF-Region-Information-To-Add,

//skip unchanged part

id-PEIPSassistanceInformation,

id-UESliceMaximumBitRateList,

id-S-NG-RANnodeUE-Slice-MBR,

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-SelectiveSCGActivationRequest,

id-SelectiveSCGActivationRequestACK

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells

//skip unchanged part

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- The IE shall be present if there is at least one  PDUSessionResourceSetupInfo-SNterminated included --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional }|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-UERadioCapabilityID					CRITICALITY reject		TYPE UERadioCapabilityID							PRESENCE optional }|

{ ID id-SourceNG-RAN-node-ID				CRITICALITY ignore		TYPE GlobalNG-RANNode-ID							PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY ignore		TYPE MDTPLMNList									PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore		TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-PSCellChangeHistory					CRITICALITY ignore		TYPE PSCellChangeHistory							PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject		TYPE IABNodeIndication								PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore		TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-CHOinformation-AddReq				CRITICALITY reject		TYPE CHOinformation-AddReq						PRESENCE optional }|

{ ID id-SCGActivationRequest				CRITICALITY ignore		TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CPAInformationRequest				CRITICALITY reject		TYPE CPAInformationRequest						PRESENCE optional }|

{ ID id-S-NG-RANnodeUE-Slice-MBR				CRITICALITY reject		TYPE UESliceMaximumBitRateList					PRESENCE optional }|

{ ID id-F1-terminatingIAB-donorIndicator		CRITICALITY reject		TYPE F1-terminatingIAB-donorIndicator			PRESENCE optional }|

{ ID id-SelectiveSCGActivationRequest		CRITICALITY reject		TYPE SelectiveSCGActivationRequest				PRESENCE optional },

...

PDUSessionToBeAddedAddReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionToBeAddedAddReq-Item

PDUSessionToBeAddedAddReq-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

s-NSSAI					S-NSSAI,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate			OPTIONAL,

sn-terminated			PDUSessionResourceSetupInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE

-- nor the PDU Session Resource Setup Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.1.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionToBeAddedAddReq-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeAddedAddReq-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RequestedFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST ACKNOWLEDGE

--

-- **************************************************************

SNodeAdditionRequestAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequestAcknowledge-IEs}},

...

}

SNodeAdditionRequestAcknowledge-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionAdmittedAddedAddReqAck	CRITICALITY ignore		TYPE PDUSessionAdmittedAddedAddReqAck		PRESENCE mandatory}|

{ ID id-PDUSessionNotAdmittedAddReqAck		CRITICALITY ignore		TYPE PDUSessionNotAdmittedAddReqAck		PRESENCE optional }|

{ ID id-SN-to-MN-Container					CRITICALITY reject		TYPE OCTET STRING								PRESENCE mandatory}|

{ ID id-admittedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes								PRESENCE optional }|

{ ID id-RRCConfigIndication					CRITICALITY reject		TYPE RRCConfigIndication						PRESENCE optional }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore		TYPE CriticalityDiagnostics					PRESENCE optional }|

{ ID id-LocationInformationSN				CRITICALITY ignore		TYPE Target-CGI									PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo		PRESENCE optional }|

{ ID id-AvailableFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE AvailableFastMCGRecoveryViaSRB3		PRESENCE optional }|

{ ID id-DirectForwardingPathAvailability	CRITICALITY ignore		TYPE DirectForwardingPathAvailability		PRESENCE optional }|

{ ID id-SCGActivationStatus					CRITICALITY ignore		TYPE SCGActivationStatus						PRESENCE optional	}|

{ ID id-CPAInformationAck					CRITICALITY ignore		TYPE CPAInformationAck							PRESENCE optional }|

{ ID id-SelectiveSCGActivationRequestACK	CRITICALITY ignore		TYPE SelectiveSCGActivationRequestACK		PRESENCE optional },

...

}

------------Start of the Next Change----------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

//skip unchanged part

#### -- S

SCGreconfigNotification ::= ENUMERATED {executed, ...}

S-NSSAIListQoE ::= SEQUENCE (SIZE(1..maxnoofSNSSAIforQMC)) OF S-NSSAI

S-BasedMDT ::= SEQUENCE {

ng-ran-TraceID				NG-RANTraceID,

iE-Extension				ProtocolExtensionContainer { {S-BasedMDT-ExtIEs} }	OPTIONAL,

...

}

S-BasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SelectiveSCGActivationCandidatePSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF SelectiveSCGActivationCandidatePSCells-item

SelectiveSCGActivationCandidatePSCells-item ::= SEQUENCE {

pscell-id						NR-CGI,

deltaToReference				ENUMERATED {true, ...}				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SelectiveSCGActivationCandidatePSCells-item-ExtIEs}}	OPTIONAL,

...

}

SelectiveSCGActivationCandidatePSCells-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SelectiveSCGActivationRequest ::= SEQUENCE {

max-no-of-pscells					INTEGER (1..maxnoofPSCellCandidates, ...),

estimatedArrivalProbability			CHO-Probability 	OPTIONAL,

refereceSCGConfiguration			OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { { SelectiveSCGActivationRequest-ExtIEs} } OPTIONAL,

...

}

SelectiveSCGActivationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SelectiveSCGActivationRequestACK ::= SEQUENCE {

candidate-pscells					SelectiveSCGActivationCandidatePSCells-list,

iE-Extensions						ProtocolExtensionContainer { { SelectiveSCGActivationRequestACK-ExtIEs} } OPTIONAL,

...

}

SelectiveSCGActivationRequestACK-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServiceType ::= ENUMERATED{

qMC-for-streaming-service,

qMC-for-MTSI-service,

qMC-for-VR-service,

...

}

SecondarydataForwardingInfoFromTarget-Item::= SEQUENCE {

secondarydataForwardingInfoFromTarget				DataForwardingInfoFromTargetNGRANnode,

iE-Extensions		ProtocolExtensionContainer { { SecondarydataForwardingInfoFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

SecondarydataForwardingInfoFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

------------Start of the Next Change----------------

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

//skip unchanged part

id-ManagementBasedMDTPLMNModificationList															ProtocolIE-ID ::= 362

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-TAINSAGSupportList																				ProtocolIE-ID ::= 364

id-SCGreconfigNotification																			ProtocolIE-ID ::= 365

id-earlyMeasurement																					ProtocolIE-ID ::= 366

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-SelectiveSCGActivationRequest																	ProtocolIE-ID ::= xx1

id-SelectiveSCGActivationRequestACK																	ProtocolIE-ID ::= xx2

END

-- ASN1STOP

------------End of the Changes--------------