3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231423

Online, 17th – 26th April, 2023

Agenda item:	 10.2.1

Source:		Lenovo

Title:		SON enhancements for SPR

Document for:		Discussion and Decision

1	Introduction

RAN3#118 meeting [1] achieved some agreements for SPR:

SPR as abbreviation for Successful PScell Change Report feature.

For SN-initiated classic PScell change the source SN node decides the T310/T312 triggers (e.g timer threshold) and the target SN node decides the T304 triggers (e.g timer threshold).

For classic addition/CPA, FFS on which node decides the T304 triggers(e.g timer threshold) and performs root cause analysis.

FFS which node decides SPR triggers and perform root cause analysis in case of MN-initiated classic PScell change /CPC user case and intra-SN classic PSCell change/CPC user case.

RAN3#119 meeting [2] achieved further agreements:

For classic addition/CPA, SN- and MN-initiated classic PSCell change/CPC, the target SN node decides the T304 trigger and performs root cause analysis.

For intra-SN classic PSCell change/CPC, the source SN decides SPR triggers of T310 and T312 and performs root cause analysis.

For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers and performs root cause analysis?

If the trigger is T312/310, the objective of SPR is to optimize lower layer issues of source PSCell and to optimize PSCell change configuration during mobility.

FFS for the trigger T304, whether the the objective of SPR is to optimize PSCell change configuration during mobility or the RACH access issue or both？

If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations.

FFS Which Node decides the triggers and which node performs root cause?

Option 1: For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers and performs root cause analysis

Option 2：For MN-initiated classic PScell change /CPC, the source SN node decides the T310/T312 triggers and performs root cause analysis

Option 3：For MN-initiated classic PScell change /CPC, the MN node decides the T310/T312 triggers based on source SN inputs and performs root cause analysis

FFS: Fetching of the SPR or forwarding mechanism (i.e. waiting RAN2’s progress).

FFS: correlation between SPR and SCGFailureInformation

In this paper, details of SON enhancements for SPR would be further discussed.

2	Discussion

For MN-initiated classicPSCell change or CPC, if SPR is triggered due to T310/T312, it was agreed that the objective of SPR is to optimize lower layer issues of source PSCell and to optimize PSCell change configuration during mobility, it indicates that classic PSCell change or CPC related parameters are set inappropriately, e.g. the threshold for triggering PSCell change or CPC procedure is improper.

RAN3#119 meeting also agreed that “If the trigger is T312/T310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations”. It is FFS on which node decides the triggers T310/T312, and which node performs root cause for MN-initiated classic PSCell change or CPC. There are three options on table:

- Option 1: For MN-initiated classic PSCell change /CPC, the MN node decides the T310/T312 triggers and performs root cause analysis.

- Option 2：For MN-initiated classic PSCell change /CPC, the source SN node decides the T310/T312 triggers and performs root cause analysis.

- Option 3：For MN-initiated classic PSCell change /CPC, the MN node decides the T310/T312 triggers based on source SN inputs and performs root cause analysis.

In R17 MRO for SCG failure, it is the node that initiates the PSCell change performs root cause analysis, this principle can be applied for SPR, e.g. for MN-initiated classic PSCell change or CPC, it is MN performs root cause analysis. Based on this principle, for MN-initiated classic PSCell change or CPC, it is suitable for the MN to decide T310/T312 trigger threshold for SPR. To compare Option 1 with Option 3, since T310/T312 trigger threshold of SPR may be a percentage value rather than an absolute value, MN can decide the percentage value (e.g. 40%, or 60%) by itself without knowing the exact timer value of T310/T312, which means that it is not necessary of MN to achieve the configured timer value of T310/T312 from source SN, in such a way, signalling coordination between MN and source SN can be avoided. Therefore, Option 1 is preferred.

Proposal 1: For MN-initiated classic PSCell change or CPC, the MN node decides the T310/T312 trigger threshold, and the MN node performs root cause analysis when SPR is triggered due to T310/T312 trigger threshold is fulfilled.

After the corresponding node decides T310/T312/T304 trigger threshold for SPR, configuration for generating SPR e.g. T310/T312/T304 trigger threshold associated with SCG would be sent to the UE. When at least one configured trigger condition for SPR is fulfilled, SPR is stored by the UE including some necessary information for analysis of the near-failure PSCell addition/change, e.g. source PSCell information in case of PSCell change/CPC, target PSCell information, SPR cause, latest measurement results, location information of the UE and time elapsed between the CPAC execution and reception of CPAC configuration in case of CPAC.

RAN3#118 meeting also discussed some other items to be included in the SPR e.g. information that PSCell change was MN-initiated or SN-initiated, C-RNTI (MN, target SN, source SN), or time between CPC execution and report retrieval, but achived no consensus. For information that PSCell change was MN-initiated or SN-initiated, since the UE can not know it without an explicit indication from the network, there is no need to include it in the SPR. For source PSCell C-RNTI as well as time elapsed since CPC execution until report retrieval, they are beneficial for MRO for the case that network has no UE context since SPR is not sent to the network immediately.

Proposal 2: It is beneficial to include source PSCell C-RNTI and time elapsed since CPC execution until report retrieval in the SPR for MRO, if SPR is not sent to the network immediately.

RAN2#120 meeting has agreed that only MN can retrieve the SPR from the UE, which means that after the PSCell addition or CPA as well as PSCell change or CPC procedure is successfully completed, an indication for availability of SPR may be transmitted to MN by the UE, based on the received indication MN may request the SPR from the UE via UE Information Request message, and then the UE may transmit the stored SPR to MN via UE Information Response message.

For SN-initiated classic PSCell change or CPC, if SPR cause is T310 or T312, source SN performs root cause analysis, e.g. MN may forward the received SPR to source SN for MRO analysis. When the source SN receives the forwarded SPR, UE context may be released in the source SN, thus the source SN can not identify the SPR for MRO. To make source SN understand the association between the UE and SPR, similar as R17 MRO for SCG failure, when the MN forwards the SPR to source SN, SN Mobility Information (e.g. which may be generated by source SN and sent to MN via the S-NODE CHANGE REQUIRED message) may also be transferred to the source SN. The existing Xn message (e.g. ACCESS AND MOBILITY INDICATION) may be reused to transfer SPR as well as SN Mobility Information.

Proposal 3: SN Mobility Information and SPR may be transferred from MN to the source SN for MRO analysis.

3	Conclusion

In this paper, details of SON enhancements for SPR are discussed. We have the following proposals:

Proposal 1: For MN-initiated classic PSCell change or CPC, the MN node decides the T310/T312 trigger threshold, and the MN node performs root cause analysis when SPR is triggered due to T310/T312 trigger threshold is fulfilled.

Proposal 2: It is beneficial to include source PSCell C-RNTI and time elapsed since CPC execution until report retrieval in the SPR for MRO, if SPR is not sent to the network immediately.

Proposal 3: SN Mobility Information and SPR may be transferred from MN to the source SN for MRO analysis.

Reference

[1] Chairman notes for RAN3#118 meeting

[2] Chairman notes for RAN3#119 meeting