3GPP TSG-RAN WG3 #119bis-e	R3-231437

E-meeting, 17th April – 26th April 2023

Agenda Item:	12.2.2.3

Source:	Lenovo

Title: 	        Discussion on issues related to AI based network energy saving

Document for:	Discussion and Approval

# 1	Introduction

In the last RAN3 meeting, the following agreements have been made related to AI based network energy saving. In this paper, we try to discuss the relevant issues.

FFS on how to calculate this EE, and which of the following 4 options should be adopted: Option 1: Indicating the value of the ratio of data volume over energy consumption directly Option 2: Define the EE metric in a more abstract way using a quantitative encoding, e.g., using EE values on a linear scale from 0 to 100. Option 3: The metric of Energy Efficiency exchanged between NG-RAN nodes is an Energy Consumption related to an additional load. And exchanged EE metric between neighboring NG-RAN nodes is defined in the interval [0, 100]. Option 4: Deliver both data volume and energy consumption over RAN interfaces to let the requesting node calculate the overall DV and over EC of the specific area and thereby drive the overall EE. RAN3#119: Introduce the metric of Energy Cost (EC) as the AI/ML metric to be shared over the Xn interface among gNBs. Adopt the below Option-3a and exchange Energy Cost (EC) upon request over the Xn interface. The metric of Energy Cost (EC) exchanged between NG-RAN nodes can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighboring node for either additional load or current load (The details to be further discussed). EC is a value at gNB level.

# 2	Discussion

According to the discussion in the last meeting, one way to use the Energy Cost (EC) is following. Before source gNB decides to switch off a cell and handover all UEs to a neighbour target gNB, source gNB can  request inferred additional energy cost from the target gNB. Then, the source gNB will take into account the estimated additional energy cost at the target gNB when decides to switch off a cell. For example, the source gNB could only switch off a cell if the estimated additional energy cost at the target gNB is less than the reduced energy cost at the source gNB.  An example procedure is illustrated in Figure 1.

To make better network energy saving decision, a gNB can request inferred additional energy cost for additional traffic load from neighbor gNB.

First of all, regarding which procedure shall be used to request the inferred additional energy cost, RAN3 has considered the following options during the last meeting email discussion:

- Option-1: modify the existing Resource Status procedure to transfer this information,

- Option-2: use the new procedure being defined to transfer the AI/ML related information

- Option-3: develop a new procedure just for this particular objective.

Since RAN3 has agreed at the same time to use the agreed new AI/ML INFORMATION REQUEST/RESPONSE/UPDATE (FFS naming) messages to collect UE performance feedback after handover, it implies the agreed new procedures can be used to convey more than prediction information. Besides, the additional energy cost is not an actual measurement, instead it is an inferred value which could be output of an AI/ML model. Thus, we believe the agreed new procedures can be used to request the additional energy cost as well.

The agreed new AI/ML INFORMATION REQUEST/RESPONSE/UPDATE (FFS naming) is extended to transfer non-prediction information, i.e., UE performance feedback

Inferred additional energy cost can be an inference result by an AI/ML model.

Use the agreed new AI/ML INFORMATION REQUEST/RESPONSE/UPDATE (FFS naming) to request and transfer the inferred additional energy cost.

To support above mentioned procedure, when requesting the additional energy cost, the source gNB should also indicate the associated additional traffic load to the target gNB. The format of the additional traffic load could be same as the format of energy cost, e.g., a value in the interval of [0,100] or actual value.

In the AI/ML INFORMATION REQUEST (FFS naming) message, when the initiating gNB requests the additional Energy Cost from its neighbour gNB, it also indicates the associated additional traffic load.

Figure 1: Exemplary procedure to request inferred additional energy cost before cell switch-off decision

In addition, companies believe the energy cost inference is at gNB level since it is not quite feasible to measure and calculate cell level energy cost. On the other hand, we see some benefits to provide cell level information for the neighbour gNB to better infer the additional energy cost considering:

- If the network energy saving decision is to switch off one cell of source gNB and handover UEs to another cell of target gNB, the inferred additional energy cost is essentially about the increased energy cost of that cell.

- Target gNB CU needs to coordinate with the corresponding DU to better understand the additional energy cost to the DU.

For example, before the network energy saving decision and requesting the inferred additional energy cost, source gNB may already have an idea to handover UEs to which target cell belonging to the target gNB. In this case, in the same message requesting inferred additional energy cost it can also indicate the potential target cell. Then, target gNB CU can coordinate with the corresponding DU to have a better understanding of the current situation and make better inference.

Target gNB can better infer the additional energy cost if it knows the potential target cell in advance.

In the AI/ML INFORMATION REQUEST (FFS naming) message, when the initiating gNB requests the additional Energy Cost from its neighbour gNB, it can also indicate the potential target cell that will be impacted.

Besides predicted DV and predicted EC, we find it also beneficial to exchange the predicted cell activation/deactivation between gNBs. For instance, after knowing one cell belonging to a neighbour gNB will probably be deactivated due to energy saving soon, a gNB will not handover UEs to that cell anymore. Otherwise, the neighbour gNB will have to handover the same UEs to another cell shortly after the UEs are handed over to the neighbour gNB.

RAN3 is suggested to support the exchange of cell activation/deactivation prediction information between gNBs over Xn interface.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	To make better network energy saving decision, a gNB can request inferred additional energy cost for additional traffic load from neighbor gNB.

Observation 2	The agreed new AI/ML INFORMATION REQUEST/RESPONSE/UPDATE (FFS naming) is extended to transfer non-prediction information, i.e., UE performance feedback

Observation 3	Inferred additional energy cost can be an inference result by an AI/ML model.

Observation 4	Target gNB can better infer the additional energy cost if it knows the potential target cell in advance.

Based on the discussion above, we propose:

Proposal 1	Use the agreed new AI/ML INFORMATION REQUEST/RESPONSE/UPDATE (FFS naming) to request and transfer the inferred additional energy cost.

Proposal 2	In the AI/ML INFORMATION REQUEST (FFS naming) message, when the initiating gNB requests the additional Energy Cost from its neighbour gNB, it also indicates the associated additional traffic load.

Proposal 3	In the AI/ML INFORMATION REQUEST (FFS naming) message, when the initiating gNB requests the additional Energy Cost from its neighbour gNB, it can also indicate the potential target cell that will be impacted.

Proposal 4	RAN3 is suggested to support the exchange of cell activation/deactivation prediction information between gNBs over Xn interface.

# Reference