3GPP TSG-RAN WG3 Meeting #119bis-e		R3-231459

Online, 17th – 26th Apr. 2023

Source:	vivo

Title:	Discussion on L1/L2-triggered Mobility

Agenda Item:	14.2

Document for:	Discussion and Decision

# Introduction

In the last meeting, RAN3 made the following agreements:

DL/UL TEID handling during LTM configuration:

For intra-DU LTM, the gNB-CU assigns a new UL GTP TEID for each DRB and provides it to the gNB-DU via UE Context Modification Request message(s). The gNB-DU assigns the new DL GTP TEIDs per DRB per candidate cell (whether it should be per candidate cell needs to be further discussed) and provides them back to the gNB-CU in UE Context Modification Response message(s).

For inter-DU LTM, the gNB-CU assigns a new UL GTP TEID for each DRB and provides it to the target gNB-DU via UE Context Setup Request message(s). The target gNB-DU assigns the new DL GTP TEIDs per DRB per candidate cell (whether it should be per candidate cell needs to be further discussed) and provides them back to the gNB-CU in UE Context Setup Response message(s).

Data transmission:

Intra-CU UP case: CU will start data transmission after LTM cells switch signaling from DU including target cell ID.

Whether new message or legacy message is FFS.

E1 aspects:

In case of CP UP separation, once CUCP receives LTM cell switch signling from (source)DU, CU CP initiates E1 bearer context modification including DL tunnel ID per DRB for target cell, and security keys corresponding to target cell (if updated) for data transmission.

The tentative flow chart to reflect above sentence is captured in section 3.6 in the Sod, to be continued based on this basis.

In this contribution, we discuss the remaining open issues for LTM(L1/L2-triggered mobility), and we are going to further discuss the procedures of LTM for both intra-DU and intra-CU inter-DU mobility scenarios.

# Discussion

Issue 1: The gNB-CU sends the suggested candidate cell(s) to the gNB-DU in UE Context Modification Request procedure in one message or multiple messages.

RAN2 has made the following agreement in RAN2#119bis-e

- For L1L2 mobility will support that candidate configurations are delta configurations on top of a reference configuration. FFS if the reference configuration is a separate reference configuration or e.g. the current configuration.

When delta configurations for candidate cells are used, if gNB-CU sends the suggested candidate cell(s) to the gNB-DU in UE Context Modification Request procedure in one message, gNB-CU needs to provide the reference configuration to the gNB-DU only once in one message to enable the gNB-DU to generate multiple candidate cell configurations. While if gNB-CU sends the suggested candidate cell(s) to the gNB-DU in UE Context Modification Request procedure in multiple messages, gNB-CU needs to provide the reference configuration in each UE Context Modification Request message. Therefore, using one message to prepare multiple candidate cell(s) leads to less signaling overhead in F1 interface when compared with multiple messages method. Hence, we prefer to prepare multiple candidate cell(s) in one message.

The procedure of preparing multiple candidate cell(s) in one message can be as below:

- For intra-DU case, gNB-CU sends a UE Context Modification Request message to gNB-DU containing the reference configuration and multiple suggested candidate cells’ id. And gNB-DU responds to gNB-CU with a UE Context Modification Response message containing multiple accepted candidate cells’ configurations.

- For intra-CU inter-DU case, gNB-CU sends a UE Context Setup Request message to the candidate gNB-DU containing the reference configuration and multiple suggested candidate cells’ id. And the candidate gNB-DU responds to gNB-CU with a UE Context Setup Response message containing multiple accepted candidate cells’ configurations.

- For intra-DU LTM case, gNB-CU can include multiple suggested candidate cells in one UE Context Modification Request message and gNB-DU responses gNB-CU with multiple accepted candidate cells in one UE Context Modification Response message.

- For intra-CU inter-DU LTM case, gNB-CU can include multiple suggested candidate cells in one UE Context Setup Request message and candidate gNB-DU response gNB-CU with multiple accepted candidate cells in one UE Context Setup Response message.

Issue 2: New message or legacy message is used for indication of initiation of LTM cell switch command.

In RAN3#118, it was agreed that gNB-DU should signal to gNB-CU during LTM execution phase. In RAN3#119, it was agreed that target cell ID is included in the message for indication of initiation of LTM from source gNB-DU to gNB-CU. We believe that UE Context Modification Required message can be used for this purpose and no new F1 message is needed to be introduced.

- Source gNB-DU indicates gNB-CU about the initiation of LTM by UE Context Modification Required message after handover command is sent.

Issue 3: DDDS on F1-U.

For the DDDS frame from source gNB-DU, in RAN2#119e, the following agreement is made,

- R2 assumes that L2 is continued whenever possible (e.g. intra-DU), without Reset, with the target to avoid data loss, and the additional delay of data recovery.

According to RAN2’s assumption, L2 is continued whenever possible, so the RLC in intra-DU mobility case may not be re-established. The DDDS from source gNB-DU to CU-UP is not needed since no PDCP data will be discarded in RLC during LTM, i.e. retransmission of lost PDCP data to serving DU is not needed.

For intra-CU inter-DU mobility case, with RLC re-established, the DDDS from source gNB-DU should be sent to CU-UP as soon as the LTM command is sent, so the CU-UP can start to forward the unsuccessfully transmitted data to target gNB-DU before handover success, this brings less data interruption.

- For intra-DU mobility case, DDDS from source gNB-DU to CU-UP is not needed for DRB whose RLC is not re-established.

- For intra-CU inter-DU mobility case, the DDDS should be sent from source gNB-DU to CU-UP when the LTM cell switch command is sent. Then the CU-UP can start forwarding the unsuccessfully transmitted data to target gNB-DU.

For the DDDS frame from target gNB-DU, we think LTM can follow the same procedure as legacy L3 handover: target gNB-DU sends initial DDDS using the new UL TEID to CU-UP after DU detects the UE access.

- For both intra-DU and intra-CU inter-DU mobility cases, target gNB-DU sends initial DDDS using the new UL TEID to CU-UP after target gNB-DU detects the UE access.

Issue 4: Early TA acquisition.

RAN3 has received a LS [3] for PDCCH-order based RACH for candidate cells:

Regarding the configuration/indication of RAR reception for PDCCH ordered-RACH, RAN1 achieved the following agreement

For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated

- If reception of RAR is not configured/indicated (without RAR)

  - TA value of candidate cell is indicated in cell switch command

  - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated

  - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order

- If reception of RAR is configured/indicated (with RAR), FFS

  - whether RAR is received from serving cell or candidate cell

    - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE

  - content of RAR

- FFS: signaling for configuration/indication of whether RAR needs to be received

- UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM

- Send LS to RAN2 and RAN3 to check the feasibility about this agreement

- Note: Definition of candidate cells is up to RAN2

As the feasibility of schemes included in the agreement above is related to the designs of RAN2 and RAN3, RAN 1 respectfully asks RAN2 and RAN3 to check the feasibility and potential impact on specs of RAN2 and RAN 3 of all options, i.e. with RAR (from serving or candidate cell) and without RAR, in this agreement.

According to the agreements from RAN1, PDCCH ordered RACH for early TA acquisition is to be supported for LTM. This means that the serving cell needs to know the CFRA resources (such as preamble index, SS/PBCH index and PRACH Mask index) that have been prepared for the UE by the candidate cells before handover execution.

Therefore, for inter-DU scenario, the CFRA resources used for PDCCH ordered RACH should be transmitted from the candidate DU to the S-DU via F1 interface.

- To support early TA acquisition for inter-DU LTM, the CFRA resources of the candidate cells needs to be transmitted from candidate DU to source DU via F1 interface. Which F1 messages to use are FFS.

Similarly, when the candidate cell acquires the TA of UE, the TA value should be transmitted from the candidate cell to the serving cell via F1 interface for inter-DU scenario in the following two cases,

- Case 1: RAR is not configured/indicated and TA value of candidate cell is indicated in cell switch command.

- Case 2: RAR is configured and but is received from serving cell.

- To support early TA acquisition for inter-DU LTM, the TA value should to be transmitted from candidate DU to S-DU via F1 interface when RAR is not configured or RAR is received from serving cell. Which F1 messages to use are FFS.

For RAR received from candidate cell, UE gets TA from candidate cell and TA value is not needed to be transmitted from candidate DU to source DU.

- Send to LS to RAN1 to indicates the RAN3 impact on early TA acquisition for inter-DU LTM.

# LTM procedures

## 3.1 Intra-DU LTM

Based on the progress of the last meeting and the proposals discussed in section 2, we show the flow chart of intra-DU LTM below,

Figure 1: Intra-DU L1/L2-triggered mobility

- The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing the cell quality measurements of serving and neighboring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate LTM configuration.

- The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing the candidate cells list.

- If the request for configuring LTM is accepted, the gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations of the candidate cells. gNB-DU may accept all or part of the target candidate cells.

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes a generated RRCReconfiguration message with the LTM configuration, which may contain a configuration per target candidate. (e.g. including the lower layer RRC configurations of the candidate cells for LTM. Details are FFS).

- The gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

- The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends lower layer measurement result to the gNB-DU. The gNB-DU decides to execute L1/L2-triggered mobility.

- The gNB-DU sends the L1/L2-triggered mobility cell switch command to the UE.

- The gNB-DU sends a UE CONTEXT MODIFICATION REQUIRED message to the gNB-CU containing the indication about the initiation of L1/L2-triggered mobility after the L1/L2-triggered mobility cell switch command is sent.

- The gNB-CU responds with a UE CONTEXT MODIFICATION CONFIRM message to the gNB-DU.

- UE performs RACH in the target cell. The gNB-DU informs the gNB-CU of which cell the UE has successfully accessed during LTM by ACCESS SUCCESS message.

- For intra-DU mobility case, RAN3 assumes gNB-DU can detect the UE access during RACH procedure in LTM, i.e. legacy mechanism is reused and no F1 impact is expected. Enhancements on F1 interface for supporting rach-less can be considered later when RAN1/2 has further conclusions for rach-less.

- The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message to release the source cell/prepared cells’ resources in the gNB-DU.

- The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

NOTE:	If subsequent LTM is to be performed, it’s up to gNB-CU whether to perform steps 14~15.

## 3.2 Intra-CU inter-DU LTM

Based on the progress of last meeting and the proposals discussed in section 2, we show the flow chart of intra-CU inter-DU LTM below,

Figure 2: Intra-CU inter-DU L1/L2-triggered mobility

- The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing the cell quality measurements of serving and neighboring cells.

- The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

- The gNB-CU determines to initiate L1/L2-triggered mobility configuration.

- The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU, containing the candidate cells list.

- If the candidate gNB-DU decides to accept the request of LTM configuration, it responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configuration for the accepted target candidate cell(s).

- The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes a generated RRCReconfiguration message with the LTM configuration, which may contain a configuration per target candidate. (e.g. including the lower layer RRC configurations of the candidate cells for LTM. Details are FFS).

- The source gNB-DU forwards the received RRCReconfiguration message to the UE.

- The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

- The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

- The UE sends the lower layer measurement result to the source gNB-DU. The source gNB-DU decides to execute L1/L2-triggered mobility to a candidate target cell.

- The source gNB-DU sends the L1/L2-triggered cell switch command to the UE. The source gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE.

- The source gNB-DU sends a UE CONTEXT MODIFICATION REQUIRED message to the gNB-CU containing the indication about the initiation of LTM after the LTM cell switch command is sent. The target cell id is included in the message.

- The gNB-CU responds to the source gNB-DU with a UE CONTEXT MODIFICATION CONFIRM message.

NOTE:	The gNB-CU can start sending DL User Data to the candidate gNB-DU after reception of the Downlink Data Delivery Status and the target cell id.

- RACH is performed at the target gNB-DU. The target gNB-DU sends an initial Downlink Data Delivery Status frame to inform the gNB-CU. The target gNB-DU also sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed.

- For intra-CU inter-DU mobility case, RAN3 assumes the target gNB-DU can detect the UE access during RACH procedure in LTM, i.e. legacy mechanism is reused and no F1 impact is expected. Enhancements on F1 interface for supporting rach-less can be considered later when RAN1/2 has further conclusions for rach-less.

- The gNB-CU may send a UE CONTEXT RELEASE COMMAND message to the gNB-DU.

- The source gNB-DU releases the UE context and responds with a UE CONTEXT RELEASE COMPLETE message to the gNB-CU.

NOTE:	If subsequent LTM is to be performed, it’s up to gNB-CU whether to perform steps 15~16.

# Conclusion

In this contribution, we discuss the issues left for L1/L2-triggered mobility. And then we discuss the procedures of LTM for both intra-DU and intra-CU inter-DU mobility scenarios. We have the following proposals,

- For intra-DU LTM case, gNB-CU can include multiple suggested candidate cells in one UE Context Modification Request message and gNB-DU responses gNB-CU with multiple accepted candidate cells in one UE Context Modification Response message.

- For intra-CU inter-DU LTM case, gNB-CU can include multiple suggested candidate cells in one UE Context Setup Request message and candidate gNB-DU response gNB-CU with multiple accepted candidate cells in one UE Context Setup Response message.

- Source gNB-DU indicates gNB-CU about the initiation of LTM by UE Context Modification Required message after handover command is sent.

- For intra-DU mobility case, DDDS from source gNB-DU to CU-UP is not needed for DRB whose RLC is not re-established.

- For intra-CU inter-DU mobility case, the DDDS should be sent from source gNB-DU to  CU-UP when the LTM cell switch command is sent. Then the CU-UP can start forwarding the unsuccessfully transmitted data to target gNB-DU.

- For both intra-DU and intra-CU inter-DU mobility cases, target gNB-DU sends initial DDDS using the new UL TEID to CU-UP after target gNB-DU detects the UE access.

- To support early TA acquisition for inter-DU LTM, the CFRA resources of the candidate cells needs to be transmitted from candidate DU to source DU via F1 interface. Which F1 messages to use are FFS.

- To support early TA acquisition for inter-DU LTM, the TA value should to be transmitted from candidate DU to S-DU via F1 interface when RAR is not configured or RAR is received from serving cell. Which F1 messages to use are FFS.

- Send to LS to RAN1 to indicates the RAN3 impact on early TA acquisition for inter-DU LTM.

- For intra-DU mobility case, RAN3 assumes gNB-DU can detect the UE access during RACH procedure in LTM, i.e. legacy mechanism is reused and no F1 impact is expected. Enhancements on F1 interface for supporting rach-less can be considered later when RAN1/2 has further conclusions for rach-less.

- For intra-CU inter-DU mobility case, RAN3 assumes the target gNB-DU can detect the UE access during RACH procedure in LTM, i.e. legacy mechanism is reused and no F1 impact is expected. Enhancements on F1 interface for supporting rach-less can be considered later when RAN1/2 has further conclusions for rach-less.

# References

- R3-226230, Solutions for L1/L2 triggered mobility. Ericsson.

- 3GPP TS TS 38.401: “NG-RAN; NG Application Protocol (NGAP)”.

- R1-2302194 LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM.