3GPP TSG-RAN WG3 #119bis-e	R3-231463

Online, 17th – 26th April 2023

Source:	CATT, CBN, China Telecom

Title:	(TP for 38.401/38.413/38.413/38.473)Discussion on multicast over RRC INACTIVE

Agenda Item:	15.3

Document for:	Discussion and decision

- Introduction

This paper discusses four aspects:

- Scenario of multicast over RRC INACTIVE;

- CU to DU RRC information for generating MCCH content.

- Delivering UE-assistance information over RAN3 interfaces (partly based on SA2 agreed CRs);

- Indicator in the group paging messages.

- Discussion

  - Scenario

During recent offline discussions there seemed to be some different understandings on the scenario of RRC INACTIVE.

The justification captured in the WID (RP-213598) is:

In Rel-17, RAN only specifies multicast for UEs in RRC_CONNECTED state, which may not fully fulfil the requirements of, e.g., Mission Critical Services, especially for cells with a large number of UEs according to TR 23.774. Also, to always keep UEs in RRC_CONNECTED state is not power efficient. It is ‎therefore important to support multicast for UEs in RRC_INACTIVE.

We can observe that there are two motivations:

- To handle the case that there are a large number of UEs in the concerned cell (i.e. the cell is congested);

- To save power.

Obviously, neither cases means there must be many UEs receiving the concerned multicast session:

- The cell may be congested by other services, e.g. unicast or other multicast session;

- Power saving is welcome regardless of how many peers are nearby.

Proposal 1: Confirm that, based on the justification captured in the WID, it is possible for the multicast-over-RRC-INACTIVE mode to be used regardless of whether there is many UEs receiving the concerned multicast session in the concerned cell.

  - CU-to-DU RRC Information

On the aspect of delivering RRC configuration for Multicast over RRC Inactive, RAN2 has agreed to adopt a “mixed” design, i.e. both per-UE RRC message and MCCH message can deliver such configuration. Although many details are still pending RAN2, one RAN3 impact can already be figured out: delivering the MRB PDCP configuration for multicast from gNB-CUs to gNB-DUs.

For broadcast the MRB PDCP configuration is delivered in the following IE type:

#### 9.3.1.225 MBS CU to DU RRC Information This IE indicates the MBS CU to DU RRC Information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Broadcast Cell List; M. >MBS Broadcast Cell Item; 1 .. <maxCellingNBDU>. >>NR CGI; M; NR CGI 9.3.1.12. >>mtch-neighbourCell; O; OCTET STRING; mtch-neighbourCell, as defined in TS 38.331[8]. MRB PDCP Config Broadcast; O; OCTET STRING; MRB-PDCP-ConfigBroadcast, as defined in TS 38.331[8]. Range bound; Explanation. maxCellingNBDU; Maximum no. cells that can be served by a gNB-DU. Value is 512..

Naturally we should introduce a similar IE for multicast. Considering that the existing “9.3.1.225 MBS CU to DU RRC Information” includes a mandatory present IE “MBS Broadcast Cell List” literally dedicated for broadcast, we had better define a new IE type rather than reusing this existing one. The newly defined IE type may be named “Multicast CU to DU RRC Information”, containing an IE “MRB PDCP Config Multicast” at least. It is pending RAN2 whether more RRC information should be contained. For safe we suggest the new IE optional present.

This new IE type can be included within “9.2.14.2 MULTICAST CONTEXT SETUP REQUEST” and “9.2.14.7 MULTICAST CONTEXT MODIFICATION REQUEST”, akin to its counterpart for broadcast.

Proposal 2: To introduce a new IE “Multicast CU to DU RRC Information” into the “9.2.14.2 MULTICAST CONTEXT SETUP REQUEST” message and the “9.2.14.7 MULTICAST CONTEXT MODIFICATION REQUEST” message, which in turn includes at least an optional “MRB PDCP Config Multicast” IE.

Considering that there may not be many UEs in RRC_CONNECTED state receiving the concerned multicast session (as in Proposal 1), the gNB-DU should rely on indicator from the gNB-CU that the PTM leg should be kept. The “Multicast CU to DU RRC Information” can be used as an implicit indicator for this purpose.

Proposal 3: The new IE “Multicast CU to DU RRC Information” can be used as an indicator that the PTM leg should be kept. It is thus proposed to capture in TS 38.401 that the gNB-DU, if receives the “Multicast CU to DU RRC Information” from the gNB-CU shall generate the MCCH message accordingly and keep PTM delivery if the session is activated.

  - Per-UE assistance information on XnAP

Last meeting there was a discussion on delivering per-UE assistance information on NGAP, and there seemed to be a consensus that a new IE should be introduced. In addition to the issue NGAP, S2-2301594 also captured that:

##### 7.2.3.Y.3 UE mobility out of RNA and within RA When a UE in RRC_INACTIVE state is receiving multicast MBS session data, if the UE moves out of the RNA and within the RA, the UE resumes the connection in the same way as specified in clause 7.2.3.7. If the target NG-RAN retrieves the UE context successfully, the following enhancements apply: - "MBS Assistance information" is stored in UE context in RAN and therefore is transferred to target RAN node using Xn signalling prcedures; - in order to support a scenario where the source RAN node does not support multicast MBS for UEs in RRC_INACTIVE state but the target RAN node does, the SMF may send the MBS assistance information in the MBS related information to the target NG-RAN via the AMF.

That is to say, it should also be supported that the assistance information is delivered over XnAP. Since SA2 has already mentioned the UE context retrieval procedure, it is natural that the assistance information is included in the XnAP handover procedure as well. Both the XnAP Retrieve UE Context Response message and the XnAP Handover Request message can contain a “9.2.1.36 MBS Session Information List” IE type, so simply introducing the new assistance information IE into each item of this IE type is sufficient.

Proposal 4: To introduce a new IE “MBS assistance information” into each item of “9.2.1.36 MBS Session Information List” of XnAP, which cover the cases of Xn-based handover and UE context retrieval. Its name and semantic description should be tagged FFS.

  - Group paging

It is widely-proposed in RAN2 that a new indicator should be introduced into Uu (group) paging message so that UEs using the “multicast over INACTIVE mode” may keep itself in RRC INACTIVE state. In last RAN3 meeting it was ever proposed that this new indicator should be introduced into the group paging messages in RAN3 specs as well.

In our understanding this new indicator (in RAN3) has two main uses:

- To distinguish Rel-17 conventional anchor node from Rel-18 anchor node. Rel-17 anchor node, unaware of the existence of “multicast over INACTIVE” mode, ordinarily expects all UEs it pages within an XnAP group paging message will soon return RRC CONNECTED state. If it finds that a UE does not resume timely (because it is a Rel-18 UE, receives the Rel-18 indicator in Uu paging and need not resume), it will ordinarily consider this UE unreachable and release the UE context, causing either an unnecessary CN paging procedure or context misalignment between the UE and the network.

- To distinguish session activation from session release. Both session activation and session release may occur for a deactivated session. For the case of session activation, as captured as an object by SA2, a UE in RRC INACTIVE state can use the “multicast over INACTIVE” mode directly and may not perform RRC resumption. For the case of session release, a UE in RRC INACTIVE state should resume and perform the entire session release procedure.

Although for both uses there are alternative solutions (mainly within RAN2 or SA2 scope), we propose introducing such indicator into RAN3 group paging messages for safe, for alignment and for simplification of RAN node behaviours.

The new indicator should indicate that the UEs are allowed to stay in RRC INACTIVE state, and its criticality should be “ignore”. That is to say, whenever a node does not support “multicast over INACTIVE” mode, it shall converse the Rel-18 paging message toward Rel-17 conventional one.

Proposal 5: To introduce an indicator, criticality “ignored”, that UEs are allowed to stay in INACITVE state into the GROUP PAGING messages in NGAP, XnAP and F1AP, in order to distinguish session activation from session release and to distinguish Rel-18 anchor nodes from Rel-17 anchor nodes.

- Conclusion

Proposal 1: Confirm that, based on the justification captured in the WID, it is possible for the multicast-over-RRC-INACTIVE mode to be used regardless of whether there is many UEs receiving the concerned multicast session in the concerned cell.

Proposal 2: To introduce a new IE “Multicast CU to DU RRC Information” into the “9.2.14.2 MULTICAST CONTEXT SETUP REQUEST” message and the “9.2.14.7 MULTICAST CONTEXT MODIFICATION REQUEST” message, which in turn includes at least an optional “MRB PDCP Config Multicast” IE.

Proposal 3: The new IE “Multicast CU to DU RRC Information” can be used as an indicator that the PTM leg should be kept. It is thus proposed to capture in TS 38.401 that the gNB-DU, if receives the “Multicast CU to DU RRC Information” from the gNB-CU shall generate the MCCH message accordingly and keep PTM delivery if the session is activated.

Proposal 4: To introduce a new IE “MBS assistance information” into each item of “9.2.1.36 MBS Session Information List” of XnAP, which cover the cases of Xn-based handover and UE context retrieval. Its name and semantic description should be tagged FFS.

Proposal 5: To introduce an indicator, criticality “ignored”, that UEs are allowed to stay in INACITVE state into the GROUP PAGING messages in NGAP, XnAP and F1AP, in order to distinguish session activation from session release and to distinguish Rel-18 anchor nodes from Rel-17 anchor nodes.

- Annex A: TP for TS 38.401

-----------Start of the Change----------

#### 8.15.1.2	Multicast MBS Session Context Establishment

Figure 8.15.1.2-1 illustrates an examplified interaction of NGAP, E1AP, F1AP and RRC protocol functions for Multicast MBS Session Context Establishment with a UE joining an active multicast session as the first UE joining in its serving gNB.

Figure 8.15.1.2-1: Multicast MBS Session Context establishment

0.	For a certain multicast MBS session (TMGI), which is currently active, no multicast MBS session context is yet established in the gNB.

1.	The UE joins the multicast session.

NOTE 1:	NAS related details for PDU Session signalling are not shown in Figure 8.15.1.2-1.

2/3. The gNB-CU-CP establishes the multicast bearer context at the gNB-CU-UP, in order to retrieve for unicast NG-U transport the GTP DL TEID, a shared resource address (GTP DL TEID).

4/5. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast transport, DL/UL GTP TEIDs are exchanged, for multicast transport, multicast address information is fetched from the 5GC. Multicast session QoS parameters are provided by 5GC.

5a/5b. The gNB-CU-CP triggers the setup of MRB resources via the E1AP MC Bearer Context Modification procedure and provides the 5GC UL TEID and Multicast session QoS parameters to the gNB-CU-UP.

6/7.	The gNB-CU-CP establishes the Multicast Context at the DU, providing MRB configuration. It may contain MBS Area Session ID information.

8/9. The gNB-CU-CP retrieves the MRB configuration for the joined UE from the gNB-DU via F1 UE Context Management procedures.

NOTE 2:	The interactions between the gNB-CU-CP and gNB-CU-UP for UE specific Berarer Context management before and after step 8/9 are not illustrated in this Figure.

10.	The gNB-DU triggers the establishment of F1-U resources, which are established either per DU or per cell or per MBS Area Session ID. The receiving gNB-CU-CP fetches a gNB side F1-U TNL address information from the gNB-CU-UP by means of a E1AP MC Bearer Context Modification procedure.

11/12. The gNB-CU-UP side of the F1-U and NG-U UP entity is established by means of the E1AP MC Bearer Context Modification procedure, providing the DU side F1-U TNL address and the 5GC NG-U TNL address to the gNB-CU-UP, which provides the gNB-CU-UP side F1-U TNL address in return.

13.	The gNB-CU-UP side F1-U TNL address is provided to the gNB-DU.

14.	In case of NG-U multicast transport, the gNB-CU-UP joins the NG-U multicast group.

15.	The gNB successfully terminates the NGAP procedure for establishing the multicast session context.

16.	The gNB-CU-CP RRC-configures each UE which has joined the multicast group.

17.	The multicast MBS media stream is provided to the UEs.

On NG-U, in case of location dependent multicast MBS Sessions, multiple shared NG-U transport tunnels may need to be setup, one per Area Session ID served by the gNB.

In case of shared NG-U termination,

-	the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs.

-	at establishment of the MC bearer context in the gNB-CU-UP, the gNB-CU- CP may request the gNB-CU-UP to either apply the available MRB configuration of the shared NG-U termination, or to apply the MRB configuration requested by the gNB-CU-CP. The gNB-CU-UP provides the MRB configuration to the gNB-CU-CP if the MRB configuration requested by the gNB-CU-CP and the available MRB configuration of the shared NG-U termination are different.

-----------Next Change----------

#### 8.15.1.X	Handling of Multicast Reception in RRC_INACTIVE state

A gNB-CU-CP may decide to move a UE which has joined an active MBS session toward RRC_INACTIVE state, and configure the UE to receive MBS packets after this state transition. Figure 8.15.1.2-1 applies for this scenario with following exceptions:

-	In Step 6, the gNB-CU-CP includes the Multicast CU to DU RRC Information IE in the F1AP Multicast Context Setup Request message. The gNB-DU then generates the MCCH message accordingly, delivers it over Uu interface, and shall not suspend the PTM delivery as long as the information provided in the Multicast CU to DU RRC Information IE is valid and the multicast session is activated.

-	Step 8 and 9 (F1AP UE context modification procedure) is not performed.

-	Instead of Step 16, an RRC Release procedure is performed between the gNB-CU and the UE. The downlink RRCRelease message may be carried either by an F1AP UE Context Release Command message (which means that an F1AP UE context release procedure is performed) or an F1AP DL RRC message transfer message, depending on the initial state of the UE.

- Annex A: TP for NGAP

-----------Start of the Change----------

#### 9.2.4.2	MULTICAST GROUP PAGING

This message is sent by the AMF and is used to notify involved UEs about the activation of a multicast MBS session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | ignore |
| MBS Service Area | O |  | 9.3.1.208 |  | YES | ignore |
| Multicast Group Paging Area List |  | 1 |  |  | YES | ignore |
| >Multicast Group Paging Area Item |  | 1..<maxnoofPagingAreas> |  |  | - |  |
| >>Multicast Group Paging Area | M |  | 9.3.1.216 |  | - |  |
| >>UE Paging List |  | 0..1 |  |  | - |  |
| >>>UE Paging Item |  | 1..<maxnoofUEsforPaging> |  |  | - |  |
| >>>>UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| >>>>Paging DRX | O |  | 9.3.1.90 |  | - |  |
| MBS Session Activation Indicator | O |  | 9.3.1.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingAreas | Maximum no. of paging areas for multicast group paging. Value is 64. |
| maxnoofUEsforPaging | Maximum no. of UEs allowed within one paging area for multicast group paging. Value is 4096. |

-----------Next Change----------

#### 9.3.1.211	MBS Session Setup Request List

This IE provides information related to MBS sessions joined by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Setup Request List |  | 1 |  |  | - |  |
| >MBS Session Setup Request Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >>MBS Session ID | M |  | 9.3.1.206 |  | - |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  | - |  |
| >>Associated MBS QoS Flow Setup Request List |  | 0..1 |  |  | - |  |
| >>>Associated MBS QoS Flow Setup Request Item |  | 1..<maxnoofMBSQoSflows> |  |  | - |  |
| >>>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>>>Associated Unicast QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>MBS Assistance Information [FFS] | O |  | 9.3.1.Y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS sessions allowed within one PDU session. Value is 32. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is 64. |

#### 9.3.1.212	MBS Session Setup or Modify Request List

This IE provides information related to MBS sessions joined by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Setup or Modify Request List |  | 1 |  |  | - |  |
| >MBS Session Setup or Modify Request Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >>MBS Session ID | M |  | 9.3.1.206 |  | - |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  | - |  |
| >>Associated MBS QoS Flow Setup or Modify Request List |  | 0..1 |  |  | - |  |
| >>>Associated MBS QoS Flow Setup or Modify Request Item |  | 1..<maxnoofMBSQoSflows> |  |  | - |  |
| >>>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>>>Associated Unicast QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>MBS QoS Flow To Release List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows to be released. | - |  |
| >>MBS Assistance Information [FFS] | O |  | 9.3.1.Y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS sessions allowed within one PDU session. Value is 32. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is 64. |

-----------Next Change----------

#### 9.3.1.X	MBS Session Activation Indicator

This IE indicates the MBS Session Activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session Activation Indicator | M |  | ENUMERATED (true, …) |  |

#### 9.3.1.Y	MBS Assistant Information

This IE is used by NG-RAN to be aware which UE is preferred to be kept in RRC Connected state, as UE level additional information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  | ENUMERATED (true, …) |  |

Editor’s Note: FFS on the name and semantic description.

-----------Next Change----------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

////////////////////////////////No change/////////////////////////////////

MaskedIMEISV,

MBS-AreaSessionID,

MBS-ServiceArea,

MBS-SessionActivationIndicator,

MBS-SessionID,

MBS-DistributionReleaseRequestTransfer,

MBS-DistributionSetupRequestTransfer,

MBS-DistributionSetupResponseTransfer,

MBS-DistributionSetupUnsuccessfulTransfer,

MBSSessionReleaseResponseTransfer,

MBSSessionSetupOrModFailureTransfer,

MBSSessionSetupOrModRequestTransfer,

MBSSessionSetupOrModResponseTransfer,

MessageIdentifier,

MDTPLMNList,

MDTPLMNModificationList,

MobilityRestrictionList,

MulticastSessionActivationRequestTransfer,

MulticastSessionDeactivationRequestTransfer,

MulticastSessionUpdateRequestTransfer,

MulticastGroupPagingAreaList,

////////////////////////////////No change/////////////////////////////////

id-ManagementBasedMDTPLMNList,

id-ManagementBasedMDTPLMNModificationList,

id-MaskedIMEISV,

id-MBS-AreaSessionID,

id-MBS-ServiceArea,

id-MBS-SessionActivationIndicator,

id-MBS-SessionID,

id-MBS-DistributionReleaseRequestTransfer,

id-MBS-DistributionSetupRequestTransfer,

id-MBS-DistributionSetupResponseTransfer,

id-MBS-DistributionSetupUnsuccessfulTransfer,

id-MBSSessionModificationFailureTransfer,

id-MBSSessionModificationRequestTransfer,

id-MBSSessionModificationResponseTransfer,

id-MBSSessionReleaseResponseTransfer,

id-MBSSessionSetupFailureTransfer,

id-MBSSessionSetupRequestTransfer,

id-MBSSessionSetupResponseTransfer,

id-MessageIdentifier,

id-MobilityRestrictionList,

id-MulticastSessionActivationRequestTransfer,

id-MulticastSessionDeactivationRequestTransfer,

id-MulticastSessionUpdateRequestTransfer,

id-MulticastGroupPagingAreaList,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- MULTICAST GROUP PAGING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST GROUP PAGING

--

-- **************************************************************

MulticastGroupPaging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastGroupPagingIEs} },

...

}

MulticastGroupPagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY ignore	TYPE MBS-SessionID							PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY ignore	TYPE MBS-ServiceArea						PRESENCE optional		}|

{ ID id-MulticastGroupPagingAreaList			CRITICALITY ignore	TYPE MulticastGroupPagingAreaList		PRESENCE mandatory	}|

{ ID id-MBS-SessionActivationIndicator			CRITICALITY ignore	TYPE MBS-SessionActivationIndicator	PRESENCE optional		},

...

}

////////////////////////////////No change/////////////////////////////////

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

////////////////////////////////No change/////////////////////////////////

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBS-AssistanceInformation,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

////////////////////////////////No change/////////////////////////////////

-- M

////////////////////////////////No change/////////////////////////////////

MaskedIMEISV ::= BIT STRING (SIZE(64))

MaximumDataBurstVolume ::= INTEGER (0..4095, ..., 4096.. 2000000)

MessageIdentifier ::= BIT STRING (SIZE(16))

MaximumIntegrityProtectedDataRate ::= ENUMERATED {

bitrate64kbs,

maximum-UE-rate,

...

}

MBS-AreaSessionID ::= INTEGER (0..65535, ...)

MBS-AssistanceInformation ::= ENUMERATED {

true,

...

}

////////////////////////////////No change/////////////////////////////////

MBS-SessionActivationIndicator ::= ENUMERATED {

true,

...

}

////////////////////////////////No change/////////////////////////////////

MBSSessionSetupRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetupRequestItem

MBSSessionSetupRequestItem ::= SEQUENCE {

mBS-SessionID									MBS-SessionID,

mBS-AreaSessionID								MBS-AreaSessionID																	OPTIONAL,

associatedMBSQosFlowSetupRequestList			AssociatedMBSQosFlowSetupRequestList												OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { MBSSessionSetupRequestItem-ExtIEs} }		OPTIONAL,

...

}

MBSSessionSetupRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation	CRITICALITY ignore	EXTENSION MBS-AssistanceInformation	PRESENCE optional	},

...

}

MBSSessionSetuporModifyRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetuporModifyRequestItem

MBSSessionSetuporModifyRequestItem ::= SEQUENCE {

mBS-SessionID										MBS-SessionID,

mBS-AreaSessionID									MBS-AreaSessionID									OPTIONAL,

associatedMBSQosFlowSetuporModifyRequestList		AssociatedMBSQosFlowSetuporModifyRequestList		OPTIONAL,

mBS-QosFlowToReleaseList 							QosFlowListWithCause								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{MBSSessionSetuporModifyRequestItem-ExtIEs}}			OPTIONAL,

...

}

MBSSessionSetuporModifyRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation	CRITICALITY ignore	EXTENSION MBS-AssistanceInformation	PRESENCE optional	},

...

}

-----------Next Change----------

### 9.4.7	Constant Definitions

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-MBS-SessionActivationIndicator						ProtocolIE-ID ::=

id-MBS-AssistanceInformation							ProtocolIE-ID ::=

END

-- ASN1STOP

-----------End of the Change--------

- Annex B: TP for XnAP

-----------Start of the Change----------

#### 9.1.1.15	RAN MULTICAST GROUP PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page UEs for a multicast session.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| MBS Session ID | M |  | 9.2.3.146 |  | YES | reject |
| UE Identity Index List |  | 1 |  |  | YES | reject |
| >UE Identity Index Item |  | 1 .. <maxnoofUEIDOindicesforMBSPaging> |  |  | – |  |
| >>CHOICE UE Identity Index Value | M |  |  |  | – |  |
| >>>Length-10 |  |  |  |  |  |  |
| >>>>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33]. | – |  |
| >>Paging DRX | O |  | UE Specific DRX 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 38.304 [33]. | – |  |
| Multicast RAN Paging Area | M |  | RAN Paging Area 9.2.3.38 |  | YES | reject |
| MBS Session Activation Indicator | O |  | 9.2.3.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDIndicesforMBSPaging | Maximum no. of UE Identity Indices for multicast group paging. Value is 4096. |

-----------Next Change----------

#### 9.2.1.36	MBS Session Information List

This IE contains NG-RAN MBS session resource context related information used at UE context transfer between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Information Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >MBS Session ID | M |  | 9.2.3.146 |  | - |  |
| >MBS Area Session ID | O |  | 9.2.3.148 | MBS Area Session ID of the UE at the NG-RAN node from which the UE context is transferred | - |  |
| >Active MBS Session Information | O |  |  |  | - |  |
| >>MBS QoS Flows to Add List |  | 1..<maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.2.3.10 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | - |  |
| >>MBS Service Area | O |  | 9.2.3.150 |  | - |  |
| >>MBS Mapping and Data Forwarding Request Info from source NG-RAN node | O |  | 9.2.1.39 |  | - |  |
| >MBS Assistance Information (FFS) | O |  | 9.2.3.Y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS Sessions. Value is 256. |
| maxnoofMBSQoSFlows | Maximum no. of QoS flows allowed within one MBS session. Value is 64. |

-----------Next Change----------

#### 9.2.3.X	MBS Session Activation Indicator

This IE indicates the MBS Session Activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session Activation Indicator | M |  | ENUMERATED (true, …) |  |

#### 9.2.3.Y	MBS Assistance Information

This IE is used by NG-RAN to be aware which UE is preferred to be kept in RRC Connected state, as UE level additional information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  | ENUMERATED (true, …) |  |

Editor’s Note: FFS on the name and semantic description.

-----------Next Change----------

9.3.4	PDU Definitions

////////////////////////////////No change/////////////////////////////////

PEIPSassistanceInformation,

UESliceMaximumBitRateList,

PagingCause,

MDTPLMNModificationList,

F1-terminatingIAB-donorIndicator,

SRB-ID,

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

MBS-SessionActivationIndicator

////////////////////////////////No change/////////////////////////////////

id-SDTPartialUEContextInfo,

id-SDTDataForwardingDRBList,

id-PEIPSassistanceInformation,

id-UESliceMaximumBitRateList,

id-S-NG-RANnodeUE-Slice-MBR,

id-ManagementBasedMDTPLMNModificationList,

id-F1-terminatingIAB-donorIndicator,

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated,

id-MBS-SessionActivationIndicator,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- RAN MULTICAST GROUP PAGING

--

-- **************************************************************

RANMulticastGroupPaging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{RANMulticastGroupPaging-IEs}},

...

}

RANMulticastGroupPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-MBS-Session-ID							CRITICALITY reject	TYPE MBS-Session-ID									PRESENCE mandatory}|

{ ID id-UEIdentityIndexList-MBSGroupPaging			CRITICALITY reject	TYPE UEIdentityIndexList-MBSGroupPaging		PRESENCE mandatory}|

{ ID id-MulticastRANPagingArea					CRITICALITY reject	TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-MBS-SessionActivationIndicator			CRITICALITY ignore	TYPE MBS-SessionActivationIndicator			PRESENCE optional },

...

}

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

////////////////////////////////No change/////////////////////////////////

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-MBS-AssistanceInformation,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

////////////////////////////////No change/////////////////////////////////

#### -- M

////////////////////////////////No change/////////////////////////////////

MBS-Area-Session-ID  ::= INTEGER (0..65535, ...)

MBS-AssistanceInformation ::= ENUMERATED {

true,

...

}

////////////////////////////////No change/////////////////////////////////

MBS-SessionActivationIndicator ::= ENUMERATED {

true,

...

}

////////////////////////////////No change/////////////////////////////////

MBS-SessionInformation-List ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBS-SessionInformation-Item

MBS-SessionInformation-Item ::= SEQUENCE {

mBS-Session-ID			MBS-Session-ID,

mBS-Area-Session-ID		MBS-Area-Session-ID														OPTIONAL,

active-MBS-SessioInformation			Active-MBS-SessionInformation							OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { MBS-SessionInformation-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-SessionInformation-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation		CRITICALITY ignore	EXTENSION MBS-SessionInformation			PRESENCE optional},

...

}

-----------Next Change----------

### 9.3.7	Constant definitions

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-MBS-SessionActivationIndicator																	ProtocolIE-ID ::=

id-MBS-AssistanceInformation																		ProtocolIE-ID ::=

END

-- ASN1STOP

-----------End of the Change--------

- Annex C: TP for F1AP

-----------Start of the Change----------

#### 9.2.14.1	MULTICAST GROUP PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to multicast group page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| UE Identity List for Paging |  | 0..1 |  |  | YES | ignore |
| >UE Identity for Paging Item |  | 1..<maxnoofUEIDforPaging> |  |  | - |  |
| >>UE Identity Index value | M |  | 9.3.1.39 |  | - |  |
| >>Paging DRX | O |  | 9.3.1.40 |  | - |  |
| Paging Cell List |  | 0..1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| MBS Session Activation Indicator | O |  | 9.3.1.X |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDforPaging | Maximum no. of UE ID for multicast group paging. Value is 4096. |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

#### 9.2.14.2	MULTICAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a multicast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Multicast MRBs To Be Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRBs to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast CU to DU RRC Information | O |  | 9.3.1.Z |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

-----------Next Change----------

#### 9.2.14.7	MULTICAST CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to modify multicast context information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| Multicast MRB To Be Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast MRB To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | O |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 0 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | O |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast MRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast CU to DU RRC Information | O |  | 9.3.1.Z |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

-----------Next Change----------

#### 9.3.1.X	MBS Session Activation Indicator

This IE indicates the MBS Session Activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session Activation Indicator | M |  | ENUMERATED (true, …) |  |

#### 9.3.1.Z	Multicast CU to DU RRC Information

This IE indicates the Multicate CU to DU RRC Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MRB PDCP Config Multicast | O |  | OCTET STRING | FFS |
| FFS |  |  |  |  |

Editor’s Note: It is pending RAN2 whether more RRC information should be contained.

-----------Next Change----------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

////////////////////////////////No change/////////////////////////////////

MBS-Area-Session-ID,

MBS-CUtoDURRCInformation,

MBSMulticastF1UContextDescriptor,

MBS-SessionActivationIndicator,

MBS-Session-ID,

MBS-ServiceArea,

Multicast-CUtoDURRCInformation,

MulticastF1UContextReferenceCU,

MulticastF1UContext-ToBeSetup-Item,

////////////////////////////////No change/////////////////////////////////

FROM F1AP-Containers

////////////////////////////////No change/////////////////////////////////

id-MBS-Area-Session-ID,

id-MBS-CUtoDURRCInformation,

id-MBS-SessionActivationIndicator,

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-Multicast-CUtoDURRCInformation,

id-MulticastF1UContextReferenceCU,

id-MulticastMBSSessionSetupList,

id-MulticastMBSSessionRemoveList,

id-MulticastMRBs-FailedToBeModified-List,

id-MulticastMRBs-FailedToBeModified-Item,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- Multicast Group Paging PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Multicast Group Paging

--

-- **************************************************************

MulticastGroupPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastGroupPagingIEs}},

...

}

MulticastGroupPagingIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MBS-Session-ID					CRITICALITY reject	TYPE MBS-Session-ID							PRESENCE mandatory	}|

{ ID id-UEIdentity-List-For-Paging-List	CRITICALITY ignore	TYPE UEIdentity-List-For-Paging-List		PRESENCE optional	}|

{ ID id-PagingCell-List					CRITICALITY ignore	TYPE PagingCell-list						PRESENCE optional	}|

{ ID id-MBS-SessionActivationIndicator	CRITICALITY ignore	TYPE MBS-SessionActivationIndicator		PRESENCE optional	},

...

}

UEIdentity-List-For-Paging-List	 ::= SEQUENCE (SIZE(1.. maxnoofUEIDforPaging)) OF ProtocolIE-SingleContainer { { UEIdentity-List-For-Paging-ItemIEs } }

UEIdentity-List-For-Paging-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentity-List-For-Paging-Item	CRITICALITY ignore	TYPE UEIdentity-List-For-Paging-Item 			PRESENCE optional }	,

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP REQUEST

--

-- **************************************************************

MulticastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupRequestIEs}},

...

}

MulticastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory  }|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID						PRESENCE mandatory  }|

{ ID id-MBS-ServiceArea					CRITICALITY reject TYPE	MBS-ServiceArea							PRESENCE optional   }|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI								PRESENCE mandatory  }|

{ ID id-MulticastMRBs-ToBeSetup-List	CRITICALITY reject	TYPE	MulticastMRBs-ToBeSetup-List		PRESENCE mandatory  }|

{ ID id-Multicast-CUtoDURRCInformation	CRITICALITY reject 	TYPE	Multicast-CUtoDURRCInformation	PRESENCE optional   },

...

}

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- MULTICAST CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

MulticastContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationRequestIEs}},

...

}

MulticastContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea						PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeSetupMod-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeSetupMod-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeModified-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeModified-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeReleased-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeReleased-List	PRESENCE optional  }|

{ ID id-Multicast-CUtoDURRCInformation		CRITICALITY reject	TYPE Multicast-CUtoDURRCInformation		PRESENCE optional  },

...

}

### 9.4.5	Information Element Definitions

////////////////////////////////No change/////////////////////////////////

#### -- M

////////////////////////////////No change/////////////////////////////////

MBS-CUtoDURRCInformation		::= SEQUENCE {

mBS-Broadcast-Cell-List		MBS-Broadcast-Cell-List,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-CUtoDURRCInformation-ExtIEs } } OPTIONAL,

...

}

MBS-CUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Broadcast-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Broadcast-Cell-Item

MBS-Broadcast-Cell-Item ::= SEQUENCE {

nRCGI						NRCGI,

mtch-neighbourCell			OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-Broadcast-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Broadcast-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastF1UContextDescriptor ::= SEQUENCE {

multicastF1UContextReferenceF1		MulticastF1UContextReferenceF1,

mc-F1UCtxtusage 	ENUMERATED {ptm, ptp, ptp-retransmission, ptp-forwarding, ...},

mbsAreaSession						MBS-Area-Session-ID				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer	{{MBSMulticastF1UContextDescriptor-ExtIEs}} OPTIONAL,

...

}

MBSMulticastF1UContextDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Multicast-CUtoDURRCInformation		::= SEQUENCE {

mRB-PDCP-Config-Multicast	OCTET STRING,

FFS

iE-Extensions				ProtocolExtensionContainer { { Multicast-CUtoDURRCInformation-ExtIEs } } OPTIONAL,

...

}

Multicast-CUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContext-ToBeSetup-Item ::= SEQUENCE {

mRB-ID                  MRB-ID,

mbs-f1u-info-at-DU      UPTransportLayerInformation,

mbsProgressInformation		MRB-ProgressInformation						OPTIONAL,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-ToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

-----------Next Change----------

MBS-SessionActivationIndicator ::= ENUMERATED {

true,

...

}

-----------Next Change----------

### 9.4.7	Constant Definitions

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-MBS-SessionActivationIndicator					ProtocolIE-ID ::=

id-Multicast-CUtoDURRCInformation					ProtocolIE-ID ::=

END

-- ASN1STOP

-----------End of the Change----------