3GPP TSG-RAN WG3 #119bis-e	R3-231465

Online, 17th – 26th April 2023

Source:	CATT

Title:	Discussion on partial success and validity time

Agenda Item:	12.2.2.1

Document for:	Discussion and decision

- Introduction

This discussion paper focuses on validity time and accuracy of prediction of load balancing, although it is also applicable for quantised predictions in other use cases.

- Discussion

  - Partial success

How to support partial success was one key issue discussed last meeting. There were two major aspects on this issue:

- Whether to introduce a “partial success allowed” indicator in the request message;

- How to indicate a partial success in the response message.

For the first aspect, our preference is “no”.

Typically AI/ML models require “enough” inputs to generate outputs, but the phrase “enough” does not mean all information requested should be provided. For example one can imagine that 5 types of information, named A, B, C, D and E for convenience here, may contribute as inputs of an AI/ML model. The best scenario is that all of the 5 types of information are available, but only A is mandatory. The model can still work if it is provided only one type between B and C, and only one type of D and E simultaneously. The model can hardly work if only A, B and C is available. Obviously, we should never introduce some complex indicator into RAN3 specs that “this request should be accepted only if A, and at least one of B and C, and at least one of D and E is available”.

Proposal 1: Do not introduce any “partial success allowed” indicator in the request message as it cannot cover some typical cases. It is sufficient enough to rely on the method that the requesting node can cancel its request if it finds the accepted items are too few to use.

For the second aspect, there are two major flavours:

- Sticking to the Resource-Status-Reporting method that each requested characteristic is represented by one bit in a bit map, and partial success is indicated by at least one bit map in the response messages.

- Using list structure instead, common in procedures other than resource status reporting initiation, with each failure item including a report characteristic and a failure cause.

We prefer to the former flavour as it seems to be accepted by more companies. In order to indicate failure causes for each characteristic, a list of cause can be introduced. (We don’t propose further introducing per-cell lists now as it is too complex.)

Proposal 2: Introducing a list into the response message (and the failure message) with each item including a cause value and a bit map of failed characteristics.

  - Validity time of prediction

In the current BL CR, the only IE related to time is the (report) periodicity copied from the Resource Status Initiation procedure. Naturally all of the metrics in the Resource Status Update are periodical ones and thus both the (report) periodicity and the average window are necessary.

However, for AI/ML we may be one-shot request and feedback as well, as hinted in the current BL CR. For one-shot request the average window is still needed—if the request node wants a prediction for just 10s a prediction for 10min will not satisfy it. So there should be (at least) one time interval in the request message. We have to call it something other than a “periodicity”. Naming it “Time Window Length” may be a good option.

And for periodical one, the time length of averaging window and report periodicity can still be the same (as for resource status reports). So we may capture in the “Semantic Description” column that the “Time Window Length” may also be used as the periodicity if the request is not one-shot, e.g.:

| Time Window Length | O |  | ENUMERATED {500ms, 1s, 2s, 5s, 10s, ...} | Also used as the reporting periodicity for all periodical reporting. |
| --- | --- | --- | --- | --- |

Proposal 3: The existing “Reporting Periodicity” IE should be renamed into e.g. “Time Window Length” in order to cover one-shot report. Nevertheless, it is also used as the periodicity for periodical reporting.

Moreover, in SON load metrics are defined that the time duration is as long as the periodicity. Assume that the time to report is t0 and the periodicity is T, the content of it is typically the average measured between t0-T and t0. The next time to report is t0+T and its content is the average between t0 and t0+T.

Now in AI/ML we are to introduce predictions, which talk about the future rather than the past. The most straightforward method could be a mirror image of load reports in SON, i.e.:

- The prediction provided at t0 is (ideally) the average between t0 and t0+T;

- The prediction provided at t0+T is (ideally) the average between t0+T and t0+2T.

However such straightforward method may not be sufficient enough.

RAN nodes generally wish any decision is suitable as long as possible, e.g. it should not be changed or rolled back within ½T. On the other hand, RAN nodes may take RRM decision at any moment, e.g. possibly at t0+¾T. If such case occurs, the RAN node will naturally wish it has got the prediction covering the duration between t0+¾T and t0+1¼T. But according to the method above, it has not. What it got is only for the duration between t0 and t0+T, received at t0.

Maybe a better approach is that, the node making such prediction provides at t0 not only the one for the duration between t0 and t0+T, but also the one for the duration between t0+T and t0+2T.

Proposal 4: Assume that the time to report prediction is t0 and the periodicity is T, in addition to the one for t0 till t0+T, the one for t0+T till t0+2T or even further is also provided if requested in the initial procedure.

  - Accuracy and confidence of prediction

Load balancing predictions (including the ones agreed last meeting and the ones tagged FFS) are all quantised ones. Theoretically the most accurate expression of accuracy/confidence of quantised predictions should be probability distributions (this is indeed available if one runs multiple models for a lot of times), but maybe we includes only the accuracy part (maybe more precisely, uncertainty) is sufficient enough.

Accuracies are ideally not constants. It can be drastically different depending on situations.

In weather forecast the accuracy can be usually fairly good, but it may not be that good if a storm, especially a tropical cyclone, is nearby. For the latter case meteorologists may run multiple models (typically in super computers) and observes that the predicted trajectories of this storm are different one another, and as one result, the predicted weather of a city are totally different.

Here in RAN AI the case can be similar. A smart AI may predict the load or energy saving metrics quite well in usual case, but if the case is not usual (e.g. holiday, demonstration) there can be much uncertainty.

So in principle, it should be supported that the prediction is provided with an accuracy IE.

Proposal 5: Every quantised prediction can be accompanied with an accuracy IE.

- Conclusion

Proposal 1: Do not introduce any “partial success allowed” indicator in the request message as it cannot cover some typical cases. It is sufficient enough to rely on the method that the requesting node can cancel its request if it finds the accepted items are too few to use.

Proposal 2: Introducing a list into the response message (and the failure message) with each item including a cause value and a bit map of failed characteristics.

Proposal 3: The existing “Reporting Periodicity” IE should be renamed into e.g. “Time Window Length” in order to cover one-shot report. Nevertheless, it is also used as the periodicity for periodical reporting.

Proposal 4: Assume that the time to report prediction is t0 and the periodicity is T, Either of the following two bullets should be supported:
-	In addition to the one for t0 till t0+T, the one for t0+T till t0+2T or even further is also provided;
-	The validity duration is longer than the period, e.g. the prediction is for t0 till t0+2T.

Proposal 5: Every quantised prediction can be accompanied with an accuracy IE.

Based on the proposal, we draft a TP on XnAP [1].

- Reference

[1] R3-231469; (TP for 38.423) Updates on non-UE associated messages to support AI/ML; CATT.