3GPP TSG-RAN WG3 #119bis-e 					                                                      R3-231476

17th – 26th April 2023

Online

Agenda item:	17.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the time-based trigger condition in NR NTN

Document for:	Discussion & decision

# 1 Introduction

RAN3#119 meeting agreed:

Introducing time-based parameters for NG HO follows legacy CHO configuration over Uu interface without any RAN2 impact.

This contribution discusses the RAN3 related aspects on the support of the Time-based Trigger Condition in NG-HO.

# 2 time-based trigger condition in NG-HO

Using Time-based trigger condition has the benefit to prepare the handover in advance, and mitigate the burst signaling issue that many UEs may be handover to target gNB at the same time due to feeder link switch over. The required Xn CR has been agreed in previous RAN3 meeting. A high level call flow is shown below.

Figure 1: CHO via Xn-HO (TS38.300 Figure 9.2.3.4.2-1: Intra-AMF/UPF Conditional Handover)

An example high level NG-HO with time-based trigger condition procedure is shown in Figure 2, which is similar to Xn-CHO (Figure 1). The main difference is the HO signaling uses NGAP message instead of XnAP message.

Figure 2: NG-HO with support Time-based trigger condition

In Xn-CHO, the source gNB only know the UE may execute the HO during T1 and T1+duration, but the source gNB cannot know the exact time when UE execute the HO, i.e. detach from source cell and synchronize to target cell. The source gNB should start the data forwarding before (or at) T1. As long as the UE remains connected with source cell, source gNB can continue send DL data to UE. For the DL data forwarded to target gNB, source gNB inform target gNB to discard those DL data that already transmitted from source cell to UE. This is supported via source gNB initiated XnAP EARLY STATUS TRANSFER message including DL Discarding IE. So target gNB only buffer the DL packets that have not been transferred to the UE.  An example is shown in Annex A – Data Forwarding example in Xn-CHO.

Observation 1-1: in Xn-CHO, Target gNB only stores those DL packets that have not been successfully transmitted to UE based on the discard request from Source gNB.

When introducing the Time-based trigger condition in NG-HO, it should not change this principle, i.e. Target gNB only stores those DL packets that have not been successfully transmitted to UE based on the discard request from Source gNB. Otherwise, target gNB has to unnecessarily buffer those DL packets that already been transmitted from source gNB to UE. Current duration is up to 10-minute. A normal gNB may set the duration to a few seconds. The UE may actually execute the handover near the end of the HO window. In other words, target gNB may have to buffer up to 10-minute DL data (in the worst case), or at least a few seconds DL data (in normal case). This can cause following issues:

- Issue 1: Significant number of DL packets need to be buffered in target gNB.

In normal HO, the target gNB only need to buffer a very short period of DL data (e.g. a few ms DL data). In this case, the target gNB may have to buffer a few seconds DL data. This issue is even severe when the time-based trigger condition is used for the handover due to feeder link switch over that many UEs need to be handover to target gNB.

- Issue 2: unnecessary retransmission.

After the UE connect with target gNB, target gNB start to send the buffered DL packet to the UE. In this case, those unnecessary DL packets have already been successfully transmitted to the UE via the source gNB to the UE. However, the target gNB cannot know the DL packets that have been successfully transmitted via source gNB. Target gNB unnecessarily retransmitted those DL packets that have already been successfully received by the UE from source gNB.

- Issue 3: add unacceptable delay to the “new” DL packet

Target gNB send the DL packets in order, i.e. first transmit the buffered DL packet, then transmit the “new” packet. When the duration is a few seconds, target gNB first transmit those DL packets received from source gNB during this period, then transmit the “new” packet received from CN. Even in case UE have already successfully received those DL packets (occurred in this a few seconds period), the “new” packets is only sent later (i.e. after this period). This adds the unacceptable a few seconds delay to the “new” packet.

Observation 1-2: without enhancement to NGAP, the data forwarding used in current NG-HO causes significant number of packets to be buffered in Target gNB, unnecessary retransmission in target gNB, and unacceptable delay added to the “new” DL packet.

To avoid the target gNB to buffer those DL packets that have already been successfully transmitted from source gNB to UE, target gNB need to be informed for those DL data packets that have already been transmitted from source gNB to UE. To support this, a “discard” mechanism similar to the XnAP DL Discarding IE needed to be introduced in NGAP, e.g. UPLINK RAN EARLY STATUS TRANSFER message and DOWNLINK RAN EARLY STATUS TRANSFER message.

Proposal 1-1: introduce a new DL Discarding IE needed to be introduced in NGAP, e.g. UPLINK RAN EARLY STATUS TRANSFER message and DOWNLINK RAN EARLY STATUS TRANSFER message.

- Coordination with SA2

The N2-based handover procedure is specified in SA2 TS23.502 Section 4.9.1.3 “Inter NG-RAN node N2 based handover”, which includes following sub-sections  (TS23.502 section/figure is also copied at Annex B - TS23.502 N2-based HO)

- 4.9.1.3.2	Preparation phase

- 4.9.1.3.3	Execution phase

- 4.9.1.3.3a	Execution phase for DAPS handover

TS23.502 Section 4.9.1.3 need to be updated. For example,

- Preparation phase: add the description for CHO preparation (e.g. refer to the similar text defined in Xn-CHO).

- Execution phase: in normal handover execution procedure (TS23.502, 4.9.1.3.3), NGAP Uplink RAN Status Transfer and Downlink RAN Status Transfer procedures are used to support data forwarding.  NGAP Uplink RAN Early Status Transfer procedure and Downlink RAN Early Status Transfer procedure are only used for DAPS handover (TS23.502, 4.9.1.3.3a). So TS23.502 need to be updated to use NGAP Uplink RAN Early Status Transfer procedure and Downlink RAN Early Status Transfer procedure for normal HO with Time-based trigger condition.

It is up to SA2 to decide on how to capture it in TS23.502. A formal request needs to be sent to SA2 to check the SA2 feedback on how to support the Time-based trigger condition in NG-HO.

Observation 1-3: coordination with SA2 is required, e.g. to update TS23.502 to support Time-based trigger condition in N2-based HO.

Proposal 1-2: RAN3 send a LS to SA2 to check how to support Time-based trigger condition in N2-based HO.

# 3 Data Forwarding in Xn-CHO with time-based trigger condition

Xn-CHO is described in TS38.300 (copied in below figure). Early data forwarding (Step 7a) is started just after the source gNB receive the RRCSetupComplete message from the UE (Step 7). Since the UE may execute the handover just after the UE send the RRCSetupComplete message, so the source gNB should start the early data forwarding as soon as source gNB receives the RRCSetupComplete message.

-- TS38.300

Figure 9.2.3.4.2-1: Intra-AMF/UPF Conditional Handover

7.	The UE sends an RRCReconfigurationComplete message to the source gNB.

7a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

8.	The UE maintains connection with the source gNB after receiving CHO configuration, and starts evaluating the CHO execution conditions for the candidate cell(s). ...

--

In case Xn-CHO with time-based trigger condition, the UE remains connected at least up to T1. The UE only execute the handover after T1. This means the source gNB can continue the UL/DL with the UE at least up to T1. If Early data forwarding is applied, the source gNB should only need to start the data forwarding at T1. This is different to current data forwarding in Xn-CHO. We propose to capture this behaviour in Stage-2 TS38.300. For example,

--  TP for TS38.300

##### 9.2.3.4.4	Data Forwarding

If late data forwarding is applied, the source NG-RAN node initiates data forwarding once it knows which target NG-RAN node the UE has successfully accessed. In that case the behavior of the Conditional Handover data forwarding follows the same behavior as defined in 9.2.3.2.3 for the intra-system handover data forwarding, except the behavior for DRBs configured with DAPS handover.

If early data forwarding is applied instead, the source NG-RAN node initiates data forwarding before the UE executes the handover, to a candidate target node of interest. When time-based trigger condition is used, the source NG-RAN node should start the data forwarding at the time indicated to the UE. The behavior of early data forwarding for the Conditional Handover follows the same principles for DRBs configured with DAPS handover in the intra-system handover as defined in 9.2.3.2.3.

--

Proposal 2-1: Capture in TS38.300 that early data forwarding should be started at T1 in Xn-CHO with time-based trigger condition.

The TP proposal can be found in the Annex - Text Proposal for BL CR 36.300.

# 4 Conclusions

In this contribution, we analyzed the support for time-based trigger condition in N2-HO and CHO recovery. Our proposals are:

Observation 1-1: in Xn-CHO, Target gNB only stores those DL packets that have not been successfully transmitted to UE based on the discard request from Source gNB.

Observation 1-2: without enhancement to NGAP, the data forwarding used in current NG-HO causes significant number of packets to be buffered in Target gNB, unnecessary retransmission in target gNB, and unacceptable delay added to the “new” DL packet.

Proposal 1-1: introduce a new DL Discarding IE needed to be introduced in NGAP, e.g. UPLINK RAN EARLY STATUS TRANSFER message and DOWNLINK RAN EARLY STATUS TRANSFER message.

Observation 1-3: coordination with SA2 is required, e.g. to update TS23.502 to support Time-based trigger condition in N2-based HO.

Proposal 1-2: RAN3 send a LS to SA2 to check how to support Time-based trigger condition in N2-based HO.

Proposal 2-1: Capture in TS38.300 that early data forwarding should be started at T1 in Xn-CHO with time-based trigger condition.

The TP proposal can be found in the Annex - Text Proposal for BL CR 36.300.

# References

- R3-226859, Summary of Offline Discussion on CB: # 41_NRNTN

- TS38.300

- TS23.502

## Text Proposal for BL CR 36.300

##### 9.2.3.4.4	Data Forwarding

If late data forwarding is applied, the source NG-RAN node initiates data forwarding once it knows which target NG-RAN node the UE has successfully accessed. In that case the behavior of the Conditional Handover data forwarding follows the same behavior as defined in 9.2.3.2.3 for the intra-system handover data forwarding, except the behavior for DRBs configured with DAPS handover.

If early data forwarding is applied instead, the source NG-RAN node initiates data forwarding before the UE executes the handover, to a candidate target node of interest. When time-based trigger condition is used, the source NG-RAN node should start the data forwarding at the time indicated to the UE. The behavior of early data forwarding for the Conditional Handover follows the same principles for DRBs configured with DAPS handover in the intra-system handover as defined in 9.2.3.2.3.

## Annex A – Data Forwarding example in Xn-CHO

Figure 3 shows an example for data forwarding in current Xn Conditional HO.

Figure 3: Example for data forwarding in Xn-CHO

- At T, S-gNB send the HOCommand to the UE. S-gNB starts the data forwarding, i.e. packet #3 to T-gNB.

- At T+1, UE remains connected with S-gNB. S-gNB send #3 to UE. S-gNB continues data forwarding, i.e. packet #4 to T-gNB.

- At T+2, UE remains connected with S-gNB. S-gNB send #4 to UE. S-gNB continues data forwarding, i.e. packet #5 to T-gNB. Since S-gNB has successfully transmitted packet #3 to UE, so S-gNB informs T-gNB to discard #3.  after T+2, T-gNB only stores packet #4 and #5.

- At T+3, UE detach from source gNB. S-gNB continue the data forwarding, i.e. #6 to T-gNB. Since S-gNB has successfully transmitted packet #4 to UE, so S-gNB informs T-gNB to discard #4.    after T+3, T-gNB only stores packet #5 and #6.

- At T+4, UE connect with T-gNB. T-gNB starts sending the buffered data packet #5 to UE.

## Annex B - TS23.502 N2-based HO

#### 4.9.1.3	Inter NG-RAN node N2 based handover

##### 4.9.1.3.2	Preparation phase

Figure 4.9.1.3.2-1: Inter NG-RAN node N2 based handover, Preparation phase

##### 4.9.1.3.3	Execution phase

Figure 4.9.1.3.3-1: inter NG-RAN node N2 based handover, execution phase

##### 4.9.1.3.3a	Execution phase for DAPS handover

This procedure applies only if at the end of the Preparation phase it has been determined that at least one DRB of the UE is subject to a DAPS related Handover.

Figure 4.9.1.3.3a-1: inter NG-RAN node N2 based DAPS handover, execution phase