3GPP TSG-RAN3 Meeting #119bis-e	R3-231518

# Online, 17 April - 26 April 2023

Agenda item:	11.2

Source: 	Xiaomi

Title: 	QoE configuration in RRC_INACTIVE/RRC_IDLE states

Document for:	Discussion & decision

Introduction

In this contribution, we’d like to discuss QoE measurement configuration and reporting in RRC_INACTIVE/RRC_IDLE state based on the following progress in last RAN3 meeting.

RAN3#119:

The RVQoE discussion on supporting AR/MR and MBS specific metrics may only be triggered based on progress of SA4.

The RRC state info when UE collects the uploaded QoE data shall not be reported in QoE report for MBS BC. MBS MC can be discussed later.

MBS BC QoE measurements can proceed after the UE switches from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED.

RAN3 to discuss which configuration information related to QoE measurement needs to be available in the new gNB.

At least the following QoE configuration related information for MBS broadcast service should be available in the new gNB:

- QoE reference

- Measurement Collection Entity Information, the detail information can be further discussed

RAN3 shall discuss which of other QoE configuration info for MBS BC QoE shall be available in the new gNB.

Measurement Configuration Application Layer ID (RRC level ID)

Service Type

Container for Application Layer Measurement Configuration (config container)

MDT Alignment Information

Area Scope of QMC (area scope)

S-NSSAI Information (slicing info)

RVQoE Information

QoE measurement type (signalling based, management based)

FFS whether to support RVQoE measurements in RRC_IDLE and RRC_INACTIVE

FFS whether new gNB can re-configure MBS BC QoE

Discussion

Leftover issues on the information needed for a new gNB

In last RAN3 meeting, it was agreed that MBS BC QoE measurements can proceed after the UE switches from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED. Regarding which configuration information related to QoE measurement needs to be available in the new gNB, the QoE reference and MCE related information is already agreed. But companies have the different views on other information since they’re based on different solutions, and we think before we had further discussion on the information and solution, we need to figure out why the information is needed for the new gNB, so we’d like to discuss what controllability that the new gNB needs to have on the already configured QMC in UE in case the UE switches from RRC_IDLE to RRC_CONNECTED.

In our understanding, the following controllability for a new gNB can be discussed：

| No. | The capability if the new gNB on QMC | The information needed for UE-based solution | The information needed for CN-based solution |
| --- | --- | --- | --- |
| 1 | The new gNB can support the transfer of the QoE report buffered in UE side when UE is in RRC_IDLE/RRC_INACTIVE state | MCE ID | MCE address |
| 2 | The new gNB can send new QoE configuration to UE, but need to make sure the m-based QoE will not override the already configured s-based QoE | QoE Reference, service type, measurement type | QoE Reference, service type, measurement type |
| 3 | new gNB can release the QoE configuration previously configured in UE, e.g. according to the scope information | Area scope and slice scope | Area scope and slice scope |
| 4 | If 3 is support, new gNB can configure the QoE configuration again if UE is in scope again | Area scope and slice scope, QoE container | Area scope and slice scope, QoE container |

Controllability 1 and 2 are already reached consensus in the previous meeting. Controllability 3 and 4 will bring more spec impacts and complexity, so in our view, we can only focus on Controllability 1 and 2, to make it simple.

But if we don’t support controllability 3 and 4, then we need to discuss how to release the already configured QoE in UE.

Proposal 1, RAN3 agree to discuss which information is needed for the new gNB based on the following purposes:

- new gNB of the UE can support the transfer of the QoE report buffered in UE side when UE is in RRC_IDLE/RRC_INACTIVE state.

- new gNB can send new QoE configuration to UE after switches from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED, and the new gNB needs to make sure the m-based QoE will not override the already configured s-based QoE.

Proposal 2, RAN3 to discuss how to release the already configured QoE in UE considering the following options:

- option 1, the release is controlled by network, i.e. for RRC_INACTIVE UE, the last serving gNB can release QoE configuration via existing paging mechanism; RRC_IDLE UE, new gNB can release QoE configuration when UE switches from RRC_IDLE to RRC_CONNECTED.

- option 2, similar to logged MDT, UE can release QoE configuration when it’s in RRC_IDLE/RRC_INACTIVE state.

Regarding the UE-based solution VS. network-based solution, it’s been discussed for several meetings, companies have different concerns, to progress, we’d like to discuss about the concerns and try to find the way forward.

|  | UE-based solution | CN-based solution |
| --- | --- | --- |
| Concerns | - Security issue | - SA2 spec impact - CN may not want store m-based QoE configuration |
| Analysis | we don’t see any security issue on UE-based solution, the transferred information is already configured in UE and it can be known by the network, besides, this is similar to logged MDT | CN-based solution will have SA2 impact, especially when the AMF connects to the new gNB is different from the one that stores the information. For s-based QoE configuration, it makes sense to store it in the CN, while for m-based QoE configuration, we think it does not make scene to store it in CN, CN may not want to know the m-based QoE configuration. |
| Way forward | Company that have concerns on the security issue should clarify what’s the security difference between MDT and QoE, then we may send LS to SA3 for further check. | If we really want to have CN-based solution, we need to check with SA2 and indicate the potential spec impacts. |

If it’s hard to down-select between UE-based solution and CN-based solution, we can also consider to have OAM-based solution, which may have minimal spec impact, if the new gNB can know the QoE reference, the new gNB can retrieve the corresponding QoE configuration from OAM by using QoE reference, OAM-based solution only need the UE to report the QoE reference.

Observation 1, the security issue for UE-based solution is not clear, considering logged MDT has similar mechanism without security issue.

Observation 2, the CN-based solution may have impact on stage 2 and stage 3 SA2 specifications.

Observation 3, it’s not appropriate to store the m-based configuration in CN, as CN may not want to know the configuration in RAN OAM.

Proposal 3, RAN3 can check with SA2 and SA3 on the above observations and potential spec impact and workload.

Observation 4, if it’s hard to down-select between UE-based solution and network-based solution, OAM-based solution can also be considered, which will have less spec impact than the other two.

RVQoE aspects for RRC_IDLE/RRC_INACTVE UEs

When talking about RVQoE for RRC_IDLE/RRC_INACTVE UEs, we think two aspects can be further discussed:

- 1. Whether RVQoE measurement will be performed when UE is in RRC_IDLE/RRC_INACTVE UEs

- 2. Whether RVQoE configuration can be re-configured when UE switch from RRC_IDLE to RRC_CONNECTED state.

We think it would be beneficial to collect RVQoE measurement for MBS broadcast service, then gNB can optimize the radio configuration. Although it is now impossible to consider the RVQoE on supporting MBS specific metrics, which is not specified yet. But other RVQoE metrics like streaming service via broadcast communication can be considered.

For the first aspect, if RVQoE measurement is performed when UE is in RRC_IDLE/RRC_INACTVE state, the next step is to discuss how to report the RVQoE report. We think SDT can be considered, if SDT is not support, UE can report the buffered RVQoE report when it reconnected to the new gNB, and the new gNB can forward the RVQoE report to the corresponding gNBs that needs the RVQoE report.

Another aspect about RVQoE is about the re-configuration of RVQoE when UE switch from RRC_IDLE to RRC_CONNECTED state, the available RVQoE metrics should be provided to the new gNB.

Observation 5: It would be beneficial for gNB to collect RVQoE measurement for MBS broadcast service to have a better resource management.

Proposal 4: RAN3 agree RVQoE measurement collection can be supported in IDLE and Inactive state.

Proposal 5: if RAN3 agreed to support RVQoE measurement collection in RRC_IDLE and RRC_INACTIVE state, RAN3 discuss how to report the RVQoE measurement.

Proposal 6, RAN3 to discuss whether and how RVQoE configuration can be re-configured when UE switch from RRC_IDLE to RRC_CONNECTED state.

Conclusion

In this contribution, we had the following observations and proposals:

Proposal 1, RAN3 agree to discuss which information is needed for the new gNB based on the following purposes:

- new gNB of the UE can support the transfer of the QoE report buffered in UE side when UE is in RRC_IDLE/RRC_INACTIVE state.

- new gNB can send new QoE configuration to UE after switches from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED, and the new gNB needs to make sure the m-based QoE will not override the already configured s-based QoE.

Proposal 2, RAN3 to discuss how to release the already configured QoE in UE considering the following options:

- option 1, the release is controlled by network, i.e. for RRC_INACTIVE UE, the last serving gNB can release QoE configuration via existing paging mechanism; RRC_IDLE UE, new gNB can release QoE configuration when UE switches from RRC_IDLE to RRC_CONNECTED.

- option 2, similar to logged MDT, UE can release QoE configuration when it’s in RRC_IDLE/RRC_INACTIVE state.

Observation 1, the security issue for UE-based solution is not clear, considering logged MDT has similar mechanism without security issue.

Observation 2, the CN-based solution may have impact on stage 2 and stage 3 SA2 specifications.

Observation 3, it’s not appropriate to store the m-based configuration in CN, as CN may not want to know the configuration in RAN OAM.

Proposal 2, RAN3 can check with SA2 and SA3 on the above observations and potential spec impact and workload.

Observation 4, if it’s hard to down-select between UE-based solution and network-based solution, OAM-based solution can also be considered, which will have less spec impact than the other two.

Observation 5: It would be beneficial for gNB to collect RVQoE measurement for MBS broadcast service to have a better resource management.

Proposal 4: RAN3 agree RVQoE measurement collection can be supported in IDLE and Inactive state.

Proposal 5: if RAN3 agreed to support RVQoE measurement collection in RRC_IDLE and RRC_INACTIVE state, RAN3 discuss how to report the RVQoE measurement.

Proposal 6, RAN3 to discuss whether and how RVQoE configuration can be re-configured when UE switch from RRC_IDLE to RRC_CONNECTED state.

Reference

[1] R3-226725, Discussion on QoE configuration in RRC_INACTIVE RRC_IDLE states, Xiaomi