3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231551

# Online, April 17th – April 26th 2023

Title:	Reply LS on RAN information exposure for XRM

Response to:	R3-230791/S2-2303813

Release:	Rel-18

Work Item:	XRM

Source:	Ericsson (to be RAN3)

To:	SA2

Cc:

Contact person:	Angelo Centonza

Angelo.Centonza(at)Ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

RAN3 thanks SA2 for their LS on new functionalities exposing NG-RAN information in the specifications under SA2 control. RAN3 has analysed the functionalities described by SA2 and would like to provide the following feedback.

Feedback on the functionality of QoS Notification Control for GBR QoS Flow:

Supporting signalling of QoS Notification Control for GBR QoS Flow via GTP-U to UPF has a potential impact on all the RAN nodes and interfaces. Signalling QoS Notification Control over the UP may deny the possibility of reporting alternative QoS parameters.

RAN3 would like to point out that QoS Notification Control for GBR QoS Flow is deliberately delayed with respect to the event that triggered it, so to create an hysteresis and avoid “ping pong” effects, hence it should not be assumed that this information is sent in real time and that it can be used for real time application rate control.

Lack of in order delivery over GTP-U may cause out of order information exposure at the UPF/AF and wrong rate adaptation decisions at the AF.

RAN3 has not identified any benefits for this feature, considering that QoS Notification control is already signalled to the 5GC over NG-C.

Feedback on the functionality of per QoS Flow via NGAP to SMF or via GTP-U to UPF:

Supporting exposure of data rate information on a per QoS Flow via NGAP to SMF or via GTP-U to UPF has a potential impact on all the RAN nodes and interfaces.

It is not feasible to assume that data rates measured per DRB by the RAN can be converted in data rates per QoS Flow, to be signalled to the CN.

RAN3 would like to point out that per QoS Flow data rate measurements are not defined nor required at the NG-RAN, however such measurements can directly be taken at the UPF.

RAN3 has not identified any benefits for this feature, considering that data rates per QoS flow can be measured already at the UPF and made available to other functions in the 5GC.

# 2	Actions

To SA2

ACTION: 	RAN3 kindly ask SA2 to take the above into account.

# 3	Dates of next RAN3 meetings

RAN3 Meeting #120	22-26 May 2023	                     Incheon, KR

RAN3 Meeting #121	21-25 August 2023	                     Toulouse, FR