3GPP TSG-RAN WG3 #119bis-e                                               R3-231553

Online, 17th – 26th April 2023

Agenda Item:	10.2.2

Source:	CATT

Title:	Discussion on MRO enhancement for CPAC

Document for:	Discussion

Introduction

In last RAN3 meeting, we made general discuss and achieved some agreements. In the document, we provide some analysis on the below topic of MRO enhancements:

MRO for CPC and CPA based on the R17 NR-DC MRO solution

MRO for MR-DC SCG failure

Discussion

2.1 MRO for CPC and CPA

In last RAN3 meeting, we have agreed the principle on which information shall be included in the SCGFailureInformation as below:

Information available in the network nodes should not be included in the SCGFailureInformation.

Based on that, we collect information which has been proposed to enhance SCG failure information in previous several meetings and then provide our analysis. There are listed as below:

- list of candidate PSCell Ids

- configured CPC execution condition(s)

- time elapsed since the last CPC configuration until SCG failure

- time elapsed since CPAC execution until SCG failure

- time elapsed between the CPAC execution towards the target PSCell and the corresponding latest CPAC configuration is received for the target PSCell

- the type of PSCell addition/change, i.e. CPA or CPC

- the target cell towards which the CPAC was executed

- the node (i.e., MN or SN) that initiates the CPC

- the latest radio measurement results, and include an indication on whether a measured neighbour cell was configured as a CPAC candidate or not

For 1),2),9), there are related to candidate PSCell list and execution conditions.

In R17 MRO for CHO, CHO candidate cell list and execution conditions are kept by network and UE for different cases. In the topic of MRO for CPC/CPA, most of time MN can keep UE context and candidate PSCell list and execution conditions when receiving SCG failure information message from Uu interface. When sending SCG FAILURE INFORMATION REPORT message, MN can send candidate PSCell list and execution conditions to SN for MRO analysis. For intra-SN conditional PSCell change case, although MN is not aware of candidate PSCell list and execution conditions, last serving SN can keep them. So, it is still not needed for UE to keep and report them.

In one word, for CHO candidate PSCell list and execution conditions, network can keep them and it is not needed for UE to keep and report them.

Proposal 1: For CHO candidate PSCell list and execution conditions, network can keep them and it is not needed for UE to keep and report them.

For 3)4)5), they are time related information.

In R17 MRO for CHO, we discussed how to define and record time related information in RLF Report which mainly includes the following time point:

a). the time when UE receives RRCReconfiguration message which includes CHO configuration.

b). the time when CHO executes.

c). the time when failure occurs.

As for MRO for CPA/CPC, the corresponding time is as below:

a). the time when UE receives RRCReconfiguration message which includes CPA/CPC configuration.

b). the time when CPA/CPC executes.

c). the time when SCG failure occurs.

For a), it is network to send RRCReconfiguration message, so, network knows the time point.

For b), when CPA/CPC executes, RRCReconfigurationComplete message which includes selectedCondRRCReconfig field is initiated from UE to network to indicate the selected PSCell. So, network is aware of the execution time point and the target cell towards which the CPAC was executed (7). Also, for (7), failedPSCellId field has been included in SCG failure information message which indicates the physical cell id and carrier frequency of the cell in which SCG failure is detected or the target PSCell of the failed PSCell change or failed PSCell addition. It can been reused in CPAC case.

RRCReconfigurationComplete message

-- ASN1START

-- TAG-RRCRECONFIGURATIONCOMPLETE-START

RRCReconfigurationComplete ::=              SEQUENCE {

rrc-TransactionIdentifier                   RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

rrcReconfigurationComplete                  RRCReconfigurationComplete-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

……

RRCReconfigurationComplete-v1700-IEs ::=    SEQUENCE {

needForGapNCSG-InfoNR-r17                   NeedForGapNCSG-InfoNR-r17                                               OPTIONAL,

needForGapNCSG-InfoEUTRA-r17                NeedForGapNCSG-InfoEUTRA-r17                                            OPTIONAL,

selectedCondRRCReconfig-r17                 CondReconfigId-r16                                                      OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                                                             OPTIONAL

}

-- TAG-RRCRECONFIGURATIONCOMPLETE-STOP

-- ASN1STOP

selectedCondRRCReconfig This field indicates the ID of the selected conditional reconfiguration the UE applied upon the execution of CPA or inter-SN CPC.

For c), SCGFailureInformation message is initiated from UE to network when SCG failure occurs. So, network is aware of the SCG failure time point.

In a word, since network is aware of the every time point during CPA/CPC procedures, it is not needed for UE to keep and report them.

Proposal 2: Since network is aware of the every time point during CPA/CPC procedures, it is not needed for UE to keep and report time related information.

For 6) and 8), the type of PSCell addition/change and the node (i.e., MN or SN) that initiates the CPC, it is consider that MN maintains UE context hence it can know 6) and 8).

As discussed above, network is aware of all of the information. So, we think it is not needed to enhance SCG failure information.

Proposal 3: It is considered that network is aware of all of the list information. It is not needed to enhance SCG failure information at this time.

Here we continue discussing the XN interface impace,according to the agreement as below:

Reusing R17 signalling mechanism to report CPA/CPC failure/ related information over Xn from MN to source SN or last serving SN

In R17, SCG Failure Information Report and SCG Failure Transfer procedure have been introduced to perform NR-DC MRO analysis. MRO for CPAC can reuse this R17 procedure.

Proposal 4: It is proposed to reuse R17 SCG Failure Information Report and SCG Failure Transfer procedure for R18 MRO for CPAC.

As discussed above, MN can keep UE context and candidate PSCell list and execution conditions when receiving SCG failure information message from Uu interface.

We think candidate PSCell list and execution conditions are needed for SN to perform MRO analysis and shall be included in SCG FAILURE INFORMATION REPORT message.

Proposal 5: It is proposed to include candidate PSCell list and execution conditions in SCG FAILURE INFORMATION REPORT message.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Proposal 1: For CHO candidate PSCell list and execution conditions, network can keep them and it is not needed for UE to keep and report them.

Proposal 2: Since network is aware of the every time point during CPA/CPC procedures, it is not needed for UE to keep and report time related information.

Proposal 3: It is considered that network is aware of all of the list information. It is not needed to enhance SCG failure information at this time.

Proposal 4: It is proposed to reuse R17 SCG Failure Information Report and SCG Failure Transfer procedure for R18 MRO for CPAC.

Proposal 5: It is proposed to include candidate PSCell list and execution conditions in SCG FAILURE INFORMATION REPORT message.

# 4. Reference

[1] Chair notes, RAN3#119