3GPP TSG-RAN WG3 #119bis                                                 R3-231558

Online, 17th – 26th April, 2023

Agenda Item:	16.3

Source:	CATT

Title:	Support Service Continuity Enhancements

Document for:	Discussion

Introduction

In last RAN3 meeting, we discussed option 1 and option 2 and agreed option 2 as a way forward [1]. In this document, we provide some further analysis based on the below agreement and further discuss pack loss during path switch.

During direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO REQ message.

At least Remote UE L2 ID and a list of candidate target relay UE IDs should be included in the XnAP HANDOVER REQUEST message.

Stage-3 details can be figured out next meeting, for example, whether the Remote UE L2 ID is already included in the inter-node message.

Following legacy HO procedure, RAN3 does not set any restriction on whether the target candidate relay UEs belong to the same target gNB or not. The same applies for the target cell.

Discussion

2.1 Path switch

a) Inter-gNB indirect-to-direct path switching

For the handover preparation/execution stage, it is same as the normal UE inter-CU handover. After switching, target CU sends a UE context release message to source CU. Source CU may send UE context modification request to gNB-DU and also send a RRC reconfiguration to source relay UE to release remote UE context e.g., release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE.

Figure 1 inter-gNB indirect-to-direct path switching

Observation 1: Target CU sends RRC reconfiguration conveyed in UE context modification request for relay UE and UE context release request for DU to release remote UE context.

Proposal 1: Take Figure 1 as baseline for inter-gNB indirect-to-direct path switching.

b) Inter-gNB direct-to-indirect path switching

Following legacy HO procedure, RAN3 does not set any restriction on whether the target candidate relay UEs belong to the same target gNB or not. The same applies for the target cell.

One open issue is about whether to allow the candidate target relay UE under the different target gNB. Basically, there is no need to restrict candidate target relay UE under the same gNB. The multiple Xn handover request messages can be sent to different target gNB based on source gNB implementation.

Proposal 2: RAN3 does not set any restriction on whether the target candidate relay UEs belong to the same target gNB or not.

Stage-3 details:

Stage-3 details can be figured out next meeting, for example, whether the Remote UE L2 ID is already included in the inter-node message.

As we discussed in the last meeting, Source gNB should send a list of candidate Relay UE L2ID in Xn handover request message. For Remote UE L2 ID, it has been covered by SUI in handover preparation, it is not necessary to introduced in Xn handover request message.

Proposal 3: Remote UE L2 ID is already included in the inter-node message, it does not need to be further introduced in Xn handover request message.

There are two options to introduce candidate target relay UE list in Xn handover request.

- Option 1:The list of candidate target relay UE can be introduced in Xn handover request directly

- Option 2: Only introduce one target relay UE e.g., the highest priority in the Xn handover request and other candidate target relay UE(s) is added in inter node message.

It is consider that the source gNB performs the initial selection of target cell and sends the highest priority target cell ID in Xn handover request and the other candidate target cell(s) is contained in inter-node message, for SL relay path switch, we also prefer this design that only containing the highest priority target relay UE in the Xn handover request and other candidate target relay UE(s) is added in inter node message. It also makes the specification more efficient and clearer in case RAN2 agrees to introduce PC5 measurement in inter-node message. Therefore, we slight prefer option 2 to option 1.

Proposal 4: Introduce one target relay UE (e.g., the highest priority in the list of candidate target relay UE) in the Xn handover request and other candidate target relay UE(s) is added in inter node message. LS is needed to RAN2 for including candidate target relay UE(s) in inter node message in Annex2.

Stage-2 flow chart:

Figure 2 inter-gNB direct-to-indirect path switching

Proposal 5: Take Figure 2 as baseline for inter-gNB direct-to-indirect path switching.

c) Intra-gNB indirect-to-indirect path switching

GNB-CU decides switch remote UE to a target relay UE which is camped in the same gNB. GNB-CU triggers UE context setup/modification procedure for target relay UE, and also triggers UE context setup procedure for remote UE. RRC reconfiguration message for release for source relay UE can be sent any time after source gNB sends RRC reconfiguration message to remote UE based on gNB implementation.

3.	The gNB sends RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission via U2N Relay UE after reception of RRCReconfiguration message from the gNB.

6.	The gNB sends RRCReconfiguration message to the U2N Relay UE to reconfigure the connection between the U2N Relay UE and the gNB. The RRCReconfiguration message to the U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration between PC5 RLC and Uu RLC).

Observation 2: For Intra-gNB indirect-to-indirect path switching, RRC reconfiguration message for release for source relay UE can be sent any time after source gNB sends RRC reconfiguration message to remote UE based on gNB implementation which align with switching from inter-gNB-DU indirect to direct path in TS38.300 in R17.

Figure 3 intra-gNB indirect-to-indirect path switching

Proposal 6: Take Figure 3 as baseline for intra-gNB indirect-to-indirect path switching.

d) Inter-gNB indirect-to-indirect path switching

This procedure of inter-gNB indirect-to-indirect path switching is similar as the inter-gNB direct-to-indirect path switching. The difference is that source gNB should send RRC reconfiguration to source relay UE to release remote UE context after receiving the indication from target gNB.

Figure 4 inter-gNB indirect-to-indirect path switching

Observation 3: Compared with inter-gNB direct-to-indirect path switching, the procedure of inter-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context.

Proposal 7: Take Figure 4 as baseline for inter-gNB indirect-to-indirect path switching.

2.2 Packet loss

Agreement:

RAN2 consider that lossless data delivery in the inter-gNB i2x cases needs to be addressed.  Solutions can be considered next meeting (including the possibility of solutions needing work from RAN3).  Solutions based on the PDCP status report mechanism are the baseline.

RAN2 discussed the lossless data delivery in the inter-gNB i2x cases. It may require some RAN3 discussion and further provide RAN3’s solutions.

Inter-gNB case 1: DL data in inter-gNB path switch/HO scenario

For inter-gNB path switch/HO scenario, DL data should be forwarded to the target gNB and be re-transmitted to the UE based on the rule in [2]:

Step 1: For DL data forwarding, the PDCP SDUs with SN number in the source NG-RAN node may be firstly forwarded to the target NG-RAN node if not acknowledged by the UE:

- for DRBs for which preservation of SN status applies, the source NG-RAN node may forward in order to the target NG-RAN node via the DRB DL forwarding tunnel all downlink PDCP SDUs with their SN corresponding to PDCP PDUs which have not been acknowledged by the UE.

Step 2: The target NG-RAN node will re-transmit the forwarded PDCP SDUs:

For RLC-AM bearers: - The target gNB re-transmits and prioritizes all downlink data forwarded by the source gNB (i.e. the target gNB should first send all forwarded PDCP SDUs with PDCP SNs, then all forwarded downlink PDCP SDUs without SNs before sending new data from 5GC), excluding PDCP SDUs for which the reception was acknowledged through PDCP SN based reporting by the UE.

If follows the legacy spec, for inter-gNB i2d/i2i case, the DL data has not been acknowledged by the relay RLC may be forwarded. The data acknowledged by Relay but lost in the source PC5 link cannot be forwarded to the target gNB, therefore such loss packet cannot be re-transmitted from the target gNB to the UE, no matter the PDCP status report is configured to be sent by the UE or not.

Observation 4: For DL data in inter-gNB i2d/i2i path switch/HO scenario, the source gNB may not forward all the un-successfully delivered PDCP SDUs to the target gNB, if assumed the DL data may loss in the source PC5 link.

There are two options from RAN3 perspective:

- Option 1: Source gNB additionally forwards PDCP SDUs which has been acknowledged by source relay UE based on source gNB implementation to the target gNB during data forwarding.

- Option 2: Target gNB identifies which packets are lost based on PDCP status report from UE, and then requesting source gNB to provide such packets via Xn-U.

For example, there are one hundred packets from 1-100. Source gNB sends 1-60 packets in source side and receives HARQ ACK from source relay UE RLC. In fact, there are only 1-50 packets are received by remote UE. When performing inter-gNB i2x path switch, source gNB will not send packet 51-60 to target gNB but send 61-100 during data forwarding.

To solve this issue, option 1 proposes to send all 1-100 packet during data forwarding or source gNB decides the start packet of data forwarding based on implementation e.g., data forwarding starts from packet 40 (40-100). Based on this, target gNB can perform data re-transmission on target according to PDCP status report.

For option 2, target gNB may found that the first packet of data forwarding is packet 60 while the PDCP status report shows that UE only successfully receive packet until 50. Target gNB will know those packets 50-60 were buffered in source relay UE. Target gNB can send an Xn message to source gNB to ask for forwarding packets from 50-60 then target gNB is able to perform data re-transmission from packet 50.

In general, option 1 has no stage-3 impact but it increases the load of Xn-U. Option 2 needs some stage-3 update that the Xn message should be sent from target gNB to source gNB and it also requires source gNB to store packet even it has been acknowledged by source relay UE. Compared with two options, the increased load in Xn-U in option 1 is acceptable because how many additional packets are forwarded to target depends on the source gNB capability/decision. Therefore, we prefer option 1 to option 2.

Proposal 8: For DL, source gNB additionally forwards PDCP SDUs which has been acknowledged by source relay UE based on source gNB implementation to the target gNB during data forwarding. Target gNB performs re-transmission based on PDCP status report from UE.

Inter-gNB case 2: UL data in inter-gNB path switch/HO scenario

For inter-gNB path switch/HO scenario, the UE should re-transmit the UL data from the oldest PDCP SN number which has not been acknowledged at RLC based on the rule in [2]:

For RLC-AM bearers: - The UE re-transmits in the target gNB all uplink PDCP SDUs starting from the oldest PDCP SDU that has not been acknowledged at RLC in the source, excluding PDCP SDUs for which the reception was acknowledged through PDCP SN based reporting by the target.

If follows the legacy spec, for i2d/i2i case, the UL data has been acknowledged by the relay RLC may not be re-transmitted by the UE to the target gNB. If the data is acknowledged by Relay but lost in the source Uu link, it may not be re-transmitted to the target gNB if its PDCP SN is small than the oldest PDCP SDU that has not been acknowledged at RLC, even the PDCP status report from the target gNB ask for the lost PDCP SDUs.

Observation 5: For UL data in inter-gNB i2d/i2i path switch/HO scenario, the remote UE may not re-transmit all the un-successfully delivered PDCP SDUs to the target gNB, even the PDCP status report from the target gNB ask for the lost PDCP SDUs, if assumed the UL data may loss during the source Uu link change, e.g. Uu RLF, Uu HO.

Remote UE re-transmits PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch, which has no further RAN3 impact.

Proposal 9: For UL, the remote UE should re-transmit all PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch

Conclusions

Based on the discussion in section 2 the followings are proposed:

Observation 1: Target CU sends RRC reconfiguration conveyed in UE context modification request for relay UE and UE context release request for DU to release remote UE context.

Proposal 1: Take Figure 1 as baseline for inter-gNB indirect-to-direct path switching.

Proposal 2: RAN3 does not set any restriction on whether the target candidate relay UEs belong to the same target gNB or not.

Proposal 3: Remote UE L2 ID is already included in the inter-node message, it does not need to be further introduced in Xn handover request message.

Proposal 4: Introduce one target relay UE (e.g., the highest priority in the list of candidate target relay UE) in the Xn handover request and other candidate target relay UE(s) is added in inter node message. LS is needed to RAN2 for including candidate target relay UE(s) in inter node message in Annex2.

Proposal 5: Take Figure 2 as baseline for inter-gNB direct-to-indirect path switching.

Observation 2: For Intra-gNB indirect-to-indirect path switching, RRC reconfiguration message for release for source relay UE can be sent any time after source gNB sends RRC reconfiguration message to remote UE based on gNB implementation which align with switching from inter-gNB-DU indirect to direct path in TS38.300 in R17.

Proposal 6: Take Figure 3 as baseline for intra-gNB indirect-to-indirect path switching.

Observation 3: Compared with inter-gNB direct-to-indirect path switching, the procedure of inter-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context.

Proposal 7: Take Figure 4 as baseline for inter-gNB indirect-to-indirect path switching.

Observation 4: For DL data in inter-gNB i2d/i2i path switch/HO scenario, the source gNB may not forward all the un-successfully delivered PDCP SDUs to the target gNB, if assumed the DL data may loss during the source PC5 link change.

Proposal 8: For DL, source gNB additionally forwards PDCP SDUs which has been acknowledged by source relay UE based on source gNB implementation to the target gNB during data forwarding. Target gNB performs re-transmission based on PDCP status report from UE.

Observation 5: For UL data in inter-gNB i2d/i2i path switch/HO scenario, the remote UE may not re-transmit all the un-successfully delivered PDCP SDUs to the target gNB, even the PDCP status report from the target gNB ask for the lost PDCP SDUs, if assumed the UL data may loss during the source Uu link change, e.g. Uu RLF, Uu HO.

Proposal 9: For UL, the remote UE should re-transmit all PDCP SDUs for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by PDCP status report in the target gNB after path switch

# 4. Reference

[1] Chair notes, RAN3#119-e

[2]	TS 38.300 NR and NG-RAN Overall Description; Stage 2

# 5. Annex 1: TP for TS38.423

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Target Relay UE L2ID | O |  | BIT STRING (SIZE(24)) |  | YES | ignore |

# 6. Annex 2: LS to RAN2

3GPP TSG-RAN WG3 Meeting #119bis	R3-23xxxx

Online, 17th – 26th April, 2023

Title:	LS on Support Service Continuity Enhancements for SL Relay

Release:	Rel-18

Work Item:	NR_SL_relay_enh

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name: 		Liu Ziqiao

E-mail Address:	liuziqiao@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall description:

RAN3 agreed to introduce a list of candidate relay UEs belonging to the same target cell in HO REQ message in the last meeting.

During direct to indirect and indirect to indirect path switch procedures, the source gNB sends a list of candidate relay UEs belonging to the same target cell in the HO REQ message.

In this meeting, RAN3 discusses the stage-3 details and further achieve an agreement that introducing one target relay UE ID in HO REQ message and other candidate relay UEs are contained in inter-node message. RAN3 respectfully asks RAN2 to add candidate relay UEs in handoverpreparationinformation message.

- 2. Actions:

RAN3 respectfully asks RAN2 to take above into account.

3. Date of next TSG RAN WG3 meetings:

TSG-RAN3 Meeting #120                               21 May - 26 May 2023	Korea