3GPP TSG-RAN WG3 #119bis                                                                                         R3-231563

Online, 17th – 26th April, 2023

Agenda Item:	21.2

Source:	CATT

Title:	Discussion on long eDRX for RRC_INACTIVE about CT4/RAN2 progress

Document for:	Discussion

- Introduction

A LS [1] is sent from CT4 about paging policy information/parameters for RRC Inactive with CN based MT communication handling.  Another LS [2] from RAN2 about eDRX beyond 10.24 sec in RRC_INACTIVE together with SDT features.

In this paper, we further discuss RAN3 impact about these two LSs.

- Discussion

  - Paging Policy Information for Network Triggered Connection

-	the Namf_MT EnableUEReachability request should allow the SMF to provide paging policy information/parameters including the PPI, the ARP and the 5QI of the QoS flow of the PDU session which triggers the Connection Resume procedure, and

-	the AMF would include the same towards the NG-RAN to enable similar NG-RAN paging as when NG-RAN pages the UE upon receiving DL packets for a UE in RRC_INACTIVE state (without CN based MT communication handling).

The paging policy information/parameters including the PPI, the ARP and the 5QI of the QoS flow of the PDU session should be included in N2 message in Figure 4.8.2.2b-1 step 2. As discussed in [3], we propose to use DL NAS transport message to indicate NG-RAN perform RAN paging. The stage3 changes as follow:

9.2.5.2	DOWNLINK NAS TRANSPORT

This message is sent by the AMF and is used for carrying NAS information over the NG interface.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Old AMF | O |  | AMF Name 9.3.3.21 |  | YES | reject |
| RAN Paging Priority | O |  | 9.3.3.15 |  | YES | ignore |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | reject |
| Mobility Restriction List | O |  | 9.3.1.85 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | O |  | 9.3.1.58 |  | YES | ignore |
| Allowed NSSAI | O |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. | YES | reject |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| UE Differentiation Information | O |  | 9.3.1.144 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| UE Radio Capability | O |  | 9.3.1.74 |  | YES | ignore |
| UE Capability Info Request | O |  | 9.3.1.192 |  | YES | ignore |
| End Indication | O |  | 9.3.3.32 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |
| Target NSSAI Information | O |  | 9.3.1.229 |  | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.54 |  | YES | ignore |
| Paging Policy Information for Network Triggered Connection | O |  | 9.3.3.x |  | YES | ignore |

9.3.3.x Paging Policy Information for Network Triggered Connection

This IE identifies paging policy information/parameters including the PPI, the ARP and the 5QI of the QoS flow of the PDU session which triggers the Connection Resume procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session ID | M |  | 9.3.1.50 |  |
| Paging Policy | O |  |  | As specified in TS 29.518 [x] |
| Allocation and Retention Priority | O |  | 9.3.1.19 |  |
| 5QI | O |  | INTEGER (0..255, …) | Indicates the standardized or pre-configured 5QI as specified in TS 23.501 [9]. |

Proposal 1: Include paging policy information/parameters including the PDU Session ID, PPI, the ARP and the 5QI of the QoS flow of the PDU session in DOWNLINK NAS TRANSPORT.

2.2 INACTIVE eDRX above 10.24sec and SDT

RAN2 would like to inform that in Rel-18, RAN2 intends to allow configuring extended DRX (eDRX) beyond 10.24 sec in RRC_INACTIVE together with SDT features (including MO and/or MT versions of SDT).

The principle of inactive eDRX above 10.24sec is that CN may be informed about RRC status transition i.e., NG-RAN indicates AMF that UE into RRC_CONNECTED. However, SDT can be transmitted in RRC_INACTIVE. It means that each UE sends SDT to NG-RAN with RRC resume message (step1 in Figure 4.8.2.2-1 in 23.501), the NG-RAN does not need to configure UE into RRC_CONNECTE states and inform AMF the RRC status transition. The further enhancement can wait for SA2 and RAN2 progress.

Proposal 2: NG-RAN does not need to inform AMF that RRC status transition for SDT. Further enhancement can wait for SA2 and RAN2 progress.

- Conclusion

Proposal 1: Include paging policy information/parameters including the PPI, the ARP and the 5QI of the QoS flow of the PDU session in DOWNLINK NAS TRANSPORT.

Proposal 2: NG-RAN does not need to inform AMF that RRC status transition for SDT. Further enhancement can wait for SA2 and RAN2 progress.

- Reference

[1] R3-231103, LS on Paging Policy Information for Network Triggered Connection Resume, CT4.

[2] R3-231113, LS on INACTIVE eDRX above 10.24sec and SDT, RAN2

[3] R3-231562, Discussion on long eDRX for RRC_INACTIVE about SA2 progress