3GPP TSG-RAN WG3 #119bis-e							R3-231581

17th – 26th April 2023

Agenda Item:	20.2

Source:	China Telecom

Title: 	Support of MT-SDT in Split Architecture

Document for:	Discussion

Introduction

The MO-SDT including RACH based SDT and CG based SDT was specified in Release 17. The Release 18 WID on MT-SDT was approved in RAN#94 meeting. The objective of this WID is listed below [1]:

- Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3]

  - MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response;

  - MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE.

In RAN3#119 meeting, the following agreements and open issues on support of split architecture were listed below:

Upon reception of DL SDT user data, the gNB-CU-UP may include the assistance information (e.g., Data size) in E1AP DL Data Notification message to gNB-CU-CP. FFS on MT-SDT indicator.

Upon reception of MT-SDT information via XnAP RAN paging message from the anchor gNB-CU, the gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message.

FFS on F1AP MT-SDT information

FFS: whether Data volume should also be provided from the CU to the DU in F1AP PAGING message.

FFS: whether MT-SDT support indication in E1 Bearer Context procedure should be defined to enable the gNB-CU-UP to include the DL data size in the E1AP DL DATA NOTIFICATION message.

FFS: whether receiving gNB-CU or receving gNB-DU decides MT-SDT Uu paging, if we agree that it is the receving gNB to make the final decision on triggering MT-SDT Uu paging.

In this paper, we will further analyse the standard impact on E1/F1 interface and provide our views.

Discussion

E1 impact

The following issues on E1 impact need to be discussed in RAN3:

- the details of assistance information in E1AP DL Data Notification

In last RAN3 meeting, RAN3 agreed to introduce an assistance information in DL DATA NOTIFICATION. However, the detail of assistance information is still FFS. DL DATA Notification aims to provide information on the detection of DL data arrival for the UE to the gNB-CU-CP. Per the latest TS37.483, this message does not differentiate DL data arrival of the SDT and non-SDT bearers. Since the last serving node cannot decide whether to trigger MT-SDT, it need to provide corresponding assistance information to neighbour nodes. From the CU-UP side, the downlink data size for SDT DRBs, is enough to be provided to the CU-CP. Here the data size for MT-SDT DRBs refers to the total amount of downlink SDT packets buffered in the CU-UP in DL DATA Notification. If “downlink data size for SDT DRBs” is agreed, there is no need to introduce another MT-SDT indication with “enum” type in assistance information.

Proposal 1: to agree “downlink data size for SDT DRBs” as the assistance information in DL DATA Notification. And There is no need to introduce another MT-SDT indication with “enum” type in assistance information.

- DRB type for MT-SDT

In Release 17 MO-SDT, a new indicator “SDT Indicator Setup” was introduced in BEARER CONTEXT SETUP REQUEST to indicate which DRB belongs to SDT DRB. One issue is whether to introduce a new type of DRB for MT-SDT. Per current specifications, DRB need to support both UL transmission and DL transmission. In view of this, it seems no strong motivation to differentiate radio bearers into UL and DL for SDT purpose. According to the following agreement in RAN2#121e meeting, our understanding is that there is no need to define a new type of DRB for MT-SDT.

6. 	RBs configured for SDT are common for MO-SDT and MT-SDT

Proposal 2: Not to specify a new type of DRB for MT-SDT in E1AP.

In addition, a new status resumeforSDT was also introduced in Release-17 to indicate the status of the Bearer Context. With this indicator enabled, the CU-CP continue to suspend the non-SDT DRBs and resume DRBs configured with SDT. According to the view in P1, there is also no need to introduce a new status of the Bearer Context for MT-SDT in DRB resume operation.

Proposal 3: Not to introduce a new status of the Bearer Context for MT-SDT in DRB resume operation.

- MT-SDT Configuration indicator

The issue on whether MT-SDT support indication is needed was discussed in last RAN3 meeting.

FFS: whether MT-SDT support indication in E1 Bearer Context procedure should be defined to enable the gNB-CU-UP to include the DL data size in the E1AP DL DATA NOTIFICATION message.

In addition, RAN2 had made the following agreement:

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

The above agreement shows that MT-SDT needs to support all features specified in Release 17 MO-SDT. From the perspective of RAN, all MT-SDT capable gNBs should support both Release 17 MO-SDT and Release 18 MT-SDT. In the view of the fact that the MT-SDT capable gNB-CU-CP can inform two modes as below for gNB-CU-UP:

  - Mode 1: Release 17 MO-SDT only

  - Mode 2: Release 18 MT-SDT

The configuration granularity of the above modes is per UE level. Moreover, in case of Mode 2 is configured to CU-UP, the CU-UP needs to inform the CU-CP about the downlink data arrival for MT-SDT via DL DATA Notification message. It is concluded that the behaviour of MT-SDT and MO-SDT is different in CU-UP. In view of this, it is necessary to introduce a new explicit indicator in BEARER CONTEXT SETUP REQUEST and BEARER CONTEXT MODIFICATION REQUEST to indicate whether MT-SDT is enabled.

Proposal 4: To introduce a new explicit indicator in E1AP to indicate whether MT-SDT is enabled.

- DL Data Volume Threshold for MT-SDT

As we know, SDT aims to optimize the small data transmission in inactive state. To distinguish whether an arrival data is small data or not, the corresponding thresholds need to be defined for UL and DL, respectively. In Release 17 MO-SDT, the UL data volume threshold, namely sdt-DataVolumeThreshold-r17, was introduced in SIB1. With this information, the SDT-capable UE can determine whether to perform SDT procedure. Similarly, we need to discuss whether DL data volume threshold is needed for MT-SDT. Our views on DL data volume threshold are listed as follow:

- Per Cell Configuration: Each NR cell can independently configure its DL data volume threshold as UL data volume threshold in MO-SDT.

- No need to broadcast: Since the MT-SDT capable UE is unclear about the data volume of incoming downlink packets, UE could not determine whether to trigger MT-SDT based on this threshold. Therefore, there is no need to broadcast this DL threshold in SIB1.

- No need to inform CU-UP about DL data volume threshold: In our understanding, given the threshold is a common value to all MT-SDT UEs, it can be configured in CU-UP via OAM. Moreover, whether to perform MT-SDT is decided by CU-CP and the responsibility of CU-UP is to provide necessary assistance information to CU-CP. Therefore, there is no need to introduce an additional IE in E1AP to indicate the DL data volume threshold.

Proposal 5: No need to introduce DL data volume threshold for MT-SDT in E1AP.

F1 impact

- Which node decides MT-SDT Uu paging?

For RRC paging message, RAN2 had made the following agreement:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

Therefore, it is natural that the gNB-CU decides the MT-SDT Uu paging and generates the corresponding RRC paging messages.

Proposal 6: The receiving gNB-CU decides MT-SDT Uu paging.

- F1AP paging information

The following issues were discussed in last RAN3 meeting, and still need to be further discussed

FFS on F1AP MT-SDT information

FFS: whether Data volume should also be provided from the CU to the DU in F1AP PAGING message.

The F1 paging message is sent by the gNB-CU and is used to request the gNB-DU to page UE. On the details of F1AP MT-SDT information, there are two options:

- Option 1: one indication for MT-SDT

- Option2: to specify a new paging cause, i.e., MT-SDT, in Paging message

In our understanding, both options could be workable and we have no strong views on the above options.

With regard to other assistant information, we think there is no need to add DL data amount of SDT DRB in this message as UE only need to evaluate radio channel quality before initiating MT-SDT procedure. Moreover, if the support of NAS signalling from CN in MT-SDT is agreed, there is also no need to indicate the data volume of SRB2 signalling to DU. In view of this, the amount of buffer MT-SDT data packets seems useless for both UE and gNB-DU.

Proposal 7: There is no need to add DL data amount of SDT DRB in this message.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: to agree “downlink data size for SDT DRBs” as the assistance information in DL DATA Notification. And There is no need to introduce another MT-SDT indication with “enum” type in assistance information.

Proposal 2: Not to specify a new type of DRB for MT-SDT in E1AP.

Proposal 3: Not to introduce a new status of the Bearer Context for MT-SDT in DRB resume operation.

Proposal 4: To introduce a new explicit indicator in E1AP to indicate whether MT-SDT is enabled.

Proposal 5: No need to introduce DL data volume threshold for MT-SDT in E1AP.

Proposal 6: The receiving gNB-CU decides MT-SDT Uu paging.

Proposal 7: There is no need to add DL data amount of SDT DRB in this message.

Reference

- RP-213583，New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR，ZTE Corporation

Text Proposal for E1AP

////////////////////////////////////////////////////////////////////////start of change ////////////////////////////////////////////////////////////////////////

- 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Serving PLMN | M |  | PLMN Identity 9.3.1.7 |  | YES | ignore |
| Activity Notification Level | M |  | 9.3.1.67 |  | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, ResumeforSDT) | Indicates the status of the Bearer Context. NOTE: This IE is not applicable to eNB-CP/eNB-UP and ng-eNB-CU-CP/ng-eNB-CU-UP | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | M |  | DRB To Setup List E-UTRAN 9.3.3.1 |  | YES | reject |
| >>Subscriber Profile ID for RAT/Frequency priority | O |  | 9.3.1.69 |  | YES | ignore |
| >>Additional RRM Policy Index | O |  | 9.3.1.70 |  | YES | Ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | M |  | 9.3.3.2 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 | Included whenever it is known by the gNB-CU-CP or by the ng-eNB-CU-CP | YES | ignore |
| Trace Activation | O |  | 9.3.1.68 |  | YES | ignore |
| NPN Context Information | O |  | 9.3.1.84 |  | YES | reject |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.89 |  | YES | ignore |
| CHO Initiation | O |  | ENUMERATED (True, …) |  | YES | reject |
| Additional Handover Information | O |  | ENUMERATED(Discard PDCP SN, …) | If set to “Discard PDCP SN”, indicates that the forwarded PDCP SNs have to be removed | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.3.1.98 |  | YES | ignore |
| gNB-CU-UP UE E1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC, no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertake or not. | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.102 |  | YES | ignore |
| SCG Activation Status | O |  | 9.3.1.105 |  | YES | ignore |
| MT-SDT indication |  |  | ENUMERATED (true, …) | Indication on whether MT-SDT is enabled | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

////////////////////////////////////////////////////////////////////////skip unchanged////////////////////////////////////////////////////////////////////////

- 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Security Information | O |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, ResumeforSDT) | Indicates the status of the Bearer Context NOTE: This IE is not applicable to eNB-CP/eNB-UP and ng-eNB-CU-CP/ng-eNB-CU-UP. | YES | reject |
| New UL TNL Information Required | O |  | ENUMERATED (required, …) | Indicates that new UL TNL information has been requested to be provided. | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Data Discard Required | O |  | ENUMERATED (required, …) | Indicate to discard the DL user data in case of RAN paging failure. | YES | ignore |
| CHOICE System | O |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | O |  | DRB To Setup Modification List E-UTRAN 9.3.3.7 |  | YES | reject |
| >>DRB To Modify List | O |  | DRB To Modify List E-UTRAN 9.3.3.8 |  | YES | reject |
| >>DRB To Remove List | O |  | DRB To Remove List E-UTRAN 9.3.3.9 |  | YES | reject |
| >>Subscriber Profile ID for RAT/Frequency priority | O |  | 9.3.1.69 |  | YES | ignore |
| >>Additional RRM Policy Index | O |  | 9.3.1.70 |  | YES | ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | O |  | PDU Session Resource To Setup Modification List 9.3.3.10 |  | YES | reject |
| >>PDU Session Resource To Modify List | O |  | 9.3.3.11 |  | YES | reject |
| >>PDU Session Resource To Remove List | O |  | 9.3.3.12 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 |  | YES | ignore |
| Activity Notification Level | O |  | 9.3.1.67 |  | YES | ignore |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC, no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertake or not. | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.102 |  | YES | ignore |
| SCG Activation Status | O |  | 9.3.1.105 |  | YES | ignore |
| SDT Continue ROHC | O |  | ENUMERATED (true, …) | Indicates ROHC should be continued for SDT DRBs. See description of sdt-DRB-ContinueROHC-r17 in TS 38.331 [10]. | YES | reject |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.129 |  | YES | ignore |
| MT-SDT indication |  |  | ENUMERATED (true, …) | Indication on whether MT-SDT is enabled | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |