3GPP TSG-RAN WG3 #119bis-e							R3-231586

Online, Apr 17th – Apr 26th, 2023

Agenda Item:	10.2.3

Source:	Ericsson

Title:	RACH Optimization enhancement

Document for:	Discussion, Decision

# 1	Introduction

During RAN3-119, discussions on RACH optimisation were carried out. The following agreement was captured:

Use non-UE associated signalling for RACH Indication over F1.

However, two TPs on F1AP and XnAP (R3-230930 and R3-230975) were discussed without any agreement.

On the other hand, an LS (R3-231112) was received from RAN2 with questions related to inclusion of PSCell ID outside of NR RACH report.

In this contribution, we discuss the open points on the TPs and share our views. Furthermore, we share our views on the received LS from RAN2 and attach a draft reply on the annex.

# RACH Optimization enhancement

## 2.1 RACH report retrieval over F1

The TP for the F1AP in R3-230930 was discussed but no agreements were reached. The following message was proposed in the TP:

#### 9.2.1.x	RACH INDICATION

This message is sent by the gNB-DU to inform the gNB-CU about one or more random access procedures performed at the gNB-DU.

Direction: gNB-DU ® gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RACH Indication List |  | 1..<maxnoofRACHIndications> |  |  | YES | ignore |
| >RACH Report List Item |  | 1 |  |  |  |  |
| >> gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | ignore |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |  |  |
| >>Random access Indication | O |  | ENUMARATED (true, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHIndications | Maximum number of RACH Indications. Value is FFS. |

During the meeting, the following comments were minuted:

- Whether the gNB-DU UE F1AP ID and Random access Indication are needed?

- The criticality of IEs in this new introduced message?

We expand on the comments minuted.

The newly added RACH Indication message is obviously dedicated to indicating RACH information and thus an additional Random Access Indication IE is not required inside the message as it does not add any value to the content the message would have without it.

The Random Access Indication IE in the RACH INDICATION message over F1 interface is not required.

On the other hand, in the proposed message, both gNB-CU UE F1AP ID and gNB-DU UE F1AP ID are included. It should be noted that UE identifier is locally unique and including only the gNB-CU UE F1AP ID is sufficient for the CU to identify the UEs. The inclusion of a single UE APID has been already followed in the Access and Mobility Indication procedure over Xn, hence it has been already discussed and proven to work.

Only gNB-CU UE F1AP ID is included in the RACH INDICATION message over F1.

Furthermore, it is important to discuss the criticality of the IEs in the message. In the TP proposed in R3-230930, all the IEs in the message have criticality “ignore”. This implies that the receiver may receive the RACH INDICATION message, but not support/understand any of its content. However, upon correctly receiving the message the receiver can ignore it without any indication to the sender. The sender therefore assumes that the message was correctly received and that the gNB-CU will trigger retrieval of RACH Reports from the UE, while the receiver has totally discarded the message. This prevents the gNB-DU to distinguish cases where the message is not supported, and for that should not be resent, and cases where the message is supported but it was received incorrectly, triggering a resend of the message.

- If all IEs in the message have criticality “ignore”, the receiver may ignore the message without any indication despite the process being successful. The sender does not understand whether to resend the message or not.

In our opinion, this is not agreeable. We propose the criticality of the RACH indication list IE to be reject in order 6to ensure correct interoperability.

The RACH indication list IE in the F1: RACH INDICATION message shall have criticality “reject”.

A TP with the proposal is provided in the annex.

## 2.2 RACH Indication over Xn

The TP for Xn in R3-230975 was also discussed without any agreements. The following message was proposed in the TP:

#### 9.1.2.x	RACH INDICATION

This message is sent by the S-NG-RAN node to inform the M-NG-RAN node that one or more RACH reports are available at the UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RACH Indication List |  | 1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>Random access Indication | M |  | ENUMRATED (true, …) |  | YES | ignore |
| >>M-NG-RAN UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| >>S-NG-RAN UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |

Following similar discussion as for the F1AP TP, the same conclusions can be derived.

The Random Access Indication IE in the RACH INDICATION message over Xn interface is not needed.

Only one NG-RAN node UE XnAP ID allocated at the M-NG-RAN node is included in the RACH INDICATION message over Xn.

The RACH indication list IE in the F1: RACH INDICATION message shall have criticality “reject”.

A TP in the annex is provided capturing the above proposals.

## 2.3 RACH partitioning feature in RA report

In this section we analyse each information so far proposed to be added in the RACH report to enable the network to further optimise RACH configurations.

Feature Priorities: namely the priority of features at the time RACH access was carried out

It needs to be reminded that RAN2 already agreed to the following:

Agree to add the following parameters into RACH report for RACH partitioning:

-     Feature or the combination of features that triggered the RACH

-     Used feature combination

With the above agreement, RAN2 agreed that the UE shall report the feature, or combination of features that triggered the UE to select a specific RACH partitioning and that triggered the UE to perform RACH access.

RAN2 also agreed that the UE shall report also the Feature Combination associated to the RACH partitioning where RACH access is carried out. In light of the agreements from RAN2 it makes only sense that the UE reports also the Feature Priority configured for features at the time of RACH access. This information, combined with the information agreed in RAN2 allows to understand the following:

- The RAN can understand which feature has triggered the UE to select a RACH partition and perform RACH access. In case many UEs are triggered by the same feature or set of features, a RACH optimisation action could be to create a dedicated RACH partition for the most demanded features

- The RAN can understand to which RACH partition the UE performed RACH access. This allows the RAN to understand which partitions are most demanded and therefore most overloaded

- The RAN can understand which feature priority was given to the feature or set of features that triggered RACH access. This allows the RAN to understand whether grouping of features is done optimally or not. For example, if a RACH partition is associated to features with different priorities and if such partition is overloaded, one RACH optimisation action may be to split the features in different priority groups

RAN3 to send an LS to RAN2 requesting the addition of feature priorities as part of the RACH Report

The set of preambles allocated to the RA partition such as the start preamble index and/or the number of preambles in the partition (RACH configuration related information) and Time between RA attempt to RA Report

This information is meant to provide the RAN with information about the RACH configuration in use at the time the RACH Report was generated.

The rationale is that if the RACH report is reported after the RACH partitions configuration has changed, the RAN would not be able to determine what RACH configuration was in use at the time the RACH report was generated. This not only prevents the RAN to derive the right optimisation action, but it might have negative effects on the system. Indeed, if the RAN assumes that the RACH configuration at the time the RACH report was generated was the same as the actual RACH configuration, the RAN may determine RACH reconfiguration actions that might have negative impacts.

The best approach to solve this issue is to let the UE include in the RACH report the start preamble index and the number of preambles in the partition the UE used at the time of performing RACH access.

An alternative approach to reporting the start preamble index and the number of preambles in the RACH partition used would be to let the UE report the time between RACH report generation and RACH report signalling to the RAN. With this time measure the RAN may be able to deduce what RACH configuration was in use at the time the RACH report was generated. However, this approach is less efficient because it carries impacts on both the UE (which has to store the new time variable) and the network (which has to store and timestamp all the RACH configurations adopted in the past).

RAN3 to send an LS to RAN2 requesting the addition of the start preamble index and the number of preambles in the partition the UE used at the time of performing RACH access

A draft LS containing proposal 7 and proposal 8 is attached in the annex.

## 2.4 SDT Use Cases

It would be useful for the UE to include SDT information in the RACH report for the purpose of revealing how much traffic was signalled as “small data”. In one example, if the amount of traffic signalled by the UE slightly exceeds the SDT threshold, the UE will transmit such traffic via normal data channels instead of via SDT transfer. It would be useful for the network to know of how much the SDT threshold has been exceeded so to optimise such threshold and allow for optimal SDT transfer utilisation. For example, the network may slightly increase the SDT threshold to trigger a much higher usage of SDT transfers.

RAN3 to send an LS to RAN2 requesting the addition in the RA report of small data traffic volume, when such small data volume exceeds the SDT threshold of a preconfigured amount

## 2.5 Inclusion of PSCell identities in the RA Report

In the received LS (R3-231112) from RAN2, the following questions were asked-

And then, RAN2 discusses the following alternatives regarding how the UE includes the PSCell identities:

-	Alt 1: Includes unique PSCell identities, i.e. if a PSCell occurs more than once in NR RA-ReportList, it is recorded only once in the list of PSCell identities

-	Alt 2: Includes the last PSCell identity (in NR RA-ReportList)

In Alt-1, UE includes the unique PSCell identities outside the NR RA-reportList container. Thus, the node fetching the RACH report from the UE can forward the container to all NG-RAN nodes serving the PSCells listed. The receiving node may extract the required information and reject the other reports.

- In Alt-1, RACH reports can be forwarded to all correct destination nodes and the receiving node can discard the redundant messages.

In Alt-2, UE includes only one PSCell identity and the node fetching the RACH report from the UE needs to forward the full set of information to the node serving the PSCell. The receiving node needs to decode and forward the RA reports to their respective destination.

- In Alt-2, RACH reports need to be forwarded to one node which re-distributes the reports to the respective destination

However, in Alt-2, the receiving NR node may not have Xn connectivity to the respective nodes containing PSCells. Since, PSCell CGI is not mandatory for the UE to include in the RA-report, it may not be possible for the NG-RAN node to re-distribute the reports. Alt-2 introduces a single point of failure and may be susceptible to error.

- Alt-2 introduces single point of failure and it is less robust compared to Alt-1 as it may incur in RA Report losses

Hence, we conclude that Alt-1 is more robust amongst the proposed alternatives by RAN2

RAN3 to indicate in the LS reply that Alt-1 is more robust and agreeable amongst the proposed alternatives.

In the Annex, TPs for both 38.423 and 38.473 is included along with the draft reply of LS to RAN2.

# Conclusion

In the previous sections we made the following observations:

Observation 1	If all IEs in the message have criticality “ignore”, the receiver may ignore the message without any indication despite the process being successful. The sender does not understand whether to resend the message or not.

Observation 2	In Alt-1, RACH reports can be forwarded to all correct destination nodes and the receiving node can discard the redundant messages.

Observation 3	In Alt-2, RACH reports need to be forwarded to one node which re-distributes the reports to the respective destination

Observation 4	Alt-2 introduces single point of failure and it is less robust compared to Alt-1 as it may incur in RA Report losses

Based on the discussion in the previous sections we propose the following:

Proposal 1	The Random Access Indication IE in the RACH INDICATION message over F1 interface is not required.

Proposal 2	Only gNB-CU UE F1AP ID is included in the RACH INDICATION message over F1.

Proposal 3	The RACH indication list IE in the F1: RACH INDICATION message shall have criticality “reject”.

Proposal 4	The Random Access Indication IE in the RACH INDICATION message over Xn interface is not needed.

Proposal 5	Only one NG-RAN node UE XnAP ID allocated at the M-NG-RAN node is included in the RACH INDICATION message over Xn.

Proposal 6	The RACH indication list IE in the F1: RACH INDICATION message shall have criticality “reject”.

Proposal 7	RAN3 to send an LS to RAN2 requesting the addition of feature priorities as part of the RACH Report

Proposal 8	RAN3 to send an LS to RAN2 requesting the addition of the start preamble index and the number of preambles in the partition the UE used at the time of performing RACH access

Proposal 9	RAN3 to send an LS to RAN2 requesting the addition in the RA report of small data traffic volume, when such small data volume exceeds the SDT threshold of a preconfigured amount

Proposal 10	RAN3 to indicate in the LS reply that Alt-1 is more robust and agreeable amongst the proposed alternatives.

TPs mirroring the above proposal are provided in the annex. An LS mirroring Proposal 7, 8 and 9 is added in the Annex. A TP mirroring Proposal 10 is available in R3-231587

# Annex –

## Text Proposals for SON BL CR for TS 38.423

------------------------------------------------------------Start of Changes------------------------------------------------------------

### 8.3.x	RACH Indication

#### 8.3.x.1		General

This message is sent by the S-NG-RAN node to the M-NG-RAN node to inform of one or more performed random access procedures at the S-NG-RAN, due to which one or more RACH reports are available at the UE.

#### 8.3.x.2		Successful Operation

Figure 8.2.x.2-1: RACH Indication procedure.

The S-NG-RAN node initiates the procedure by sending the RACH INDICATION message to M-NG-RAN node.

The RACH INDICATION message contains information concerning one or more performed random access procedures and existence of one or more RACH report at the UE.

Upon reception of the RACH INDICATION message the M-NG-RAN node may fetch the RACH report from the UE.

#### 8.3.x.3 	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Start of the Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.x	RACH INDICATION

This message is sent by the S-NG-RAN node to inform the M-NG-RAN node that one or more RACH reports are available at the UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RACH Indication List |  | 1 |  |  | YES | reject |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | reject |
| >> UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |

### 9.3.4	PDU Definitions

(unchanged part skipped)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- RACH INDICATION

--

-- **************************************************************

RachIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RachIndication-IEs}},

...

}

RachIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-RachIndicationList						CRITICALITY reject		TYPE RachIndicaionList							PRESENCE mandatory },

...

}

### 9.3.5	Information Element definitions

(unchanged part skipped)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

RachIndicationList ::= SEQUENCE (SIZE(1..maxnoofRachReports)) OF RachIndication-Item

RachIndication-Item ::= SEQUENCE {

uEAssistantIdentifier			NG-RANnodeUEXnAPID		PRESENCE mandatory },

iE-Extensions						ProtocolExtensionContainer { { RachIndication-Item-ExtIEs} }	OPTIONAL,

...

}

RachIndication-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

### 9.3.7	Constant definitions

(unchanged part skipped)

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

id-rachIndicaion												ProcedureCode ::= xx

-----------------------------------------------  Next ASN.1 Change -------------------------------------------------------

(unchanged part skipped)

id-UEAssistantIdentifier																			ProtocolIE-ID ::= 361

id-ManagementBasedMDTPLMNModificationList															ProtocolIE-ID ::= 362

id-F1-terminatingIAB-donorIndicator																	ProtocolIE-ID ::= 363

id-TAINSAGSupportList																				ProtocolIE-ID ::= 364

id-SCGreconfigNotification																			ProtocolIE-ID ::= 365

id-earlyMeasurement																					ProtocolIE-ID ::= 366

id-BeamMeasurementsReportConfiguration																ProtocolIE-ID ::= 367

id-CoverageModificationCause																			ProtocolIE-ID ::= 368

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-ChannelOccupancyTimePercentageUL				ProtocolIE-ID ::= xx1
id-EnergyDetectionThresholdUL					ProtocolIE-ID ::= xx2

id-RachIndicationList							ProtocolIE-ID ::= xx

END

-- ASN1STOP

-----------------------------------------------End of all changes-----------------------------------------------

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

## Text Proposals for SON BL CR for TS 38.473

------------------------------------------------------------Start of Changes------------------------------------------------------------

### 8.2.x		RACH Indication

#### 8.2.x.1		General

This procedure is initiated by the gNB-DU to inform the gNB-CU about the occurrences of successful random access procedures in the gNB-DU.

The procedure uses non-UE associated signalling.

#### 8.2.x.2		Successful Operation

Figure 8.2.x.2-1: RACH Indication procedure.

The gNB-DU initiates the procedure by sending the RACH INDICATION message to the gNB-CU. The RACH INDICATION message contains information concerning one or more successful random access procedures occurring in the gNB-DU.  Upon reception of the RACH INDICATION message, the gNB-CU may trigger retrieval of RACH Reports from the UE.

#### 8.2.x.3 	Abnormal Conditions

Not applicable.

<<< next change >>>

#### 9.2.1.x	RACH INDICATION

This message is sent by the gNB-DU to inform the gNB-CU about one or more random access procedures performed at the gNB-DU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| RACH Indication List |  | 1 |  |  | YES | reject |
| >RACH Report List Item |  | 1..<maxnoofRACHIndications> |  |  |  |  |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHIndications | Maximum number of RACH Indications. Value is 64. |

### 9.4.4	PDU Definitions

(unchanged part skipped)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

[snip]

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

PosSItypeList,

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

UlTxDirectCurrentMoreCarrierInformation,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

RachIndicationList

FROM F1AP-IEs

[snip]

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-PosSItypeList,

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-UlTxDirectCurrentMoreCarrierInformation,

id-SRSPosRRCInactiveQueryIndication,

id-RachIndicationList,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

[snip]

-- **************************************************************

--

#### -- RACH Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- RACH Indication

--

-- **************************************************************

RachIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{ RachIndication-IEs}},

...

}

RachIndication-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-RachIndicationList						CRITICALITY reject		TYPE RachIndicaionList							PRESENCE mandatory },

...

}

END

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

(unchanged part skipped)

[snip]

RachIndicationList ::= SEQUENCE (SIZE(1..maxnoofRachReports)) OF RachIndication-Item

RachIndication-Item ::= SEQUENCE {

uEAssistantIdentifier			GNB-CU-UE-F1AP-ID		PRESENCE mandatory },

iE-Extensions						ProtocolExtensionContainer { { RachIndication-Item-ExtIEs} }	OPTIONAL,

...

}

RachIndication-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RANAC ::= INTEGER (0..255)

RAN-MeasurementID ::= INTEGER (1.. 65536, ...)

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

(unchanged part skipped)

[snip]

id-SRSPosRRCInactiveConfig							ProtocolIE-ID ::= 674

id-SDTBearerConfigurationQueryIndication			ProtocolIE-ID ::= 675

id-SDTBearerConfigurationInfo						ProtocolIE-ID ::= 676

id-UL-GapFR2-Config									ProtocolIE-ID ::= 677

id-ConfigRestrictInfoDAPS							ProtocolIE-ID ::= 678

id-UE-MulticastMRBs-Setup-List						ProtocolIE-ID ::= 679

id-UE-MulticastMRBs-Setup-Item						ProtocolIE-ID ::= 680

id-MulticastF1UContextReferenceCU					ProtocolIE-ID ::= 681

id-PosSItypeList									ProtocolIE-ID ::= 682

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-UlTxDirectCurrentMoreCarrierInformation			ProtocolIE-ID ::= 689

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 690

id-RachIndicationList							ProtocolIE-ID ::= xx -- assigned by MCC

id-ChannelOccupancyTimePercentageUL					ProtocolIE-ID ::= 901 -- assigned by MCC

END

## LS to RAN2 on RACH optimisation

3GPP TSG-RAN WG3 Meeting #119bis-e	R3-23xxxx

Online, Online 17th Apr – 26th Apr 2023

Title:	LS on RACH Optimisation

Release:	Rel-18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name: 		Angelo Centonza

E-mail Address:	angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description:

As part of the work on SON for NG-RAN, RAN3 discussed enhancements to the RACH optimisation functions.

As part of such discussions RAN3 identified potential enhancements to the RACH report collected by UEs and signalled to the NG-RAN. Such enhancements are listed below:

- Enable the addition in the RACH Report of the feature priority of each feature in the feature combination used by the UE at the time RACH access is triggered. This enables the NG-RAN to determine whether any optimisation is needed with respect to how features with different priorities are combined in the same feature combination associated to a RACH partition

- Enable the addition in the RACH Report of RACH partition configuration information. This information consists of the start preamble index and the number of preambles in the partition for which the RACH Report was generated. This enables the NG-RAN to determine the RACH partition configuration at the time the RACH Report was generated. This enhancement is needed because the RACH partitions configurations may change with time and a RACH Report generated under certain RACH partition configurations may be reported to the NG-RAN after such RACH partition configurations have changed.

- 2. Actions:

RAN3 respectfully asks RAN2 to take it the above into account and to provide feedback on whether the proposed additions are feasible and can be specified in Rel18.

3. Date of next TSG RAN WG3 meetings:

3GPP RAN2#120	from 2023-05-22	to 2023-05-26		Incheon , KR