3GPP TSG-RAN WG3 Meeting #119-bis-e	R3-231587

# Online, April 17th – April 26th 2023

Title:	Reply LS on RACH enhancement for R18 SONMDT

Response to:	R3-231112(R2-2302066)

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	Ericsson (to be RAN3)

To:	TSG RAN WG2

Cc:	-

Contact Person:

Name:		Angelo Centonza

E-mail Address:	Angelo.Centonza (at) Ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

In the LS R3-231112/R2-2302066, it mentions:

RAN2 made the following agreements:

1: To have “a list of SN RA report entries as a single NR container (i.e. NR RA-ReportList)”.

And then, RAN2 discusses the following alternatives regarding how the UE includes the PSCell identities:

-	Alt 1: Includes unique PSCell identities, i.e. if a PSCell occurs more than once in NR RA-ReportList, it is recorded only once in the list of PSCell identities

-	Alt 2: Includes the last PSCell identity (in NR RA-ReportList)

RAN3 reply:

RAN3 thanks RAN2 for the questions. RAN3 has performed an analysis on both Alt-1 and Alt-2 proposed by RAN2 and concluded the following:

Alt-1 has the advantage that the entire report can be forwarded to all NG-RAN nodes serving PSCells included in the report. Respective recipient nodes are to discard irrelevant information. In Alt-2, the report is to be forwarded to a single NG-RAN node. It is the responsibility of the recipient node to forward the report to other NG-RAN nodes. Alt-2 creates a single point of failure at the NG-RAN node that has to redistribute the RA Reports. If the PSCell CGI is not included in the RA Report, the re-distribution NG-RAN node would not be able to forward the RA Report correctly.

In light of this discussion, RAN3 has reached the following decision:

Amongst the alternatives proposed by RAN2, Alt-1 is more robust and agreeable in RAN3.

# 2. Actions:

To RAN WG2 group.

ACTION: 	RAN3 respectfully asks RAN2 to take the above feedback into account

# 3	Dates of next RAN3 meetings

RAN3 Meeting #120	22-26 May 2023	                     Incheon, KR

RAN3 Meeting #121	21-25 August 2023	                     Toulouse, FR