3GPP RAN WG3 Meeting #119bis-e	R3-231637

# Online, 17th – 26th April, 2023

Agenda Item:	9.2.4

Source:	Ericsson, Qualcomm Incorporated, CATT

Title:		Discussion on support of preconfigured GAPs for different configured BWP for RedCap UE

Document for:	Discussion

# Introduction

In Rel-17 RedCap, BWP switching can happen in the following cases.

- RRC Reconfiguration based.

- Inactivity Timer expiry

- or DCI switch based among already configured BWPs to UE.

In last RAN3#119 meeting, a F1 CR [1] was agreed to introduce a list of candidate ServingCellMOs with the NCD-SSB information in the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST messages so that gNB-DU can select related measObjectId as servingCellMO for NCD-SSB. Based on the signalled information, the gNB-DU will indicate the list of ServingCellMO-encoded-in-CGC by UE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages, so that CU knows which MeasObjectIDs will be used by gNB-DU and gNB-CU can free up the resources for those unused measObjectIds. This solution covers the case when RRC Reconfiguration based BWP switch is performed.

The case of BWP switch via DCI has yet to be discussed for NCD-SSB case. RAN3 should discuss whether there will be any additional F1AP impacts for determination of Measurement GAPs by gNB-CU in case of BWP DCI switching.

# Discussion

In DCI based BWP switching scenario, with the introduction of preConfGapStatus-r17 IE in BWP-DownlinkDedicated IE in RRC (see extract from TS 38.331 below), the UE will autonomously know which Measurement GAP (MG) to apply after the active BWP associated with NCD-SSB is changed via DCI. Below the configuration option available in RRC, which is per BWP Config:

BWP-DownlinkDedicated IE -> preConfGapStatus-r17 : preConfGapStatus Indicates whether the pre-configured measurement gaps (i.e. the gaps configured with preConfigInd) are activated or deactivated upon the switch to this BWP. If this field is configured, the UE shall apply network-controlled mechanism for activation and deactivation of the pre-configured measurement gaps, otherwise the UE shall apply the autonomous activation/deactivation mechanism, as specified in TS 38.133 [14]. The first/leftmost bit corresponds to the measurement gap with gap ID 1, the second bit corresponds to measurement gap with gap ID 2, and so on. Value 0 indicates that the corresponding pre-configured measurement gap is deactivated while value 1 indicates that the corresponding pre-configured measurement gap is activated. The UE shall ignore the bit if the corresponding measurement gap is not a pre-configured measurement gap.

Further, in TS 38.133-i00, there is the following description:

### 9.1.7 Pre-configured measurement gap #### 9.1.7.1 Introduction A UE capable of Pre-configured measurement gap (Pre-MG) pattern can be configured with a Pre-MG pattern via RRC signalling [2]. The gap interruption requirements in Section 9.1.2 apply to Pre-MG when Pre-MG is activated, and no gap interruption is expected when Pre-MG is deactivated. - The requirements apply for NR standalone operation with single carrier and NR CA. #### 9.1.7.2 Requirements applicability The requirements related to pre-configured measurement gap apply provided: - UE indicates support of preconfiguredUE-AutonomousMeasGap [2] and/or preconfiguredNW-ControlledMeasGap [2], and - either a single per-UE measurement gap is pre-configured by the network, or one or two per-FR measurement gaps are pre-configured by the network, and - one of measurement gap patterns among measurement gap patterns #0 ~ #25 is configured for pre-configured measurement gap, and - UE is in NR SA with single carrier or with NR CA. A measurement gap is configured as pre-configured measurement gap if preConfigInd is indicated by network in the configuration message of the measurement gap. If UE indicates support of only preconfiguredNW-ControlledMeasGap [2], UE can expect the network to configure preConfGapStatus.

In TS 38.331 h40 6.3.3 UE capability information elements is defined as follows:

MeasAndMobParametersCommon ::= SEQUENCE { ... -- R4 19-3-2 pre-configured measurement gap preconfiguredUE-AutonomousMeasGap-r17 ENUMERATED {supported} OPTIONAL, -- R4 19-3-1 pre-configured measurement gap preconfiguredNW-ControlledMeasGap-r17 ENUMERATED {supported} OPTIONAL, ... ]]}

And in TS 38.306 h40 section 4.2.9

| preconfiguredUE-AutonomousMeasGap-r17 Indicates whether the UE supports the preconfigured measurement gap with UE-autonomous mechanism for activation and deactivation as specified in TS 38.133 [5]. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| preconfiguredNW-ControlledMeasGap-r17 Indicates whether the UE supports the preconfigured measurement gap with network-controlled mechanism for activation and deactivation as specified in TS 38.133 [5]. | UE | No | No | No |

Therefore, based on UE capability, the understanding of the framework is if UE indicates support of only preconfiguredNW-ControlledMeasGap, the UE can expect the network to configure all possible preconfigured measurement gaps that will be activated by the UE.

- Based on RAN2 and RAN4 definition of preConfGapStatus-r17 IE, for network-controlled mode, the UE expects the network to configure all possible preconfigured measurement gaps that will be activated by the UE in case of active BWP switching,

It is not clear however how both gNB-CU and gNB-DU can know the different pre-configured MGs for all the BWPs which will be used by UE, so that gNB-CU can prepare the RRC message to UE. In fact, gNB-CU needs to configure multiple measurement gaps in advance, covering all possible gaps considering the list of BWPs that can be configured active via DCI at some point in time, so that the gNB-CU can provide a presetting via RRC signaling to the UE. The UE will use this RRC pre-setting according to TS 38.331 to know which gaps it should apply when active BWP has been switched via DCI without any further RRC reconfiguration from the gNB-CU.

- To support the case of active BWP switching, the gNB-CU needs to provide in one RRC message all MGs required for multiple BWPs to UE, which UE will use to know which gap it should apply when active BWP is switched via DCI.

Considering the design of F1AP, we see that this signalling over F1 to support this case can be done as follows:

- First, the gNB-DU selects multiple BWPs for UE (i.e. BWPs are encoded in CellGroupConfig) and if DU may ask UE to change active BWP via DCI later, it should provide the selected BWPs (for frequencies outside and inside the BWP) to gNB-CU in UE CONTEXT SETUP RESPONSE/ UE CONTEXT MODIFICATION RESPONSE message.

- After receiving above response, if gNB-CU decides to configure preconfigured measurement gaps for all configured BWPs, gNB-CU should send UE CONTEXT MODIFICATION REQUEST to gNB-DU, e.g. in the CU to DU RRC Information IE, listing the BWP IDs and the measurement frequency config to generate preconfigured gaps.

Notes:

- After receiving above response, if gNB-CU determines also that measurement GAP is required for UE’s current active BWP, in the same UE CONTEXT MODIFICATION REQUEST, gNB-CU can still ask gNB-DU to generate it in legacy F1 IEs.

- Otherwise, gNB-CU will not to use the legacy IEs (in UE CONTEXT MODIFICATION REQUEST) to ask gNB-DU to generate measurement GAP for UE’s current active BWP, but only configuring preconfigured measurement GAP for UE’s current active BWP.

- Upon receiving the UE CONTEXT MODIFICATION REQUEST message with preconfigured measurement GAP generation request, gNB-DU generates different pre-configured measurement GAP for all BWPs in CellGroupConfig in BWP-change-via-DCI scenario.

- In case of success, the gNB-DU sends UE CONTEXT MODIFICATION RESPONSE message to gNB-CU with updated CellGroupConfig and measurement GAP configuration. This allows CU to send all MGs in single RRC Reconfig message to UE.

The flow-chart below presents the overall signalling where the new additions are in red:

Figure 1 : Sequence flow for preconfigured Measurement GAPs

With this signalling it can be possible to support the case of active BWP switching via DCI. Steps 6-8 can be skipped in case CU does not request DU to generate measurement gaps. However, in case the CU asks the DU to generated preconfigured MGs, two RRC Reconfiguration messages (step 4 and step 8) will have to be sent.

An option to avoid the first RRC Reconfiguration in step 4, could be to add exception text in F1AP procedures so that the gNB-CU does not trigger any RRC Reconfiguration when gNB-CU receives CGC in step 3 from gNB-DU for the case of   BWP specific servingCellMO configured for a RedCap UE. We note that exceptions for not sending the CellGroupConfig already exist in the case of SDT in TS 38.473 in the UE Context setup and modification procedures. However, it should be checked whether the second CGC could correspond to full configuration or to a delta config.

- To support the case of active BW switching via DCI for RedCap UE, the following actions are needed:

  - The gNB-DU must inform gNB-CU of the list of all configured BWPs information

  - The gNB-CU informs gNB-DU to generate different pre-configured measurement GAP for all the configured BWPs in CGC based on the information received previously from gNB-DU.

  - The gNB-DU schedules the list of preconfigured Meas GAPs to support network controlled meas preconfigured GAPs and signals the information to gNB-CU.

  - Add exception text in F1AP that the gNB-CU does not trigger UE RRC Reconfiguration for a RedCap UE configured with BWP specific ServingCellMO. Based on this, the gNB-CU sends the preconfigured MGs to UE via one RRC Reconfiguration signaling message.

Note: it should be checked whether the second CGC could correspond to full configuration or to a delta config.

A CR to F1AP capturing the proposed signalling is proposed in [2]

# Conclusions and Proposals

Our observations and proposals are summarized below.

- Based on RAN2 and RAN4 Rel-17 definition of preConfGapStatus-r17 IE, for network-controlled mode, the UE expects the network to configure all possible preconfigured measurement gaps that will be activated by the UE in case of active BWP switching,

- To support the case of active BWP switching, the gNB-CU needs to provide in one RRC message all MGs required for multiple BWPs to UE, which UE will use to know which gap it should apply when active BWP is switched via DCI.

- To support the case of active BW switching via DCI for RedCap UE, the following actions are needed:

  - The gNB-DU must inform gNB-CU of the list of all configured BWPs information

  - The gNB-CU informs gNB-DU to generate different pre-configured measurement GAP for all the configured BWPs in CGC based on the information received previously from gNB-DU.

  - The gNB-DU schedules the list of preconfigured Meas GAPs to support network controlled meas preconfigured GAPs and signals the information to gNB-CU.

  - Add exception text in F1AP that the gNB-CU does not trigger UE RRC Reconfiguration for a RedCap UE configured with BWP specific ServingCellMO Based on this, the gNB-CU sends the preconfigured MGs to UE via one RRC Reconfiguration signaling message.

A CR to F1AP capturing the proposed signalling is proposed in [2]

# References

R3-231063, Correction to support NCD-SSB RedCap requirements in F1AP, CR to TS 38.473 v17.3.0, Ericsson, CATT, China Telecom, Qualcomm Inc., ZTE, Huawei, Nokia, Nokia Shanghai Bell

R3-231637, Support of preconfigured Measurement GAPs for RedCap UE, CR to TS 38.473 v17.4.1, Ericsson