3GPP TSG-RAN WG3 Meeting #119bis-e  	R3-231655

E-meeting, 17 April – 26 April 2023

Agenda item:	12.2.1

Source:	Nokia, Nokia Shanghai Bell, Orange

Title:	(TP for TS 38.300) AI/ML General Aspects

Document for:	Text Proposal

# 1 	Introduction

In earlier meetings the following has been agreed:

RAN3 #117e

Define a new procedure over Xn which can be used for AI/ML related information, e.g., predicted information

RAN3 #117bis-e

Procedures used for AI/ML support in the NG-RAN shall be use case agnostic.

The cases of i) Model Training and Model Inference at the NG-RAN and ii) Model Training at OAM and Model Inference at the RAN, make use of the same procedures, with the exception that procedures for exchange of training data and feedback data (which is not related to model performance feedback) will be different for i) and ii).

Introduce a new Class 1 procedure for initiating the reporting of AI/ML Related Information and a Class 2 procedure for Data Reporting of AI/ML Related Information.

Legacy information that are used to support AI/ML are transferred via existing legacy procedures (no need to signal them via other procedures)

RAN3 #118

WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”.

RAN3 #119

The agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE performance feedback reporting.

The agreed new class2 non-UE associated procedure (AI/ML INFORMATION UPDATE, which name is FFS) is used for UE performance feedback reporting.

The metric of Energy Cost (EC) exchanged between NG-RAN nodes can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighboring node for either additional load or current load (The details to be further discussed). EC is a value at gNB level.

In this contribution we provide a discussion for the above WA (RAN3#118) and provide a TP for BL CR to TS 38.300 based on the above agreements.

# 2 	Discussion

The motivation behind capturing the WA “Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”.” was to keep transparent in the request and in the response messages of the introduced AI/ML procedures whether the requested information is needed for input or feedback of an AI/ML algorithm or whether it was produced as part of model output. In our view the purpose for which data is requested is not needed and can in fact create confusion since feedback information is another form of input information to an AI/ML algorithm. In addition, input information for an AI/ML algorithm is output information for another. However, we also think that the term “data type agnostic” is not very clear to reflect the actual intention. It is possible to determine from the request message whether a model output is requested that involves predictions. However, the procedure shall not reflect explicitly the purpose for which data will be used and whether it will be used for input or for feedback. Therefore, we propose to confirm the working assumption with a slight modification in the text.

Proposal 1: The procedures used for AI/ML support in the NG-RAN shall be “data type agnostic” in the sense that the purpose for which data is requested (e.g., input, output, feedback) shall not be indicated.

Based on the above agreements and proposal we propose a TP for BL CR to TS 38.300.

Proposal 2: Accept the TP for BL CR to TS 38.300 in the Annex.

# 3 	Conclusion

In this contribution we make the following proposals:

Proposal 1: The procedures used for AI/ML support in the NG-RAN shall be “data type agnostic” in the sense that the purpose for which data is requested (e.g., input, output, feedback) shall not be indicated.

Proposal 2: Accept the TP for BL CR to TS 38.300 in the Annex.

# Annex – TP for BL CR for TS 38.300

<<< start of changes >>>

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

<Unchanged Text Omitted>

EC	Energy Cost

<Unchanged Text Omitted>

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

ML	Machine Learning

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

<<< next change >>>

## X.X AI/ML for NG-RAN

### X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

### X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN.

•	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN.

The two scenarios make use of the same signalling procedures except for signalling needed for training and feedback data in case AI/ML Model Training is located in the OAM. All signalling procedures used for AI/ML within the NG-RAN are use case and data type agnostic, in that the purpose for which AI/ML information is requested (e.g., input, output, feedback) is not indicated. The AI/ML function may use any suitable signalling procedure, including specific AI/ML procedures, to exchange AI/ML information between NG-RAN nodes.

<<< next change >>>

### X.X.3 AI/ML Optimization

#### X.X.3.1 Support for AI/ML Load Balancing

The aim of this function is to exchange, over network interfaces, information (input, output or feedback) enabling AI/ML Model Training and Inference for AI/ML-optimized load balancing decisions.

#### X.X.3.2 Support for AI/ML Mobility Enhancements

The aim of this function is to exchange, over network interfaces, information (input, output or feedback) enabling AI/ML model training and inference for AI/ML-optimized mobility decisions.

#### X.X.3.3 Support for AI/ML Energy Saving

The aim of this function is to exchange, over network interfaces, information (input, output or feedback) enabling AI/ML model training and inference for AI/ML-optimized energy saving decisions. In order to evaluate the impact of an AI/ML action, an Energy Cost (EC) metric is defined. The metric of EC exchanged between NG-RAN nodes can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighboring node for either additional load or current load.

<<< end of changes >>>