3GPP TSG-RAN WG3 Meeting #119bis-e 	R3-231660

E-meeting, 17 April – 26 April 2023

Agenda item:	12.3

Source:	Nokia, Nokia Shanghai Bell, Qualcomm

Title:	MDT Data Collection Continuity for NG-RAN AI/ML

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #119, we made the following WA:

WA: There are the benefits in enabling UEs to continuously collect MDT measurement across RRC states and to provide to the network such continuous time series of data for the purpose of AI/ML Training/Retraining. The evaluation of gains vs impacts related to the use cases in R18 coming from potential solutions needs to be further discussed.

Solutions on how to achieve Continuous MDT are FFS

In this contribution we provide our views related to the above WA and open point.

# 2	Background

During the study item, we captured in TR 37.817 that “if existing UE measurements are needed by a gNB for AI/ML-based network energy saving, RAN3 shall reuse the existing framework (including MDT and RRM measurements).” In earlier meetings, the topic of continuous data collection across RRC states has been brought up by certain companies. The motivation behind the various proposals is different. Some companies propose that it is necessary for a UE to log MDT location information continuously in order to predict UE trajectory based on detailed location information. The motivation behind this requirement is that in case some location information is missing, a node cannot properly make a trajectory prediction or execute model inference, e.g. when model implementations are based on LSTM (Long Short-Term Memory) neural networks. Another proponent of continuous MDT focuses on the continuous collection of location data (for the UE trajectory prediction) and proposes to ensure sufficient continuity by introducing location data recording interval. Finally, another related idea on continuity focuses on configuring immediate and logged MDT in the same Trace Activation from core network to UE. The motivation in this latter case is to enable calibration of UE measurements among RRC states.

# 3	Benefits of Continuous Measurement Data Across RRC states

In some scenarios the collection of continuous or uninterrupted series of measurements can be beneficial. One such example is trajectory prediction which is an enabler for the Rel-18 use cases, in particular, for the AI/ML Mobility use case. Other examples could be found in use cases not covered in Rel-18, like MIMO optimization. Training an ML Model for time-series forecasting may require a time series of measurements. To improve prediction accuracy and to be able to increase the prediction horizon, data sets need to contain a large number of measurements.

Observation 1: There exist some ML Algorithms, e.g., based on time-series forecasting that may be used to predict UE trajectory, that require a large number of continuous measurement series to make a prediction (forecast).

For example, an ML Model predicting detailed-location trajectory may use detailed location information from UEs, provided e.g., through MDT or RRM measurements. To be able to characterize detailed-location trajectories not only for RRC connected, but also for UEs that have transited to RRC idle or inactive states detailed location information may need to be collected across RRC states from the UEs.

Observation 2: To train an ML Model that predicts detailed-location trajectory, network may need to collect detailed location information across RRC states from the UEs.

Current MDT framework in fact already supports continuous data collection with different variations. Signaling-based MDT methods enable core network to configure a given UE, identified e.g., by its IMSI, IMEI, SUPI, both with Logged MDT and Immediate MDT, if UE capability supports them both. In this way, the operator could target data collection from a specific UE of interest. This configuration could enable the operator to determine the reasons behind performance degradation for a specific UE, by continuously monitoring its measurements across RRC states.

Observation 3: Signaling-based MDT allows operator to monitor a specific UE continuously across RRC states in order to determine performance degradation and to improve performance.

Furthermore, for Management-based MDT, a UE in RRC idle state that is configured with Logged MDT, indicates to network data availability when it goes to RRC connected state. At this point, the network may, if needed, subsequently configure the UE with Immediate MDT. Similarly if a UE is in RRC connected state and switches to RRC idle or inactive it may provide the network with logged MDT information to deliver to network when it goes back to RRC connected state, including the full reference of the logged MDT session (TR/TRSR) . If a UE is transitioning among RRC connected and RRC idle/inactive states within the same NG-RAN node, based on the provided MDT session references, the current MDT framework therefore enables the network to constitute a continuous (uninterrupted) measurement series by linking the Immediate MDT data and the Logged MDT data.  In inter-gNB scenarios, the TCE can constitute continuous measurement series based on the availability of permanent UE ID. As it can be observed, a UE can be configured by the operator with two different trace sessions, one for Logged MDT and one for Immediate MDT, and the UE can still be identified by the operator.

Observation 4: If a UE is transitioning among RRC connected and RRC idle/inactive states within the same NG-RAN node, current MDT framework allows a UE to provide the network with continuous reporting across RRC states. In inter-gNB scenarios, the TCE can constitute continuous measurement series based on the availability of permanent UE ID.

In practice, continuity in time in the training data set may not be always guaranteed. Some measurements may be missing as part of the data collection process because they could not be provided. Furthermore, the time-granularity with which some measurements are collected may be coarse either intentionally (to minimize the impacts produced by the load of the populated data) or because finer granularity cannot be collected. In practice, data collection poses signaling overhead in a network and therefore in most practical scenarios, the actual collected data sets may be small compared to the size of the problem that needs to be solved. Data science has derived different techniques to overcome such issues. For example, data augmentation is a technique that can be used to increase the size and diversity of the data set by transformations of the original data. As another example, another technique is to produce synthetic data that can be created artificially without using the original sets of data.  Interpolation may be used as part of these techniques for AI/ML where new data points are created to fill in gaps in data sets that take values within a certain range. How to realize these techniques depends on each ML Model and is up to implementation. Data will be prepared (augmented, labelled, processed) tailored to a particular ML Model.

Observation 5: Data preparation and continuity in the data collection is part of the Model Training entity and is achieved by implementation using various techniques depending on the details of the ML Model and the available data.

On another separate aspect, for AI/ML data collection, an equally important aspect to data continuity is data diversity. Even though, as discussed, data continuity can be created using Data Science techniques in the Model Training, data diversity also determines the quality of data collection. A good training data set shall contain data characterizing as broadly as possible the optimization problem. As an example, having a large amount of UE measurements over the same area or with very similar values (e.g., RSRP or location) is not very useful and is also costly in terms of signaling load and required storage for the collected information. Diversity depends also on the ML Model and may depend on an area (e.g., which percentage of an area should be covered by the data collection), time (e.g., how to evenly collect data per time unit), sources (e.g., data should represent as broadly as possible a UE distribution and not be collected from the same UE).

Observation 6: Diversity in the data collection is an equally important requirement for creation of a good Training Data Set.

# 4	Impacts of Continuous MDT across RRC states

As discussed in the previous section, continuity in the data collection is already supported in the current MDT framework based on linking immediate MDT measurement logs and logged MDT measurement logs in the NG-RAN or in the TCE. Any new requirement for continuous UE logging across RRC states would impact the UE and would not be applicable for pre-Rel-18 UEs.

In practice, data collection provided by Rel-18 UEs only would result in data collected from a limited number of UEs, and hence result in a significant downside when it comes to fulfilling the diversity requirement for creation of a good training dataset, without providing any corresponding benefit.

Observation 7: Any new Rel-18 UE requirement for data collection would negatively impact training dataset diversity.

Diversity in the data provision has been one of the principles in Management-based MDT where a large number of UEs is targeted so that data collection is maximized. MDT measurements may be collected from all legacy UEs in RRC connected mode (immediate MDT), and from all UEs in RRC idle/inactive capable of legacy logged MDT.

In addition, if a UE is configured to log (continuously) under a NG-RAN node goes to RRC idle and gets back to RRC connected state at a different NG-RAN node, it is not possible, nor relevant, to identify the UE at the old NG-RAN node, and such requirement would pose a security risk for the UE  linked to the introduction of a permanent UE ID in the RAN.

In addition, the motivation to force continuous logging is unclear based on our use cases. All the RAN3 Rel. 18 use cases in NG-RAN AI/ML are connected mode use cases. For this reason, when it comes to RSRP measurements provided by the UE it is unlikely that the corresponding AI/ML models that generate Model Inference in RRC Connected mode will find RSRP measurements from RRC Idle state to be a useful input. When it comes to location information, the trajectory prediction under discussion for Mobility Optimization is cell-based predicted UE Trajectory, which can be obtained using UE History Information from UE and neighbouring nodes. We believe that consecutive detailed UE location information is not needed to train an ML Model for cell-based UE Trajectory prediction.

Observation 8: Continuous MDT for detailed location information does not have clear benefits for the solutions related to the Rel.18 NG-RAN AI/ML use cases e.g., for cell-based UE Trajectory prediction.

For cell-based UE trajectory prediction, instead of detailed location information, sequences of cells a UE has visited will need to be propagated to the node that performs the Model Training. It therefore seems to us more appropriate that for the predicted UE Trajectory use case, we need to maintain continuity in the sequences of cells a UE has visited across RRC states.

Observation 9: For cell-based UE trajectory prediction, cell-based continuity information is needed across RRC states.

On another aspect, we believe that MDT could not possibly solve the issue of continuity in the UE measurements, if that were required. Logging across RRC states, faces also the issues of the limited UE memory. Given that the actual time a UE spends in RRC Idle or Inactive state is not under network control, there are no guarantees that the UE memory will not be exceeded before it transitions to RRC connected state where it can indicate data availability to the network. If the memory is exceeded, then the UE will replace older entries in the log with the newer ones. The same problem exists if UE logs information while in RRC Connected or RRC Idle states.

Observation 10: Due to the limited UE memory, a UE in RRC idle or Inactive state may replace older entries with newer information creating discontinuity in the actual reported measurements.

For the above reasons, we think that continuous MDT enhancements should not be considered in Rel.18.

Proposal: Do not consider continuous MDT enhancements for cell-based UE Trajectory prediction.

# 5 	Conclusion

In this paper we make the following observations and proposal:

Observation 1: There exist some ML Algorithms, e.g., based on time-series forecasting that may be used to predict UE trajectory, that require a large number of continuous measurement series to make a prediction (forecast).

Observation 2: To train an ML Model that predicts detailed-location trajectory, network may need to collect detailed location information across RRC states from the UEs.

Observation 3: Signaling-based MDT allows operator to monitor a specific UE continuously across RRC states in order to determine performance degradation and to improve performance.

Observation 4: If a UE is transitioning among RRC connected and RRC idle/inactive states within the same NG-RAN node, current MDT framework allows a UE to provide the network with continuous reporting across RRC states. In inter-gNB scenarios, the TCE can constitute continuous measurement series based on the availability of permanent UE ID.

Observation 5: Data preparation and continuity in the data collection is part of the Model Training entity and is achieved by implementation using various techniques depending on the details of the ML Model and the available data.

Observation 6: Diversity in the data collection is an equally important requirement for creation of a good Training Data Set.

Observation 7: Any new Rel-18 UE requirement for data collection would negatively impact training dataset diversity.

Observation 8: Continuous MDT for detailed location information does not have clear benefits for the solutions related to the Rel.18 NG-RAN AI/ML use cases e.g., for cell-based UE Trajectory prediction.

Observation 9: For cell-based UE trajectory prediction, cell-based continuity information is needed across RRC states.

Observation 10: Due to the limited UE memory, a UE in RRC idle or Inactive state may replace older entries with newer information creating discontinuity in the actual reported measurements.

Proposal: Do not consider continuous MDT enhancements for cell-based UE Trajectory prediction.