3GPP TSG-RAN WG3 #119bis-e                                                                                R3-231671

Online, 17th – 26th April, 2023

Title: 	Further discussion on MT-SDT

Source: 	CATT

Agenda item:	20.2

Document for:	Discussion and Decision

- Introduction

In the last RAN3 meeting, we initially discussed the support of MT-SDT, some aggreements have been achieved, and some details are still open, as below:

MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling. Upon reception of DL SDT user data, the gNB-CU-UP may include the assistance information (e.g., Data size) in E1AP DL Data Notification message to gNB-CU-CP. FFS on MT-SDT indicator. When receiving DL SDT data, the anchor gNB may send MT-SDT information IE to the neighbour gNBs within the RNA, via XnAP RAN paging message. The encoding and the name of MT-SDT information IE is FFS. The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging. Upon reception of MT-SDT information via XnAP RAN paging message from the anchor gNB-CU, the gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message. FFS on F1AP MT-SDT information FFS: whether Data volume should also be provided from the CU to the DU in F1AP PAGING message. FFS: whether and how XnAP RTRV UE CTXT REQ message (that carries MT-SDT resume indication) FFS: whether MT-SDT support indication in E1 Bearer Context procedure should be defined to enable the gNB-CU-UP to include the DL data size in the E1AP DL DATA NOTIFICATION message. FFS: whether receiving gNB-CU or receving gNB-DU decides MT-SDT Uu paging, if we agree that it is the receving gNB to make the final decision on triggering MT-SDT Uu paging. FFS: MT-SDT assistance information sent from the anchor in the XnAP: RAN paging message and other alignment with RAN2 progress.

In this contribution, we will further discuss the details on supporting of the MT-SDT, and provide the observations and proposals accordingly.

- Discussion

  - MT-SDT Information over Xn/F1/E1

For the XnAP, some of the companies prefer to allow the new gNBs to further decide whether to trigger MT-SDT Uu Paging, and this is already an agreement.

The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging.

However, some of the companies preferred not to change the decision made by the anchor gNB. A potential compromise is to include a MT-SDT indicator as mandatory info in MT-SDT information, and optionally include some other assistance info. If MT-SDT Information is not provided, which means the anchor has decided to follow the legacy behaviors as handling of non-SDT in inactive. If only the MT-SDT indicator is included in the MT-SDT information, which means the receiving gNB should follows the indicator to do MT-SDT Uu Paging. If MT-SDT Indicator and the assistance info are provided, which means the receiving gNB may decide to trigger MT-SDT Uu paging or not by implementation.

For the encoding of the MT-SDT information IE, SDT Support Request IE used in UE Context Retrieval procedure could be referred.

9.2.3.163	SDT Support Request

This IE indicates that the UE requested for SDT and may include additional assistance information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SDT Indicator | M |  | ENUMERATED (true,…) |  |
| SDT assistant information | O |  | ENUMERATED (single packet, multiple packets, …) | “Single packet” indicates no subsequent SDT transmission is expected; “Multiple packets” indicates subsequent SDT transmission is expected. |

9.2.3.xxx	MT-SDT Information

This IE indicates that the the RAN Paging is for MT-SDT and may include additional assistance information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Indicator | M |  | ENUMERATED (true,…) |  |
| MT-SDT assistant information | O |  | FFS |  |

Proposal 1: In Xn Paging, MT-SDT information should include a MT-SDT Indicator, and optionally include MT-SDT assistance info.

On the MT-SDT assistance information, we understand there’s no need to distinguish signalling or data over Xn. single or multiple packects could be included to indicate single or multiple signalling and/or data have been received by the last serving gNB. Besides that, data volume of the pending DL data/signalling could also be considered.

Proposal 2: In Xn Paging, MT-SDT assistance information may include single or multiple DL packets indicator, and/or data volume of the DL data/signalling.

In CU-DU split case, as gNB-DU is responsible for generating the Uu Paging on top of the F1 Paging message, it’s agreed that gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message. However, it’s FFS on the F1AP MT-SDT information.

Upon reception of MT-SDT information via XnAP RAN paging message from the anchor gNB-CU, the gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message. FFS on F1AP MT-SDT information FFS: whether Data volume should also be provided from the CU to the DU in F1AP PAGING message. FFS: whether receiving gNB-CU or receving gNB-DU decides MT-SDT Uu paging, if we agree that it is the receving gNB to make the final decision on triggering MT-SDT Uu paging.

If the final decision is made in gNB-CU, the gNB-CU only needs to provide a MT-SDT indicator to gNB-DU in F1AP. Or else, the MT-SDT assistance info should also be provided to gNB-DU, in this case, the MT-SDT Information defined for Xn Paging could be reused for F1 Paging. To make things easier, we propose to align with XnAP.

Proposal 3: F1AP MT-SDT information could align with the MT-SDT information used for Xn Paging, which means the assistance info may also be provided to gNB-DU.

In CP UP split architecture, when the MT-SDT data arrives, the gNB-CU-UP should indicate the arrival of DL data via DL DATA NOTIFICATION message. The existing notification is on PDU session and QoS flow level, we understand that without any enhancement on DL DATA NOTIFICATION, the gNB-CU-CP could deduce that whether the DL data is received in the SDT bearer(s), as the gNB-CU-CP knows the mapping between data bearers and QoS flows, and which data bearers are applicable to SDT.

Observation 1: Without any enhancement on DL DATA NOTIFICATION in E1AP, gNB-CU-CP could know the DL data is received in the SDT bearer(s).

However, gNB-CU-CP does not know the data volum received in each Qos flow, it may not be able to decide to perform MT-SDT procedure or legacy non-SDT procedure. Thus, it’s benefical to indicate the data volume received on the SDT bearer(s), per UE notification or per bearer notification are both fine, per UE notification is sufficient. For the data received on the non-SDT bearers, no need to provide the data volume towards the gNB-CU-CP.

Observation 2: Without more assistance information e.g. data volume of the DL data received in the CU-UP, gNB-CU-CP may not able to decide to perform MT-SDT procedure or legacy non-SDT procedure.

Proposal 4: Indicates gNB-CU-CP the Data volume received on the SDT bearers in the gNB-CU-UP.

  - MT-SDT overall procedures

FFS: whether and how XnAP RTRV UE CTXT REQ message (that carries MT-SDT resume indication)

To response the Paging for MT-SDT, UE may initiate the RRCResumeRequest message, and RAN2 agreed to introduce a new Resume cause, one code point MT-SDT indication. Then we should consider whether need to explicitly indicate the last serving gNB the RRC Resume is for MT-SDT?

Currently, the RRC Resume Cause introduced in XnAP RETRIEVE UE CONTEXT REQUEST is only used for RNA update case, as below:

9.2.3.61 RRC Resume Cause The purpose of the RRC Resume Cause IE is to indicate to the old NG-RAN node the reason for the RRC Connection Resume as received from the UE in the ResumeCause defined in TS 36.331 [14] and TS 38.331 [10]. In this version of the specification, this is limited to the case of RNA update. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RRC Resume Cause; M; ENUMERATED (rna-Update, ...).

Observation 3: In existing XnAP, the RRC Resume Cause is limited to the case of RNA update.

In the last RAN2 meeting, they agreed that during RRCResume for MT-SDT, no need to check availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT).

Agreements - Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT) - UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures - If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

As above, the 1st RRC resume is for MT-SDT, MO-SDT or non-SDT, we do not need to consider the concurrent case, e.g. MT+MO SDT. This means when RRCResume is initiated for MT-SDT, no UL data or signalling is accompanied.

Proposal 5: No need to consider the concurrent case, i.e. MT+MO SDT when initiating the RRC Resume procedure. No UL data or signalling is accompanied when RRCResume is initiated for MT-SDT.

Following the agreements of RAN2, in case the RRC resume cause is MT-SDT, there’s no UL data or signalling received together with the RRCResumeRequest. In the UE Context Retrieve Request, the existing SDT Indicator in the SDT Support Request IE could be reused, the other assistance info is not included.

Proposal 6: No need to explicitly indicate the resume cause is MT-SDT in the XnAP RETRIEVE UE CONTEXT REQUEST message, reusing of the existing SDT Indicator is sufficient.

Samilar to the MO-MDT procedure as we discussed in Rel-17, the last serving gNB may decide to relocate the UE context or not for MT-SDT transmission. Correspondingly, we provide the overall procedures “MT-SDT with UE Context Relocation” and “MT-SDT with UE Context Relocation” separately in the sub-sections.

Proposal 7: the last serving gNB could decide to relocate the UE context or not for MT-SDT transmission, both MT-SDT with and without UE context relocation should be considered.

As above, the oveall procedure for MT-SDT could be illustrated with the figure below:

Figure 1: MT-SDT overall procedure

- DL SDT data and/or DL SDT signalling are received at the last serving gNB for the UE in RRC_INACTIVE state.

- RAN Paging is triggered, the receiving gNB takes account the MT-SDT information received in RAN Paging from the last serving gNB to decide whether to trigger the MT-SDT Paging towards the UE.

- If the UE has been successfully reached, it may initiate the SDT procedure as described in 18.2 or 18.3 with the following difference:

- The UE indicates the RRC Resume Request is for MT-SDT, without UL data/signalling.

Corresponding TP for BL CR to TS 38.300 is provided in [3], taking account the comments received during the offline and online discussion the last meeting, also taking account the RAN2 agreements. RAN3 is requested to discuss and agree the TP.

Proposal 8: Discuss and agree the TP for MT-SDT BL CR to TS 38.300 [3].

3. Conclusion

In this contribution, we discussed how to make support of MT-SDT. Based on the discussion above, we provide the following observations and proposals:

Proposal 1: In Xn Paging, MT-SDT information should include a MT-SDT Indicator, and optionally include MT-SDT assistance info.

Proposal 2: In Xn Paging, MT-SDT assistance information may include single or multiple DL packets indicator, and/or data volume of the DL data/signalling.

Proposal 3: F1AP MT-SDT information could align with the MT-SDT information used for Xn Paging, which means the assistance info may also be provided to gNB-DU.

Observation 1: Without any enhancement on DL DATA NOTIFICATION in E1AP, gNB-CU-CP could know the DL data is received in the SDT bearer(s).

Observation 2: Without more assistance information e.g. data volume of the DL data received in the CU-UP, gNB-CU-CP may not able to decide to perform MT-SDT procedure or legacy non-SDT procedure.

Proposal 4: Indicates gNB-CU-CP the Data volume received on the SDT bearers in the gNB-CU-UP.

Observation 3: In existing XnAP, the RRC Resume Cause is limited to the case of RNA update.

Proposal 5: No need to consider the concurrent case, i.e. MT+MO SDT when initiating the RRC Resume procedure. No UL data or signalling is accompanied when RRCResume is initiated for MT-SDT.

Proposal 6: No need to explicitly indicate the resume cause is MT-SDT in the XnAP RETRIEVE UE CONTEXT REQUEST message, reusing of the existing SDT Indicator is sufficient.

Proposal 7: the last serving gNB could decide to relocate the UE context or not for MT-SDT transmission, both MT-SDT with and without UE context relocation should be considered.

Proposal 8: Discuss and agree the TP for MT-SDT BL CR to TS 38.300 [3].

4. Reference

[1] RP-213583 New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR

[2] RAN2#121 Chair Notes

[3] R3-231672 (TP for MT-SDT BL CR to TS 38.300) Support of MT-SDT