# 3GPP TSG-WG3 Meeting #119bis              	   R3-231794

# Online, Apr 16th - 27th , 2023

# Source: 			CMCC

# Title:  (TP to TS 38.401)Considerations on multi-path for SL relay

# Agenda item:	16.4

# Document for: 	Discussion

Introduction

In last RAN2 and RAN3 meeting, we discussed multi-path adding/release/path change procedure for inter-DU and inra-DU cases for scenario 1. Some conclusions are agreed as follow. In this paper, we further discuss other procedure for both scenario 1 and scenario 2.

Agreements:

Add Inter-DU Direct Path Addition and Inter-DU Indirect Path Addition procedures in TS 38.401. FFS on other procedures.

The direct path and indirect path cannot be configured for a remote UE simultaneously in this release, depending on RAN2 decision.

For inter-DU case, legacy DC based data split/duplication mechanism can be reused as baseline for split DRB/SRB.

The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition.

Previous progress：

For Scenario 1, the responsibility of gNB-CU and gNB-DU in Rel-17 SL relay can be reused as a baseline. Whether to enhance the responsibility of gNB-CU and gNB-DU in Rel-18 is FFS.

For the responsibility of gNB-CU and gNB-DU in Scenario 2, the RAN3 waits for RAN2’s progress on protocol stack for Scenario 2.

WA: The RAN3 will specify the details of the path change procedure after introducing the procedure of the direct/indirect path addition.

Discussion

Intra-DU direct path adding procedure for scenario 1

Fig.1 intra-DU direct path adding for multi-path in scenario 1

- Remote UE reports trigger event relative information to gNB-CU. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

- gNB-CU decides to add direct path to the remote UE under same gNB-DU.

- gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message to gNB-DU, which contains the information to add new direct path.

- gNB-DU responses with the UE CONTEXT MODIFICATION RESPONSE message to gNB-CU, which contains the configurations used for direct path.

- gNB-CU reconfigures the remote UE and relay UE.

- The remote UE accesses gNB-CU via configured direct path.

Intra-DU indirect path adding procedure for scenario 1

Fig.2 intra-DU indirect path adding for multi-path in scenario 1

- 1. Remote UE reports trigger event relative information to gNB-CU. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

- 2. gNB-CU decides to add indirect path to the remote UE under same gNB-DU.

- 3. gNB-CU reconfigures the relay UE.

- 4. gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message to gNB-DU, which contains the information to add indirect path.

- 5. gNB-DU responses with the UE CONTEXT MODIFICATION RESPONSE message to gNB-CU, which contains the configurations used for indirect path.

- 6. gNB-CU reconfigures the remote UE.

- 7. The remote UE establishes PC5 connection with relay UE.

- 8. The remote UE accesses gNB-CU via the indirect path.

Inter-DU indirect path adding procedure for scenario 2

In last meeting some cases about direct and indirect path add/ release are discussed in RAN3, and overall procedure of inter-DU direct/indirect path adding are agreed.

For scenario 2, the relative discussion is on going in RAN2 parallel with scenario 1. We suggest discuss the inter-DU indirect path adding procedure for scenario 2.

As in Fig.3 and Fig.4, remote UE with direct path adds the indirect path for multi-path for scenario 2,

Fig.3 inter-DU indirect path adding for multi-path in scenario 2

- 1. Remote UE reports trigger event relative information to gNB-CU (FFS), or gNB-CU triggered indirect path adding with gNB implementation (FFS). The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs. (FFS)

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under a different gNB-DU (i.e., gNB-DU2).

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU2 and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU2, which contains the indirect path configuration at least.

5.	The gNB-DU2 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU1. The contents in the RRCReconfiguration message may include at least indirect path addition configuration, bearer mapping and the associated radio bearer(s).

7.	gNB-DU1 sends the RRCReconfiguration message to the remote UE.

8.	The remote UE establishes non-3GPP connection with relay UE.

9.	The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU1.

10.	The gNB-DU1 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

Intra-DU indirect path adding procedure for scenario 2

Fig.4 intra-DU indirect path adding for multi-path in scenario 2

- 1. Remote UE reports trigger event relative information to gNB-CU. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

- 2. gNB-CU decides to add indirect path to the remote UE under same gNB-DU.

- 3. gNB-CU reconfigures the relay UE.

- 4. gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message to gNB-DU, which contains the information to add indirect path.

- 5. gNB-DU responses with the UE CONTEXT MODIFICATION RESPONSE message to gNB-CU, which contains the configurations used for indirect path.

- 6. gNB-CU reconfigures the remote UE.

- 7. The remote UE establishes non-3GPP connection with relay UE.

- 8. The remote UE accesses gNB-CU via the indirect path.

Proposal 1: Agree the TP to TS 38.401 as in the Annex.

Protocol stack aspect

In RAN2 #119bis, the details about the UE-to-UE link and relay UE to gNB link for scenario 2 are discussed. Those following progress may have impacts on RAN3:

- Do not specify adaptation layer over UE-to-UE link for scenario 2 in RAN2.

- For Scenario 2, different Uu logical channels are configured for identification of data directed to/originating from the relay UE and data relayed from/to the remote UE over the Uu link of the indirect path, as in Rel-17.

- UE identification is not needed over Uu link in Scenario 2, if relay UE serves only one remote UE and different Uu RLC channels can be assumed for the remote UE and the relay UE.

- Bearer identification except LCID is not needed in L2 PDU over Uu link in Scenario 2. Only 1:1 bearer mapping is supported over Uu link for the indirect path.  FFS how to configure the mapping.

- Without the adaptation layer over Uu link in scenario 2, a PDCP PDU can be delivered to an intended PDCP entity or RLC entity for support of more than one RB over Uu link e.g. by configuring 1:1 bearer mapping and different Uu RLC channels for relay UE local traffic and relay traffic for PDU delivery.

- Do not specify adaptation layer over Uu link for scenario 2 in RAN2.

For Scenario 1, the responsibility of gNB-CU and gNB-DU in Rel-17 SL relay can be reused as a baseline. Comparing with scenario 1, some difference in scenario 2 can be considered during the discussion on gNB-CU and gNB-DU responsibility:

- adaption layer for both the UE to UE link and UE to gNB link are not need, local remote UE ID allocation is not needed.

- the UE to UE link is non-standard interface

- Remote UE is connected to one relay UE in this release

- Remote UE bearer and relay UE RLC channel is 1:1 mapping

For the responsibility of gNB-CU and gNB-DU in Scenario 2, the RAN3 waits for RAN2’s progress on protocol stack for Scenario 2

- Discovery is not needed, resource pool is not needed.

- Regarding to 1-5) difference in scenario 2, the responsibility of the gNB-DU and gNB-CU can be listed as:

gNB-CU’s responsibility:

  - remote UE and relay UE context maintenance

  - remote UE bearer mapping

  - Relaying Uu RLC channel management

  - E2E QoS split management for relaying

  - Data split/duplication conducted on the PDCP layer

gNB-DU’s responsibility:

  - Determine the RLC/MAC/PHY Configuration for the relaying Uu RLC channels of relay UE

Proposal 2: It is proposed to agree the responsibility of the gNB-DU and gNB-CU for scenario 2 as follow:

gNB-CU’s responsibility:

  - remote UE and relay UE context maintenance

  - remote UE bearer mapping

  - Relaying Uu RLC channel management

  - E2E QoS split management for relaying

  - Data split/duplication conducted on the PDCP layer

gNB-DU’s responsibility:

  - Determine the RLC/MAC/PHY Configuration for the relaying Uu RLC channels of relay UE

Impact of UE association in scenario 2

In scenario 2, the link between remote UE and relay UE is assumed as ideal connection. However, gNB should identify the association between the 2 UEs before the indirect path adding. As we agreed in SI, this relation between remote UE and relay UE is assumed as pre-configured or static. In RAN2 #121 meeting, it agreed to deprioritize association mechanism between remote UE and relay UE from CN to RAN. Another details about UE association have not been fully discussed for lack of time. In our understanding, gNB pre-configures the associated UE information to an authorized UE. The UE association issues may have impacts on RAN3, for example, how to define the UE identity for Relay UE, the UE association establishment/ release / modification. RAN3 can wait for RAN2 progress on that.

- Proposal 3: Ask RAN3 to wait for RAN2 progress on UE association issues in scenario 2.

Conclusions

According the above discussion we have following proposals:

Proposal 1: Agree the TP to TS 38.401 as in the Annex.

Proposal 2: It is proposed to agree the responsibility of the gNB-DU and gNB-CU for scenario 2 as follow:

gNB-CU’s responsibility:

  - remote UE and relay UE context maintenance

  - remote UE bearer mapping

  - Relaying Uu RLC channel management

  - E2E QoS split management for relaying

  - Data split/duplication conducted on the PDCP layer

gNB-DU’s responsibility:

  - Determine the RLC/MAC/PHY Configuration for the relaying Uu RLC channels of relay UE

- Proposal 3: Ask RAN3 to wait for RAN2 progress on UE association issues in scenario 2.

Annex

- (TP to TS 38.401 on sidelink relay enhancement)

8.xx.3 Intra-DU direct path adding procedure for scenario 1

Fig. 8.xx.3 intra-DU direct path adding for multi-path in scenario 1

- Remote UE reports trigger event relative information to gNB-CU. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

- gNB-CU decides to add direct path to the remote UE under same gNB-DU.

- gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message to gNB-DU, which contains the information to add new direct path.

- gNB-DU responses with the UE CONTEXT MODIFICATION RESPONSE message to gNB-CU, which contains the configurations used for direct path.

5.gNB-CU reconfigures the remote UE and relay UE.

6. The remote UE accesses gNB-CU via configured direct path.

8.xx.4 Intra-DU indirect path adding procedure for scenario 1

Fig.8.xx.4 intra-DU indirect path adding for multi-path in scenario 1

- 1. Remote UE reports trigger event relative information to gNB-CU. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

- 2. gNB-CU decides to add indirect path to the remote UE under same gNB-DU.

- 3. gNB-CU reconfigures the relay UE.

- 4. gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message to gNB-DU, which contains the information to add indirect path.

- 5. gNB-DU responses with the UE CONTEXT MODIFICATION RESPONSE message to gNB-CU, which contains the configurations used for indirect path.

- 6. gNB-CU reconfigures the remote UE.

- 7. The remote UE establishes PC5 connection with relay UE.

- 8. The remote UE accesses gNB-CU via the indirect path.

8.xx.5 Inter-DU indirect path adding procedure for scenario 2

Fig.8.xx.5 inter-DU indirect path adding for multi-path in scenario 2

- 1. Remote UE reports trigger event relative information to gNB-CU (FFS), or gNB-CU triggered indirect path adding with gNB implementation (FFS). The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs. (FFS)

2.	The gNB-CU decides to add the indirect path via relay UE to remote UE under a different gNB-DU (i.e., gNB-DU2).

3.	The reconfiguration to relay UE is performed among relay UE, gNB-DU2 and gNB-CU if relay UE is in RRC_CONNECTED state. The gNB-CU sends an RRCReconfiguration message to the relay UE. If the relay UE is in RRC_IDLE/INACTIVE state, this step is skipped.

4.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message for the remote UE to the gNB-DU2, which contains the indirect path configuration at least.

5.	The gNB-DU2 responds to the gNB-CU with a UE CONTEXT SETUP RESPONSE message.

6.	gNB-CU sends the DL RRC MESSAGE TRANSFER message for remote UE by including the RRCReconfiguration message to gNB-DU1. The contents in the RRCReconfiguration message may include at least indirect path addition configuration, bearer mapping and the associated radio bearer(s).

7.	gNB-DU1 sends the RRCReconfiguration message to the remote UE.

8.	The remote UE establishes non-3GPP connection with relay UE.

9.	The remote UE may complete the indirect path addition procedure by sending the RRCReconfigurationComplete message to the gNB-DU1.

10.	The gNB-DU1 sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

8.xx.6 Intra-DU indirect path adding procedure for scenario 2

Fig.8.xx.6 intra-DU indirect path adding for multi-path in scenario 2

- 1. Remote UE reports trigger event relative information to gNB-CU. The remote UE may report Uu measurement results of neighboring cells and one or multiple candidate relay UEs.

- 2. gNB-CU decides to add indirect path to the remote UE under same gNB-DU.

- 3. gNB-CU reconfigures the relay UE.

- 4. gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message to gNB-DU, which contains the information to add indirect path.

- 5. gNB-DU responses with the UE CONTEXT MODIFICATION RESPONSE message to gNB-CU, which contains the configurations used for indirect path.

- 6. gNB-CU reconfigures the remote UE.

- 7. The remote UE establishes non-3GPP connection with relay UE.

- 8. The remote UE accesses gNB-CU via the indirect path.