3GPP TSG-RAN WG3 Meeting #119-bise                                                                  R3-231801

Apr 17- Apr 26, 2023

Agenda Item:	12.2.2.3

Source:	CMCC

Title:	Open Issues on AI/ML for NG-RAN Energy Saving

Document for:	Discussion and Decision

1	Introduction

During meeting #118, we got the common understanding on exchanging the EE on Xn interface, and the following options are proposed:

FFS on how to calculate this EE, and which of the following 4 options should be adopted:

Option 1: Indicating the value of the ratio of data volume over energy consumption directly

Option 2: Define the EE metric in a more abstract way using a quantitative encoding, e.g., using EE values on a linear scale from 0 to 100.

Option 3: The metric of Energy Efficiency exchanged between NG-RAN nodes is an Energy Consumption related to an additional load. And exchanged EE metric between neighboring NG-RAN nodes is defined in the interval [0, 100].

Option 4: Deliver both data volume and energy consumption over RAN interfaces to let the requesting node calculate the overall DV and over EC of the specific area and thereby drive the overall EE.

However during meeting #119, we can’t get consensus on the above options, and an enhanced option (option-3a) is proposed, and finally got consensus on the following opinions:

- Introduce the metric of Energy Cost (EC) as the AI/ML metric to be shared over the Xn interface among gNBs.

- Adopt the below Option-3a and exchange Energy Cost (EC) upon request over the Xn interface.

- The metric of Energy Cost (EC) exchanged between NG-RAN nodes can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighboring node for either additional load or current load (The details to be further discussed). EC is a value at gNB level.

The details on how to define the additional load or current load needs further discussion.

In this contribution, we provide further considerations on open issues on AI/ML for NG-RAN Energy Saving, on the procedure to exchange the information, and how to define the load and energy consumption metrics.

2	Discussion

In previous meetings, we all agree that the energy saving aims to improve the overall energy cost in an area. When the source node begins to collect energy cost data for AI/ML Training model and AI/ML Inference model, it may request the data from neighbouring node. It is a typical AI/ML Information Exchange.

Sometimes, the real time actual Energy Consumption is hard to collect and used as input for AI/ML Training model, according to different spectrum, different data volume, different service types, different scenarios. So the predicated/estimated Energy Consumption  value based on a given offload assumption under current environment can be provided as input.

We also suggest to use the new class 1 procedure to request/response and update the Energy Consumption of neighbouring nodes. This procedure is designed for AI/ML information exchange, and we can expand “Report Characteristics IE” to indicate different kind of data. And based on the assumption the procedure should be “data agnostic”, it is better to use this unified procedure for different kinds of input/feedbacks data exchange through Xn interface between neighbouring nodes.

Proposal 1: The source node can request the Energy Consumption through AI/ML Information Request message. Define a new bit in “Reply Characteristics IE” to indicate it is a “Energy Consumption Request”.

And the target node can use AI/ML Information Response to indicate whether it can or can’t provide the Energy Consumption, or it can provide a predicated or actual Energy Consumption to the source node. If can, the target node can begin to calculate the Energy Consumption, and then send it back to source node through AI/ML Information Update message.

Proposal 2: The source NG-RAN node could carry the load type (additional load or current load) and load metric in the request message to target NG-RAN node.

Proposal 3: The target NG-RAN node could report the predicated or actual value of Energy Consumption through AI/ML Information Update message. And in AI/ML Information Response message, the target node could carry the EC value type (predicated value/actual value/no information provided). Later, the target node can report the Energy Consumption value through AI/ML Information Update message.

The input Data Volume from source node can be an absolute value to represent current total load or delta value to represent an additional load. The output Energy Consumption from target node can be an actual value to represent its current real energy consumption or an inferred value to represent its predicated energy consumption increased by given offload.

Proposal 4: If source node sends an additional load value, the target can provide a predicated value, based on its inference on the increased Energy Consumption value if it takes the additional load from the source node.

Proposal 5：If source node sends a current total load, the target can provide a predicated value, based on its inference on the increased Energy Consumption value if it takes all these traffic from the source node.

Proposal 6: If the target node won’t do the predication, it can also choose to provide the actual Energy Consumption value.

Both the predicated/estimated and the actual Energy Consumption should be provided as an absolute value, instead of an index value, which can’t be defined and understood exactly same by different nodes. AI/ML Training/Inference should base on an accurate value which can be understood exactly same by different nodes, otherwise, OAM can do the work without any data exchange procedures between neighbouring nodes.

And the metric of load and Energy Consumption (EC) is not confirmed yet. In previous meeting, we discussed to use TS28.552 as a base line. Re-use the definition in SA5 is a good choice to start. And also it is agreed that the calculation granularity in node level is enough and feasible.

Proposal 7： The load metric can refer to the definition of DL PDCP SDU Data Volume Measurements in TS28.552 as a base line, it provide the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. It can start from the node level.

Proposal 8: Energy Consumption (EC) is also per node level measurement, can refer to the definition in TS28.552 as a base line. It may include Average Power Consumption for the NG-RAN node.

3	Summary

In this contribution, we provide additional input considerations on open issues on AI/ML for NG-RAN Energy Saving, and , following proposals are made:

Proposal 1: The source node can request the Energy Consumption through AI/ML Information Request message. Define a new bit in “Reply Characteristics IE” to indicate it is a “Energy Consumption Request”.

Proposal 2: The source NG-RAN node could carry the load type (additional load or current load) and load metric in the request message to target NG-RAN node.

Proposal 3: The target NG-RAN node could report the predicated or actual value of Energy Consumption through AI/ML Information Update message. And in AI/ML Information Response message, the target node could carry the EC value type (predicated value/actual value/no information provided). Later, the target node can report the Energy Consumption value through AI/ML Information Update message.

Proposal 4: If source node sends an additional load value, the target can provide a predicated value, based on its inference on the increased Energy Consumption value if it takes the additional load from the source node.

Proposal 5：If source node sends a current total load, the target can provide a predicated value, based on its inference on the increased Energy Consumption value if it takes all these traffic from the source node.

Proposal 6: If the target node won’t do the predication, it can also choose to provide the actual Energy Consumption value.

Proposal 7：The load metric can refer to the definition of DL PDCP SDU Data Volume Measurements in TS28.552 as a base line, it provide the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. It can start from the node level.

Proposal 8: Energy Consumption (EC) is also per node level measurement, can refer to the definition in TS28.552 as a base line. It may include Average Power Consumption for the NG-RAN node.

4	Reference

[1] 	3GPP RP-220617 TR 37.817-200, CMCC

[2]      3GPP TS28.552:  Management and orchestration; 5G performance measurements (Release 17)