3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231809

Online, 17 April - 26 April 2023

Agenda item:	15.3

Source:	CMCC

Title:	Multicast Reception in RRC_INACTIVE state

Document for:	Discussion

Introduction

In the previous meetings, we show our interpretation of potential effect on RAN3 of two methods that provides PTM configuration for inactive reception. However, which method on providing PTM configuration for inactive reception is decided by RAN2. After checking the conclusions from RAN2#120 meeting, they give their answer on this issue and the conclusions are as follows:

We will have a mixed approach and we start with the following:

    - When NW configures UE to continue the multicast reception in INACTIVE state, NW provides the PTM configuration for the activated multicast session via the RRC dedicated signalling, at least for the serving cell (FFS other cases).

    - MCCH is used in case there is a need to indicate a PTM configuration in case there is a need for change in PTM config or during mobility beyond serving cell / gNB. FFS session status change and other indications.

    - We assume that the UE can only receive multicast service after it joined the session.

    - FFS whether MCCH configuration is initially provided to the UE via dedicated signalling.

During last meeting, we only focus on the interpretation of assistance information provided from SA2 and there is no time to discuss this mixed approach. Therefore, the conclusions are as follows:

RAN3#119:

Support a per UE per MBS session indication from CN to RAN.

It is FFS which interpretation on the indication from CN to RAN should be adopted in RAN3:

Interpretation 1: UE is preferred to be kept in RRC connected when receiving the related MBS session data.

Interpretation 2: The IE indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception.

In this contribution, combined with the latest agreements in RAN2 and RAN3, we will continue to analyse potential impact of the mixed approach on RAN3 and state our interpretation on the indication from CN to RAN.

Discussion

PTM Configuration for Inactive Reception

Two options for PTM configuration delivery are discussed for a long time in RAN2. One option is to deliver PTM configurations using RRC dedicated signalling. The other option is to deliver PTM configuration provided in MCCH-like channel and information regarding MCCH scheduling provided via SIB. From the mixed approach proposed by RAN2, we boldly assume that network will provide PTM configuration to UE using RRC dedicated signalling in case that gNB decides to change the RRC state from connected to inactive, and provide PTM configuration to UE using MCCH-based solution in case that PTM configuration is required to update. Based on our superficial assumptions on the scenario of two PTM configurations, different PTM configurations should be adapted for various scenarios. From RAN3 perspective, main goal is to evaluate network interface impacts from potential scenarios for assignment of PTM configuration.

Observation 1: From RAN3 perspective, main goal is to evaluate network interface impacts from potential scenarios for assignment of PTM configuration.

Obviously, if a UE is constantly served by a gNB without change in PTM configuration, there will be no impact on network interface. Thus, RAN3 should focus on the case where there is a need for change in PTM configuration or mobility beyond serving cell.

Observation 2: RAN3 should focus on the case where there is a need for change in PTM configuration or mobility beyond serving cell.

Assistance Information

During last meeting, there was a discussion on how to interpret the indication from CN to NG-RAN node of the MBS assistant information that goes on for a long time without reaching any consensus. Before RAN3#119bis-e meeting, an offline discussion is initiated among interested companies and so far there is also no compromise between companies . Two interpretations are proposed in last meeting:

Interpretation 1: UE is preferred to be kept in RRC connected when receiving the related MBS session data.

Interpretation 2: The IE indicates that the UE requires preferential treatment within the multicast group, guaranteeing steady and prompt provision of system resources for data transmission and reception.

From our view, we slightly prefer to adopt the interpretation 2, since RAN is the node who decides the RRC state for  UE and what the 5GC only can aware is CM states. We agree some proponents advocated in offline discussion, the information provided from 5GC to RAN shall never influence RAN internal properties directly, which should be abstract traffic/service characteristics or descriptions. We suggest to send a LS to SA2 telling them the divergent understanding from RAN3 and the consensus already made by RAN3. After obtaining the LS, we hope that SA2 can rediscuss the semantic of the IE and correct their expression on previous LS.

Proposal 1: Prefer to adopt the interpretation 2, since RAN is the node who decides the RRC state for  UE and what the 5GC only can aware is CM states.

Proposal 2: Send a LS to SA2 telling them the divergent understanding from RAN3 and the consensus already made by RAN3.

Mobility Scenario

For mobility case, RAN3 reached the conclusion that “NG-RAN signaling supports service continuity for UEs receiving multicast session data in RRC_INACTIVE, i.e., a UE is able to continue multicast reception without RRC state transitioning after cell reselection in RRC_INACTIVE state if the configuration of the new cell is available for the UE.” Combined the conclusion of RAN2 and RAN3, UE will keep receiving multicast session data in RRC_INACTIVE to support service continuity through monitoring MCCH after cell reselection based on the premise that the configuration of the new cell is available for the UE.

Proposal 3: Based on the premise that the configuration of the new cell is available for the UE, UE will keep receiving multicast session data in RRC_INACTIVE to support service continuity through monitoring MCCH after cell reselection.

During last meeting, RAN2 reached the following agreements:

Multicast MCCH configuration is provided via new SIB.

Optionally, Multicast MCCH configuration for the serving cell can also be provided in dedicated signalling. Understanding is we are not optimizing mobility case because of this.

Serving cell will not provide the PTM configuration of neighbour cells from other gNBs.

FFS whether the network can provide PTM configuration for intra-gNB cells.

Combined the above agreements in RAN2, once UE receiving multicast in RRC_INACTIVE reselects to a new cell, it can monitor SIB to get MCCH configuration, as Rel-17 broadcast does. Followed by this way, all UEs can obtain the multicast MCCH configuration, regardless of MBS-exclusive UE. Apart from the way of monitoring new SIB, RAN2 also states that UE also can be configured to acquire the multicast MCCH configuration from the dedicated signalling.

If UE wants to acquire the multicast MCCH configuration of its neighbouring in advance, necessary signalling enhancement over Xn will be performed between serving gNB and neighbour gNBs. For inter-gNB case, once UE reselects to a neighbour cell, it will use the pre-prepared configuration. However, if the configuration for a neighbour cell is updated, a change notification should be transferred to the serving cell, which will have signalling impact on XnAP. From our perspective, exchange of multicast MCCH configuration of neighbour gNBs in advance is worth discussing. To achieve the coordination, there will be enhancement on Xn to support mobility beyond serving cells.

Proposal 4: There will be potential enhancement on Xn to support mobility beyond serving cells, if exchange of MCCH configuration of neighbour gNBs in advance is supported.

F1 impact

In previous meeting, we agree that “During an active multicast session, the gNB-DU shall keep the PTM transmission when delivering respective multicast data to RRC_INACTIVE UEs. FFS on how the gNB is aware that it delivers multicast service to UEs in RRC inactive.” For unicast service, the gNB-CU may send the UE Context Release Command message to UE if gNB-CU wants to release a UE into RRC_INACTIVE. Once receiving the message, gNB-DU would release the UE context. For multicast service, if UEs configured by CU are switched to RRC_INACTIVE, gNB-DU should follow the basic principle by releasing the UEs context and keep the PTM transmission. Therefore, we need to face the issue on how the gNB-DU is aware that it should keep delivering multicast service to UEs in RRC_INACTIVE. There would be F1 impact from gNB-CU to gNB-DU. It is easily conceivable that an indication notifies gNB-DU to keep delivering multicast service to UEs from gNB-CU should be introduced. Whether introducing a new signalling for indication or using existing F1AP message to carry the indication could be further discussed.

Proposal 5: An indication notifies gNB-DU to keep delivering multicast service to UEs from gNB-CU should be introduced.

Proposal 6: Whether introducing a new signalling for indication or using existing F1AP message to carry the indication could be further discussed.

Once gNB-DU obtaining the indication, combined with the mixed approach proposed by RAN2, we discuss the impacts of two options for PTM configuration delivery separately on F1AP. gNB-DU would not aware the multicast PTM configuration if adopting RRC dedicated signalling. Since the RRC dedicated signalling is included in RRC-Container in DL RRC MESSAGE TRANSFER message, gNB-DU would transparently transfer the container to UE when receiving from CU. If adopting MCCH-based solution, gNB-DU could generate multicast MCCH configuration with the help of some multicast configuration from CU.

Observation 3: If adopting RRC dedicated signalling to deliver PTM configuration, gNB-DU would not aware the multicast PTM configuration.

Observation 4: If adopting MCCH-based solution to deliver PTM configuration, gNB-DU could generate multicast MCCH configuration with the help of some multicast configuration from CU.

Followed by the Broadcast Context Management procedure, it is proposed to introduce a new multicast CU to DU RRC Information IE in Multicast Context Management procedure. Sharing the same function with the MBS CU to DU RRC Information IE which is shown below, this new IE indicates the multicast CU to DU RRC Information and the detail content of this IE depends on RAN2 discussion.

- 9.3.1.225 MBS CU to DU RRC Information This IE indicates the MBS CU to DU RRC Information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Broadcast Cell List; M. >MBS Broadcast Cell Item; 1 .. <maxCellingNBDU>. >>NR CGI; M; NR CGI 9.3.1.12. >>mtch-neighbourCell; O; OCTET STRING; mtch-neighbourCell, as defined in TS 38.331[8]. MRB PDCP Config Broadcast; O; OCTET STRING; MRB-PDCP-ConfigBroadcast, as defined in TS 38.331[8]. Range bound; Explanation. maxCellingNBDU; Maximum no. cells that can be served by a gNB-DU. Value is 512..

Proposal 7: It is proposed to introduce a new multicast CU to DU RRC Information IE in Multicast Context Management procedure.

Conclusions

In this paper, we have discussed the multicast reception in RRC_INACTIVE state. The proposals are listed below:

Observation 1: From RAN3 perspective, main goal is to evaluate network interface impacts from potential scenarios for assignment of PTM configuration.

Observation 2: RAN3 should focus on the case where there is a need for change in PTM configuration or mobility beyond serving cell.

Observation 3: If adopting RRC dedicated signalling to deliver PTM configuration, gNB-DU would not aware the multicast PTM configuration.

Observation 4: If adopting MCCH-based solution to deliver PTM configuration, gNB-DU could generate multicast MCCH configuration with the help of some multicast configuration from CU.

Proposal 1: Prefer to adopt the interpretation 2, since RAN is the node who decides the RRC state for  UE and what the 5GC only can aware is CM states.

Proposal 2: Send a LS to SA2 telling them the divergent understanding from RAN3 and the consensus already made by RAN3.

Proposal 3: Based on the premise that the configuration of the new cell is available for the UE, UE will keep receiving multicast session data in RRC_INACTIVE to support service continuity through monitoring MCCH after cell reselection.

Proposal 4: There will be potential enhancement on Xn to support mobility beyond serving cells, if exchange of MCCH configuration of neighbour gNBs in advance is supported.

Proposal 5: An indication notifies gNB-DU to keep delivering multicast service to UEs from gNB-CU should be introduced.

Proposal 6: Whether introducing a new signalling for indication or using existing F1AP message to carry the indication could be further discussed.

Proposal 7: It is proposed to introduce a new multicast CU to DU RRC Information IE in Multicast Context Management procedure.

Reference

- R3-23xxxx, Proposal on the way forward on assistant information from CN to NG-RAN node, CATT;