3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231822

Online, Apr. 17 – 26, 2023

Title: 	(TP for AI&ML BLCR for TS 38.300) Further discussions on common issues and Stage 2 updates on the introduction of RAN AI/ML

Source: 	Huawei

Agenda item:	12.2.1

Document Type:	Discussion and decision

1. Introduction

In last RAN3#119 meeting, Rel-18 WI on RAN AI&ML was further discussed, some common understandings were achieved and some agreements/working assumptions for both Stage 2 and Stage 3 were reached as well [1] [2] [3] [4]. In this paper, further general considerations on the remaining common issues, e.g. the details of the agreed new procedure, further Stage 2 descriptions, etc., are discussed with some suggestions being proposed.

2. Background

In RAN3#117bis-e meeting, some to-be-continued issues were identified [5] which in our understanding are not use case specific ones:

-	Whether to capture in Stage 2 specifications message sequence charts to support AI/ML in NG-RAN depends on further progress and it is FFS.

-	WA: The new procedure is introduced to exchange AI/ML related information is data type agnostic, namely it can be used to transfer AI/ML data.

-	It’s FFS on whether more new procedures needed to transfer different types of AL/ML data (e.g., feedback, measurements for training/inference).

-	The exact information to be included in this new procedure need to be discussed on a case by case basis.

-	In case new requirements are identified, it is FFS to tackle Solutions for AI/ML information exchange over the NG interface in R18

Note that, in the above list of FFSs, the latter has been solved in the last RAN3 meeting (RAN3#119), where it was agreed that [1]

Solutions for AI/ML information exchange over the NG interface are not considered as part of Rel-18.

while, especially for the FFSs above highlighted in yellow, discussion is still open. Moreover, in RAN3#118 meeting, there was a follow-up Stage 2 discussion which led to the following WA [6]:

WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”.

In our understanding, those open issues above are common ones which should apply to all the three use cases. In the rest of this paper, we will try to have further discussions and analysis on each of them, and share our suggestions.

3. Discussion

3.1	Remaining open issues for the agreed procedure

In RAN3#117bis-e meeting, it was further agreed that the new procedure over Xn used for AI/ML related information shall be use case agnostic, and the remaining open issues could be seen above section 3.1 which are copied below:

-	WA: The new procedure is introduced to exchange AI/ML related information is data type agnostic, namely it can be used to transfer AI/ML data.

-	It’s FFS on whether more new procedures needed to transfer different types of AL/ML data (e.g., feedback, measurements for training/inference).

-	The exact information to be included in this new procedure need to be discussed on a case by case basis.

The main issue here is what kind of info could be included/transferred using this new procedure, in addition to the predicted information which was agreed in RAN3#117-e meeting.

Observation 1: The main open issue for the agreed new procedure are, what info else could be included/transferred using this new procedure, in addition to predicted information.

Such issue has been extensively discussed in RAN3#118 and RAN3#119 meetings, but there was no strong majority among companies in having AI/ML support in NG-RAN via data type agnostic procedures (this is still a WA). Taking all the discussions happened so far, we could see that the info to be exchanged over Xn concerning the support of the AI/ML function mainly would include predicted info (e.g. resource, trajectory, traffic, power consumption, etc.) as well as performance feedback info (e.g. energy efficiency, throughput, packet delay, packet loss, etc.); some of these info are use case specific, e.g. energy efficiency related info which is mainly about the use case of energy saving, while some of them are common, e.g. trajectory related info. In our understanding, as long as there is a request and response mechanism for AI/ML info exchange, there is no need to introduce additional mechanism and procedures for requesting and reporting performance feedback info. This is also motivated by the agreements achieved in RAN3#119 meeting concerning the LB use case [1] [2], in particular the ones highlighted in green:

The agreed class1 procedure (AI/ML INFORMATION REQUEST/RESPONSE, the name needs further discussion) is used to configure UE performance feedback reporting.

Introduce into the agreed new request message (AI/ML INFORMATION REQUEST, the name needs further discussion), an indication that UE performance feedback is provided after handover event. Whether the indication is in implicit or explicit way needs to be further discussed.

Introduce a trigger indication in the HO request message to indicate that UE performance feedback is requested after HO completion. The details of indication need to be discussed.

The agreed new class2 non-UE associated procedure (AI/ML INFORMATION UPDATE, which name is FFS) is used for UE performance feedback reporting.

Partial reporting mechanism is supported in the agreed AI/ML information procedures (AI/ML INFORMATION REQUEST/RESPONSE/UPDATE, which name is FFS). The solutions need to be discussed.

Of course, for the details of such info, as mentioned just now, some are use case specific and some are common, the details should be discussed on use case basis as decided in RAN3#117bis-e meeting.

Observation 2: The information included/transferred using this new procedure mainly could include predicted info and performance feedback info, some of which are use case specific while some are common.

Proposal 1: The new agreed procedure could be used for transferring prediction info and performance feedback info as well, the details should be discussed on use case basis.

Proposal 1bis: Turn this WA into agreement: “WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”.”

Taking a further step, we could see that among such information, most of them are cell-level or node-level, which could be handled by a non-UE-associated procedure; while some of them are UE-specific, i.e. handover failure and cell-based UE trajectory prediction.

For handover failure indication, actually this is an existing mechanism which could be conveyed through the HO failure message or made available from the HO failure report in SON or from the UE RLF report; while for the cell-based UE trajectory prediction information RAN3 agreed in RAN3#118 meeting that such info is transferred to the target gNB via the Handover Request (i.e., the existing procedure).

With the analysis above, we could see that most of info to be exchanged over Xn could be handled by the agreed new non-UE-associated procedure; while for the UE specific info, what we could identify on the table up to now is handover failure info, which could either be handle by the existing mechanism or the agreed new non-UE-associated procedure. Thus, we think whether a dedicated UE-associated procedure is needed is still pending on whether new UE-specific info are identified and whether such new info could not be transferred with the agreed new non-UE-associated procedure, and such new info should be use case dependent.

Observation 3: Most of info to be exchanged over Xn are cell- or node-level, which could be handled by the agreed new non-UE-associated procedure.

Observation 3bis: For the identified UE-specific info so far, i.e. handover failure, they could be transferred either by the agreed new non-UE-associated procedure or by the existing mechanisms.

Proposal 2: Before deciding to introduce a new UE-associated procedure, new UE-specific info should be investigated on use case basis.

As could be seen from the analysis above, for the identified UE-specific info so far, i.e. handover failure, it could be handled either by the non-UE associated message or by the existing mechanism. Meaning while, it is obvious that some existing procedures would anyway be used, for example, to exchange some HO failure info over existing SON report or exiting HO failure message; on the other hand, we would also agree that which existing procedure to be used should also be discussed use case by use case.

Proposal 3: The existing procedure should be used for transferring UE-specific info, which existing procedure to be used should be discussed use case by use case also depending on the type of UE-specific info to be transferred.

3.2	Stage 2 update

In RAN3#117bis-e meeting, some Stage 2 TPs were agreed, with more descriptions on general principles and mechanisms for the support of the RAN AI/ML function, while there were further agreements which could be further captured in the Stage 2 spec, including:

Procedures used for AI/ML support in the NG-RAN shall be use case agnostic.

The cases of i) Model Training and Model Inference at the NG-RAN and ii) Model Training at OAM and Model Inference at the RAN, make use of the same procedures, with the exception that procedures for exchange of training data and feedback data (which is not related to model performance feedback) will be different for i) and ii).

Legacy information that are used to support AI/ML are transferred via existing legacy procedures (no need to signal them via other procedures)

In addition to the above, also the following Stage 2 agreement from RAN3#119 should be captured:

Solutions for AI/ML information exchange over the NG interface are not considered as part of Rel-18.

Moreover, we also think some of the proposals in this paper should also be captured in Stage 2 spec, e.g. proposal 1/3.

Proposal 4: RAN3 to agree Stage 2 updates to TS 38.300 capturing further agreements reached in RAN3#117bis-e, RAN3#119 and RAN3#119bis-e meetings.

The corresponding TP – capturing agreements up to RAN3#119 meeting – could be seen in Annex-2.

3.3	LS to SA5

In previous RAN3 meetings, a general issue was also mentioned, i.e., what kind of AI/ML performance feedback info would help to evaluate/improve the trained AI/ML model performance/accuracy. On this issue RAN3 reached some agreements, see below:

From RAN3#117-e

For all three use cases: The following information should be specified as a start point on the basis of TR37.819:

- Predicted resource status information over Xn

- UE performance (e.g, UL/DL throughput, packet delay, packet loss)

From RAN3#117Bbis-e

Support the following UE performance information to be sent for feedback purposes: Average Packet Delay, Average UE Throughput DL, Average UE Throughput UL, Average Packet Error Rate.

In our understanding, we think the overall Energy Cost – i.e., the metric agreed in RAN3#119 meeting [1] [4] to be exchanged over Xn among gNBs for the ES use case – evaluated after the applicability of a certain AI/ML based ES strategy/action could also be used as performance feedback for energy saving case, see our paper in [7].

With these agreements and proposals, for the scenario where AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB, performance feedback info should be provided to OAM, so that OAM could take further actions accordingly, thus we think an LS should be sent to SA5 from RAN3 to make them aware of the RAN3 agreements. The draft LS could be seen in the Annex-1.

Proposal 5: Agree to send an LS to SA5 to make them aware of the RAN3 agreements on performance feedback info.

4. Conclusion

Based on the discussion in this paper, we reach the following observations and proposals.

Observation 1: The main open issue for the agreed new procedure are, what info else could be included/transferred using this new procedure, in addition to predicted information.

Observation 2: The information included/transferred using this new procedure mainly could include predicted info and performance feedback info, some of which are use case specific while some are common.

Observation 3: Most of info to-be-exchanged over Xn are cell or node level, which could be handled by the agreed new non-UE-associated procedure.

Observation 3bis: For the identified UE-specific info so far, i.e. handover failure, they could be transferred either by the agreed new non-UE-associated procedure or by the existing mechanisms.

Proposal 1: The new agreed procedure could be used for transferring prediction info and performance feedback info as well, the details should be discussed on use case basis.

Proposal 1bis: Turn this WA into agreement: “WA: Procedures used for AI/ML support in the NG-RAN shall be “data type agnostic”.”

Proposal 2: Before deciding to introduce a new UE-associated procedure, new UE-specific info should be investigated on use case basis.

Proposal 3: The exiting procedure should be used for transferring UE-specific info, which existing procedure to be used should be discussed use case by use case also depending on the type of UE-specific info to be transferred.

Proposal 4: RAN3 to agree Stage 2 updates to TS 38.300 capturing further agreements reached in RAN3#117bis-e and RAN3#119 meetings.

Proposal 5: Agree to send an LS to SA5 to make them aware of the RAN3 agreements on performance feedback info.

5. Reference

- 3GPP RAN3#119 Chairman’s notes (RAN3_119_agenda_20230303_EOM2.doc)

- R3-231023, Summary of Offline Discussion on AIRAN_LB, ZTE

- R3-230856, Summary of Offline Discussion 18_AIRAN2_ME, Nokia

- R3-231003, Summary of Offline Discussion on CB: # 19_AIRAN3_ES, Verizon

- 3GPP RAN3#117bis-e Chairman’s notes (RAN3_117bis-e_agenda_20221018_EOM1.doc)

- 3GPP RAN3#118 Chairman’s notes (RAN3_118_agenda_20221118_1710.doc)

- R3-231826, (TP for AI&ML BLCR for 38.423) Further discussions on remaining open issues for energy saving using AI/ML, Huawei

6. Annex - 1

Title:	[Draft] LS on R18 RAN AI/ML about performance feedback info

Response to:

Release:	Rel-18

Work Item:	AI/ML for NG-RAN (NR_AIML_NGRAN-Core)

Source:	Huawei [will be RAN3]

To:	SA5

Cc:

Contact person:	Yang Xudong

yangxudong@huawei.com

+86-21-38900808

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

As SA5 has already learned that, for this WI on AI/ML for NG-RAN, RAN3 agreed the following two scenarios:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

RAN3 would like to inform SA5 that RAN3 further agreed to introduce some performance feedback information which is collected after action is taken according to inference results, such feedback information will be provided to model training side which could help to improve the performance/accuracy of the trained model, see below:

- UE performance information: Average Packet Delay, Average UE Throughput DL, Average UE Throughput UL, Average Packet Error Rate.

- Node-level performance information: overall Energy Cost (EC)

For the latter, RAN3 defined EC as the metric to be exchanged between NG-RAN nodes over Xn corresponding to an inferred energy consumption related to an additional load or an actual energy consumption value from a neighboring node for either additional load or current load (The details to be further discussed). For the UE performance information, instead, RAN3 understands that definitions of such information are already available in SA5 specifications.

The above-listed information should be provided from NG-RAN to OAM for the scenario where AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

2	Actions

To SA5

ACTION: 	RAN3 would like to ask SA5 to take the agreements into account and take further actions accordingly if SA5 believes it is needed, and update RAN3 if any further progress is made.

3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#120			2023-05-22 – 2023-05-26	Incheon, South Korea

RAN3#121			2023-08-21 – 2023-08-25	Toulouse, France

RAN3#121bis-e		2023‑10‑09	2023‑10‑18		Online

7. Annex - 2: TP to TS 38.300

******************************Start of  Change*****************************************

X.X AI/ML for NG-RAN

X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

For the scenario where AI/ML model training is located in the OAM, the inputs will be sent from NG-RAN directly to OAM (which is in the scope of SA5). For the scenario where the necessary AI/ML related information is to be exchanged among NG-RAN nodes, such information, e.g. AI/ML predictions as mentioned above, could be transferred either via use case agnostic AI/ML dedicated procedures or existing procedures, depending on the considered use case, or performance feedback info that could be exchanged via AI/ML dedicated procedures in a request and response way.

For the Mobility Optimization use case, the AI/ML predictions – i.e., cell-based UE trajectory predictions – are transferred to target gNB via the Handover Request (existing procedure).

For the Load Balancing use case, UE performance feedback reporting is configured through an AI/ML dedicated request/response procedure, as mentioned above, while actual reporting is performed through an AI/ML dedicated updated procedure.

For the Energy Saving use case, the metric of Energy Cost (EC) is introduced as the AI/ML metric to be shared among gNBs; the EC is a value at gNB level and can be an inferred energy consumption related to an additional load or an actual energy consumption value from a neighbour gNB related to either an additional load or the current load.

Solutions for AI/ML information exchange over the NG interface are not considered in this release of the specification.

******************************End of  Change*****************************************