3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231830

Online, 17 April – 26 April 2023

Title: 	Discussion on QoE measurement in NR-DC

Source: 	China Unicom

Agenda item:	11.3

Document Type:	Discussion

Introduction

The Work Item on QoE reporting has been agreed in RAN#96 (RP-221803) with the following objectives:

- Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2]. - Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture, and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity. Note 1: The QoE measurements are not performed separately for each leg. - Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios. - Specify the QoE measurement continuity in mobility scenarios in NR-DC. - Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

This paper further discusses how to configure the QoE measurement and RAN-Visible QoE measurement in NR-DC based on the agreements and FFS of the previous meetings.

Discussion

QoE configuration in NR-DC

In Rel17, RRC identifier is defined, an RRC identifier conveyed in the RRC signalling is used to identify the application layer measurement configuration and report between the gNB and the UE. The RRC identifier is mapped to the QoE reference in the gNB, and the gNB forwards the application layer measurement report to MCE together with the QoE reference. In RAN3#119 meeting, it has the following agreement and leave a FFS:

The MN is responsible for RRC ID allocation for m-based sessions configured by the MN or SN, and notifies the allocated RRC ID(s) to the SN.

FFS on whether a pool of RRC ID is split between MN and SN or whether it is per measurement.

RAN3 has agreed that the MN is responsible for RRC ID allocation for m-based QoE configuration configured by MN or SN, but leave the FFS for how to allocate the RRC ID. The RRC ID is unique in the QoE configurations in one UE, so only MN allocate the RRC ID can avoid the repetition for the RRC ID for the different QoE measurements which is send by different node (MN or SN). From this perspective, one RRC ID per measurement for both MN and SN, or two RRC IDs which is split between MN and SN, both of these two modes are all applicable, but for RAN visible QoE reporting which are generated by the MN or SN,if there are two separate RAN visible QoE configurations, two separate RRC IDs for MN and SN are needed. Because when the RAN visible QoE reporting is reported to the MN or SN, if should have a clear distinction for the RAN visible QoE reports which are corresponding with different QoE configurations.

Proposal 1: A pool of RRC ID is split between MN and SN is better for QoE configuration for NR-DC.

QoE reporting in NR-DC

Generally speaking, the node configured for QoE measurement will receive the corresponding QoE measurement report. But in RAN overload scenario, the configured node does not have enough resources to provide the UE reporting its QoE report, and another leg may help the situation. Coordination may be needed for the MN and SN to decide which node to receive the QoE reporting, and it is better for MN to command the UE to switch the reporting leg since the it can be send along with the QoE configuration.

Proposal 2: For RAN overload scenario, it is preferred for MN to command the UE to switch the reporting leg.

In RAN3#117bis meeting, it has the following agreement:

The coordination between the MN and the SN should support at least the following (details to be further discussed):

- Indication about switching the reporting leg.

The coordination procedure between MN and SN should be supported when MN and SN overload, the node which is overload should initiate the coordination procedure, and the peer node can admit the QoE measurements report according to it’s own uplink resource status, and which message can be used for the coordination procedure, what information should be included in the message need to be further studied in RAN3.

Proposal 3: For RAN overload scenario, RAN3 should further discuss the details about the coordination procedure.

RAN visible QoE configuration in NR-DC

In Rel-17, the RVQoE configuration is configured according the available RVQoE metrics sending from OAM/AMF, for NR-DC scenario, both the MN and SN need to generate the RVQoE configurations, so the available  RVQoE metrics should be send to the other node during the RVQoE configuration coordination.

Proposal 4: The node that received the QoE configuration from the AMF/OAM can send to the other node the list of available RVQoE metrics during RVQoE configuration coordination.

In last meeting, there has the following FFS:

The node which sends the initial RVQoE configuration to UE and the node which sends the legacy QoE configuration to UE should be the same?

Since RVQoE configuration is not always send the same time with legacy QoE configuration, it is no need to send the RVQoE configuration the same node with legacy QoE configuration. It is more flexible to send the RVQoE configuration and legacy QoE configuration independently.

Proposal 5: RVQoE configuration and legacy QoE configuration can be send to UE independently.

RAN visible QoE reporting in NR-DC

In the previous meetings, RAN3 have the following agreements:

The MN can receive RVQoE reports directly from the UE.

The SN can receive RVQoE reports directly from the UE.

UE can send RVQoE report to MN, MN then forward the RVQoE report to SN if needed, and vice versa.

RAN3 has agreed that both MN and SN can receive RVQoE reports directly from the UE, and can forward the QoE reports to the peer node if needed. There is a case that the UE has no direct SRB to transmit the QoE directly to SN, and it need to send to MN, and then MN forward the RVQoE reports to SN, the existing Xn message can be considered.

Proposal 6: RVQoE report for SN can be sent to MN first if there is no SRB (SRB3 or new SRB) between UE and SN,  and MN can forward the RVQoE report to SN.

Proposal 7: RRC Transfer procedure can be considered to transfer the RVQoE report between MN and SN.

Alignment of QoE measurements and radio related measurement in NR-DC

In R17, the NG-RAN Trace ID of MDT will send to gNB in QoE configuration, the NG-RAN node includes the MDT session identifiers (Trace Reference and Trace Recording Session Reference) to the corresponding QoE report  for correlation. In R18, For the alignment of QoE measurements and radio related measurement in NR-DC, how to correlate the QoE report with both MN and SN MDT report should be discussed.

Proposal 8: In NR-DC, how to correlate the QoE report with MDT report of both MN and SN should be discussed.

Conclusion

Proposal 1: A pool of RRC ID is split between MN and SN is better for QoE configuration for NR-DC.

Proposal 2: For RAN overload scenario, it is preferred for MN to command the UE to switch the reporting leg.

Proposal 3: For RAN overload scenario, RAN3 should further discuss the details about the coordination procedure.

Proposal 4: The node that received the QoE configuration from the AMF/OAM can send to the other node the list of available RVQoE metrics during RVQoE configuration coordination.

Proposal 5: RVQoE configuration and legacy QoE configuration can be send to UE independently.

Proposal 6: RVQoE report for SN can be sent to MN first if there is no SRB (SRB3 or new SRB) between UE and SN,  and MN can forward the RVQoE report to SN.

Proposal 7: RRC Transfer procedure can be considered to transfer the RVQoE report between MN and SN.

Proposal 8: In NR-DC, how to correlate the QoE report with MDT report of both MN and SN should be discussed.