3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231839

# Online, 17 – 26 August 2023

Agenda item:		23.2

Source:	ZTE

Title:	Discussion on positioning enhancements

Document for:		Discussion and Decision

Introduction

The following is the objectives of the R18 Positioning WI:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]:

  - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1].

    - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum.

    - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

  - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

  - Specify support of resource allocation for SL PRS:

    - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

      - For resource allocation mechanism for SL PRS in Scheme 2:

        - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

        - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

    - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1].

      - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

  - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

  - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2]

  - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

- Specify the error modelling parameters, signalling, and procedures to support UE-based and LMF-based integrity of RAT-dependent positioning methods [RAN2, RAN3].

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including:

  - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4]

    - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning.

      - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap.

    - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs

  - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3].

    - SRS for positioning configurations in multiple cells [RAN2, RAN1].

      - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work.

    - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3].

    - SRS for positioning activation/request procedure(s) [RAN2, RAN1].

  - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2].

  - Specify solutions for alignment between eDRX and PRS configurations [RAN2].

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs)

  - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2].

    - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs.

  - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4].

  - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2].

    - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

  - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

- Define extensions of signalling, protocol, and procedure for NR positioning enhancement, as needed for the above objectives [RAN3].

In this contribution, we focus on the positioning enhancement which are related to RAN3, e.g., sidelink positioning, integrity of RAT-dependent positioning and carrier positioning method.

Discussion

Sidelink positioning

Sidelink positioning and ranging service authorizations

According to the objective of WI, RAN3 should discuss how to specify signalling to NG-RAN for sidelink positioning and ranging service authorizations. Following the conclusion in TR 23.700 :

===============================TR 23.700================================

For KI #1 and KI #8, the followings are taken as conclusions:

-	The procedures and signalling specified for ProSe in clause 6.2 of TS 23.304 [4] and for V2X in clause 6.2 of TS 23.287 [3] are used to provision the Ranging/SL positioning service authorization and policy/parameter provisioning to the UE.

-	The specific Ranging/SL positioning parameters to UE include:

-	When the UE is served by NG-RAN, the PLMNs in which the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	When the UE is not served by NG-RAN, indicates whether the UE is authorized to perform Ranging/SL positioning service over PC5 reference point for acquiring relative distance, relative direction, high accuracy relative distance and high accuracy relative direction.

-	The mapping between Ranging/SL positioning services (e.g. ProSe identifiers, V2X service types) and Ranging/SL positioning QoS parameters. Ranging/SL Positioning QoS parameters include RSPP transport QoS, i.e. PQI if RSPP is transported over PC5-U, and positioning QoS.

NOTE 1:	Details of the RSPP transport QoS and the positioning QoS will be coordinated with RAN WGs during the normative phase. Whether RSPP is transported over PC5-U depends on the outcome of KI#4.

NOTE 2: Positioning QoS parameters can also be dynamically provided in the Ranging/SL Positioning service request by the service application.

-	The authorized Ranging/SL positioning role per PLMN, i.e. target UE, reference UE, SL Positioning Client UE, SL Positioning Server UE and located UE.

-	When the UE is not served by NG-RAN, indicates the authorized Ranging/SL positioning role, i.e. target UE, reference UE, SL Positioning Client UE, SL Positioning Server UE, and located UE.

-	The discovery parameters for Ranging/SL positioning service depends on the conclusions of KI#3.

NOTE 3:	These parameters can be preconfigured in ME/UICC. If same policy/parameters are provided by PCF, the policy/parameters provided by PCF have higher priority than preconfigured parameters.

-	The procedures and signalling specified for ProSe in clause 6.6 of TS 23.304 [4] and for V2X in clause 6.5 of TS 23.287 [3] are used to provision the Ranging/SL positioning service authorization to RAN.

-	The specific Ranging/SL positioning service authorization to RAN include:

-	The "Ranging/SL positioning service authorized" indication based on UE subscription from UDM, indicating the UE is authorized to use Ranging/SL positioning service in the serving PLMN.

Absolute positioning. Relative positioning. Ranging.

-	QoS parameters used by NG-RAN.

NOTE 4:	What QoS parameters of Ranging/SL positioning used by NG-RAN will be specified during normative phase.NOTE 5:	The parameters list is not exhausted. Additional parameters can be added during normative phase.

===============================TR 23.700================================

The highlighted content indicates that the authorization procedures and signalling used for ProSe and V2X can be used as a foundation for authorizing the Ranging/SL positioning service. Therefore, it is suggested that the registration procedure, service request procedure, and handover procedure be enhanced to support the authorization of Ranging/SL positioning.

- Introduce the Ranging/SL positioning service authorization information in the following NGAP message:

  - INITIAL CONTEXT SETUP REQUEST

  - UE CONTEXT MODIFICATION REQUEST

  - HANDOVER REQUEST

  - PATH SWITCH REQUEST ACKNOWLEDGE

- Introduce the Ranging/SL positioning service authorization information in the following XnAP message:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

As seen in the TS 23.586 [2], the description of functional entities of AMF involve provision the NG-RAN with indication about the UE authorization status and PC5 QoS parameters related to Ranging/SL positioning over PC5.

===============================TS 23.586================================

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the AMF performs the following functions:

-	Select a PCF supporting Ranging/SL positioning Policy/Parameter provisioning based on indication of Ranging/SL positioning Capability as part of the 5G ProSe Capability/V2X capability in the "5GMM capability" in the Registration Request.

-	Store the Ranging/SL positioning Capability.

-	Forward the Ranging/SL positioning Capability of Target UE to LMF.

-	Forward the Ranging/SL positioning Capability to PCF.

-	Obtain from UDM the subscription information related to Ranging/SL positioning and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about Ranging/SL Positioning over PC5 (i.e. as Ranging/SL positioning-enabled UE);

-	Provision the NG-RAN with PC5 QoS parameters related to Ranging/SL positioning over PC5.

Editor's note:	It is FFS what the Ranging/SL positioning QoS parameters are, whether the Ranging/SL positioning QoS parameters are needed and how to use the Ranging/SL positioning QoS parameters.

==============================TS 23.586================================

Hence, following are proposed to involve in the

- A "Ranging/SL positioning service authorized" indication

- The authorized Ranging/SL positioning PC5 QoS parameters

- The authorized Ranging/SL positioning role, e.g., target UE, anchor UE, reference UE, assistance UE, etc.

- The following information could be considered as authorization information related to Ranging/SL positioning over PC5:

- A "Ranging/SL positioning service authorized" IE

- The authorized Ranging/SL positioning parameters.

Sildeink positioning PRS configuration

Regarding SL-PRS resource allocation, both scheme 1 and scheme 2 should be introduced for supporting SL positioning/ranging:

  - Scheme 1: Network-centric operation SL-PRS resource allocation

  - Scheme 2: UE autonomous SL-PRS resource allocation

Scheme 1 is for in/partial UEs to perform mode 1-like solution, i.e., network to allocate and distribute SL-PRS configuration; scheme 2 is for UE to perform mode 2-like autonomous SL-PRS resource allocation, which can be applied to in coverage partial coverage and out of coverage UEs. That is to say, in/partial coverage UEs can adopt scheme 1 or scheme 2, but out of coverage UE can only take scheme 2. From RAN3 perspective, we should focus on the coordination between NG-RAN and LMF for SL-PRS configuration.

- Focus on the coordination between NG-RAN and LMF for SL-PRS configuration with respect to scheme 1 in RAN3.

The following agreement were achieved regarding scheme 1 SL-PRS resource allocation:

Agreement in RAN1#110b-e Regarding Scheme 1 SL-PRS resource allocation, a transmitting UE receives a SL-PRS resource allocation signaling from the network. Consider one or more of the following options: - Opt. 1: through higher layers from the LMF - Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB - Up to further discussion which one or more of these shall be applicable Agreement in RAN1#112 Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further).

For the design of NG-RAN UE SL positioning architecture for scheme 1 SL-PRS resource allocation, LMF should also be part of sidelink positioning to manage the support of different location services for target UEs. Especially when UEs in one positioning session have more than one serving NG-RAN nodes (e.g. gNBs), it is necessary to use LMF to interact with multiple NG-RAN nodes for assistance data information transfer, the capability of NG-RAN nodes transfer and location information transfer. However, current sidelink communication does not need LMF’s participation and clearly LMF does not have any sidelink related information. In order to support LMF’s involvement for sidelink positioning and hybrid positioning, sidelink related information transfer between LMF and gNB should be supported (Figure 1 shows one possible signaling procedure).

Figure 1: Sidelink related information transfer between LMF and gNB

- Sidelink related information, e.g., SL-PRS resource allocation configuration should be informed from NG-RAN node to LMF, via a new NRPPa procedure.

For the periodic SL-PRS configuration (with shares the significant part of current LPP), there exists a issue. Different UEs may belong to different gNB’s reach, when serving gNB provides SL-PRS configuration to scheme 1 UE for transmitting, the scheme 1 UEs with different serving gNBs may possibly acquire SL-PRS configuration with some resource conflict. This will lead to an issue that, UEs will have interface with each other when sending the SL-PRS according to the overlapping SL-PRS configuration.

Figure 2. Example of different scheme 1 UEs in the same sidelink positioning session under different gNB’s coverage

For example in figure 2, UE 1 to UE m are all in coverage and in a same sidelink positioning session. In which, UE 1 and UE 2 have the common serving gNB as gNB 1, UE 3 has the serving gNB as gNB 2, and so on. UE 1 and UE 2 is configured to adopt scheme 1 and can receive the SL-PRS configuration from gNB 1, UE 3 is configured to adopt scheme 1 and can receive the SL-PRS configuration from gNB 2. UE 1 and UE 2 will not have resource conflict due to the serving gNB 1 can allocate well; however UE 1/UE 2 and UE 3 will have SL-PRS resource conflict since gNB1 and gNB 2 will not coordinate SL-PRS configuration of these three UEs.

- The periodic SL-PRS configuration, used for UE transmission, may cause resource conflict and interface problems as UEs belonging to different gNBs may acquire the same configuration.

- RAN3 is kindly asked to discuss how to resolve the SL-PRS resource conflict between different scheme 1 UEs in the same sidelink positioning session

Specifically, regarding how to resolve the SL-PRS resource conflict between different scheme 1 UEs in the same sidelink positioning session, we have the following solutions and corresponding analyses as shown in Table 1.

Table 1: LMF resolving SL-PRS resource conflict between different scheme 1 UEs

| Solution | Details |
| --- | --- |
| Solution 1: LMF to coordinate SL-PRS configuration of different gNBs | (1) LMF needs to send a request message via NRPPa to trigger each gNB to provide the SL-PRS configuration for scheme 1 UE (2) Each gNB provides the response message to LMF containing the configured SL-PRS configuration of each scheme 1 UE. (3a) LMF resolves the interference by LMF’s implementation, for example LMF deletes some of the overlapping SL-PRS resources of some of the gNBs. LMF provides the non-overlapped SL-PRS configuration to mode 1 UEs via LPP. (3b) LMF sends the feedback (e.g. collected all gNBs’ SL-PRS configuration, or muting pattern) to each gNB, gNB distribute the modified SL-PRS configuration to each scheme 1 UE. |
| Solution 2: LMF to coordinate SL-PRS configuration of different gNBs. | (1) LMF needs to send a request message via NRPPa provide the requested SL-PRS configuration and to trigger each gNB to provide the SL-PRS configuration for scheme 1 UE (2) Each gNB provides the response message to LMF containing the configured SL-PRS configuration of each scheme 1 UE. |
| Solution 3: LMF ask gNBs to coordinate with each other | (1) LMF sends a request message to gNB(s) and the request message is to request gNB(s) to coordinate SL-PRS configuration with other gNB (s) (2) The gNB sets up Xn interface with the other gNB(s) as requested by LMF, and the gNB sends the SL-PRS configuration of itself to other gNB via Xn interface. (3) After receiving one gNB’s SL-PRS configuration, the other gNB(s) can configure non-overlapped SL-PRS configuration. |

Among solution 1, 2 and solution 3, we prefer solution 1 and 2 since solution 3 includes some Xn interface design where even the legacy Rel-16/17 positioning do not require the positioning information interaction between gNBs. Solution 1 and 2 only need to reuse the NRPPa procedure between LMF and gNB. Considering the UE may usually be configured with periodic, aperiodic or semi-persistent SL-PRS together, we think a unified design is desired, that is, regardless of periodic/aperiodic/semi-persistent SL-PRS (SL-PRS time-domain behavior), serving gNB of the UE should configure the SL-PRS configuration for UE. 3a) in Solution1 can not be accepted because RAN1 agreed that SL-PRS resource allocation is configured from gNB rather LMF.

Above all, if LMF is able to decide the SL-PRS configuration to avoid the conflict, solution 2 can be acceptable, otherwise, solution 1 with 3b) is acceptable.

- For scheme 1 SL-PRS resource allocation, support SL-PRS configuration transfer between LMF and gNB in order to coordinate SL-PRS resources among UEs from different gNBs. The solution to be further discussed.

Integrity of RAT-dependent positioning

According to RAN1’s agreements during the SI phase, the mapping between error sources and positioning methods are discussed. In RAT-dependent positioning, the main task for RAN3 is to design signaling structure of different error source distributions agreed by RAN1/2.

UL assistance data (NG-RAN node - LMF)

RAN1#110, RAN1#110-bis and RAN1#111 have provided error sources of UL assistance data in 38.455 as follows:

Agreement in RAN1#110 For LMF-based positioning integrity mode, ARP location (e.g., ARPLocationInformation in TS 38.455) is an error source for UL-AoA. FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity) Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 FFS : Whether the error statistics of ARP location is available at the gNB Other error sources are not precluded Agreement in RAN1#110 For LMF-based positioning integrity mode, at least inter-TRP synchronization is an error source for UL-TDOA. FFS : Specification impact of inter-TRP synchronization as an error source for UL-TDOA Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 Agreement in RAN1#110-bis For LMF-based positioning integrity mode, TRP location (e.g., Geographical Coordinates in TS 38.455) is an error source for DL-TDOA, DL-AoD, UL-TDOA, UL-AoA and multi-RTT. oNote : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 oFFS : Specification impact of TRP location as an error source for LMF-based positioning integrity mode oFFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity) Agreement in RAN1#111 For LMF-based positioning integrity mode, for UL-TDOA, inter-TRP synchronization error can be caused in part by errors in SFN initialization time. Note: Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857.

TRP location is provided per TRP in NRPPa message; ARP location is configured per TRP since one TRP can associate multiple ARPs, and inter-TRP synchronization is indicated in the 38.455 as SFN Initialisation Time in the IE TRP information. Similar to the DL assistance data, the error source of ARP location and inter-TRP synchronization can have their corresponding error distribution parameters, and the error distribution parameters of these two error sources can be configured per TRP information in 38.455.

Since in RAN2#121, the following agreement is achieved toward the TRP related error source bound provision to LMF:

Agreement: Any interaction between the LMF and NG-RAN to support determination of error sources is in RAN3 scope. Other aspects of determining the TRP error sources are left to deployment and implementation.

Since the TRP error sources is the static information of TRP in NG-RAN node, introducing the TRP error sources for ARP location, TRP location and inter-TRP synchronization is a straightforward and simple way.

- Introduce the error sources for ARP location, TRP location and inter-TRP synchronization in the TRP information IE.

TRP measurments

RAN1#110 and RAN1#110-b have provided error sources of UL positioning measurements as follows:

Agreement in RAN1#110 For LMF-based positioning integrity mode, at least the followings are error sources for timing related measurements : - RSTD measurement is an error source for DL-TDOA - RTOA measurement is an error source for UL-TDOA - UE Rx-Tx time difference measurement is an error source for Multi-RTT - gNB Rx-Tx time difference measurement is an error source for Multi-RTT FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 Agreement in RAN1#110 For LMF-based positioning integrity mode, at least angle of arrival measurement is an error source for UL-AoA FFS : Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) FFS: The error can be expressed as the error of the AoA/ZoA in LCS or GCS or the error of a defined function of AoA/ZoA in LCS. Note : Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 Agreement in RAN1#110-b - Study the following alternatives for expression of angle of arrival measurement error for determination of positioning integrity for UL-AoA, and down select between Alt 1 and Alt 2: - Alt. 1 : No conversion (e.g., the measurement error is expressed as error in AoA or ZoA in LCS/GCS) - Alt. 2 : conversion function ( defined function of AoA/ZoA in LCS) - FFS : Distribution of AoA measurement error for an NLOS/LOS link - FFS : Other Details (e.g., mean, standard deviation)

For UL/UL+DL positioning (UL-TDOA, UL-AoA and multi-RTT), TRPs measure the SRS and perform measurement result, then each gNB gathers TRPs measurements and reports to LMF. There may be error sources existing at TRP/gNB side, for example TRP measurement error. Thus, Rel-18 should also support NG-RAN nodes to report error distribution parameters for TRP measurement error along with TRP measurement report. As the agreement indicated above, the error source of RTOA measurement, gNB Rx-Tx time difference measurement and angle of arrival measurement should have their corresponding error bound. Similar as DL measurements, since each TRP in 38.455 only reports one measurement for each measurement type, the error distribution parameters of RTOA measurement, gNB Rx-Tx time difference measurement and angle of arrival should be associated with multiple TRP’s measurements, to be specific, associated with each MEASUREMENT RESPONSE message or MEASUREMENT REPORT message in 38.455.

- For error source of RTOA measurement, gNB Rx-Tx time difference measurement and angle of arrival measurement, the error distribution parameters of the each error source should be associated with each MEASUREMENT RESPONSE message or MEASUREMENT REPORT message in 38.455.

The corresponding TPs to 38.413, 38.423, 38.455 and 38.473 are provided in the Annex.

Conclusion

We propose the following observations and proposals:

Sidelink positioning:

- Introduce the Ranging/SL positioning service authorization information in the following NGAP message:

  - INITIAL CONTEXT SETUP REQUEST

  - UE CONTEXT MODIFICATION REQUEST

  - HANDOVER REQUEST

  - PATH SWITCH REQUEST ACKNOWLEDGE

- Introduce the Ranging/SL positioning service authorization information in the following XnAP message:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

- The following information could be considered as authorization information related to Ranging/SL positioning over PC5:

- A "Ranging/SL positioning service authorized" IE

- The authorized Ranging/SL positioning parameters

- Focus on the coordination between NG-RAN and LMF for SL-PRS configuration with respect to scheme 1 in RAN3.

- Sidelink related information, e.g., SL-PRS resource allocation configuration should be informed from NG-RAN node to LMF, via a new NRPPa procedure.

- RAN3 is kindly asked to discuss how to resolve the SL-PRS resource conflict between different scheme 1 UEs in the same sidelink positioning session

- For scheme 1 SL-PRS resource allocation, support SL-PRS configuration transfer between LMF and gNB in order to coordinate SL-PRS resources among UEs from different gNBs. The solution to be further discussed.

Integrity of RAT-dependent positioning:

- Introduce the error sources for ARP location, TRP location and inter-TRP synchronization in the TRP information IE.

- For error source of RTOA measurement, gNB Rx-Tx time difference measurement and angle of arrival measurement, the error distribution parameters of the each error source should be associated with each MEASUREMENT RESPONSE message or MEASUREMENT REPORT message in 38.455.

- References

[1] TR23.700-86

[2] TS23.856

- Annex

- TP to 38.413

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

*************Skip unchanged part*************

- 9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Old AMF | O |  | AMF Name 9.3.3.21 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | C-ifPDUsessionResourceSetup |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| GUAMI | M |  | 9.3.3.3 |  | YES | reject |
| //Skip unchanged part |  |  |  |  |  |  |
| QMC Configuration Information | O |  | 9.3.1.223 |  | YES | ignore |
| Target NSSAI Information | O |  | 9.3.1.229 |  | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.231 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.233 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Ranging/SL positioning authorized | O |  | 9.3.1.xxx |  | YES | ignore |
| Ranging/SL positioning UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |
| Ranging/SL positioning PC5 QoS Parameters | O |  | 9.3.1.zzz | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |

*************Skip unchanged part*************

- 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the AMF to provide UE Context information changes to the NG-RAN node.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| RAN Paging Priority | O |  | 9.3.3.15 |  | YES | ignore |
| //Skip unchanged part |  |  |  |  |  |  |
| QMC Configuration Information | O |  | 9.3.1.223 |  | YES | ignore |
| QMC Deactivation | O |  | 9.3.1.222 |  | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.231 |  | YES | ignore |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.243 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.233 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Ranging/SL positioning authorized | O |  | 9.3.1.xxx |  | YES | ignore |
| Ranging/SL positioning UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |
| Ranging/SL positioning PC5 QoS Parameters | O |  | 9.3.1.zzz | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |

*************Skip unchanged part*************

- 9.2.3.4	HANDOVER REQUEST

This message is sent by the AMF to the target NG-RAN node to request the preparation of resources.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| Handover Type | M |  | 9.3.1.22 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| UE Aggregate Maximum Bit Rate | M |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| //Skip unchanged part |  |  |  |  |  |  |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.3.1.220 |  | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.231 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.233 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Ranging/SL positioning authorized | O |  | 9.3.1.xxx |  | YES | ignore |
| Ranging/SL positioning UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |
| Ranging/SL positioning PC5 QoS Parameters | O |  | 9.3.1.zzz | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |

*************Skip unchanged part*************

- 9.2.3.9	PATH SWITCH REQUEST ACKNOWLEDGE

This message is sent by the AMF to inform the NG-RAN node that the path switch has been successfully completed in the 5GC.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| UE Security Capabilities | O |  | 9.3.1.86 |  | YES | reject |
| Security Context | M |  | 9.3.1.88 |  | YES | reject |
| New Security Context Indicator | O |  | 9.3.1.55 |  | YES | reject |
| //Skip unchanged part |  |  |  |  |  |  |
| Time Synchronisation Assistance Information | O |  | 9.3.1.220 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.233 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.243 |  | YES | ignore |
| Ranging/SL positioning authorized | O |  | 9.3.1.xxx |  | YES | ignore |
| Ranging/SL positioning UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |
| Ranging/SL positioning PC5 QoS Parameters | O |  | 9.3.1.zzz | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |

*************Skip unchanged part*************

- 9.3.1.xxx	5G ProSe Authorized

This IE provides information on the authorization status of the UE to use the Ranging/SL positioning services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ranging/SL positioning-enabled UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for Ranging/SL positioning |

- 9.3.1.zzz	Ranging/SL positioning PC5 QoS Parameters

This IE provides information on the Ranging/SL positioning PC5 QoS parameters of the UE’s sidelink communication for Ranging/SL positioning services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ranging/SL positioning PC5 QoS Flow List |  | 1 |  |  |
| > Ranging/SL positioning PC5 QoS Flow Item |  | 1..<maxnoofPC5QoSFlows> |  |  |
| >>PQI | M |  | INTEGER (0..255, …) | PQI is a special 5QI as specified in TS 23.501 [9]. |
| >> Ranging/SL positioning PC5 Flow Bit Rates |  | 0..1 |  | Only applies for GBR QoS flows. |
| >>>Guaranteed Flow Bit Rate | M |  | Bit Rate 9.3.1.4 | Guaranteed Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9]. |
| >>>Maximum Flow Bit Rate | M |  | Bit Rate 9.3.1.4 | Maximum Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9]. |
| >>Range | O |  | ENUMERATED (m50, m80, m180, m200, m350, m400, m500, m700, m1000, …) | Only applies for groupcast. |
| Ranging/SL positioning PC5 Link Aggregate Bit Rates | O |  | Bit Rate 9.3.1.4 | Only applies for Non-GBR QoS flows. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flows allowed towards one UE. Value is 2048. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

- TP to 38.423

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

*************Skip unchanged part*************

- 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| *************Skip unchanged part************* |  |  |  |  |  |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Ranging/SL positioning authorized | O |  | 9.2.3.xxx |  | YES | ignore |
| Ranging/SL positioning PC5 QoS Parameters | O |  | 9.2.3.zzz | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |

*************Skip unchanged part*************

- 9.1.1.9	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information – Retrieve UE Context Response | M |  | 9.2.1.13 |  | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Ranging/SL positioning authorized | O |  | 9.2.3.xxx |  | YES | ignore |
| Ranging/SL positioning PC5 QoS Parameters | O |  | 9.2.3.zzz | This IE applies only if the UE is authorized for Ranging/SL positioning services | YES | ignore |

*************Skip unchanged part*************

- 9.2.3.xxx	5G ProSe Authorized

This IE provides information on the authorization status of the UE to use the Ranging/SL positioning services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ranging/SL positioning-enabled UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for Ranging/SL positioning |

- 9.2.3.zzz	Ranging/SL positioning PC5 QoS Parameters

This IE provides information on the Ranging/SL positioning PC5 QoS parameters of the UE’s sidelink communication for Ranging/SL positioning services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ranging/SL positioning PC5 QoS Flow List |  | 1 |  |  |
| > Ranging/SL positioning PC5 QoS Flow Item |  | 1..<maxnoofPC5QoSFlows> |  |  |
| >>PQI | M |  | INTEGER (0..255, …) | PQI is a special 5QI as specified in TS 23.501 [9]. |
| >> Ranging/SL positioning PC5 Flow Bit Rates |  | 0..1 |  | Only applies for GBR QoS flows. |
| >>>Guaranteed Flow Bit Rate | M |  | Bit Rate 9.3.1.4 | Guaranteed Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9]. |
| >>>Maximum Flow Bit Rate | M |  | Bit Rate 9.3.1.4 | Maximum Bit Rate for the PC5 QoS flow. Details in TS 23.501 [9]. |
| >>Range | O |  | ENUMERATED (m50, m80, m180, m200, m350, m400, m500, m700, m1000, …) | Only applies for groupcast. |
| Ranging/SL positioning PC5 Link Aggregate Bit Rates | O |  | Bit Rate 9.3.1.4 | Only applies for Non-GBR QoS flows. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flows allowed towards one UE. Value is 2048. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

- TP to 38.455

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

- 9.1.1.xx	 SIDELINK POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request SL-PRS positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SL-PRS Transmission Characteristics (FFS) | O |  | 9.2.XX |  | YES | ignore |

- 9.1.1.yy	SIDELINK POSITIONING INFORMATION RESPONSE

This message is sent by the NG-RAN node to provide SL-PRS positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SL-PRS Configuration (FFS) | O |  | 9.2.YY |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

- 9.1.1.zz	SIDELINK POSITIONING INFORMATION FAILURE

This message is sent by the NG-RAN node to indicate that the sidelink positioning information cannot be provided.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

- 9.1.1.mm	POSITIONING INFORMATION UPDATE

This message is sent by the NG-RAN node to indicate that a change in the SL-PRS configuration.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SL-PRS Configuration (FFS) | O |  | 9.2.YY |  | YES | ignore |

*************Skip unchanged part*************

- 9.2.36	Relative Time 1900

This information element indicates the initialisation time (e.g. SFN Initalisation Time for a cell, requested time for an action, etc).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Time 1900 | M |  | BIT STRING (SIZE(64)) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |  |  |  |
| Error Distribution | O |  | 9.2.xx |  | YES | ignore |

*************Skip unchanged part*************

- 9.2.38	UL Angle of Arrival

This information element contains the uplink Angle of Arrival measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival | M |  | INTEGER(0..3599) | TS 38.133 [16] |  |  |
| Zenith Angle of Arrival | O |  | INTEGER(0..1799) | TS 38.133 [16] |  |  |
| LCS to GCS Translation | O |  | 9.2.69 | If absent, the azimuth and zenith are provided in GCS. |  |  |
| Error Distribution | O |  | 9.2.xx |  | YES | ignore |

- 9.2.39	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] | - |  |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP Rx TEG Information | O |  | 9.2.85 |  | YES | ignore |
| Error Distribution | O |  | 9.2.xx |  | YES | ignore |

*************Skip unchanged part*************

- 9.2.40	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] | - |  |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP TEG Information | O |  | 9.2.80 |  | YES | ignore |
| Error Distribution | O |  | 9.2.xx |  | YES | ignore |

*************Skip unchanged part*************

- 9.2.46	Geographical Coordinates

This information element contains the geographical coordinates for the TRP and any associated ARP(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE TRP Position Definition Type | M |  |  |  | - |  |
| >Direct |  |  |  |  |  |  |
| >>CHOICE Accuracy | M |  |  |  |  |  |
| >>>normal accuracy |  |  |  |  |  |  |
| >>>>TRP Position | M |  | NG-RAN Access Point Position 9.2.10 | The configured estimated geographical position of the antenna of the cell/TRP. |  |  |
| >>>>Error Distribution | O |  | 9.2.xx |  | YES | ignore |
| >>>high accuracy |  |  |  |  |  |  |
| >>>>TRP High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.2.49 | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. |  |  |
| >>>>Error Distribution | O |  | 9.2.xx |  | YES | ignore |
| >Referenced |  |  |  |  |  |  |
| >>Reference Point | M |  | 9.2.51 | The reference point is used to derive the TRP position |  |  |
| >>CHOICE Type | M |  |  |  |  |  |
| >>>Geodetic |  |  |  |  |  |  |
| >>>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.48 | The configured estimated relative geodetic coordinate of the antenna of the cell/TRP |  |  |
| >>>>Error Distribution | O |  | 9.2.xx |  | YES | ignore |
| >>>Cartesian |  |  |  |  |  |  |
| >>>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.50 | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP |  |  |
| >>>>Error Distribution | O |  | 9.2.xx |  | YES | ignore |
| DL-PRS Resource Coordinates | O |  | 9.2.47 | DL-PRS Resource Coordinates relative to the TRP coordinate | - |  |
| ARP Location Information | O |  | 9.2.76 |  | Yes | ignore |
| Error Distribution | O |  | 9.2.xx |  | YES | ignore |

- 9.2.XX	 Error distribution

This IE contains error distribution information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Mean | O |  | FFS |  |
| Standard Deviation | O |  | FFS |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

- TP to 38.473

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Start of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

- 9.2.12.xx	SIDELINK POSITIONING INFORMATION REQUEST

This message is sent by the CU to DU to request SL-PRS positioning information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SL-PRS Transmission Characteristics (FFS) | O |  | 9.2.XX |  | YES | ignore |

- 9.2.12.yy	SIDELINK POSITIONING INFORMATION RESPONSE

This message is sent by the DU to CU to provide SL-PRS positioning information.

- Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SL-PRS Configuration (FFS) | O |  | 9.2.YY |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

- 9.2.12.zz	SIDELINK POSITIONING INFORMATION FAILURE

This message is sent by the gNB-DU to indicate that the sidelink positioning information cannot be provided.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

- 9.2.12.mm 	SIDELINK POSITIONING INFORMATION UPDATE

This message is sent by the gNB-DU to indicate that a change in the SL-PRS configuration.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SL-PRS Configuration (FFS) | O |  | 9.2.YY |  | YES | ignore |

*************Skip unchanged part*************

- 9.3.1.167	UL Angle of Arrival

This information element contains the uplink Angle of Arrival measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival | M |  | INTEGER(0..3599) | TS 38.133 [16] |  |  |
| Zenith Angle of Arrival | O |  | INTEGER(0..1799) | TS 38.133 [16] |  |  |
| LCS to GCS Translation | O |  | 9.3.1.241 | If absent, the azimuth and zenith are provided in GCS. |  |  |
| Error Distribution | O |  | 9.3.1.xxx |  | YES | ignore |

- 9.3.1.168	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 |  |  |  |  |  |  |
| >>k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] | - |  |
| >k1 |  |  |  |  |  |  |
| >>k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] | - |  |
| >k2 |  |  |  |  |  |  |
| >>k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] | - |  |
| >k3 |  |  |  |  |  |  |
| >>k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] | - |  |
| >k4 |  |  |  |  |  |  |
| >>k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] | - |  |
| >k5 |  |  |  |  |  |  |
| >>k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] | - |  |
| Additional Path List | O |  | 9.3.1.169 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.3.1.248 |  | YES | ignore |
| TRP Rx TEG Information | O |  | 9.3.1.280 |  | YES | ignore |
| Error Distribution | O |  | 9.3.1.xxx |  | YES | ignore |

*************Skip unchanged part*************

- 9.3.1.170	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 |  |  |  |  |  |  |
| >>k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] | - |  |
| >k1 |  |  |  |  |  |  |
| >>k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] | - |  |
| >k2 |  |  |  |  |  |  |
| >>k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] | - |  |
| >k3 |  |  |  |  |  |  |
| >>k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] | - |  |
| >k4 |  |  |  |  |  |  |
| >>k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] | - |  |
| >k5 |  |  |  |  |  |  |
| >>k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] | - |  |
| Additional Path List | O |  | 9.3.1.169 |  | - |  |
| Extended Additional Path List | O |  | 9.3.1.248 |  | YES | ignore |
| TRP TEG Information | O |  | 9.3.1.253 |  | YES | ignore |
| Error Distribution | O |  | 9.3.1.xxx |  | YES | ignore |

*************Skip unchanged part*************

- 9.3.1.183	Relative Time 1900

This information element indicates the initialisation time (e.g. SFN Initalisation Time for a cell, requested time for an action, etc).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Relative Time 1900 | M |  | BIT STRING (SIZE(64)) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |  |  |  |
| Error Distribution | O |  | 9.2.xx |  | YES | ignore |

- 9.3.1.184	Geographical Coordinates

This information element contains the geographical coordinates for the TRP and any associated ARP(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE TRP Position Definition Type | M |  |  |  | - |  |
| >Direct |  |  |  |  |  |  |
| >>CHOICE Accuracy | M |  |  |  |  |  |
| >>>normal accuracy |  |  |  |  |  |  |
| >>>>TRP Position | M |  | NG-RAN Access Point Position 9.3.1.174 | The configured estimated geographical position of the antenna of the cell/TRP. |  |  |
| >>>>Error Distribution | O |  | 9.3.1.xxx |  | YES | ignore |
| >>>high accuracy |  |  |  |  |  |  |
| >>>>TRP High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.3.1.190 | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. |  |  |
| >>>>Error Distribution | O |  | 9.3.1.xxx |  | YES | ignore |
| >Referenced |  |  |  |  |  |  |
| >>Reference Point | M |  | 9.3.1.188 | The reference point is used to derive the TRP position |  |  |
| >>CHOICE Type | M |  |  |  |  |  |
| >>>Geodetic |  |  |  |  |  |  |
| >>>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.3.1.186 | The configured estimated relative geodetic coordinate of the antenna of the cell/TRP |  |  |
| >>>>Error Distribution | O |  | 9.3.1.xxx |  | YES | ignore |
| >>>Cartesian |  |  |  |  |  |  |
| >>>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.3.1.187 | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP |  |  |
| >>>>Error Distribution | O |  | 9.3.1.xxx |  | YES | ignore |
| DL-PRS Resource Coordinates | O |  | 9.3.1.185 | DL-PRS Resource Coordinates relative to the TRP coordinate | - |  |
| ARP Location Information | O |  | 9.3.1.243 |  | Yes | ignore |
| Error Distribution | O |  | 9.3.1.xxx |  | YES | ignore |

- 9.3.1.xxx	 Error distribution

This IE contains error distribution information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Mean | O |  | FFS |  |
| Standard Deviation | O |  | FFS |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<End of changes>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>