3GPP TSG-RAN WG3 Meeting #119bis-e	R3-231842

# Online, 17 -26 April 2023

Agenda item:		12.3

Source:	ZTE, Lenovo, Samsung

Title:	Discussion on MDT enhancement for AI/ML data collection

Document for:		Discussion and Decision

Introduction

In the previous meeting, the following working assumption was agreed, and the solution needs to be discussed.

WA: There are the benefits in enabling UEs to continuously collect MDT measurement across RRC states and to provide to the network such continuous time series of data for the purpose of AI/ML Training/Retraining. The evaluation of gains vs impacts related to the use cases in R18 coming from potential solutions needs to be further discussed.

Solutions on how to achieve Continuous MDT are FFS

In this contribution, we focus on the analysis on benefits of the continuous AI/ML data collection and provide the solutions to support the continuous MDT data collection.

Discussion

Several companies still have concerns about the benefits of continuous data collection for AI. From the perspective of AI/ML training data, having a continuous series of data is crucial to building an accurate training dataset. For time-series based AI/ML models like LSTM, the input data for both model inference and model training is a time series of continuous information. To illustrate this point, let's see the Figure 1. The data collection is requested from T0 to T7, but it cannot be guaranted that UE (user equipment) will not change its state during that time. If the state of the UE does change, the data collection will be interrupted, which in turn interrupts the data collection necessary for AI/ML training and inference.

Figure 1. Continuous UE measurements from T0 to T7.

Training dataset may be composed of a series of data, as shown below:

Data1: {(x1, t1), (x2, t2), …, (x5, t5)}

Data2: {(x2, t2), (x3, t3), …, (x6, t6)}

Data3: {(x3, t3), (x4, t4), …, (x7, t7)}

…

DataN:{(xN,tN), (xN+1, tN+1), …,(xN+4, tN+4)}}

For example, if the reporting of requested measurements is interrupted across UE RRC states, the training dataset will be invalid, or we may need to apply normalization techniques to the dataset to fill in the gaps caused by the interruption. This is why enhancing MDT to support continuous reporting is crucial.

Observation 1: Based on the current collection mechanisms, the continuous data collection for AI/ML will be interrupted when UE state is changed.

Proposal 1: Enhancing MDT to support continuous reporting is important to build an accurate training dataset.

Currently, existing MDT mechanisms can serve as the baseline and there is no need to define a new signaling to transfer UE measurements. For model training and inference, the input data should be consecutive and a time series of information per a certain granularity. In the case of AI/ML-based mobility optimization, as well as for other AI/ML use cases, input information would be retrieved from the UE side. However, the current MDT mechanism is unable to support the collection of input information for AI/ML model training and inference.

For example, when a UE enters a connected state from an idle/inactive state, continuous data collection would be interrupted. When a UE is in the idle/inactive state, it collects data based on the logged MDT configuration. But when it enters the connected state, the logged MDT configuration is interrupted, and measurements are reported to the RAN directly, causing the interruption of continuous measurements and invalidating the training dataset, as discussed before.

To meet the continuous input data requirement for AI/ML model training and inference, existing MDT mechanisms need to be enhanced to support the collection of input information from the UE. This enhancement would ensure that the input data is uninterrupted even when the UE changes its state, thus allowing for more accurate and effective AI/ML model training and inference.

Proposal 1: Current MDT mechanism should be enhanced in order to support continuous AI/ML data collection.

Currently, following are the input information from UE across three AI/ML based use cases, captured in the TR37.817:

===============================TR37.817===============================

From the UE:

-	UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available

-	UE Mobility History Information

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

===============================TR37.817===============================

Now we agree that cell-level UE trajectory is considered as a start point, and geographic UE trajectory is FFS, when we discuss the solution of the UE trajectory prediction. However, it does not imply the input information used for UE trajectory prediction should only be historical cell-level UE trajectory information.

In fact, the accuracy of UE trajectory prediction can be improved by incorporating fine-grained input information, such as UE geographic location, in addition to historical cell-level UE trajectory information. Therefore, while cell-level UE trajectory is a good starting point, it's important to consider other sources of input information for more accurate UE trajectory prediction.

Proposal 2: The accuracy of UE trajectory prediction can be improved by incorporating fine-grained input information, such as UE geographic location, in addition to historical cell-level UE trajectory information.

UE geographic location, as the input information from UE used for trajectory prediction, the UE location from UE side needs the requirement of continuous data collection for AI/ML based load balancing, AI/ML based energy saving, and AI/ML based mobility optimization.

Proposal 3: Enhance the MDT reporting mechanisms to support collect continuous UE geographic location.

Hence, when UE location information needs to be used as input for model training or inference in NG-RAN node, the historical UE location information, for a past period of time needs to be collected. To facilitate this, it is recommended to introduce an indication in the logged MDT measurement configuration to inform the UE whether to collect continuous historical information (such as historical cell information, coordinates, latitude, longitude, altitude, etc.) regardless of the UE state, whether it is idle, inactive, or connected.

Following are the signaling procedures for the continuous data collection:

Figure 1. Signaling procedures for the continuous data collection

Proposal 4: Introduce an indication in the logged MDT measurement configuration to indicate UE whether to collect continuous information (e.g., historical cell information, latitude, longitude, altitude, velocity, etc.) across various RRC states (connected, inactive, idle).

The MDT configuration would also include the measurement period and duration, which would determine how frequently the UE should collect and report the location information.

Proposal 5: The MDT configuration also includes measurement periodicity, and measurement duration.

In a previous meeting, some companies raised concerns that the approach of collecting continuous historical information for AI/ML model training and inference would burden the UE since it may store a large amount of measurement data. However, these concerns are not justified.

Firstly, the measurement duration is used to determine the time range for a measurement. This means that UE will not store a large quantity of measurements as the duration would specify the amount of data collected for each measurement. Therefore, this means that the UE will only store measurements for the measurement duration specified in the MDT configuration.

Observation 2: UE will only store measurements for the measurement duration specified in the MDT configuration.

Secondly, this approach is based on logged measurements. When the UE enters the connected state, the measurements stored in the UE are reported to the RAN directly, and if the continuous collection indication is available, the UE will continue to collect the measurements.

Observation 3: When the UE enters the connected state, the measurements stored in the UE are reported to the RAN directly so that the measurement may not always be stored in the UE.

Proposal 6: Collecting continuous data information does not bring the burdens to be UE.

Our solution is to introduce a "continuous MDT" indication, and the logged MDT configuration can also include a measurement duration parameter. When the UE moves to idle mode, it can start collecting the logged MDT measurements based on this configuration. And then when the UE moves from IDLE/INACTIVE to CONNECTED, UE continues to collect the measurements by itself based on the configuration of measurement duration. Once the measurement duration has elapsed, the UE can stop collecting the measurements and report them to the RAN. In addition, supporting company brings another solution to support the continuous data collection.

[1] proposes to introduce a "continuous MDT" flag in the logged MDT configuration. UE moves to idle mode and collects the logged MDT measurements. And then when the UE moves from IDLE/INACTIVE to CONNECTED, the UE indicates to the RAN, as part of the RRC Setup procedure, that it is configured with Continuous MDT. This allows the RAN to immediately configure the UE with Immediate MDT measurements.

There may some issues with the approach above:

- First, when the network side re-initiate the configuration, the network side may not know the time that the UE side has already measured, which could result in unnecessary measurement workloads.

- Secondly, in the case of mobility, this is particularly challenging in scenarios where different vendor OAMs are used, e.g., whether the configuration by the new RAN side is consistent with the previous configuration.

- Thirdly, the reconfiguration from RAN will still result in measurement interruption since the immediate MDT configuration is re-triggered.

Hence, since MDT measurement configurations are stored and performed in the UE, our approach is the most straightforward and simple solution with minor standard impact.

Proposal 7: Introducing an indication in the logged MDT measurement configuration to indicate UE to let UE collect measurement across various RRC state is the most straightforward and simple solution with minor standard impact.

Since the MDT enhancement needs to coordinate other groups, and there are the impacts on OAM and Uu interface, the LS should be sent to ask RAN2/SA5 whether the continuous AI/ML related information reporting from UE should be supported and whether the solution proposed by RAN3 is feasible.

Proposal 8: Send LS to RAN2/SA5 on the MDT enhancement to support continuous AI/ML related information reporting from UE to check the feasibility.

The corresponding TP is in [3].

Conclusion

We propose the following observations:

Observation 1: Based on the current collection mechanisms, the continuous data collection for AI/ML will be interrupted when UE state is changed.

Observation 2: UE will only store measurements for the measurement duration specified in the MDT configuration.

Observation 3: When the UE enters the connected state, the measurements stored in the UE are reported to the RAN directly so that the measurement may not always be stored in the UE.

We propose the following proposals:

Proposal 1: Enhancing MDT to support continuous reporting is important to build an accurate training dataset.

Proposal 2: The accuracy of UE trajectory prediction can be improved by incorporating fine-grained input information, such as UE geographic location, in addition to historical cell-level UE trajectory information.

Proposal 3: Enhance the MDT reporting mechanisms to support collect continuous UE geographic location.

Proposal 4: Introduce an indication in the logged MDT measurement configuration to indicate UE whether to collect continuous information (e.g., historical cell information, latitude, longitude, altitude, velocity, etc.) across various RRC states (connected, inactive, idle).

Proposal 5: The MDT configuration also includes measurement periodicity, and measurement duration.

Proposal 6: Collecting continuous data information does not bring the burdens to be UE.

Proposal 7: Introducing an indication in the logged MDT measurement configuration to indicate UE to let UE collect measurement across various RRC state is the most straightforward and simple solution with minor standard impact.

Proposal 8: Send LS to RAN2/SA5 on the MDT enhancement to support continuous AI/ML related information reporting from UE to check the feasibility.

- References

- R3-230457, MDT Enhancements for AI/ML, Ericsson, Deutsche Telekom, InterDigital

[2] R3-231844, [DRAFT] LS on the MDT enhancement to support continuous AI/ML related information reporting from UE, ZTE

[3] R3-231843, (TP to TS38.413) MDT Enhancements for continuous data collection, ZTE, Lenovo, Samsung