3GPP TSG-RAN WG3 Meeting #120	R3-232516

Incheon, Republic of Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1734 | rev | 3 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BLCR) X2AP CR on IoT NTN Functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT, Nokia, Nokia Shanghai Bell, Qualcomm, ZTE, CMCC, Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh |  |  |  |  |  | Date: |  |  | 2023-05-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN2 has agreed to support time-based CHO in IoT NTN, which is based on the R17 NR NTN solution. In R17 NR NTN, start time and duration to network signaling are added in the CHO signaling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Handover window start and duration are added to the HANDOVER REQUEST message, within the Conditional Handover Information Request IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for time-based CHO in X2AP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2, 9.1.1.1, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Add co-signer Rev2: Version for RAN3#119bises Rev3: Version for RAN3#120 |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source eNB initiates the procedure by sending the HANDOVER REQUEST message to the target eNB. When the source eNB sends the HANDOVER REQUEST message, it shall start the timer TRELOCprep.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message, the target eNB shall consider that the request concerns a conditional handover and shall include the Conditional Handover Information Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

UNCHANGED PART OMITTED

If the Maximum Number of CHO Preparations IE is included in Conditional Handover Information Acknowledge IE contained in the the HANDOVER REQUEST ACKNOWLEDGE message, then the source eNB should not prepare more candidate target cells for a CHO for the same UE towards the target eNB than the number indicated in the Maximum Number of CHO Preparations IE.

If the Estimated Arrival Probability IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target eNB may use the information to allocate necessary resources for the incoming CHO.

If the Conditional Handover Time Based Information IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target eNB may use this information to allocate necessary resources for the incoming CHO. If the Conditional Handover Time Based Information IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message but the UE does not appear in the candidate cell before the handover window duration indicated, the target eNB shall consider the conditional handover as canceled.

If the EPC Handover Restriction List Container IE is included in the HANDOVER REQUEST message, the target eNB shall, if supported, store this information in the UE context and shall use it as specified in TS 36.300 [15].

UNCHANGED PART OMITTED

****************** NEXT CHANGE ***************

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Basic Mobility Procedures

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source eNB to the target eNB to request the preparation of resources for a handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| GUMMEI | M |  | 9.2.16 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – |  |
| >UE Security Capabilities | M |  | 9.2.29 |  | – |  |
| >AS Security Information | M |  | 9.2.30 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – |  |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – |  |
| >E-RABs To Be Setup List |  | 1 |  |  | – |  |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >>>Bearer Type | O |  | 9.2.92 |  | YES | reject |
| >>>DAPS Request Information | O |  | 9.2.154 |  | YES | ignore |
| >>>Ethernet Type | O |  | 9.2.157 |  | YES | ignore |
| >>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>Security Indication | O |  | 9.2.181 |  | YES | reject |
| >RRC Context | M |  | OCTET STRING | Includes the RRC HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9], or the RRC HandoverPreparationInformation-NB message as defined in 10.6.2 of TS 36.331 [9]. | – |  |
| >Handover Restriction List | O |  | 9.2.3 |  | – |  |
| >Location Reporting Information | O |  | 9.2.21 | Includes the necessary parameters for location reporting | – |  |
| >Management Based MDT Allowed | O |  | 9.2.59 |  | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | YES | ignore |
| >UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.97 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| >EPC Handover Restriction List Container | O |  | 9.2.153 |  | YES | ignore |
| >Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.159 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.171 |  | YES | reject |
| >IMS voice EPS fallback from 5G | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| UE History Information | M |  | 9.2.38 | Same definition as in TS 36.413 [4] | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.33 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source eNB provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 36.331 [9]) | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.78 |  | YES | ignore |
| UE Context Reference at the SeNB | O |  |  |  | YES | ignore |
| >Global SeNB ID | M |  | Global eNB ID 9.2.22 |  | – |  |
| >SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | – |  |
| >SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | – |  |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| V2X Services Authorized | O |  | 9.2.93 |  | YES | ignore |
| UE Context Reference at the WT | O |  |  |  | YES | ignore |
| >WT ID | M |  | 9.2.95 |  | – |  |
| >WT UE XwAP ID | M |  | 9.2.96 |  | – |  |
| NR UE Security Capabilities | O |  | 9.2.107 |  | YES | ignore |
| UE Context Reference at the SgNB | O |  |  |  | YES | ignore |
| >Global en-gNB ID | M |  | 9.2.112 |  | – |  |
| >SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the SgNB. | – |  |
| Aerial UE subscription information | O |  | 9.2.129 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.136 |  | YES | ignore |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >New eNB UE X2AP ID | C-ifCHOmod |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | – |  |
| >New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Conditonal Handover Time Based Information | O |  |  | This IE only applies to NTN. |  |  |
| >>Handover Window Start | M |  | FFS | Pending to RAN2 final agreement | – |  |
| >>Handover Window Duration | M |  | FFS | Pending to RAN2 final agreement | – |  |
| NR V2X Services Authorized | O |  | 9.2.158 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.160 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |

Editor’s note:	This CR is a mirror as NR NTN and could be updated.

| Range bound | Explanation |
| --- | --- |
| maxnoofBearers | Maximum no. of E-RABs. Value is 256 |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

****************** NEXT CHANGE ***************

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

X2AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

UNCHANGED PART OMITTED

id-MIMOPRBusageInformation,

id-Additional-Measurement-Timing-Configuration-List,

id-ServedCellSpecificInfoReq-NR,

id-RAT-Restrictions,

id-PSCell-UE-HistoryInformation,

id-SensorMeasurementConfiguration,

id-CHOTimeBasedInformation,

maxnoofBearers,

maxCellineNB,

maxEARFCN,

maxEARFCNPlusOne,

newmaxEARFCN,

maxInterfaces,

UNCHANGED PART OMITTED

CHOinformation-REQ ::= SEQUENCE {

cho-trigger						CHOtrigger,

new-eNB-UE-X2AP-ID				UE-X2AP-ID											OPTIONAL

-- This IE shall be present if the cho-trigger IE is present and set to "CHO-replace" --,

new-eNB-UE-X2AP-ID-Extension	UE-X2AP-ID-Extension								OPTIONAL,

cHO-EstimatedArrivalProbability	CHO-Probability										OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CHOinformation-REQ-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-REQ-ExtIEs X2AP-PROTOCOL-EXTENSION ::={

{ID id-CHOTimeBasedInformation	CRITICALITY ignore	EXTENSION CHOTimeBasedInformation		PRESENCE optional},

...

}

CHOTimeBasedInformation ::= SEQUENCE {

cHO-HOWindowStart			CHO-HandoverWindowStart,

cHO-HOWindowDuration		CHO-HandoverWindowDuration,

iE-Extensions				ProtocolExtensionContainer { {CHOTimeBasedInformation-ExtIEs} }	OPTIONAL,

...

}

CHOTimeBasedInformation-ExtIEs	X2AP-PROTOCOL-EXTENSION ::= {

...

}

CHOinformation-ACK ::= SEQUENCE {

requestedTargetCellID			ECGI,

maxCHOpreparations				MaxCHOpreparations											OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-ACK-ExtIEs} }	OPTIONAL,

...

}

CHOinformation-ACK-ExtIEs X2AP-PROTOCOL-EXTENSION ::={

...

}

CandidateCellsToBeCancelledList ::= SEQUENCE (SIZE (1..maxnoofCellsinCHO)) OF ECGI

CHO-Probability ::= INTEGER (1..100)

CHO-HandoverWindowStart ::= FFS

CHO-HandoverWindowDuration ::= FFS

CNsubgroupID ::= INTEGER (0..7, ...)

UNCHANGED PART OMITTED

****************** NEXT CHANGE ***************

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

UNCHANGED PART OMITTED

id-SecurityIndication														ProtocolIE-ID ::= 435

id-SecurityResult															ProtocolIE-ID ::= 436

id-RAT-Restrictions															ProtocolIE-ID ::= 437

id-SCGreconfigNotification													ProtocolIE-ID ::= 438

id-MIMOPRBusageInformation													ProtocolIE-ID ::= 439

id-SensorMeasurementConfiguration											ProtocolIE-ID ::= 440

id-AdditionalListofForwardingGTPTunnelEndpoint								ProtocolIE-ID ::= 441

id-CHOTimeBasedInformation													ProtocolIE-ID ::= a

END

-- ASN1STOP

****************** END CHANGES ***************