3GPP TSG-RAN WG3 Meeting #120	             R3-232521

Incheon, Korea, 22nd – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.300 on AI/ML for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, CATT, Samsung, Lenovo, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-Core |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Further stage 2 updates on the introduction of RAN AI&ML |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3#117bis: - Split the section for AI/ML for NG-RAN into two subsections: General, Mechanisms and Principles - Further update the description RAN3#117: - Add the abbreviation of AI and ML - Introduce a section for AI/ML for NG-RAN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AI/ML for NG-RAN will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: - Capture the agreed TP in R3-225211 Rev1: - Capture the agreed TP in R3-226056 Rev2: - resubmit to RAN3#119 |  |  |  |  |  |  |  |  |

******************************************************Start of 1st Change****************************************************

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

<Unchanged Text Omitted>

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

ML	Machine Learning

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

******************************************************End of 1st Change****************************************************

******************************************************Start of 2nd Change****************************************************

X.X AI/ML for NG-RAN

X.X.1 General

AI/ML for NG-RAN, as a RAN internal function, is achieved by using Artificial Intelligence (AI) and Machine Learning (ML) techniques.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

X.X.2 Mechanisms and Principles

The AI/ML function requires inputs from neighbour NG-RAN nodes (e.g. predicted information, feedback information, measurements) and/or UEs (e.g. measurement results), in support to AI/ML processes such as AI/ML Model Inference and AI/ML Model Training.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

For the deployments of RAN intelligence, following scenarios may be supported:

•	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

•	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

******************************************************End of Change********************************************************