3GPP TSG-RAN WG3 Meeting #120	R3-232693

Incheon, Korea, 22 – 26 May 2023

Agenda item:	10.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on the MRO for the fast MCG recovery and for CPAC

Document for:	Discussion

# 1	Introduction

At the last meeting, RAN3 discussed MRO for various features, including MRO for the fast MCG recovery and MRO for CPAC.

In case of MRO for the fast MCG recovery, RAN3 reformulated definitions for some of the events related to the fast MCG recovery that MRO is supposed to address. These events are:

(a) SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running).

(b) The signalling delay is longer than the time the UE waits for the response (T316 expired).

(f1) The SCG fails or is deactivated yet before the UE sends the MCGFailureInformation.

So far, detection of these issues is supposed to be based on reporting from the UE, as requested in [1]. In this paper, we consider if such detection is possible also for pre-Rel.18 UEs.

In case of MRO for CPAC, RAN3 formulated definitions of failure events related to CPAC that interest MRO. Based on these definitions, RAN3 may start considering signalling solution.

# 2	Discussion

## 2.1	MRO for the fast MCG recovery

Let us consider the cases (a) and (b): if there was no data transmission when MCG failed, the MN may not be aware of the RLF until it receives the MCGFailureInformation from the SN. When it received the information, it prepares MCG reconfiguration and sends it to the UE. If the UE does not reconnect after T316 expires, the MN may assume it is lost (and sends Release towards the SN) – but technically, it does not know if the SCG failed yet before the MCG reconfiguration was delivered (case a), or the SN was overloaded and thus processing delay cause the reconfiguration be delayed beyond T316.

Observation 1-1: Without the Rel.18 information from the UE, the MN can’t identify scenario (a) from scenario (b).

Scenario f1 can likely be recognised though: if the MN configured fast MCG recovery to the UE, it did not receive SCG deactivation information from the SN, but receives the RRC Reestablishment from the UE (or the RLF INFORATION from one of its neighbours) without receiving the MCGFailureInformation, then the MN may assume SCG failed yet before the UE was able to send the MCGFailureInformation to the SN.

Observation 1-2: Case f1 can likely be detected and identified in the MN for pre-Rel.18 UEs.

Let us then focus on (a) and (b). The key difference between cases (a) and (b) is the fact that in case (a), the SN will likely be able to make full attempt to send the MCG reconfiguration to the UE before T316 expires, but without any response from the UE, while in case (b), it will not be able to start sending it before T316 expires or will not complete all the retransmissions before T316 expires.

The information that the SN completed full attempt of delivering MCG reconfiguration to the UE when T316 was still running, or could not do so, is only available in the SN – if the SN knows the configured T316. It can then inform the MN that the timer run out after full set of retransmission without success (i.e. SCG likely failed while T316 was running – case (a)), or that the timer expired before it had time to start sending the MCG reconfiguration to the UE, or before all the retransmissions were done (i.e. the timer was too short for the delay in overall signalling – case (b)).

Proposal 1-1: In order to enable identification of cases (a) and (b) for UEs that do not support Rel.18 reporting, the SN should be informed about the T316 value configured in the UE, and then enabled to report the result of the reconfiguration attempt back to the MN.

Providing the information about the T316 timer to the SN enables the latter to detect the situation where the UE is lost. In this case, the DC operation can’t be continued and the SN must be released. Therefore, the SN does not need to wait for the Release from the MN, but may start the Release on its own. The assistance information can be therefore included in the SN-initiated Release – or, if the MN sent it first, in the response to the MN’s Release.

Proposal 1-2: Assistance information from the SN can be included in the SN-initiated Release or in the response to the MN-initiated Release procedures.

Furthermore, in scenario (b), the SN will try sending the MCG reconfiguration even if the UE does not await it any more – the timer has already expired. The only way to stop it is to send the Release from the MN, but this will also be delayed, because MN’s implementation will surely have time buffers and will not react immediately when the T316 expires (besides, it can start counting time from the moment it received the MCGFailureInformation, which means it will be badly delayed). If the SN will try to send the MCG reconfiguration to the UE until it receives Release from the MN, it is very likely to waste badly radio resources.

Observation 1-3: delivery of the T316 information to the SN enables it to perform MRO optimisation and to avoid waste radio resources.

## 2.2	MRO for CPAC

At RAN3 #119, RAN3 agreed to reuse existing procedures defined in Rel.17 for the classic SCG mobility:

Reusing R17 signalling mechanism to report CPA/CPC failure/ related information over Xn from MN to source SN or last serving SN.

The Rel.17 solution assumes following steps:

- MN receives the SCG failure information from the UE;

- MN verifies with the last serving SN if there was any intra-SN PSCell change execution, which could fail;

- If there was none, but there was recent SN-initiated SN change execution, the MN forwards the information to the source SN;

Step (1) is the same in case of CPAC. Step (2) is meant to verify if there was any intra-SN PSCells chang and this is relevant also in case of CPAC. One may observe that if the last SN change was CPC, there could be intra-SN classic PSCell change in the target SN. If the UE is pre-Rel.18, the MN will not be able to identify the type of intra-SN change. Nonetheless, if there was such change, and failed, the target SN will know it.

Observation 2-1: The MN shall check with the last serving SN if there was any intra-SN PSCell change (classic or CPC) also in case of MRO for CPAC.

Step (3) is needed if there was a recent SN-initiated inter-SN CPC execution and is used to inform the source SN about the failure. In case of CPC, this node must also be notified.

Observation 2-2: Also in case of MRO for CPAC, the MN shall inform the source node if there was recent SN-initiated CPC and no further intra-SN PSCell change in the target SN.

One difference, as compared to the MRO for the classic SCG mobility, is the fact that the responsibility of the CPC execution is split between the source SN and the target SN, depending on the best PSCell reported from the UE. To correctly identify the scenario, the list of PSCells proposed to be prepared and those prepared shall be first checked in the MN – but the MN, which is assumed to keep UE context, knows both lists and can verify if the best reported PSCell belongs to them (the MN can also read the measurements – RAN2 agreed that MR-DC is to be addressed first). Therefore, the mechanism must enable consultation with both of the SNs – but the existing signalling enables it and thus it can be used also in Rel.18.

Observation 2-3: The existing mechanism enables correct detection of a CPC to wrong target PSCell (the MN must be enhanced to check the best PSCell with the stored lists of all and prepared target PSCells).

Finally, another new aspect is CPA: in case of MRO for classic SCG mobility, the Addition has not been considered. In Rel.18, two scenarios for CPA are defined: too early CPA or CPA to wrong target PSCell. In case of too early CPA, the problem is on the MN side, thus there is no need to consult any SN – no signalling is needed.

Observation 2-4: In case of too early CPA, the problem is on the MN side and thus no signalling is needed.

In case of CPA to wrong target PSCell, the problem may be either on the side of the MN, or the target SN. The MN must also verify that there was no intra-SN cell change after the addition. Therefore, the steps 1-2 from the classic Rel.17 must be applied, only step 3 is not needed.

Observation 2-5: To identify a CPA to wrong cell, the MN must consult the reported failure with the target SN (also, to exclude intra-SN PSCell change). The existing signalling can be reused.

Proposals 2-1: Based on the analysis, RAN3 concludes that no stage-3 changes are needed to handle the MRO for CPAC.

One may consider also stage-2 changes in the description of the mechanism. The Rel.17 description is provided in the TS 37.340:

MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

One may observe, that the description defines “responsible” nodes. Therefore, it can cover also cases that were not considered in Rel.17, e.g. addition or CPC to wrong PSCell.

Observation 2-6: The existing stage-2 description of the overall mechanism is general enough to cover also handling of CPAC scenarios, including CPA.

Based on this, we can conclude:

Proposal 2-2: No further work is needed in RAN3 to enable detection of MRO scenarios for CPAC.

However, MRO is not only detection, but also correction. We have already discussed some options in [1], let us remind it:

An additional problem concerns the selection of the list of allowed PSCells and the final target PSCell. Currently, both are done based on measurements reported from the UE at the moment when CPC is initiated. However, at the moment the UE executes the configured CPC, the situation may be slightly different, but not so much that the CPC would need to be re-initialised. Therefore, the Target SN may prepare wrong cells out of the allowed set, and if the failure is detected it still won’t be able to make a better decision in future – because the decision is always made based on possibly outdated measurements. Furthermore, the Target SN is allowed to decide how many PScells to prepare up to a maximum number indicated by the initiating node. For example, even if two allowed PSCells are indicated by the initiating node and the maximum number of prepared PScClls is set also to two, it is permissible that the Target SN prepares just one PSCell.

Proposal 2-3: RAN3 shall consider if the initiating node can support the Target SN in selecting the right target PSCell(s).

The initiating node may process the SCGFailureInformation messages sent by multiple UEs and determine which PSCells make most sense to be prepared in the future. This information can then be indicated to the Target SN to aid it in preparing the right PSCells via the preparation messages.

Proposal 2-4: The initiating node may indicate additional information to Target SN via the preparation messages to aid correct PSCell selection for preparation.

Such information may include a prioritisation of certain PScells over others and/or the probability that a given PSCell was the correct one in the past,

Proposal 2-5: The additional information may comprise of a prioritisation of certain PSCells over others and/or the probability that a given PSCell was the correct one in the past.

# 3	Conclusions

In this paper, we have addressed two areas where MRO is considered in Rel.18. In case of MRO for the fast MCG recovery, we conclude what follows:

Observation 1-1: Without the Rel.18 information from the UE, the MN can’t identify scenario (a) from scenario (b).

Observation 1-2: Case f1 can likely be detected and identified in the MN for pre-Rel.18 UEs.

Proposal 1-1: In order to enable identification of cases (a) and (b) for UEs that do not support Rel.18 reporting, the SN should be informed about the T316 value configured in the UE, and then enabled to report the result of the reconfiguration attempt back to the MN.

Proposal 1-2: Assistance information from the SN can be included in the SN-initiated Release or in the response to the MN-initiated Release procedures.

Observation 1-3: delivery of the T316 information to the SN enables it to perform MRO optimisation and to avoid waste radio resources.

A TP implementing the above solution for the MRO for the fast MCG recovery is proposed in the Annex below.

Then, in the context of MRO for CPAC, we conclude that:

Observation 2-1: The MN shall check with the last serving SN if there was any intra-SN PSCell change (classic or CPC) also in case of MRO for CPAC.

Observation 2-2: Also in case of MRO for CPAC, the MN shall inform the source node if there was recent SN-initiated CPC and no further intra-SN PSCell change in the target SN.

Observation 2-3: The existing mechanism enables correct detection of a CPC to wrong target PSCell (the MN must be enhanced to check the best PSCell with the stored lists of all and prepared target PSCells).

Observation 2-4: In case of too early CPA, the problem is on the MN side and thus no signalling is needed.

Observation 2-5: To identify a CPA to wrong cell, the MN must consult the reported failure with the target SN (also, to exclude intra-SN PSCell change). The existing signalling can be reused.

Proposals 2-1: Based on the analysis, RAN3 concludes that no stage-3 changes are needed to handle the MRO for CPAC.

Observation 2-6: The existing stage-2 description of the overall mechanism is general enough to cover also handling of CPAC scenarios, including CPA.

Proposal 2-2: No further work is needed in RAN3 to enable detection of MRO scenarios for CPAC.

Proposal 2-3: RAN3 shall consider if the initiating node can support the Target SN in selecting the right target PSCell(s).

Proposal 2-4: The initiating node may indicate additional information to Target SN via the preparation messages to aid correct PSCell selection for preparation.

Proposal 2-5: The additional information may comprise of a prioritisation of certain PSCells over others and/or the probability that a given PSCell was the correct one in the past.

# References

- R3-231226, RAN3 #119-bis

# Annex 1 – Text proposal for TS 38.423

The text proposal below concerns changes to enable MRO for the fast MCG recovery.

First change – omitted text not changed

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node ® S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ..., true-with-timer) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Source NG-RAN Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source NG-RAN node or the source SN. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| Configured T316 value | C-ifFastMCGrecovery |  | FFS |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |
| ifFastMCGrecovery | This IE shall be present if the Requested Fast MCG recovery via SRB3 IE is present and set to "true-with-timer". |

Next change – omitted text not changed

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node ® S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node |  | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  |  | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. |  | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.3.49 |  |  | – |  |
| >S-NG-RAN node Security Key | O |  | 9.2.3.51 |  |  | – |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.3.17 |  |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  |  | – |  |
| >Lower Layer presence status change | O |  | 9.2.3.60 |  |  | – |  |
| >PDU Session Resources To Be Added List |  | 0..1 |  |  |  | – |  |
| >>PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  |  | – |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  | – |  |
| >>>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  |  | – |  |
| >>>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  |  | – |  |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. |  | YES | ignore |
| >PDU Session Resources To Be Modified List |  | 0..1 |  |  |  | – |  |
| >>PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Info – SN terminated IE nor the PDU Session Resource Modification Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.3.4 apply. |  | – |  |
| >>>PDU Session ID | M |  | 9.2.3.18 |  |  | – |  |
| >>>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  |  | – |  |
| >>>PDU Session Resource Modification Info – SN terminated | O |  | 9.2.1.9 |  |  | – |  |
| >>>PDU Session Resource Modification Info – MN terminated | O |  | 9.2.1.11 |  |  | – |  |
| >>>S-NSSAI | O |  | 9.2.3.21 |  |  | YES | reject |
| >>>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.2.3.82 | Expected UE Activity Behaviour for the PDU Session. |  | YES | ignore |
| >PDU Session Resources To Be Released List | O |  | PDU session List with Cause 9.2.1.26 |  |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10]. |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. |  | YES | ignore |
| Requested Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested to be released. |  | YES | ignore |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  |  | YES | ignore |
| Additional DRB IDs | O |  | DRB List 9.2.1.29 |  | Indicates additional list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 |  | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) |  | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 |  | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  |  | YES | reject |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ..., true-with-timer) |  | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| Requested Fast MCG recovery via SRB3 Release | O |  | ENUMERATED (true, ...) |  | Indicates that resources for fast MCG recovery via SRB3 are requested to be released. | YES | ignore |
| SN triggered | O |  | ENUMERATED (TRUE ...) |  |  | YES | ignore |
| Target Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 |  | Indicates the target node ID of the handover procedure decided by the M-NG-RAN node. | YES | ignore |
| PSCell History Information Retrieve | O |  | ENUMERATED (query, ...) |  | Indicates that the SN UE history information is requested. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  |  | YES | ignore |
| CHO Information SN Modification | O |  |  |  |  | YES | ignore |
| >Conditional Reconfiguration | M |  | ENUMERATED (intra-MN-CHO, ...) |  |  | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  |  | YES | ignore |
| Conditional PSCell Addition Information Modification Request | O |  |  |  | This IE may be sent to the target SN. | YES | ignore |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..8, ...) |  | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | Indicates the arrival probability for the UE towards the candidate target SN. |  |  |
| Conditional PSCell Change Information Update | O |  |  |  | This IE may be sent to the source SN. | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  |  | – |  |
| >>>Candidate PSCell List |  | 1 |  |  |  | – |  |
| >>>>Candidate PSCell Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |  | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 |  | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | ignore |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.2.3.169 |  |  | YES | ignore |
| Configured T316 value | C-ifFastMCGrecovery |  | FFS |  |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

| Condition | Explanation |
| --- | --- |
| ifFastMCGrecovery | This IE shall be present if the Requested Fast MCG recovery via SRB3 IE is present and set to "true-with-timer". |

Next change – omitted text not changed

#### 9.1.2.15	S-NODE RELEASE REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to the M-NG-RAN node to confirm the request to release S-NG-RAN node resources.

Direction: S-NG-RAN node ® M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU sessions To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released List – SN terminated | O |  | PDU Session List with data forwarding request info 9.2.1.24 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; S-SN provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| Fast MCG Recovery MRO information |  | 0..1 |  |  | YES | Ignore |
| >Delivery attempts before T316 expired | M |  | INTEGER (0..FFS) |  | - | - |
| >Value of T316 at the max no of delivery attempts | O |  | FFS |  | - | - |

Next change – omitted text not changed

#### 9.1.2.17	S-NODE RELEASE REQUIRED

This message is sent by the S-NG-RAN node to request the release of all resources for a specific UE at the S-NG-RAN node.

Direction: S-NG-RAN node ® M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU sessions To Be Released |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources to be released List – SN terminated | O |  | PDU session List with data forwarding request info 9.2.1.24 |  | – |  |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message as defined in TS 38.331 [10]. | YES | ignore |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | ignore |
| Fast MCG Recovery MRO information |  | 0..1 |  |  | YES | Ignore |
| >Delivery attempts before T316 expired | M |  | INTEGER (0..FFS) |  | - | - |
| >Value of T316 at the max no of delivery attempts | O |  | FFS |  | - | - |

Remaining text not changed