3GPP TSG-RAN WG3 Meeting #120	R3-232821

Incheon, Korea, 22 May – 26 May, 2023

Title: 	(TP for MDT BL CRs for TS 38.413 and TS 38.423): Remaining issues for MDT in NPN

Source: 	Huawei

Agenda item:	10.2.4

Document Type:	Discussion

1. Introduction

In the last RAN3 #119b meeting [1], the MDT configuration enhancements for both PNI-NPN and SNPN are discussed, while there are still some remaining issues:

FFS: Check the company's preference: in case of PNI-NPN area scope is not present, whether how to interpret the old area scope choices when it set as cell base, TA based, TAI based, and PLMN wide with following cases: -only represents the PN area, i.e, only including non-CAG cells -can represent both PN and NPN area, i.e, including both non-CAG cells and CAG cells. If the latter is preferred, whether further clarification is needed to avoid misunderstanding. If old area scope choices only represents the PN area , FFS whether there is a need to preserve the legacy functionality of the NGAP "PLMN wide" choice, namely, this choice is equivalent to absence of area scope over Xn and RRC, i.e. MDT is performed within the MDT PLMN List without any restriction. FFS on set the range of maxnoofCAGforMDT to 256 or 12. FFS introduce the "SNPN based", "SNPN cell based", and "SNPN TA based" choice IEs into the existing CHOICE structure, this would allow for the specification of interested SNPNs, interested cells of SNPNs, or interested TAs of SNPNs. FFS Whether there is a need to introduce a NGAP "SNPN wide" choice.

In this paper, we provide our opinions about the remaining issues in PNI-NPN and SNPN, respectively.

2. Discussion

2.1 Remaining issues for PNI-NPN

According to the current RAN3 progress, in the MDT configuration over NGAP interface, the IE PNI-NPN Area Scope of MDT is optional. Thus, here comes the question that if the IE PNI-NPN Area Scope of MDT is not present, how the old area scope choices mean, i.e. whether they only represent the PN area or both PN and PNI-NPN area. Based on the latest released specification, the old area scope choices are originally designed to describe the PN network area configurations, while there is no area scope related information about PNI-NPN. Considering the compatibility to the earlier releases, in case that PNI-NPN area scope is not present, only the PN cells should be the target cells for MDT measurements.

In case the PNI-NPN area scope is not present, the old area scope choices only represent the PN area

Another remaining issue is that how to support the PLMN wide option for PNI-NPN and SNPN. The straightforward way is to introduce two new branches in current choice structure on both NGAP and XnAP, named as PNI-NPN wide and SNPN wide. The reason is  because the area scope IE in XnAP is implemented as optional IE by mistake. While majority companies prefer to keep the area scope IE in XnAP optional. And consequently, two CRs for XnAP have been agreed at last meeting specifying that if the area scope IE is absent on XnAP, it means the MDT task is referring to the PLMN wide branch on NGAP. Such procedural text is only applicable for public network. Therefore, dedicated new branches for PNI-NPN wide and SNPN wide are necessary. Otherwise, there is no way to specify the receiving node behaviour when the area scope is absent.

To add new branches of PNI-NPN wide and SNPN wide in the area scope IE in both NGAP and XnAP.

For the range of maxnoofCAGforMDT, some companies argued that since a UE can be configured by at most 12 CAG IDs in a cell, the OAM should also at most configure 12 CAG IDs in the MDT configuration. We need to note that in TS 38.331, it has been claimed that the IE maxNPN-r16 refers to the maximum number of NPNs broadcast and reported by UE at establishment.

However, in TS 38.413, it has been clearly claimed that the allowed PNI-NPN for a UE in a PLMN can be set to at most 256, as quoted as the follows.

#### 9.3.3.45 Allowed PNI-NPN List This IE contains information on allowed UE mobility in PNI-NPN including allowed PNI-NPNs and whether the UE is allowed to access non-CAG cells for each PLMN. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Allowed PNI-NPN Item; 1..<maxnoofEPLMNs+1>. >PLMN Identity; M; 9.3.3.5. >PNI-NPN Restricted; M; ENUMERATED (restricted, not-restricted, …); If set to “restricted”, indicates that the UE is not allowed to access non-CAG cells for this PLMN.. >Allowed CAG List per PLMN; 1..<maxnoofAllowedCAGsperPLMN>. >>CAG ID; M; 9.3.3.43. Range bound; Explanation. maxnoofEPLMNs+1; Maximum no. of equivalent PLMNs plus one serving PLMN. Value is 16.. maxnoofAllowedCAGsperPLMN; Maximum number of CAGs per PLMN in UE’s Allowed PNI-NPN list. Value is 256..

In our opinion, for a UE configured with logged MDT configuration, it may move among a number of cells, while those cells may support NPN networks with different CAG IDs. If the UE is configured to perform logged MDT in at most 12 CAGs, the valid measurement results will be significantly declined. Therefore, in order to collect enough MDT measurement results, the range of maxnoofCAGforMDT should be 256.

The range of maxnoofCAGforMDT should be 256.

2.2 Remaining issues for SNPN

For the area scope enhancements to support MDT in SNPN, the following options are discussed at last meeting:

- Option A: add a list of SNPN into the existing Choice structure.

- Option B: add a list of SNPN outside the existing Choice structure.

- Option C: add “SNPN cell based” and “SNPN TA based” choice into the existing Choice structure.

- Option D: add “SNPN wide” choice into the existing Choice structure.

- Option E: reuse the legacy Choice structure for SNPN, i.e, OAM configures the cell list or TA list within interested SNPN(s), there is no need to introduce a SNPN list for MDT.

- Option F: add NID under each existing branch in the area scope IE.

In the current SNPN network, one SNPN will still be regarded as isolated from other networks. If a UE is configured with MDT in its registered SNPN network, the UE can only collect related MDT measurement in the registered SNPN. Meanwhile, the UE will be de-registered from the previously registered SNPN. In this case, if a UE moves from an SNPN network to PN, the UE will delete any stored MDT configuration and related measurement results for previous registered SNPN. And it may receive a new MDT configuration from the PN if needed.

However, so far the WI about supporting equivalent SNPN in RP-230788 [2] has been approved within Rel-18, which includes the following two KIs:

The objective of this work item is to perform normative work for RAN functionalities required to support 5GS functions of the SA2 WID “Enhanced support of Non-Public Networks Phase 2”, with the following features being of RAN relevance: - Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection [RAN3, RAN2] - Support for non-3GPP access for SNPN [RAN3]

In our opinion, with the equivalent SNPN taken into consideration, the UE is able to connect and camp on multiple SNPN networks, without the register and de-register procedures. Therefore, the UE can perform MDT in multiple SNPN s with one MDT configuration. In this case, an NID list should be included in the area scope of MDT.  Therefore, at least option A is needed.

We think that there is no need for operators to configure the UE to collect MDT data in both PNs and SNPNs at the same time. With this understanding, option B is excluded.

Regarding option C, further clarification is needed on whether such finer granularity of area scope is needed for MDT in SNPN. Considering that the coverage of one SNPN is restricted to one or few factories, the need of option C seems weak. But we are acceptable to option C if majority prefers.

RAN3 to adopt include an NID list in the area scope of MDT and support individual area scope configurations per SNPN network.

Based on above proposals and the previous agreements for MDT in PNI-NPN, a TP to 37.320 is provided in [3].

To agree on the stage 2 TP for TS 37.320 in [3].

3. Conclusion

Based on the discussion in this paper, we propose the following:

- In case the PNI-NPN area scope is not present, the old area scope choices only represent the PN area

- To add new branches of PNI-NPN wide and SNPN wide in the area scope IE in both NGAP and XnAP.

- The range of maxnoofCAGforMDT should be 256.

- RAN3 to adopt include an NID list in the area scope of MDT and support individual area scope configurations per SNPN network.

- To agree on the stage 2 TP for TS 37.320 in [3].

The TPs for MDT BL CRs for TS 38.413 and TS 38.423 are provided in the annex.

The stage 2 TP for TS 37.320 is provided in [3] .

5. Reference

- RAN3_119bis-e_agenda_20230426_EOM1.

- RP-230788, New WID on Further Enhancement of Private Network Support for NG-RAN.

- R3-232822	(TP for MDT BLCR for TS 37.320): MDT support in NPN

Annex – TP of TS 38.413

#### 9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN wide |  |  | NULL | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAI | M |  |  |  | - |  |
| >PNI-NPN Based |  |  |  |  | YES | Ignore |
| >>CHOICE Area Scope of PNI-NPN Based MDT |  |  |  |  |  |  |
| >>>PLMN wide |  |  |  |  |  |  |
| >>>CAG List for MDT |  | 1..<maxnoofCAGforMDT> |  |  |  |  |
| >>>>PLMN ID |  |  | 9.3.3.5 |  |  |  |
| >>>>CAG ID | M |  | 9.3.3.43 |  |  |  |
| >SNPN Based MDT |  |  |  |  |  |  |
| >>SNPN List for MDT |  |  | 1..<maxnoofSNPNforMDT> | maxnoofSNPNforMDT = 1 in this version of the specification |  |  |
| >>>PLMN ID | M |  | 9.3.3.5 |  |  |  |
| >>>NID | M |  | 9.3.3.42 |  |  |  |
| >>>CHOICE Area Scope of MDT | M |  |  |  |  |  |
| >>>>Cell based |  |  |  |  |  |  |
| >>>>>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |
| >>>>>>NR CGI | M |  | 9.3.1.7 |  |  |  |
| >>>>TA based |  |  |  |  |  |  |
| >>>>> TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>>>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. |  |  |
| >>>>PLMN wide |  |  | NULL |  |  |  |
| >>>>TAI based |  |  |  |  |  |  |
| >>>>> TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>>>>TAI | M |  |  |  |  |  |
| CHOICE MDT Mode | M |  |  |  | - |  |
| >Immediate MDT |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  | - |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  | - |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  | - |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  | - |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | This IE is defined in TS 38.331 [18]. | - |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | This IE is defined in TS 38.331 [18]. Unit: [minute]. | - |  |
| >>CHOICE Report Type | M |  |  |  | - |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  | - |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. | YES | ignore |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | - |  |
| PNI-NPN Area Scope of MDT | O |  | 9.3.3.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofCAGforMDT | Maximum no. of CAG IDs for MDT scope. Value is 256. |

| Condition | Explanation |
| --- | --- |
| C-ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| C-ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| C-ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| C-ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| C-ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

Editor´s note 1: The text in the Semantics Descriptions may be revisited if needed

Next Change

#### 9.3.3.x	PNI-NPN Area Scope of MDT

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Area Scope of PNI-NPN Based MDT |  |  |  |  |
| >PLMN wide |  |  |  |  |
| >CAG List for MDT |  | 1..< maxnoofCAGforMDT > |  |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  |
| >>CAG ID | M |  | 9.3.3.43 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCAGforMDT | Maximum no. of CAG IDs for MDT area scope. Value is 256. |

Next Change

    - 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-ConfiguredTACIndication,

id-CurrentQoSParaSetIndex,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-EndpointIPAddressAndPort,

id-EnergySavingIndication,

id-ExtendedPacketDelayBudget,

id-ExtendedRATRestrictionInformation,

id-ExtendedReportIntervalMDT,

id-ExtendedSliceSupportList,

id-ExtendedTAISliceSupportList,

id-ExtendedUEIdentityIndexValue,

id-EUTRA-PagingeDRXInformation,

id-GlobalCable-ID,

id-GlobalRANNodeID,

id-GlobalTNGF-ID,

id-GlobalTWIF-ID,

id-GlobalW-AGF-ID,

id-GUAMIType,

id-IncludeBeamMeasurementsIndication,

id-IntersystemSONInformationRequest,

id-IntersystemSONInformationReply,

id-IntersystemResourceStatusUpdate,

id-LastEUTRAN-PLMNIdentity,

id-LastVisitedPSCellList,

id-LocationReportingAdditionalInfo,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-ExcessPacketDelayThresholdConfiguration,

id-M7ReportAmount,

id-MaximumIntegrityProtectedDataRate-DL,

id-MBS-AreaSessionID,

id-MBS-QoSFlowsToBeSetupList,

id-MBS-QoSFlowsToBeSetupModList,

id-MBS-QoSFlowToReleaseList,

id-MBS-ServiceArea,

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

id-MDTConfiguration,

id-MicoAllPLMN,

id-NetworkInstance,

id-NGAPIESupportInformationRequestList,

id-NGAPIESupportInformationResponseList,

id-NID,

id-NR-CGI,

id-NRNTNTAIInformation,

id-NPN-MobilityInformation,

id-NPN-PagingAssistanceInformation,

id-NPN-Support,

id-NR-PagingeDRXInformation,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-OnboardingSupport,

id-PagingAssisDataforCEcapabUE,

id-PagingCauseIndicationForVoiceService,

id-PDUSessionAggregateMaximumBitRate,

id-PduSessionExpectedUEActivityBehaviour,

id-PDUSessionPairID,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceReleaseResponseTransfer,

id-PDUSessionType,

id-PEIPSassistanceInformation,

id-PSCellInformation,

id-PNI-NPNBasedMDT,

id-PNI-NPN-AreaScopeofMDT,

id-SNPNBasedMDT,

id-QMCConfigInfo,

id-QosFlowAddOrModifyRequestList,

id-QosFlowFailedToSetupList,

id-QosFlowFeedbackList,

id-QosFlowParametersList,

id-QosFlowSetupRequestList,

id-QosFlowToReleaseList,

id-QosMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-SuccessfulHandoverReportList,

id-UEContextReferenceAtSource,

id-RAT-Information,

id-RedundantCommonNetworkInstance,

id-RedundantDL-NGU-TNLInformationReused,

id-RedundantDL-NGU-UP-TNLInformation,

id-RedundantDLQosFlowPerTNLInformation,

id-RedundantPDUSessionInformation,

id-RedundantQosFlowIndicator,

id-RedundantUL-NGU-UP-TNLInformation,

id-SCTP-TLAs,

id-SecondaryRATUsageInformation,

id-SecurityIndication,

id-SecurityResult,

id-SgNB-UE-X2AP-ID,

id-S-NSSAI,

id-SONInformationReport,

id-SourceNodeID,

id-SourceNodeTNLAddrInfo,

id-SourceTNLAddrInfo,

id-SurvivalTime,

id-TNLAssociationTransportLayerAddressNGRAN,

id-TAINSAGSupportList,

id-TargetHomeENB-ID,

id-TargetRNC-ID,

id-TraceCollectionEntityURI,

id-TSCTrafficCharacteristics,

id-UEHistoryInformationFromTheUE,

id-UERadioCapabilityForPaging,

id-UERadioCapabilityForPagingOfNB-IoT,

id-UL-NGU-UP-TNLInformation,

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

id-UsedRSNInformation,

id-UserLocationInformationTNGF,

id-UserLocationInformationTWIF,

id-UserLocationInformationW-AGF,

id-EarlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-TAI,

id-HFCNode-ID-new,

id-GlobalCable-ID-new,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofBluetoothName,

maxnoofBPLMNs,

maxnoofCAGSperCell,

maxnoofCAGforMDT,

maxnoofSNPNforMDT,

maxnoofCandidateCells,

maxnoofCellIDforMDT,

maxnoofCellIDforQMC,

maxnoofCellIDforWarning,

maxnoofCellinAoI,

maxnoofCellinEAI,

maxnoofCellsforMBS,

maxnoofCellsingNB,

maxnoofCellsinngeNB,

maxnoofCellsinNGRANNode,

maxnoofCellinTAI,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEmergencyAreaID,

maxnoofEAIforRestart,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofE-RABs,

maxnoofErrors,

maxnoofExtSliceItems,

maxnoofForbTACs,

maxnoofFreqforMDT,

maxnoofMBSFSAs,

maxnoofMBSQoSFlows,

maxnoofMBSServiceAreaInformation,

maxnoofMBSAreaSessionIDs,

maxnoofMBSSessions,

maxnoofMBSSessionsofUE,

maxnoofMDTPLMNs,

maxnoofMRBs,

maxnoofMultiConnectivity,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighPCIforMDT,

maxnoofNGAPIESupportInfo,

maxnoofNGConnectionsToReset,

maxNRARFCN,

maxnoofNRCellBands,

maxnoofNSAGs,

maxnoofPagingAreas,

maxnoofPC5QoSFlows,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofPLMNforQMC,

maxnoofQosFlows,

maxnoofQosParaSets,

maxnoofRANNodeinAoI,

maxnoofRecommendedCells,

maxnoofRecommendedRANNodes,

maxnoofAoI,

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo,

maxnoofReportedCells,

maxnoofSensorName,

maxnoofServedGUAMIs,

maxnoofSliceItems,

maxnoofSNSSAIforQMC,

maxnoofSuccessfulHOReports,

maxnoofTACs,

maxnoofTACsinNTN,

maxnoofTAforMDT,

maxnoofTAforQMC,

maxnoofTAIforInactive,

maxnoofTAIforMBS,

maxnoofTAIforPaging,

maxnoofTAIforRestart,

maxnoofTAIforWarning,

maxnoofTAIinAoI,

maxnoofTargetS-NSSAIs,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofUEAppLayerMeas,

maxnoofUEsforPaging,

maxnoofWLANName,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofThresholdsForExcessPacketDelay

<<< Skip unchanged>>>

#### -- A

<<< Skip unchanged>>>

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

pLMNWide					NULL,

tAIBased					TAIBasedMDT,

choice-Extensions		ProtocolIE-SingleContainer { {AreaScopeOfMDT-NR-ExtIEs} }

}

AreaScopeOfMDT-NR-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PNI-NPNBasedMDT		CRITICALITY ignore	TYPE PNI-NPNBasedMDT		PRESENCE optional },

{ ID id-SNPNBasedMDT		CRITICALITY ignore	TYPE SNPNBasedMDT			PRESENCE optional }

...

}

#### -- C

<<< Skip unchanged>>>

CAGListForMDT ::= SEQUENCE {

plmnID				PLMNIdentity,

cAGID				CAG-ID,

iE-Extensions		ProtocolExtensionContainer { {CAGListforMDT-ExtIEs} }	OPTIONAL,

...

}

CAGListForMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::={

...

}

SNPNListforMDT ::= SEQUENCE {

plmnID				PLMNIdentity,

nID					NID,

areaScopeOfMDT		AreaScopeOfMDT-NR

iE-Extensions		ProtocolExtensionContainer { {SNPNListforMDT-ExtIEs} }	OPTIONAL,

...

}

SNPNListforMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::={

...

}

#### AreaScopeOfPNI-NPNBasedMDT ::= CHOICE {

#### pLMNWide			NULL,

#### cAGListForMDT		CAGListForMDT,

#### choice-Extensions	ProtocolIE-SingleContainer { {AreaScopeOfPNI-NPNBasedMDT -ExtIEs} }

#### }

#### -- M

<<< Skip unchanged>>>

MDT-Configuration-NR ::= SEQUENCE {

mdt-Activation				MDT-Activation,

areaScopeOfMDT				AreaScopeOfMDT-NR,

mDTModeNr					MDTModeNr,

signallingBasedMDTPLMNList 	MDTPLMNList												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-NR-ExtIEs} } 	OPTIONAL,

...

}

MDT-Configuration-NR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PNI-NPN-AreaScopeOfMDT		CRITICALITY ignore	EXTENSION PNI-NPN-AreaScopeOfMDT		PRESENCE optional		},

...

}

#### -- P

<<< Skip unchanged>>>

PNI-NPNBasedMDT ::= SEQUENCE {

areaScopeOfPNI-NPNBasedMDT				AreaScopeOfPNI-NPNBasedMDT,

iE-Extensions		ProtocolExtensionContainer { { PNI-NPNBasedMDT-ExtIEs} } 	OPTIONAL,

...

}

SNPNBasedMDT ::= SEQUENCE (SIZE(1..maxnoofSNPNforMDT)) OF SNPNBasedMDT-Item

PNI-NPN-AreaScopeofMDT ::= SEQUENCE (

areaScopeOfPNI-NPNBasedMDT				AreaScopeOfPNI-NPNBasedMDT,

iE-Extensions	ProtocolExtensionContainer { { PNI-NPN-AreaScopeofMDT-ExtIEs} } 	OPTIONAL,

...

}

PNI-NPNBasedMDT-Item ::= CHOICE {

pLMNWide				NULL,

cAGListForMDT		CAGListForMDT,

choice-Extensions		ProtocolExtensionContainer { {PNI-NPNBasedMDT-Item-ExtIEs} }	OPTIONAL,

...

}

PNI-NPNBasedMDT-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SNPNBasedMDT-Item ::= SEQUENCE {

plmnID				PLMNIdentity,

nID					NID,

areaScopeOfMDT		AreaScopeOfMDT-NR

iE-Extensions		ProtocolExtensionContainer { {PNI-NPNBasedMDT-Item-ExtIEs} }	OPTIONAL,

...

}

SNPNBasedMDT-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

<<< Skip unchanged>>>

maxNRARFCN									INTEGER ::= 3279165

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofSNPNforMDT							INTEGER	::= 1

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

<<< Skip unchanged>>>

id-NGAPIESupportInformationResponseList					ProtocolIE-ID ::= 356

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-PNI-NPN-AreaScopeofMDT								ProtocolIE-ID ::= XXX

id-PNI-NPNBasedMDT                                      ProtocolIE-ID ::= YYY

id-SNPNBasedMDT                                      	ProtocolIE-ID ::= ZZZ

END

-- ASN1STOP

End of Changes

Annex – TP of TS 38.423

#### 9.2.3.126	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…) |  |
| CHOICE Area Scope of MDT-NR | O |  |  |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope for MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |
| >>Cell ID List for MDT-NR |  | 1 .. <maxnoofCellIDforMDT> |  |  |
| >>>NR CGI | M |  | 9.2.2.7 |  |
| >TA based |  |  |  | If PNI-NPN Area Scope for MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |
| >>TA List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |
| >>>TAC | M |  | OCTET STRING (SIZE (3)) | The TAI is derived using the current serving PLMN. |
| >TAI based |  |  |  | If PNI-NPN Area Scope for MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |
| >>TAI List for MDT |  | 1 .. <maxnoofTAforMDT> |  |  |
| >>>TAI | M |  | 9.2.3.20 |  |
| >PNI-NPN based |  |  |  |  |
| >> CHOICE Area Scope of MDT |  |  |  |  |
| >>>PLMN wide |  |  |  |  |
| >>>CAG List for MDT |  | 1..<maxnoofCAGforMDT> |  |  |
| >>>>PLMN ID | M |  | 9.2.2.4 |  |
| >>>>CAG ID | M |  | 9.2.2.66 |  |
| >SNPN Based MDT |  |  |  |  |
| >>SNPN List for MDT |  |  | 1..<maxnoofSNPNforMDT> | maxnoofSNPNforMDT = 1 in this version of the specification |
| >>>PLMN ID | M |  | 9.2.2.4 |  |
| >>>NID | M |  | 9.2.2.65 |  |
| >>>CHOICE Area Scope of MDT | M |  |  |  |
| >>>>Cell based |  |  |  |  |
| >>>>>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |
| >>>>>>NR CGI | M |  | 9.2.2.7 |  |
| >>>>TA based |  |  |  |  |
| >>>>> TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  |
| >>>>>>TAC | M |  | 9.2.2.5 | The TAI is derived using the current serving PLMN. |
| >>>>PLMN wide |  |  | NULL |  |
| >>>>TAI based |  |  |  |  |
| >>>>> TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  |
| >>>>>>TAI | M |  |  |  |
| CHOICE MDT Mode | M |  |  |  |
| >Immediate MDT-NR |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [43]. First Bit = M1, Second Bit= M2, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". This version of the specification does not use bits 3. |
| >>M1 Configuration | C-ifM1 |  | 9.2.3.128 |  |
| >>M4 Configuration | C-ifM4 |  | 9.2.3.129 |  |
| >>M5 Configuration | C-ifM5 |  | 9.2.3.130 |  |
| >>MDT Location Information | O |  | BITSTRING(SIZE(8)) | Each position in the bitmap represents requested location information as defined in TS 37.320 [43]. First Bit = GNSS Other bits are reserved for future use and are ignored if received. Value "1" indicates "activate" and value "0" indicates "do not activate". The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to "1". |
| >>M6 Configuration | C-ifM6 |  | 9.2.3.131 |  |
| >>M7 Configuration | C-ifM7 |  | 9.2.3.132 |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.11 |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.12 |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  |
| >Logged MDT-NR |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 and ms61440, infinity) | This IE is defined in TS 38.331 [10]. The value "infinity" represents one shot logging, i.e., only one log per event in the logged MDT report. |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90, 120) | This IE is defined in TS 38.331 [10]. Unit: [minute]. |
| >>CHOICE Report Type | M |  |  |  |
| >>>Periodical |  |  |  |  |
| >>>Event Triggered |  |  |  |  |
| >>>>Logged Event Trigger Config | M |  | 9.2.3.137 |  |
| >>Bluetooth Measurement Configuration | O |  | 9.2.3.134 |  |
| >>WLAN Measurement Configuration | O |  | 9.2.3.135 |  |
| >>Sensor Measurement Configuration | O |  | 9.2.3.136 |  |
| >>Area Scope of Neighbour Cells | O |  | 9.2.3.140 |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [10]. |
| Signalling based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  |
| PNI-NPN Area Scope of MDT | O |  | 9.2.3.x |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofCAGforMDT | Maximum no. of CAG IDs for MDT scope. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to "1". |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to "1". |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to "1". |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to "1". |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to "1". |

Editor´s note 1: The text in the Semantics Descriptions may be revisited if needed

#### 9.2.3.x	PNI-NPN Area Scope of MDT

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Area Scope of MDT |  |  |  |  |
| >PLMN wide |  |  |  |  |
| >CAG List for MDT |  | 1..< maxnoofCAGforMDT > |  |  |
| >>PLMN Identity | M |  | 9.2.2.4 |  |
| >>CAG ID | M |  | 9.2.2.66 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCAGforMDT | Maximum no. of CAG IDs for MDT area scope. Value is 256. |

/*********************Next change********************************/

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedReportIntervalMDT,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-IntendedTDD-DL-ULConfiguration-NR,

id-MaxIPrate-DL,

id-SecurityResult,

id-OldQoSFlowMap-ULendmarkerexpected,

id-PDUSessionCommonNetworkInstance,

id-PDUSession-PairID,

id-BPLMN-ID-Info-EUTRA,

id-BPLMN-ID-Info-NR,

id-DRBsNotAdmittedSetupModifyList,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-ULForwardingProposal,

id-DRB-IDs-takenintouse,

id-SplitSessionIndicator,

id-NonGBRResources-Offered,

id-MDT-Configuration,

id-TraceCollectionEntityURI,

id-NPN-Broadcast-Information,

id-NPNPagingAssistanceInformation,

id-NPNMobilityInformation,

id-NPN-Support,

id-LTEUESidelinkAggregateMaximumBitRate,

id-NRUESidelinkAggregateMaximumBitRate,

id-ExtendedRATRestrictionInformation,

id-QoSMonitoringRequest,

id-QoSMonitoringDisabled,

id-QosMonitoringReportingFrequency,

id-DAPSRequestInfo,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-NBIoT-UL-DL-AlignmentOffset,

id-TDDULDLConfigurationCommonNR,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRCellPRACHConfig,

id-Redundant-UL-NG-U-TNLatUPF,

id-Redundant-DL-NG-U-TNLatNG-RAN,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-Additional-Redundant-UL-NG-U-TNLatUPF-List,

id-RedundantCommonNetworkInstance,

id-TSCTrafficCharacteristics,

id-RedundantQoSFlowIndicator,

id-Additional-PDCP-Duplication-TNL-List,

id-RedundantPDUSessionInformation,

id-UsedRSNInformation,

id-RLCDuplicationInformation,

id-CSI-RSTransmissionIndication,

id-UERadioCapabilityID,

id-secondary-SN-UL-PDCP-UP-TNLInfo,

id-pdcpDuplicationConfiguration,

id-duplicationActivation,

id-NPRACHConfiguration,

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-QoS-Mapping-Information,

id-AdditionLocationInformation,

id-dataForwardingInfoFromTargetE-UTRANnode,

id-Cause,

id-SecurityIndication,

id-RRCConnReestab-Indicator,

id-SourceDLForwardingIPAddress,

id-SourceNodeDLForwardingIPAddress,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-BeamMeasurementIndicationM1,

id-Supported-MBS-FSA-ID-List,

id-MBS-SessionAssociatedInformation,

id-MBS-SessionInformation-List,

id-SliceRadioResourceStatus-List,

id-CompositeAvailableCapacitySupplementaryUplink,

id-SSBOffsets-List,

id-NG-RANnode2SSBOffsetsModificationRange,

id-NR-U-Channel-List,

id-NR-U-ChannelInfo-List,

id-MIMOPRBusageInformation,

id-UEAssistantIdentifier,

id-IAB-MT-Cell-List,

id-NoPDUSessionIndication,

id-permutation,

id-UL-GNB-DU-Cell-Resource-Configuration,

id-DL-GNB-DU-Cell-Resource-Configuration,

id-tdd-GNB-DU-Cell-Resource-Configuration,

id-Additional-Measurement-Timing-Configuration-List,

id-SurvivalTime,

id-Local-NG-RAN-Node-Identifier,

id-Neighbour-NG-RAN-Node-List,

id-FiveGProSeUEPC5AggregateMaximumBitRate,

id-Redcap-Bcast-Information,

id-UESliceMaximumBitRateList,

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-PNI-NPNBasedMDT,

id-PNI-NPN-AreaScopeofMDT,

id-SNPNBasedMDT,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEPLMNsplus1,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

maxnoofForbiddenTACs,

maxnoofMBSFNEUTRA,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighbours,

maxnoofNIDs,

maxnoofNRCellBands,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofProtectedResourcePatterns,

maxnoofQoSFlows,

maxnoofQoSParaSets,

maxnoofRANAreaCodes,

maxnoofRANAreasinRNA,

maxnoofSCellGroups,

maxnoofSCellGroupsplus1,

maxnoofSliceItems,

maxnoofExtSliceItems,

maxnoofSNPNIDs,

maxnoofsupportedTACs,

maxnoofsupportedPLMNs,

maxnoofTAI,

maxnoofTAIsinAoI,

maxnoofTNLAssociations,

maxnoofUEContexts,

maxNRARFCN,

maxNrOfErrors,

maxnoofRANNodesinAoI,

maxnooftimeperiods,

maxnoofslots,

maxnoofExtTLAs,

maxnoofGTPTLAs,

maxnoofCHOcells,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofRACHReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationstate,

maxnoofBluetoothName,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofTAforMDT,

maxnoofWLANName,

maxnoofSensorName,

maxnoofNeighPCIforMDT,

maxnoofFreqforMDT,

maxnoofNonAnchorCarrierFreqConfig,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofUEIDIndicesforMBSPaging,

maxnoofMBSFSAs,

maxnoofMBSQoSFlows,

maxnoofMRBs,

maxnoofCellsforMBS,

maxnoofMBSServiceAreaInformation,

maxnoofTAIforMBS,

maxnoofAssociatedMBSSessions,

maxnoofMBSSessions,

maxnoofSuccessfulHOReports,

maxnoofPSCellsPerSN,

maxnoofNR-UChannelIDs,

maxnoofCellsinCHO,

maxnoofCHOexecutioncond,

maxnoofServingCells,

maxnoofBHInfo,

maxnoofTLAsIAB,

maxnoofTrafficIndexEntries,

maxnoofBAPControlPDURLCCHs,

maxnoofServedCellsIAB,

maxnoofDUFSlots,

maxnoofSymbols,

maxnoofHSNASlots,

maxnoofRBsetsPerCell,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofPSCellCandidates,

maxnoofTargetSNs,

maxnoofUEAppLayerMeas,

maxnoofSNSSAIforQMC,

maxnoofCellIDforQMC,

maxnoofPLMNforQMC,

maxnoofTAforQMC,

maxnoofMTCItems,

maxnoofCSIRSconfigurations,

maxnoofCSIRSneighbourCells,

maxnoofCSIRSneighbourCellsInMTC,

maxnoofNeighbour-NG-RAN-Nodes,

maxnoofSRBs,

maxnoofSMBR,

maxnoofNSAGs,

maxnoofRBsetsPerCell1,

maxnoofTargetSNsMinusOne,

maxnoofThresholdsForExcessPacketDelay,

maxnoofCAGforMDT,

maxnnofSNPNforMDT,

<<< Skip unchanged>>>

#### -- A

<<< Skip unchanged>>>

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

tAIBased					TAIBasedMDT,

...,

pNI-NPNBased				PNI-NPNBasedMDT,

sNPNBased					SNPNBasedMDT,

}

<<< Skip unchanged>>>

CAGListforMDT ::= SEQUENCE {

plmnID				PLMNIdentity,

cAGID				CAG-Identifier,

iE-Extensions		ProtocolExtensionContainer { {CAGListforMDT-ExtIEs} }	OPTIONAL,

...

}

CAGListforMDT-ExtIEs XnAP-PROTOCOL-EXTENSION ::={

...

}

SNPNListforMDT ::= SEQUENCE {

plmnID				PLMNIdentity,

nID					NID,

areaScopeOfMDT		AreaScopeOfMDT-NR

iE-Extensions		ProtocolExtensionContainer { {SNPNListforMDT-ExtIEs} }	OPTIONAL,

...

}

SNPNListforMDT-ExtIEs XnAP-PROTOCOL-EXTENSION ::={

...

}

AreaScopeOfPNI-NPNBasedMDT ::= CHOICE {

pLMNWide			NULL,

cAGListForMDT		CAGListForMDT,

choice-Extensions	ProtocolIE-SingleContainer { {AreaScopeOfPNI-NPNBasedMDT -ExtIEs} }

}

<<< Skip unchanged>>>

PNI-NPNBasedMDT ::= SEQUENCE {

areaScopeOfPNI-NPNBasedMDT				AreaScopeOfPNI-NPNBasedMDT,

iE-Extensions		ProtocolExtensionContainer { { PNI-NPNBasedMDT-ExtIEs} } 	OPTIONAL,

...

}

SNPNBasedMDT ::= SEQUENCE (SIZE(1..maxnoofSNPNforMDT)) OF SNPNBasedMDT-Item

PNI-NPN-AreaScopeofMDT ::= SEQUENCE {

areaScopeOfPNI-NPNBasedMDT				AreaScopeOfPNI-NPNBasedMDT,

iE-Extensions	ProtocolExtensionContainer { { PNI-NPN-AreaScopeofMDT-ExtIEs} } 	OPTIONAL,

...

}

PNI-NPNBasedMDT-Item ::= CHOICE {

pLMNWide			NULL,

cAGListForMDT		CAGListForMDT

,

iE-Extensions		ProtocolExtensionContainer { {PNI-NPNBasedMDT-Item-ExtIEs} }	OPTIONAL,

...

}

PNI-NPNBasedMDT-Item-ExtIEs XnAP-PROTOCOL-EXTENSION ::= {

...

}

SNPNBasedMDT-Item ::= SEQUENCE {

plmnID				PLMNIdentity,

nID					NID,

areaScopeOfMDT		AreaScopeOfMDT-NR

iE-Extensions		ProtocolExtensionContainer { {PNI-NPNBasedMDT-Item-ExtIEs} }	OPTIONAL,

...

}

SNPNBasedMDT-Item-ExtIEs XnAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

<<< Skip unchanged>>>

maxnoofTargetSNsMinusOne					INTEGER ::= 7

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofSNPNforMDT							INTEGER ::= 1

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

<<< Skip unchanged>>>

id-PNI-NPN-AreaScopeofMDT														ProtocolIE-ID ::= XXX

id-PNI-NPNBasedMDT                                      						ProtocolIE-ID ::= YYY

id-SNPNBasedMDT																	ProtocolIE-ID ::= ZZZ

END

-- ASN1STOP

End of Changes