3GPP TSG-RAN WG3 Meeting #120	R3-232866

Incheon, KR, 22 May – 26 May, 2023

Title: 	(TPs to TS 38.423, 37.463 BL CRs) Consideration on MT-SDT

Source: 	Huawei

Agenda item:	20.2

Document Type:	Other

# 1. Introduction

Based on the discussion in [1], we provide the TPs to TS 38.423, TS 37.463 BL CRs for MT-SDT on the following proposals:

Proposal 1: The XnAP: MT-SDT information IE carried in the XnAP: RAN PAGING message includes an MT-SDT indicator mandatorily (to indicate the data/signalling is SDT only) and optionally the data size of the SDT Data/signalling.

Proposal 3: The MT-SDT indicator IE is not needed in the MT-SDT Information IE in the E1AP: DL DATA NOTIFICATION message.

# 2. Reference

[1] R3-232865 (TPs to TS 38.401, 38.473 BL CRs) Consideration on MT-SDT, Huawei

# Annex 5: TP to TS 38.423 BL CR

#### 9.2.3.xxx	MT-SDT Information

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Indicator | M |  | ENUMERATED (true, …) |  |
| MT-SDT Data Size | O |  | INTEGER (1…96000, …) | Indicates the total data size for all SDT bearers. Unit: byte. |

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

*** skip unchanged ***

-- M

MaxNrofRS-IndexesToReport::= INTEGER (1..64, ...)

*** skip unchanged ***

MessageOversizeNotification ::= SEQUENCE {

maximumCellListSize								MaximumCellListSize,

iE-Extension				ProtocolExtensionContainer { {MessageOversizeNotification-ExtIEs}}	OPTIONAL,

...

}

MessageOversizeNotification-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MaximumCellListSize ::= INTEGER(1..16384, ...)

MT-SDT-Information ::= SEQUENCE {

mt-SDT-Indicator			MT-SDT-Indicator,

mT-SDT-DataSize 			MT-SDT-DataSize					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs} } OPTIONAL,

...

}

MT-SDT-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MT-SDT-DataSize	::= INTEGER (1..96000, ...)

MT-SDT-Indicator ::= ENUMERATED {true, ...}

MultiplexingInfo 	::=	SEQUENCE{

iAB-MT-Cell-List 	IAB-MT-Cell-List,

iE-Extensions		ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL,

...

}

MultiplexingInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

# Annex 4: TP to TS 37.463 BL CR

#### 9.3.1.xxx	MT-SDT Information

This IE provides the assistant information for MT-SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Data Size | M |  | INTEGER (1..96000,…) | Indicates the total data size for all SDT bearers. Unit: byte. |