3GPP TSG-RAN WG3 #120                                                                                              R3-232899

Incheon, KR, 22nd – 26th May, 2023

Agenda Item:	21.2

Source:	CATT

Title:	(TP for 38.413) Discussion on long eDRX for RRC_INACTIVE

Document for:	Discussion

- Introduction

The discussion part can be seen in R3-232898 Discussion on long eDRX for RRC_INACTIVE.

- TP on 38.413

### 8.3.X	MT Communication Handling

#### 8.3.X.1	General

The purpose of the MT Communication Handling procedure is to request the AMF to activate or deactivate the CN based MT Communication handling for a UE in RRC_INACTIVE state with extended DRX beyond 10.24 seconds as specified in TS 23.501 [9]. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.X.2-1: MT Communication Handling procedure. Successful operation.

The NG-RAN node initiates the procedure by sending the MT COMMUNICATION HANDLING REQUEST message to the AMF.

If the NR Paging eDRX Cycle for RRC INACTIVE IE is included in the MT COMMUNICATION HANDLING REQUEST message, the AMF shall, if supported,  applying MT communication handling as specified in TS 38.304 [12] and TS 23.501 [9].

If the RRC state IE is included in the MT COMMUNICATION HANDLING REQUEST message, the AMF shall take it into account as specified in TS 23.501 [9].

If the UE state IE set “reachable” is included in the MT COMMUNICATION HANDLING REQUEST message, the AMF shall consider the UE is now reachable for downlink data and/or signalling as specified in TS 23.501 [9].

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.Y	DL Data Notification

#### 8.3.Y.1	General

This procedure is initiated by the AMF to indicate that there is DL data or DL signaling buffered in 5GC for the UE.

The procedure uses UE associated signalling.

#### 8.3.Y.2	Successful Operation

Figure 8.5.2.2-1: DL Data Notification

The AMF initiates the DL Data Notification procedure by sending the DL DATA NOTIFICATION message to the NG-RAN node.

Upon reception of the DL DATA NOTIFICATION message, the NG-RAN node shall perform RAN Paging for the UE in RRC_INACTIVE state expect SDT with long eDRX>10.24s.

Editor’s Note: The description of DL Data Notification message about SDT with long eDRX>10.24 can be added after SA2 and/or SDT WI has further progress.

If Paging Policy Information for Network Triggered Connection IE is included in DL DATA NOTIFICATION message, the NG-RAN shall take it into account when performing RAN Paging for the UE in RRC_INACTIVE state.

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.X1	MT COMMUNICATION HANDLING REQUEST

This message is sent by the NG-RAN node to the AMF to request CN based MT communication handling for UEs in RRC_INACTIVE state with long eDRX beyond 10.24 seconds as specified in TS 23.501 [9].

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NR Paging eDRX Cycle for RRC INACTIVE | O |  | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) | This IE shall present in case NG-RAN request MT communication handling | YES | reject |
| RRC State | M |  | 9.3.1.92 |  | YES | ignore |
| UE State | O |  | ENUMERATED (reachable, …) | This IE is present in case of small data transmission | YES | ignore |

<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.X4	DL DATA NOTIFICATION

This message is sent by the AMF to indicate the there is MT data or signaling buffered in 5GC for the UE.

Direction: AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Paging Policy Information for Network Triggered Connection | O |  | 9.3.4.X |  | YES | ignore |

9.3.4.x Paging Policy Information for Network Triggered Connection

This IE identifies paging policy parameters including the PPI, the ARP and the 5QI, and QFI of the QoS flow of the PDU session for Connection Resume procedure in RRC Inactive with CN based MT communication handling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session ID | M |  | 9.3.1.50 |  |
| QoS Flow Identifier | O |  | 9.3.1.51 |  |
| Paging Policy | O |  |  | As specified in TS 29.518 [x] |
| Allocation and Retention Priority | O |  | 9.3.1.19 |  |
| 5QI | O |  | INTEGER (0..255, …) | Indicates the standardized or pre-configured 5QI as specified in TS 23.501 [9]. |