3GPP RAN WG3 Meeting #120	R3-233053

Incheon, KR, 22nd – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | Draft CR | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SDT with UE context relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In section 18.2 of SDT with UE context relocation, there are green arrows after step 4 and before step 5 for sending UL small data to CN and to UE before even the Path Switch Request procedure takes place. However this is not correct. Since the UPF maintains the NG-AP UE associated connection with the last serving gNB and there has not been any path switch procedure, the receiving gNB cannot send data to UPF as it does not have the UL TEID and there is securiy issue as the UL data shall be forwarded to UPF after Path switch when AMF sends the security indication to RAN. In RAN3#119bis-e, it was argued by some companies that user data can be sent between UPF and new gNB, as specified in TS 38.300 section 9.2.3.2.1 C-Plane Handling, where in the Figure 9.2.3.2.1-1: Intra-AMF/UPF Handover, we can see the signalling flow of a basic handover scenario where neither the AMF nor the UPF are changed. The user data is sent from UPF to target gNB via source gNB between steps 7 and 8 and between steps 8b and 9. We can understand that the last serving gNB acts as relay to transmit the user data, but the connection between UPF to new gNB is not directly present as NG-C interface is not established before the path switch procedure. We can also compare with what is specified in TS 23.502 clauses 4.9.1.2.2/3/4 and that UL data cannot be transmitted directly from new gNB to UPF before the path switch procedure. Therefore the arrows in section 18.2 of TS 38.300 showing that UL small data and subsequent UL small data can be sent directly from Receiving gNB to UPFare wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Delete the green arrows of UL small data and Subsequent UL small data between steps 4 and 5 in Figure 18.2-1. - Delete the text: "The UL SDT data, if any, is delivered from the receiving gNB to the UPF." in step 3 Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has isolated impacted on the SDT procedure. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incoherence with legacy data transmission, where UL data is only exchanged after Patch Switch procedure, to acquire new UL TEID and possilbe new security indication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 18.2	SDT with UE context relocation

The overall procedure for SDT procedure over RACH with UE context relocation is illustrated in the figure 18.2-1.

Figure 18.2-1. RA-based SDT with UE context relocation

1.	The UE sends an RRCResumeRequest as well as UL SDT data and/or UL SDT signalling to the receiving gNB.

2.	The receiving gNB identifies the last serving gNB using the I-RNTI and retrieves the UE context by means of Xn-AP Retrieve UE Context procedure. The receiving gNB indicates that the UE request is for an SDT and may also provide SDT assistance information (e.g., single packet, multiple packets).

3.	The last serving gNB decides to relocate UE context and responds with the RETRIEVE UE CONTEXT RESPONSE message.

4-6.	The receiving gNB decides to keep UE in RRC_INACTIVE state for SDT. If loss of DL user data buffered in the last serving gNB shall be prevented, the receiving gNB provides forwarding addresses via the Xn-U ADDRESS INDICATION message. The receiving gNB also initiates NG-AP Path Switch procedure to establish a NG UE associated signalling connection to the serving AMF. After the Path Switch procedure, the buffered UL NAS PDU, if any, is delivered from the receiving gNB to the AMF. And then, the subsequent UL/DL SDT data and/or signalling are transferred between UE and core network via the receiving gNB.

7.	After the SDT transmission is completed, the receiving gNB generates and sends the RRCRelease message including the suspend indication to the UE to complete the SDT procedure and continue in RRC_INACTIVE state.

NOTE:	In case DL non-SDT data or DL non-SDT signalling arrives, or the UE assistance information (i.e. UL non-SDT data arrival indication) is received from the UE, the receiving gNB may decide to directly send the UE to RRC_CONNECTED state by sending the RRCResume message.

8.	The receiving gNB indicates to the last serving gNB to remove the UE context by sending the XnAP UE CONTEXT RELEASE message. The XnAP UE CONTEXT RELEASE message can be sent after step 6.